| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | MAS 37 |
| Source:* | Andreas Kraft, Deutsche Telekom, a.kraft@telekom.de Ingo Friese, Deutsche Telekom; Ingo.Friese@telekom.de Harald Trzcionka, Deutsche Telekom, Harald.Trzcionka@telekom.de Andreas Neubacher, Deutsche Telekom, andreas.neubacher@t-mobile.at |
| Date:* | 2017-09-18 |
| Reason for Change/s:* | Extension of Smart Meter models |
| CR against: Release* | Rel. 4 |
| CR against: WI* | Active WI-0084 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | oneM2M- TS-0023-V4.0.1 |
| Clauses * | 5.3.1.x, 5.5.2.x |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR provides changes, on the Smart meter models introduced in the CRs MAS-2018-0089R01, MAS-2018-0090R02, MAS-2018-0091R01, MAS-2018-0092R01, MAS-2018-0093R01, and MAS-2018-0094. It tries to take the proposed changes of MAS-2018-0115 into account.

### -----------------------Start of change 1-------------------------------------------

#### 5.3.1.25 connectivity

This ModuleClass provides capabilities to monitor network connectivity.

Table 5.3.1.25-1: DataPoints of connectivity ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| rsrp | xs:integer | R | false | dBm | Reference signal received power (RSRP) is a measurement of the received power level in an LTE cell network. |
| rsrq | xs:integer | R | true | dB | Reference signal received quality (RSRQ) indicates the quality of the received reference signal. RSRQ is defined as the ratio N×RSRP/(E-UTRA carrier RSSI), where N is the number of RB’s of the E-UTRA carrier RSSI measurement bandwidth. The measurements in the numerator and denominator shall be made over the same set of resource blocks. |
| cellID | xs:integer | R | true |  | Serving Cell ID in case Network Bearer Resource is a Cellular Network. |
| rssi | xs:integer | R | true | dBm | In telecommunications, received signal strength indicator (RSSI) is a measurement of the power present in a received radio signal. |
| signalECL | xs:integer | R | true |  | Based on measurements of the reference signal’s received power, the UE will select an entry coverage enhancement level (ECL) to camp into the cell. The coverage level will determine the Narrowband Physical Random Access Channel (NPRACH) resources used by the device and will inform the eNB of the device receiver sensitivity conditions. |
| sinr | xs:integer | R | true | dB | Signal to interference plus noise ratio (SINR) is commonly used in wireless communication as a way to measure the quality of wireless connections. |
| pci | xs:string | R | true |  | Physical Cell ID is an indentification of a cell at physical layer. |
| dailyActivityTime | xs:integer | R | true | s | Daily communication time (Starts at 00:00h) |
| dailyNumberOfConnections | xs:integer | R | true |  | Daily number of connections (Starts at 00:00h) |
| commFreqValue | xs:integer | R | true | MHz | Communication frequency value (commFreqValue) is the transmission frequency of the wireless signal. |
| currentCycleBeginn | xs:datetime | R | true |  | A timestamp that indicates the beginning of the current cycle for counting the transfer volumina and transmission errors. |
| currentCycleVolume | xs:integer | R | true | bytes | Number of bytes transferred since currentCycleBeginn |
| currentCycleTransmissionErrors | xs:integer | R | true |  | Number of transmission errors since currentCycleBeginn |
| minimumCommunicationLatency | xs:integer | R | true | s | The minimum time delay between the last communication attempt |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

#### slcReportInfo

This ModuleClass provides information of status of the street light controller.

Table 5.3.1.81-2: DataPoints of slcReportInfo ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| voltageFrequency | xs:float | R | true | Hz | The frequency of voltage. |
| switchStatus | xs:boolean | RW | false |  | This value indicates the status of light switch. ”False”: off, “True”: on. |
| onlineStatus | xs:boolean | R | true |  | This value indicates the communication status of light. ”False”: offline, “True”: online. |
| lightVoltage | xs:float | R | false | V | The voltage of the street light controller. |
| lightCurrent | xs:float | R | false | A | The current of the street light controller. |
| lightDimmingValue | xs:integer | RW | false | % | The dimming value of the street light. |
| lightPowerFactor | xs:float | R | false |  | The light power factor is the ratio of active power to apparent power. Power factor is a factor that measures the efficiency of electrical equipment. The value range is [0,1]. |
| lightActivePower | xs:float | R | true | kW | Active power is the electrical power required to keep the electrical equipment running normally, that is, the electrical power that converts electrical energy into other forms of energy, such as mechanical, optical, thermal, and so on. |
| lightReactivePower | xs:float | R | true | kVA | The reactive power is the electrical power required to establish an alternating magnetic field and induced magnetic flux. |
| lightApparentPower | xs:float | R | true | kVA | This value indicates the apparent power that is mainly used to calculate the energy consumption of the street light. |
| lightPolarizationAxis | xs:float | R | true | degree | The angle of the polarization axis in case street light devices support this feature. The PolarizationAxis is measured starting from Editors Note: DESCRIPTION NEEDED |
| colourTemperature | xs:integer | R | true | K | The current colour temperature of the street lights. |
| lampTechnology | xs:string | R | true |  | A string that indicates the type of lamp technology that is used in the street lamps, e.g. “LED”, “Tungsten”, etc |

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

5.3.1.61 magneticSensorParameters

### This ModuleClass provides capabilities to set service parameters for parking detectors.

Table 5.3.1.61-3: DataPoints of magneticSensorParameters ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| detectionInterval | xs:integer | RW | false | s | Time interval for detecting the geomagnetic field strength. |
| initialMagnetic | xs:float | RW | true | G | The value of initial geomagnetic field strength, which is usually set when the parking detector is initially installed. It may be reset later if the geomagnetic field strength of the local environment is affected by other factors. The value is used as the reference threshold to determine the parkingStatus. When the parking detector detects that the strength of the magnetic field is greater than the initialMagnetic, the parkingStatus is set to “true”, otherwise, the parkingStatus is set to “false”. |
| magneticSensitivityLevel | xs:integer | RW | true |  | The level of detection sensitivity. It’s implementation specific. |
| highMagneticAlarm | xs:boolean | R | false |  | The alarm of high magnetic interference. The alarm threshold is implementation specific. |

#### 5.3.1.62 parkingStatus

This ModuleClass provides the status of the parking detector.

Table 5.3.1.62-4: DataPoints of parkingStatus ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| parkingStatus | xs:boolean | R | false |  | This value indicates the status of the parking space. - ”False”: not occupied. - “True”: occupied. |

### -----------------------End of change 3---------------------------------------------

### -----------------------Start of change 4-------------------------------------------

#### deviceMagneticParkingMonitor

A parking detector is used to detect the state of the parking space. When the vehicle enters the parking space, the detector measure the change of the geomagnetic field strength, and report the state of the parking space to a server.

Table 5.5.2.2-5: Modules of deviceMagneticParkingMonitor Device model

| Module Instance Name | Module Class Name | Optional | Description |
| --- | --- | --- | --- |
| battery | battery | true | See clause Error: Reference source not found. |
| connectivity | connectivity | true | See clause . |
| periodicalReportConfig | periodicalReportConfig | true | See clause Error: Reference source not found. |
| ParkingStatus | ParkingStatus | false | See clause . |
| magneticSensorParameters | magneticSensorParameters | false | See clause . |

### -----------------------End of change 4---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?