| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | PRO 29 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2017-05-24 |
| Reason for Change/s:* | A developer may hard to understand specification. This contribution describes an essential field of MQTT CONNECT packet to use in oneM2M. |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / WI-0049 Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0010-V2.5.0 |
| Clauses * | 6.3 Connecting MQTT |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The clause describes how to establish MQTT session which is used CONNECT Packet.

Because of  beginning description of MQTT Control packet, MQTT message format move from the clause 6.4.1 to clause 6.3.

This contribution proposes to describe briefly fixed header, variable header and payload based on MQTT message format.

Atfer that,  it re-organized existing consideration in the name of ‘Application of MQTT CONNECT Packet’

This contribution proposes to use formal term of ‘MQTT CONNECT Packet’ as specifiec in OASIS MQTT Version 3.1.1  and keep consistency with clause 6.4 instead of MQTT CONNECT protocol packet,

The Consolas font means ‘name of MQTT fields’.

R01 revises

- Delete clause of (6.3.1) fixed header of MQTT CONNECT Packet

- Re-draw of figure based on MQTT specification.

### -----------------------Start of change 1-------------------------------------------

## 6.3	Connecting to MQTT

In order to communicate, the two client parties (AE and CSE or CSE and CSE) shall connect to a common MQTT server. The MQTT server shall be hosted in one of the two nodes or shall exist as an independent external entity, following one of the two scenarios shown in clause 5.2.

Once each party has located the address of the MQTT server, it then connects to it using the standard MQTT CONNECT Control Packet.

- An MQTT Control Packet consists of up to three parts: a fixed header, a variable header, a payload as shown in Figure 6.3-1.

- Figure 6.3-1 Format of MQTT Control Packet

### 6.3.1 Variable header of MQTT CONNECT Packet

A variable header for the MQTT CONNECT Packet consists of four fields in the following order: Protocol Name, Protocol Level, Connect Flags, and Keep Alive as shown in Figure 6.3.1-1

- Figure 6.3.1-1 Variable header of MQTT CONNECT Packet

The value of Protocol Name field is “MQTT”. The value of the Protocol Level field for the MQTT version 3.1.1 of the protocol is 4 (0x04). The Connect Flags is shown in Figure 6.3.1-2. The Keep Alive is a time interval measured in seconds.

- Figure 6.3.1-2 Connect Flags of variable header for MQTT CONNECT Packet

### 6.3.2 Payload of MQTT CONNECT Packet

A payload for the MQTT CONNECT Packet is determined by the Connect Flags in the variable header. These fields may consist of Client Identifier, Will Topic, Will Message, User Name, Password.

Mandatory fields to establish MQTT session for oneM2M are:

- Client Identifier

Optional fields to eatablish MQTT session for oneM2M are:

- Keep Alive

- User Name

- Password

It shall not be set the fields to establish MQTT session for oneM2M.

- Will Topic

- Will Message

An example of payload of MQTT CONNECT Packet is shown in Figure 6.3.2-1.

Figure 6.3.2-1 Example of of MQTT CONNECT Packet

### 6.3.4 Application of MQTT CONNECT Packet

The following considerations apply:

- Connect Flags:

  - A client shall set the "Clean Session" flag in the MQTT CONNECT Packet to false. This means that MQTT Session state related to that client will be retained by the MQTT Server in the event of a disconnection (deliberate or otherwise) of that client.

  - A client shall not set the "Will Flag", “Will QoS”, “Will Retain” so Will Message and Will Topic shall not be present in the payload.

- Client Identifier: The CONNECT Packet contains a Client Identifier as described in clause A.3.2. The Client Identifier have to be unique at least among all clients that connect to a given MQTT server instance (this is a requirement imposed by the MQTT protocol). This condition will be satisfied if an AE uses its AE-ID and a CSE uses its CSE-ID. See clause 7 of oneM2M TS-0001 [Error: Reference source not found] for a discussion of these Identifiers. The prefix A:: or C:: shall be added to the ID to show whether it is an AE-ID or a CSE-ID as these ID spaces are not distinct

- The AE-ID or CSE-ID may not be known during the initial registration process, in which case the client shall use some other appropriate unique ID.

- Keep Alive: A client may choose to provide a non-zero MQTT Keep Alive value or to provide a Keep Alive of 0 (this disables the MQTT KeepAlive).

- User Name and Password: The MQTT server may require that a client provides a User Name and a Password (or other credential). When MQTT server authenticates by user name and password, the corresponding user name flag and password flag in the CONNECT shall be set to 1. For more information see clause Error: Reference source not found.

A client might choose to keep the MQTT connection open permanently (restarting it as soon as possible after any unforeseen connection loss), it might choose to connect only when it wants to act as an Originator, or it might choose to connect based on the <schedule> associated with a relevant oneM2M resource.

Once a client has connected to the MQTT server it can then communicate (subject to authorization policies) with any other client connected to its server. There is no need for it to create another connection if it wants to communicate with a different counter-party.

When a client determines that it no longer wishes to participate in an MQTT Session with its MQTT Server it shall perform the following steps:

- Disconnect from that server, if it is currently connected.

- Reconnect with the cleanSession flag set to true.

- Disconnect again.

These steps delete any state that the MQTT server might be holding on behalf of the client.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?