| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | PRO 29 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2017-05-24 |
| Reason for Change/s:* | Developers may have difficult to understand specification. |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / WI-0049 Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0010-V2.5.0 |
| Clauses * | 6.4.1 Request and Receiving Messages |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes to use formal term of ‘MQTT PUBLISH Packet’ as specifiec in OASIS MQTT Version 3.1.1

The Consolas font means ‘name of MQTT fields’.

MQTT message format move to clause 6.3 which starts description of MQTT packet.

.

R01 is revised based on ad hoc sssion discussion

- Packet type for SUBSCRIBE is 3 which is deleted 0x03. The 0x03 may understand one-byte-value while packet type is 4 bit.

- Figures are re-drawn based on MQTT specification to prevent confusion of bit order.

- The value of QoS Level depends on importance of contance instance. An important content instance should be set to 1. Some cotent instance may be set to 0. However, QoS level 2 should not be used.

| QoS value | Bit 2 | Bit 1 | Description |
| --- | --- | --- | --- |
| 0 | 0 | 0 | At most once delivery |
| 1 | 0 | 1 | At least once delivery |
| 2 | 1 | 0 | Exactly once delivery |
| - | 1 | 1 | Reserved – must not be used |

-----------------------Start of change 1-------------------------------------------

### 6.4.1	Request and Response Messages

MQTT does not have a data model to describe or constrain the content of its Application Message payloads (to that extent it is similar to a TCP socket).  Mca and Mcc request messages shall be serialized into XML, JSON or CBOR following the serialization process defined in clause 6.5.

The packet type field in figure 6.4.1.1-1 is used to define the MQTT Control Packet type. It is a 4-bit field which may take the values listed in the Table 6.4.1-1. When a oneM2M Request/Response message is bound to MQTT, its packet type shall have value 3, i.e. the Request/Response message is delivered in an MQTT PUBLISH Packet.

Table 6.4.1-1: MQTT Control Packet Types

| Reserved | 0 | Reserved for future use |
| --- | --- | --- |
| CONNECT | 1 | Client request to connect to server |
| CONNACK | 2 | Connect acknowledgement |
| PUBLISH | 3 | Publish message |
| PUBACK | 4 | Publish message acknowledgement |
| PUBREC | 5 | Publish received (QoS=2) |
| PUBREL | 6 | Publish release (QoS=2) |
| PUBCOMP | 7 | Publish complete (QoS=2) |
| SUBSCRIBE | 8 | Client subscribe request |
| SUBACK | 9 | Subscribe acknowledgement |
| UNSUBSCRIBE | 10 | Client unsubscribe request |
| UNSUBACK | 11 | Unsubscribe acknowledgement |
| PINGREQ | 12 | Ping request |
| PINGRESP | 13 | Ping response |
| DISCONNECT | 14 | Client disconnection request |
| Reserved | 15 | Reserved for future use |

#### 6.4.1.1	Fixed header of MQTT PUBLISH Packet

A fixed header of MQTT PUBLISH Packet consists of RETAIN, QoS Level, DUP flag, Packet Type, Remaining Length fields as shown in Figure 6.4.1.1-1.

The QoS Level represents the QoS level of the MQTT PUBLISH Packet with possible values of 0, 1 or 2. However, since oneM2M messages are idempotent, the QoS Level should not be set to QoS 2.

Figure 6.4.1.1-1 Fixed header of MQTT PUBLISH Packet

NOTE:	MQTT packets are subjected to a theoretical maximum message size of 256MB, but it is good practice not to send packets that are bigger than a 100kB. If a larger amount of data needs to be sent, it should be segmented into multiple PUBLISH packets.

6.4.1.2	Variable header of MQTT PUBLISH Packet

A variable header for the MQTT PUBLISH Packet consists of two fields in the following order: Topic Name, Packet Identifier.

Figure 6.4.1.2-1 Variable header for PUBLISH Packet

The Topic Name identifies the information channel to which payload data is published. A Topic Name for oneM2M Request Message is specified at clause 6.4.2. A Topic Filter for listening for and responding to a Request is specified at clause 6.4.3.

The Packet Identifier field is only present in PUBLISH Packets where the QoS level is 1 or 2.

#### 6.4.1.3	Payload of MQTT Control PUBLISH Packet

A payload for the MQTT PUBLISH Packet is oneM2M Requst Message or Response Message as specified at clause 6.5.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?