| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | PRO 29 |
| Source:* | Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com |
| Date:* | 2017-05-12 |
| Reason for Change/s:* | Developers may have difficult to understand specification. |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: PRO-2017-0106R01 STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0010-V2.5.0 |
| Clauses * | 6.4.5 Request/Response Message Flow |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The specification of MQTT Version 3.1.1, published by OASIS at 29 October 2014, describes as follows.

The CONNACK Packet is the packet sent by the Server in response to a CONNECT Packet received from a Client. The first packet sent from the Server to the Client MUST be a CONNACK Packet [MQTT-657 3.2.0-1].

If the Client does not receive a CONNACK Packet from the Server within a reasonable amount of time, the Client SHOULD close the Network Connection. A "reasonable" amount of time depends on the type of application and the communications infrastructure.

A SUBACK Packet is sent by the Server to the Client to confirm receipt and processing of a SUBSCRIBE Packet. A SUBACK Packet contains a list of return codes, that specify the maximum QoS level that was granted 1011 in each Subscription that was requested by the SUBSCRIBE.

This contribution proposes to use formal terms:

0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000 MQTT CONNECT Packet,

MQTT SUBSCRIBE Packet,

0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000 MQTT PUBLISH Packet as specifiec in OASIS MQTT Version 3.1.1.

The Consolas font means ‘name of MQTT fields’.

In Figure 6.4.5-2, the AE and CSE is not appropriate. oneM2M message can send any entity. So Originator and Receiver are suitable for message exchange between entities.

### -----------------------Start of change 1-------------------------------------------

### 6.4.5	Request/Response Message Flow

Figure 6.4.5-1: Initiating Process in MQTT binding

In the MQTT protocol, each client shall subscribe to the MQTT server to receive messages. As shown in figure 6.4.5.1-1, the AE or CSE initiates the MQTT binding process by trying to connect to the MQTT server, as described in clause 6.3.

After each MQTT client successfully connects to the server, it shall subscribe to the MQTT server. The Topic Names which each MQTT client subscribes to are "/oneM2M/req/+/<receiver>" (to receive requests) and "/oneM2M/resp/<originator>/+" (to receive replies) where <receiver> and <originator> are both set equal to the ID (SP-relative-AE-ID or SP-relative-CSE-ID as appropriate).

Accordingly the plus sign (‘+' U+002B) wildcard and the SP-relative-AE-ID or SP-relative-CSE-ID are used in the Topic Name within the MQTT SUBSCRIBE Packet. This enables the MQTT client to receive the PUBLISH Packets whose target it is. Therefore, through this process, the AE or CSE receives messages if the request/response messages are published to oneM2M/req/<originator>/<receiver> or oneM2M/resp/<originator>/<receiver>.

Figure 6.4.5-2: Request/Response message delivery over MQTT

As an example, figure 6.4.5-2 illustrates the request/response message delivery over MQTT protocol between Originator and Receiver via Mca/Mcc reference point in oneM2M. The message flow is as follows.

In this scenario, the Originator wants to send a Request message to the Receiver. The Originator’s MQTT client library sends an MQTT PUBLISH Packet to the MQTT server with "oneM2M/req/ SP-relative-AE-ID/SP-relative-CSE-ID" as the Topic Name.

The MQTT PUBLISH packet shall include {"op", "fr", "to", "rqi"} and any other optional parameters in accordance with operation (e.g. CREATE, RETRIEVE, UPDATE, DELETE, NOTIFY) specified in clause 7.2.1.1 of oneM2M TS-0004 [Error: Reference source not found] in its payload.

When the MQTT server receives the MQTT PUBLISH Packet from the MQTT client, the server refers to the Topic Name and delivers the message to the intended MQTT client. Finally, the MQTT client library delivers the message to the Receiver.

Regarding the Response message, the MQTT message is sent using "oneM2M/resp/<originator>/<receiver>" as its Topic Name. In order to send a Response message to the Originator, the Receiver may use the "fr" information from the received request message or the Topic Name of the MQTT PUBLISH Packet related with the received request message.

After that, the Receiver sends an MQTT PUBLISH Packet to the MQTT server with "oneM2M/resp/ SP-relative-AE-ID/SP-relative-CSE-ID" as the Topic Name in this scenario. The MQTT PUBLISH Packet dhall include {"rsc", "rqi"} and any other optional message parameters in accordance with operation (e.g. CREATE, RETRIEVE, UPDATE, DELETE, NOTIFY) specified in clause 7.2.1.2 of oneM2M TS-0004 [Error: Reference source not found] in its payload.

-----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?