| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | PRO#29 |
| Source:* | Datang |
| Date:* | 2017-05-22 |
| Contact:* | Wei Zhou, Datang, zhouwei@catt.cn Hui Xu, Datang, xuhui@catt.cn |
| Reason for Change/s:* | Content for distributed authorization |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <Work Item number> MNT maintenace / <Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V3.1.0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR:<> : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

### -----------------------Start of change 1---------------------------------------------

##### m2m:resourceType

Table 6.3.4.2.1-1: Interpretation of resourceType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscriptionProfile |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | trafficPattern |  |
| 34 | dynamicAuthorizationConsultation |  |
| xx | authorizationDecision |  |
| xx | authorizationPolicy |  |
| xx | authorizationInformation |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | trafficPatternAnnc |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| NOTE: See clause Error: Reference source not found "Request message parameter data types". |  |  |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

##### m2m:memberType

Used for memberType attribute in <group> resource.

Table 6.3.4.2.11-2: Interpretation of memberType

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | mixed | A mixture of all the resource types (except mixed itself). |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscription |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | trafficPattern |  |
| 34 | dynamicAuthorizationConsultation |  |
| xx | authorizationDecision |  |
| xx | authorizationPolicy |  |
| xx | authorizationInformation |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | trafficPatternAnnc |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 20001 | oldest |  |
| 20002 | latest |  |
| NOTE: See clause Error: Reference source not found "Resource Type group". |  |  |

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3---------------------------------------------

##### m2m:authorizationDecision

Used for decision attribute of <authorizationDecision> resource.

Table 6.3.4.2.xx-3: Interpretation of authorizationDecision

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | PERMIT | The requested access is permitted. |
| 2 | DENY | The requested access is denied. |
| NOTE: See clause Error: Reference source not found "Resource Type authorizationDecision". |  |  |

##### m2m:authorizationStatus

Used for status attribute of <authorizationDecision> resource.

Table 6.3.4.2.zz-4: Interpretation of authorizationStatus

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | OK | It indicates the access control policy evaluation, access control policy retrieval, or access control information retrieval process is successful. |
| 2 | NOT_APPLICABLE | It indicates there is no applicable access control policy or requested access control information. |
| 3 | MISSING_ATTRIBUTE | It indicates that some access control information necessary to make an access control decision are not available, e.g. roles or tokens. |
| 4 | SYNTAX_ERROR | It indicates there is a syntax error in access control information or an access control policy, e.g. invalid tokens or access control rules. |
| 5 | PROCESSING_ERROR | It indicates there is an error occurred during the access control policy evaluation, access control policy retrieval, or access control information retrieval process. |
| NOTE: See clause Error: Reference source not found "Resource Type authorizationDecision". |  |  |

##### m2m:acpCombiningAlgorithm

Used for combiningAlgorithm attribute of <authorizationPolicy> resource.

Table 6.3.4.2.yy-5: Interpretation of acpCombiningAlgorithm

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | PERMIT_OVERRIDES | When evaluating multiple access control policies, if a single "Permit" result is encountered, then the combined result is "Permit". |
| NOTE: See clause Error: Reference source not found "Resource Type authorizationPolicy". |  |  |

### -----------------------End of change 3---------------------------------------------

### -----------------------Start of change 4---------------------------------------------

#### m2m:ipAddress

Table 6.3.5.xx-6: Type Definition of m2m:ipAddress

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| ipv4Address | m2m:ipv4 | 0..1 | IPv4 address |
| ipv6Address | m2m:ipv6 | 0..1 | IPv6 address |

#### m2m:setOfPermissions

Used for policies attribute of <authorizationPolicy> resource.

Table 6.3.5.xx-7: Type Definition of m2m:setOfPermissions

| Element Path | Element Data Type | Mutiplicity | Note |
| --- | --- | --- | --- |
| privileges | m2m:setOfAcrs | 0..n |  |

### -----------------------End of change 4---------------------------------------------

### -----------------------Start of change 5---------------------------------------------

#### Usage

This type is used by the following resource types:

<delivery>, <eventConfig>, <execInstance>, <m2mServiceSubscriptionProfile>, <mgmtCommand>, <request>, <serviceSubscribedNode>, <statsCollect>, <statsConfig>, <subscription>, <serviceSubscribedAppRule>, <notificationTargetMgmtPolicyRef>, <notificationTargetPolicy>, <policyDeletionRules>, <dynamicAuthorizationConsultation>, <role>, <token>, <authorizationDecision>, <authorizationPolicy> <authorizationInformation>

### -----------------------End of change 5---------------------------------------------

### -----------------------Start of change 6---------------------------------------------

### Resource Type <CSEBase>

#### 7.4.3.1	Introduction

A <CSEBase> resource shall represent a CSE. This <CSEBase> resource shall be the root for all the resources that are residing on the CSE. The detailed description can be found in clause 9.6.3 in TS-0001 [Error: Reference source not found].

Table 7.4.3.1-8: Data type definition of <CSEBase> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| CSEBase | CDT-CSEBase-v3_1_0.xsd |  |

The value of the parentID attribute for the <CSEBase> resource shall be an empty string since the <CSEBase> resource does not have a parent.

Table 7.4.3.1-9: Resource Specific Attributes of <CSEBase> resource

| Attribute Name | Data Type | Default Value and Constraints |
| --- | --- | --- |
| cseType | m2m:cseTypeID | No default |
| CSE-ID | m2m:ID | No default |
| supportedResourceType | list of m2m:resourceType | No default |
| pointOfAccess | m2m:poaList | No default |
| nodeLink | xs:anyURI | No default |
| dynamicAuthorizationConsultationIDs | list of xs:anyURI | No default |
| e2eSecInfo | m2m:e2eSecInfo | No default |

Table 7.4.3.1-10: Child resources of <CSEBase> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <remoteCSE> | [variable] | 0..n | Clause Error: Reference source not found |
| <remoteCSEAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <node> | [variable] | 0..n | Clause Error: Reference source not found |
| <AE> | [variable] | 0..n | Clause Error: Reference source not found |
| <container> | [variable] | 0..n | Clause Error: Reference source not found |
| <group> | [variable] | 0..n | Clause Error: Reference source not found |
| <accessControlPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <mgmtCmd> | [variable] | 0..n | Clause Error: Reference source not found |
| <locationPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <statsConfig> | [variable] | 0..n | Clause Error: Reference source not found |
| <statsCollect> | [variable] | 0..n | Clause Error: Reference source not found |
| <request> | [variable] | 0..n | Clause Error: Reference source not found |
| <delivery> | [variable] | 0..n | Clause Error: Reference source not found |
| <schedule> | [variable] | 0..1 | Clause Error: Reference source not found |
| <m2mServiceSubscriptionPolicy | [variable] | 0..n | Clause Error: Reference source not found |
| <serviceSubscribedAppRule> | [variable] | 0..n | Clause Error: Reference source not found |
| <notificationTargetPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <dynamicAuthorizationConsultation> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainer> | [variable] | 0..n | Clause Error: Reference source not found |
| <timeSeries> | [variable] | 0..n | Clause Error: Reference source not found |
| <role> | [variable] | 0..n | Clause Error: Reference source not found |
| <token> | [variable] | 0..n | Clause 7 |
| <authorizationDecision> | [variable] | 0..n | Clause 7 |
| <authorizationPolicy> | [variable] | 0..n | Clause 7 |
| <authorizationInformation> | [variable] | 0..n | Clause 7 |

### -----------------------End of change 6---------------------------------------------

### -----------------------Start of change 7---------------------------------------------

### Resource Type <authorizationDecision>

#### 7.4.43.1	Introduction

The <authorizationDecision> resource represents an access control decision.

The detailed description can be found in clause 9.6.42 in TS-0001 [6].

Table 7.4.43.1-11: Data type definition of <authorizationDecision> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| authorizationDecision | CDT-authorizationDecision-v3_0_0.xsd (TBD) |  |

Table 7.4.41.1-12: Universal/Common Attributes of <authorizationDecision> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | NP | NP |
| announcedAttribute | NP | NP |
| dynamicAuthorizationConsultationIDs | NP | NP |

Table 7.4.41.1-13: Resource Specific Attributes of <authorizationDecision> resource

| Attribute Name |  | Request Optionality |  |  |  | Data Type |  | Default Value and Constraints |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Create |  | Update |  |  |  |  |
| decision | O |  | O |  | m2m:authorizationDecision |  | NULL |  |
| status | O |  | O |  | m2m:authorizationStatus |  | NULL |  |
| to | O |  | O |  | xs:anyURI |  | NULL |  |
| from | O |  | O |  | m2m:ID |  | NULL |  |
| operation | O |  | O |  | m2m:operation |  | NULL |  |
| requestedResourceType | O |  | O |  | m2m:resourceType |  | NULL |  |
| filterUsage | O |  | O |  | m2m:filterCriteria |  | NULL |  |
| roleIDs | O |  | O |  | List of m2m:roleID |  | NULL |  |
| tokenIDs | O |  | O |  | List of m2m:tokenID |  | NULL |  |
| tokens | O |  | O |  | List of m2m:dynAuthJWT |  | NULL |  |
| requestTime | O |  | O |  | m2m:timestamp |  | NULL |  |
| originatorLocation | O |  | O |  | m2m:locationRegion |  | NULL |  |
| originatorIP | O |  | O |  | m2m:ipAddress |  | NULL |  |

Table 7.4.41.1-14: Child Resources of <authorizationDecision> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |

#### 7.4.40.2	<authorizationDecision> resource specific procedure on CRUD operations

##### 7.4.40.2.0	Introduction

This clause describes <authorizationDecision> resource specific behaviour for CRUD operations.

##### Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

The Hosting CSE shall check whether a PDP process is bound to the <authorizationDecision> resource. If it is true, the Hosting CSE shall trigger the PDP process, otherwise no other changes from the generic procedures in clause Error: Reference source not found.

The triggered PDP process shall perform as follows:

- Checking if the access control decision request satisfies the following conditions. If it is true, continue to do the following steps, otherwise stop this process and set the error response code with “BAD_REQUEST”.

- Only to, from, operation, resourceType, filterUsage, roleIDs, tokenIDs, tokens, requestTime, originatorLocation, and/or originatorIP resource attributes are in the updating list.

- All the mandatory resource attributes (i.e. to, from and operation attributes) used for constructing an access control decision request are in the updating list.

- The formats of the updating values are all correct.

- Deleting any existing resource specific attributes before performing the resource update operation, and then performing the resource update operation.

- Constructing an access control decision request with the updated resource attributes.

- Obtaining applicable access control policies and information according to the access control decision request. If it is no successful, the status attribute shall be set as follows:

- If the process cannot obtain applicable access control policies or information without error, update the status attribute with “NOT_APPLICABLE” and go to step 6.

- If the process cannot obtain applicable access control policies or information with error, update the status attribute with “INDETERMINATE” and go to step 6.

NOTE:	How to obtain the applicable access control policies or information is out of scope of the specification.

- Evaluating the access control decision request against access control policies as specified in oneM2M TS-0003 [7] and updating the decision and status attribute with the evaluation result as follows:

- If some access control information requested by the evaluation process is not provided, update the status attribute with “MISSING_ATTRIBUTE” and go to step 6.

- If there are some errors in access control policies, update the status attribute with “SYNTAX_ERROR” and go to step 6.

- If there is an error appears during the evaluation, update the status attribute with “PROCESSING_ERROR” and go to step 6.

- If the access control policy evaluation is successful, update the decision attribute with the evaluation result (i.e. “ PERMIT” or “DENY”) and status attribute with “OK” and go to step 6.

- Generating an UPDATE response using the decision and/or status attribute and returning it back to the requester.

- All the resource specific attributes may be deleted after the response being sent in order to avoid information leak.

##### Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### Resource Type <authorizationPolicy>

#### 7.4.43.1	Introduction

The <authorizationPolicy> resource represents a set of access control policies.

The detailed description can be found in clause 9.6.43 in TS-0001 [6].

Table 7.4.43.1-15: Data type definition of <authorizationPolicy> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| authorizationPolicy | CDT-authorizationPolicy-v3_0_0.xsd (TBD) |  |

Table 7.4.41.1-16: Universal/Common Attributes of <authorizationPolicy> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | NP | NP |
| announcedAttribute | NP | NP |
| dynamicAuthorizationConsultationIDs | NP | NP |

Table 7.4.41.1-17: Resource Specific Attributes of <authorizationPolicy> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| policies | O | O | m2m:setOfPermissions | NULL |
| combiningAlgorithm | O | O | m2m:acpCombiningAlgorithm | NULL |
| status | O | O | m2m:authorizationStatus | NULL |
| to | O | O | xs:anyURI | NULL |

Table 7.4.41.1-18: Child Resources of <authorizationPolicy> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |

#### 7.4.40.2	<authorizationPolicy> resource specific procedure on CRUD operations

##### 7.4.40.2.0	Introduction

This clause describes <authorizationPolicy> resource specific behaviour for CRUD operations.

##### Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

The Hosting CSE shall check whether a PRP process is bound to the <authorizationPolicy> resource. If it is true, the Hosting CSE shall trigger the PRP process, otherwise no other changes from the generic procedures in clause Error: Reference source not found.

The triggered PRP process shall perform as follows:

- Checking if the access control policy request satisfies the following conditions. If it is true, continue to do the following steps, otherwise stop this process and set the error response code with “BAD_REQUEST”.

- Only to resource attribute is in the updating list.

- The format of the updating value is correct.

- Deleting any existing resource specific attributes before performing the resource update operation, and then performing the resource update operation.

- Constructing an access control policy request with the updated resource attributes.

- Obtaining applicable access control policies and policy combining algorithm according to the access control policy request and set the policies, combiningAlgorithm and status attributes as follows:

- If the process cannot obtain applicable access control policies without error, update the status attribute with “NOT_APPLICABLE” and go to step 6.

- If there is an error appears during the access control policy retrieval process, update the status attribute with “PROCESSING_ERROR” and go to step 6.

- If the access control policy retrieval is successful, update the policies attribute with retrieved access control policies, combiningAlgorithm attribute with policy combining algorithm and status attribute with “OK” and go to step 6.

NOTE:	How to obtain the applicable access control policies is out of scope of the specification.

- Generating an UPDATE response using the policies, combiningAlgorithm and/or status attributes and returning it back to the requester.

- All the resource specific attributes may be deleted after the response being sent in order to avoid information leak.

##### Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### Resource Type <authorizationInformation>

#### 7.4.43.1	Introduction

The <authorizationInformation> resource represents access control information.

The detailed description can be found in clause 9.6.44 in TS-0001 [6].

Table 7.4.43.1-19: Data type definition of <authorizationInformation> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| authorizationInformation | CDT-authorizationInformation-v3_0_0.xsd (TBD) |  |

Table 7.4.41.1-20: Universal/Common Attributes of <authorizationInformation> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlInformationIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | NP | NP |
| announcedAttribute | NP | NP |
| dynamicAuthorizationConsultationIDs | NP | NP |

Table 7.4.41.1-21: Resource Specific Attributes of <authorizationInformation> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| status | O | O | m2m:authorizationStatus | NULL |
| from | O | O | m2m:ID | NULL |
| roleIDs | O | O | List of m2m:roleID | NULL |
| tokenIDs | O | O | List of m2m:tokenID | NULL |

Table 7.4.41.1-22: Child Resources of <authorizationInformation> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <role> | [variable] | 0..n | Clause Error: Reference source not found |
| <token> | [variable] | 0..n | Clause Error: Reference source not found |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |

#### 7.4.40.2	<authorizationInformation> resource specific procedure on CRUD operations

##### 7.4.40.2.0	Introduction

This clause describes <authorizationInformation> resource specific behaviour for CRUD operations.

##### Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

The Hosting CSE shall check whether a PIP process is bound to the <authorizationInformation> resource. If it is true, the Hosting CSE shall trigger the PIP process, otherwise no other changes from the generic procedures in clause Error: Reference source not found.

The triggered PIP process shall perform as follows:

- Checking if the access control information request satisfies the following conditions. If it is true, continue to do the following steps, otherwise stop this process and set the error response code with “BAD_REQUEST”.

- Only from, roleIDs and/or tokenIDs resource attributes are in the updating list.

- All the mandatory resource attributes (i.e. from attribute) used for constructing an access control information request are in the updating list.

- The format of the updating value is correct.

- Deleting any existing resource specific attributes and child resources before performing the resource update operation, and then performing the resource update operation.

- Constructing an access control information request with the updated resource attributes.

- Obtaining requested access control information (e.g. <role> and/or <token> resources)  according to the access control information request and create <role> and/or <token> resources and set the status attribute as follows:

- If the process cannot obtain requested access control information without error, update the status attribute with “NOT_APPLICABLE” and go to step 6.

- If there is an error appears during the access control information retrieval process, update the status attribute with “PROCESSING_ERROR” and go to step 6.

- If the access control information retrieval is successful, create corresponding <role> and/or <token> resources and update the status attribute with “OK” and go to step 6.

NOTE:	How to obtain the requested access control information is out of scope of the specification.

- Generating an UPDATE response using the <role> and/or <token> resources and/or status attribute and returning it back to the requester.

- All the resource specific child resource and attributes may be deleted after the response being sent in order to avoid information leak.

##### Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### -----------------------End of change 7---------------------------------------------

### -----------------------Start of change 8---------------------------------------------

## Short names

### Introduction

XML, JSON and CBOR representations require encoding of the names of primitive parameters, resource attributes, (in the case of XML) resource types and complex data types members. Whenever a protocol binding transfers such a name over a oneM2M reference point, it shall use a shortened form of that name, rather than the full name that is used elsewhere in this and other oneM2M specifications. Short names enable payload reduction on involved telecommunication interfaces.

The mapping between the full names and their shortened form is given in the clauses 8.2.2 to 8.2.5.

These names are case-sensitive. A oneM2M implementation shall use the letter casing given in these clauses.

### Primitive parameters

In protocol bindings primitive parameter names shall be translated into short names of Table 8.2.2-23.

Table 8.2.2-23: Primitive parameter short names

| Parameter Name | XSD long name | Occurs in | Short Name |
| --- | --- | --- | --- |
| Operation | operation | Request | op |
| To | to | Request, Response | to |
| From | from | Request, Response | fr |
| Request Identifier | requestIdentifier | Request, Response | rqi |
| Resource Type | resourceType | Request | ty |
| Content | primitiveContent | Request, Response | pc |
| Role IDs | roleIDs | Request | rids |
| Originating Timestamp | originatingTimestamp | Request, Response | ot |
| Request Expiration Timestamp | requestExpirationTimestamp | Request | rqet |
| Result Expiration Timestamp | resultExpirationTimestamp | Request, Response | rset |
| Operation Execution Time | operationExecutionTime | Request | oet |
| Response Type | responseType | Request | rt |
| Result Persistence | resultPersistence | Request | rp |
| Result Content | resultContent | Request | rcn |
| Event Category | eventCategory | Request, Response | ec |
| Delivery Aggregation | deliveryAggregation | Request | da |
| Group Request Identifier | groupRequestIdentifier | Request | gid |
| Filter Criteria | filterCriteria | Request | fc |
| Discovery Result Type | discoveryResultType | Request | drt |
| Response Status Code | responseStatusCode | Response | rsc |
| Tokens | tokens | Request | tkns |
| Token IDs | tokenIDs | Request | tids |
| Token Request Indicator | tokenRequestIndicator | Request | tqi |
| Local Token IDs | localTokenIDs | Request | ltids |
| Group Request Target Members | groupRequestTargetMembers | Request | grtm |
| Assigned Token Identifiers | assignedTokenIdentifiers | Response | ati |
| Token Request Information | tokenRequestInformation | Response | tqf |
| Content Status | contentStatus | Response | cnst |
| Content Offset | contentOffset | Response | cnot |
| Authorization Signature Indicator | authorSignIndicator | Request | asi |
| Authorization Signature Request Information | authorSignReqInfo | Request, Response | asri |
| Authorization Signatures | authorSigns | Request | aus |
| Authorization Relationship Indicator | authorRelIndicator | Request | auri |

XML serialized representations of primitives employ root element names to differentiate between request and response primitive types (see clause Error: Reference source not found). These root element names shall be translated into short names as in Table 8.2.2-24.

Table 8.2.2-24: Primitive root element short names

| Root Element Name | Occurs in | Short Name |
| --- | --- | --- |
| requestPrimitive | Request | rqp |
| responsePrimitive | Response | rsp |

### Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

Table 8.2.3-25: Resource attribute short names (1/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| accessControlPolicyIDs | All except accessControlPolicy, contentInstance | acpi |
| announcedAttribute | accessControlPolicy, AE, container, contentInstance, group, locationPolicy, mgmtObj, node, remoteCSE, schedule, semanticDescriptor, trafficPattern | aa |
| announceTo | accessControlPolicy, AE, container, contentInstance, group, locationPolicy, mgmtObj, node, remoteCSE, schedule, semanticDescriptor, trafficPattern | at |
| creationTime | All | ct |
| expirationTime | All except contentInstance, CSEBase | et |
| labels | All (optional) | lbl |
| lastModifiedTime | All | lt |
| Link | All | lnk |
| parentID | All | pi |
| resourceID | All | ri |
| resourceType | All | ty* |
| stateTag | container, contentInstance, delivery, request | st |
| resourceName | All | rn |
| privileges | accessControlPolicy | pv |
| selfPrivileges | accessControlPolicy | pvs |
| App-ID | AE | api |
| AE-ID | AE | aei |
| appName | AE | apn |
| pointOfAccess | AE, CSEBase, remoteCSE | poa |
| ontologyRef | AE, container, contentInstance, semanticDescriptor. flexContainer, timeSeries | or |
| nodeLink | AE, CSEBase, remoteCSE | nl |
| contentSerialization | AE | csz |
| creator | container, contentInstance,eventConfig, group, pollingChannel, statsCollect, statsConfig, subscription, semanticDescriptor, notificationTargetPolicy, flexContainer, timeSeries | cr |
| maxNrOfInstances | container, timeSeries | mni |
| maxByteSize | container, timeSeries | mbs |
| maxInstanceAge | container, timeSeries | mia |
| currentNrOfInstances | container, timeSeries | cni |

Table 8.2.3-26: Resource attribute short names (2/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| currentByteSize | container | cbs |
| locationID | container | li |
| disableRetrieval | container | disr |
| contentInfo | contentInstance | cnf |
| contentSize | contentInstance | cs |
| contentRef | contentInstance | conr |
| containerDefinition | flexContainer | cnd |
| primitiveContent | request | pc* |
| content | contentInstance, timeSeriesInstance | con |
| cseType | CSEBase, remoteCSE | cst |
| CSE-ID | CSEBase, remoteCSE, service SubscribedNode | csi |
| supportedResourceType | CSEBase | srt |
| notificationCongestionPolicy | CSEBase | ncp |
| source | delivery | sr |
| target | delivery, request | tg |
| lifespan | delivery | ls |
| eventCat | delivery | ec |
| deliveryMetaData | delivery | dmd |
| aggregatedRequest | delivery | arq |
| eventID | eventConfig, statsCollect | evi |
| eventType | eventConfig | evt |
| evenStart | eventConfig | evs |
| eventEnd | eventConfig | eve |
| operationType | eventConfig | opt |
| dataSize | eventConfig | ds |
| execStatus | execInstance | exs |
| execResult | execInstance | exr |
| execDisable | execInstance | exd |
| execTarget | execInstance, mgmtCmd | ext |
| execMode | execInstance, mgmtCmd | exm |
| execFrequency | execInstance, mgmtCmd | exf |
| execDelay | execInstance, mgmtCmd | exy |
| execNumber | execInstance, mgmtCmd | exn |
| execReqArgs | execInstance, mgmtCmd | exra |
| execEnable | mgmtCmd | exe |
| memberType | group | mt |
| currentNrOfMembers | group | cnm |
| maxNrOfMembers | group | mnm |
| memberIDs | group | mid |
| membersAccessControlPolicyIDs | group | macp |
| memberTypeValidated | group | mtv |
| consistencyStrategy | group | csy |
| semanticSupportIndicator | group | ssi |
| groupName | group, subscription | gn |
| locationSource | locationPolicy | los |
| locationUpdatePeriod | locationPolicy | lou |
| locationTargetID | locationPolicy | lot |
| locationServer | locationPolicy | lor |
| locationContainerID | locationPolicy | loi |
| locationContainerName | locationPolicy | lon |
| locationStatus | locationPolicy | lost |
| description | mgmtCmd, mgmtObj, all management resources from firmware | dc |
| cmdType | mgmtCmd | cmt |
| mgmtDefinition | mgmtObj, all management resources from firmware | mgd |
| objectIDs | mgmtObj | obis |

Table 8.2.3-27: Resource attribute short names (3/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| objectPaths | mgmtObj | obps |
| nodeID | node | ni |
| hostedCSELink | node | hcl |
| mgmtClientAddress | node | mgca |
| CSEBase | remoteCSE | cb* |
| M2M-Ext-ID | remoteCSE | mei |
| Trigger-Recipient-ID | remoteCSE | tri |
| requestReachability | remoteCSE | rr |
| triggerReferenceNumber | remoteCSE | trn |
| descendantCSEs | remoteCSE | dcse |
| originator | request | org |
| metaInformation | request | mi |
| requestStatus | request | rs |
| operationResult | request | ors |
| operation | request | op* |
| requestID | request | rid |
| scheduleElement | schedule | se |
| deviceIdentifier | serviceSubscribedNode | di |
| ruleLinks | serviceSubscribedNode | rlk |
| statsCollectID | statsCollect | sci |
| collectingEntityID | statsCollect | cei |
| collectedEntityID | statsCollect | cdi |
| devStatus | areaNwkDeviceInfo | ss |
| statsRuleStatus | statsCollect | srs |
| statModel | statsCollect | sm |
| collectPeriod | statsCollect | cp |
| eventNotificationCriteria | subscription | enc |
| expirationCounter | subscription | exc |
| notificationURI | subscription | nu |
| groupID | subscription | gpi |
| notificationForwardingURI | subscription | nfu |
| batchNotify | subscription | bn |
| rateLimit | subscription | rl |
| preSubscriptionNotify | subscription | psn |
| pendingNotification | subscription | pn |
| notificationStoragePriority | subscription | nsp |
| latestNotify | subscription | ln |
| notificationContentType | subscription | nct |
| notificationEventCat | subscription | nec |
| subscriberURI | subscription | su |
| version | firmware, software, token | vr |
| URL | firmware, software | url |
| update | firmware | ud |
| updateStatus | firmware | uds |
| install | software | in |
| uninstall | software | un |
| installStatus | software | ins |
| activate | software | act |
| deactivate | software | dea |
| activeStatus | software, areaNwkInfo | acts |
| memAvailable | memory | mma |
| memTotal | memory | mmt |

Table 8.2.3-28: Resource attribute short names (4/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| areaNwkType | areaNwkInfo | ant |
| listOfDevices | areaNwkInfo | ldv |
| devId | areaNwkDeviceInfo | dvd |
| devType | areaNwkDeviceInfo | dvt |
| areaNwkId | areaNwkDeviceInfo | awi |
| sleepInterval | areaNwkDeviceInfo | sli |
| sleepDuration | areaNwkDeviceInfo | sld |
| listOfNeighbors | areaNwkDeviceInfo | lnh |
| batteryLevel | battery | btl |
| batteryStatus | battery | bts |
| deviceLabel | deviceInfo | dlb |
| manufacturer | deviceInfo | man |
| model | deviceInfo | mod |
| deviceType | deviceInfo | dty |
| fwVersion | deviceInfo | fwv |
| swVersion | deviceInfo | swv |
| hwVersion | deviceInfo | hwv |
| capabilityName | deviceCapability | can |
| attached | deviceCapability | att |
| capabilityActionStatus | deviceCapability | cas |
| enable | deviceCapability, allJoynSvcObject | ena |
| disable | deviceCapability | dis |
| currentState | deviceCapability | cus |
| reboot | reboot | rbo |
| factoryReset | reboot | far |
| logTypeId | eventLog | lgt |
| logData | eventLog | lgd |
| logStatus | eventLog | lgst |
| logStart | eventLog | lga |
| logStop | eventLog | lgo |
| firmwareName | firmware | fwn |
| softwareName | software | swn |
| cmdhPolicyName | cmdhPolicy | cpn |
| mgmtLink | cmdhPolicy, activeCmdhPolicy, cmdhDefaults, cmdhNetworkAccessRules, cmdhNwAccessRule | cmlk |
| activeCmdhPolicyLink | activeCmdhPolicy | acmlk |
| order | cmdhDefEcValue, cmdhLimits | od |
| defEcValue | cmdhDefEcValue | dev |
| requestOrigin | cmdhDefEcValue, cmdhLimits | ror |
| requestContext | cmdhDefEcValue, cmdhLimits | rct |
| requestContextNotification | cmdhDefEcValue, cmdhLimits | rctn |
| requestCharacteristics | cmdhDefEcValue, cmdhLimits | rch |
| applicableEventCategories | cmdhNetworkAccessRules | aecs |
| applicableEventCategory | cmdhEcDefParamValues, cmdhBuffer | aec |
| defaultRequestExpTime | cmdhEcDefParamValues | dqet |
| defaultResultExpTime | cmdhEcDefParamValues | dset |
| defaultOpExecTime | cmdhEcDefParamValues | doet |
| defaultRespPersistence | cmdhEcDefParamValues | drp |
| defaultDelAggregation | cmdhEcDefParamValues | dda |
| limitsEventCategory | cmdhLimits | lec |
| limitsRequestExpTime | cmdhLimits | lqet |
| limitsResultExpTime | cmdhLimits | lset |
| limitsOpExecTime | cmdhLimits | loet |
| limitsRespPersistence | cmdhLimits | lrp |
| limitsDelAggregation | cmdhLimits | lda |
| targetNetwork | cmdhNwAccessRule, trafficPattern | ttn |

Table 8.2.3-29: Resource attribute short names (5/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| minReqVolume | cmdhNwAccessRule | mrv |
| spreadingWaitTime | cmdhNwAccessRule | swt |
| backOffParameters | cmdhNwAccessRule | bop |
| otherConditions | cmdhNwAccessRule | ohc |
| maxBufferSize | cmdhBuffer | mbfs |
| storagePriority | cmdhBuffer | sgp |
| applicableCredIDs | serviceSubscribedAppRule | apci |
| allowedApp-IDs | serviceSubscribedAppRule | aai |
| allowedAEs | serviceSubscribedAppRule | aae |
| allowedRole-IDs | serviceSubscribedAppRule | ari |
| notificationTargetURI | notificationTargetMgmtPolicyRef | ntu |
| notificationlPolicyID | notificationTargetMgmtPolicyRef | npi |
| action | notificationTargetPolicy | ac |
| policyLabel | notificationTargetPolicy | plbl |
| rulesRelationship | notificationTargetPolicy | rrs |
| creator | notificationTargetPolicy | cr |
| deletionRules | policyDeletionRules | dr |
| deletionRulesRelation | policyDeletionRules | drr |
| dynamicAuthorizationConsultationIDs | All resources having an accessControlPolicyID attribute | daci |
| dynamicAuthorizationEnabled | dynamicAuthorizationConsultation | dae |
| dynamicAuthorizationPoA | dynamicAuthorizationConsultation | dap |
| dynamicAuthorizationLifetime | dynamicAuthorizationConsultation | dal |
| descriptorRepresentation | semanticDescriptor | dcrp |
| semanticOpExec | semanticDescriptor | soe |
| descriptor | semanticDescriptor | dsp |
| relatedSemantics | semanticDescriptor | rels |
| periodicInterval | timeSeries | pei |
| missingDataDetect | timeSeries | mdd |
| missingDataMaxNr | timeSeries | mdn |
| missingDataList | timeSeries | mdlt |
| missingDataCurrentNr | timeSeries | mdc |
| missingDataDetectTimer | timeSeries | mdt |
| dataGenerationTime | timeSeriesInstance | dgt |
| sequenceNr | timeSeriesInstance | snr |
| providedToNSE | trafficPattern | ptn |
| periodicIndicator | trafficPattern | pri |
| periodicDurationTime | trafficPattern | pdt |
| periodicIntervalTime | trafficPattern | pit |
| stationaryIndication | trafficPattern | sti |
| dataSizeIndicator | trafficPattern | dsi |
| validityTime | trafficPattern | vdt |
| roleID | role | rlid |
| roleName | role | rlnm |
| tokenLink | role | rltl |
| tokenID | token | tkid |
| tokenObject | token | tkob |
| issuer | token, role | tkis |
| holder | token, role | tkhd |
| notBefore | token, role | tknb |
| notAfter | token, role | tkna |
| tokenName | token | tknm |
| audience | token | tkau |
| permissions | token | tkps |
| extension | token | tkex |
| e2eSecInfo | CSEBase, remoteCSE, AE | esi |

Table 8.2.3-30: Resource attribute short names (6/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| serviceName | genericInterworkingService | gisn |
| operationName | genericInterworkingOperationInstance | gion |
| inputDataPointLinks | genericInterworkingService, genericInterworkingOperationInstance | giip |
| outputDataPointLinks | genericInterworkingService, genericInterworkingOperationInstance | giop |
| inputLinks | genericInterworkingOperationInstance | giil |
| outputLinks | genericInterworkingOperationInstance | giol |
| operationState | genericInterworkingOperationInstance | gios |
| direction | allJoynApp | dir |
| objectPath | allJoynSvcObject | ajop |
| interfaceIntrospectXmlRef | allJoynInterface | ajir |
| input | allJoynMethodCall | inp |
| callStatus | allJoynMethodCall | clst |
| output | allJoynMethodCall | out |
| currentValue | allJoynProperty | crv |
| requestedValue | allJoynProperty | rqv |
| decision | authorizationDecision | aud |
| status | authorizationDecision, authorizationPolicy, authorizationInformation | aus |
| to | authorizationDecision, authorizationPolicy | aut |
| from | authorizationDecision, authorizationInformation | auf |
| requestedResourceType | authorizationDecision | aurr |
| operation | authorizationDecision | auo |
| filterUsage | authorizationDecision | aufu |
| roleIDs | authorizationDecision, authorizationInformation | auri |
| tokenIDs | authorizationDecision, authorizationInformation | auti |
| tokens | authorizationDecision | auts |
| requestTime | authorizationDecision | aurt |
| originatorLocation | authorizationDecision | auol |
| originatorIP | authorizationDecision | auip |
| policies | authorizationPolicy | aup |
| combiningAlgorithm | authorizationPolicy | auca |
| NOTE: * marked short names have been already assigned in Table 8.2.2-1. |  |  |

### Resource types

In protocol bindings resource type names shall be translated into short names of Table 8.2.4-31.

Table 8.2.4-31: Resource and specialization type short names

| Resource Type Name | Short Name |
| --- | --- |
| accessControlPolicy | acp |
| accessControlPolicyAnnc | acpA |
| AE | ae |
| AEAnnc | aeA |
| container | cnt |
| containerAnnc | cntA |
| contentInstance | cin |
| contentInstanceAnnc | cinA |
| CSEBase | cb |
| delivery | dlv |
| eventConfig | evcg |
| execInstance | exin |
| group | grp |
| groupAnnc | grpA |
| locationPolicy | lcp |
| locationPolicyAnnc | lcpA |
| m2mServiceSubscriptionProfile | mssp |
| mgmtCmd | mgc |
| node | nod |
| nodeAnnc | nodA |
| pollingChannel | pch |
| remoteCSE | csr |
| remoteCSEAnnc | csrA |
| request | req |
| schedule | sch |
| scheduleAnnc | schA |
| serviceSubscribedAppRule | asar |
| serviceSubscribedNode | svsn |
| statsCollect | stcl |
| statsConfig | stcg |
| subscription | sub |
| firmware | fwr |
| firmwareAnnc | fwrA |
| software | swr |
| softwareAnnc | swrA |
| memory | mem |
| memoryAnnc | memA |
| areaNwkInfo | ani |
| areaNwkInfoAnnc | aniA |
| areaNwkDeviceInfo | andi |
| areaNwkDeviceInfoAnnc | andiA |
| battery | bat |
| batteryAnnc | batA |
| deviceInfo | dvi |
| deviceInfoAnnc | dviA |
| deviceCapability | dvc |
| deviceCapabilityAnnc | dvcA |
| reboot | rbo * |
| rebootAnnc | rboA |
| eventLog | evl |
| eventLogAnnc | evlA |
| cmdhPolicy | cmp |
| activeCmdhPolicy | acmp |
| cmdhDefaults | cmdf |
| cmdhDefEcValue | cmdv |
| cmdhEcDefParamValues | cmpv |
| cmdhLimits | cml |
| cmdhNetworkAccessRules | cmnr |
| cmdhNwAccessRule | cmwr |
| cmdhBuffer | cmbf |
| notificationTargetMgmtPolicyRef | ntpr |
| notificationTargetPolicy | ntp |
| policyDeletionRules | pdr |
| dynamicAuthorizationConsultation | dac |
| semanticDescriptor | smd |
| semanticDescriptorAnnc | smdA |
| timeSeries | ts |
| timeSeriesAnnc | tsA |
| timeSeriesInstance | tsi |
| timeSeriesInstanceAnnc | tsiA |
| trafficPattern | trpt |
| trafficPatternAnnc | trptA |
| role | rol |
| token | tk |
| genericInterworkingService | gis |
| genericInterworkingServiceAnnc | gisA |
| genericInterworkingOperationInstance | gio |
| genericInterworkingOperationInstanceAnnc | gioA |
| svcObjWrapper | ajsw |
| svcObjWrapperAnnc | ajswA |
| svcFwWrapper | ajfw |
| svcFwWrapperAnnc | ajfwA |
| allJoynApp | ajap |
| allJoynAppAnnc | ajapA |
| allJoynSvcObject | ajso |
| allJoynSvcObjectAnnc | ajsoA |
| allJoynInterface | ajif |
| allJoynInterfaceAnnc | ajifA |
| allJoynMethod | ajmd |
| allJoynMethodAnnc | ajmdA |
| allJoynMethodCall | ajmc |
| allJoynMethodCallAnnc | ajmcA |
| allJoynProperty | ajpr |
| allJoynPropertyAnnc | ajprA |
| authorizationDecision | aud* |
| authorizationPolicy | aup* |
| authorizationInformation | aui |
| NOTE: * marked short names have been already assigned in attribute Tables 8.2.3-1 to 8.2.3-5. |  |

### Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-32: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| createdBefore | filterCriteria, eventNotificationCriteria | crb |
| createdAfter | filterCriteria, eventNotificationCriteria | cra |
| modifiedSince | filterCriteria, eventNotificationCriteria | ms |
| unmodifiedSince | filterCriteria, eventNotificationCriteria | us |
| stateTagSmaller | filterCriteria, eventNotificationCriteria | sts |
| stateTagBigger | filterCriteria, eventNotificationCriteria | stb |
| expireBefore | filterCriteria, eventNotificationCriteria | exb |
| expireAfter | filterCriteria, eventNotificationCriteria | exa |
| labels | filterCriteria, eventNotificationCriteria | lbl * |
| resourceType | filterCriteria, accessControlObjectDetails | ty * |
| sizeAbove | filterCriteria, eventNotificationCriteria | sza |
| sizeBelow | filterCriteria, eventNotificationCriteriay | szb |
| contentType | filterCriteria | cty |
| limit | filterCriteria | lim |
| attribute | filterCriteria, eventNotificationCriteria | atr |
| contentFilterSyntax | filterCriteria | cfs |
| contentFilterQuery | filterCriteria | cfq |
| level | filterCriteria | lvl |
| offset | filterCriteria | ofst |
| notificationEventType | eventNotificationCriteria | net |
| operationMonitor | eventNotificationCriteria, notificationEvent | om |
| representation | notificationEvent | rep |
| filterUsage | filterCriteria | fu |
| eventCatType | eventCat | ect |
| eventCatNo | eventCat | ecn |
| number | batchNotify | num |
| duration | batchNotify | dur |
| notification | aggregatedNotification, Request Primitive Content | sgn |
| notificationEvent | notification | nev |
| verificationRequest | notification | vrq |
| subscriptionDeletion | notification | sud |
| subscriptionReference | notification | sur |
| creator | notification | cr* |
| notificationForwardingURI | notification | nfu* |
| IPEDiscoveryRequest | notification | idr |
| filterCriteria | IPEDiscoveryRequest | fc* |
| operation | operationMonitor, dynAuthDasRequest | op* |
| operations | operationMonitor | ops |
| originator | operationMonitor, IPEDiscoveryRequest, dynAuthDasRequest | or* |
| accessId | externalID | aci |
| MSISDN | externalID | msd |
| action | actionStatus | can |
| status | actionStatus | sus |
| childResource | All except execInstance, announced resource, management resources from firmware | ch |
| accessControlRule | privileges, selfPrivileges | acr |
| accessControlOriginators | accessControlRule | acor |
| accessControlOperations | accessControlRule | acop |
| accessControlContexts | accessControlRule | acco |
| accessControWindow | accessControlContexts | actw |
| accessControlIpAddresses | accessControlContexts | acip |
| ipv4Addresses | accessControlIpAddress | ipv4 |
| ipv6Addresses | accessControlIpAddress | ipv6 |
| accessControlLocationRegion | accessControlContexts | aclr |
| countryCode | accessControlLocationRegion | accc |
| circRegion | accessControlLocationRegion | accr |
| name | attribute, anyArgType, mgmtLinkRef, childResourceRef, contentRef | nm* |
| specializationID | childResourceRef, accessControlObjectDetails | spid |
| value | attribute | val |
| type | anyArgType | typ |
| maxNrOfNotify | rateLimit | mnn |
| timeWindow | rateLimit | tww |
| scheduleEntry | scheduleElement | sce |
| aggregatedNotification | Request Primitive Content | agn |
| attributeList | Request Primitive Content | atrl |
| securityInfo | Request Primitive Content, Response Primitive Content | seci |
| aggregatedResponse | Response Primitive Content | agr |
| resource | Response Primitive Content | rce |
| URIList | Response Primitive Content | uril |
| debugInfo | Response Primitive Content | dbg |
| anyArg | resetArgsType, rebootArgsType, uploadArgsType, downloadArgsType, softwareInstallArgsType softwareUpdateArgsType, softwareUninstallArgsType, execReqArgsListType | any |
| fileType | downloadArgsType | ftyp |
| URI | resourceWrapper, dynAuthTokenReqInfo | uri |
| URL | downloadArgsType | url* |
| username | uploadArgsType, downloadArgsType, softwareUpdateArgsType, softwareUninstallArgsType, | unm |
| password | uploadArgsType, downloadArgsType, softwareUpdateArgsType, softwareUninstallArgsType, | pwd |
| filesize | downloadArgsType | fsi |
| targetFile | downloadArgsType | tgf |
| delaySeconds | downloadArgsType | dss |
| successURL | downloadArgsType | surl |
| startTime | downloadArgsType | stt |
| completeTime | downloadArgsType | cpt |
| UUID | softwareInstallArgsType softwareUpdateArgsType, softwareUninstallArgsType, | uuid |
| executionEnvRef | softwareInstallArgsType softawareUpdateArgsType, softwareUninstallArgsType, | eer |
| version | softwareUninstallArgsType, tokenClaimSet | vr* |
| reset | execReqArgsListType | rst |
| reboot | execReqArgsListType | rbo* |
| upload | execReqArgsListType | uld |
| download | execReqArgsListType | dld |
| softwareInstall | execReqArgsListType | swin |
| softwareUpdate | execReqArgsListType | swup |
| softwareUninstall | execReqArgsListType | swun |
| tracingOption | deliveryMetaData | tcop |
| tracingInfo | deliveryMetaData | tcin |
| responseTypeValue | responseTypeInfo | rtv |
| notificationURI | responseTypeInfo | nu |
| timeOfDay | deletionContexts | tod |
| locationRegions | deletionContexts | lr |
| URIReference | contentRef | urir |
| semanticsFilter | filterCriteria | smf |
| missingDataList | timeSeries | mdl |
| missingData | eventNotificationCriteria | md |
| tokenID | tokenClaimSet, dynAuthLocalTokenIdAssignments | tkid |
| holder | tokenClaimSet | tkhd* |
| issuer | tokenClaimSet | tkis* |
| notBefore | tokenClaimSet | tknb* |
| notAfter | tokenClaimSet | tkna* |
| tokenName | tokenClaimSet | tknm* |
| audience | tokenClaimSet | tkau* |
| permissions | tokenClaimSet | tkps* |
| extension | tokenClaimSet | tkex* |
| permission | tokenPermissions | pm |
| resourceIDs | tokenPermission | ris |
| privileges | tokenPermission, setOfPermissions | pv* |
| roleIDs | tokenPermission | rids* |
| localTokenIdAssignment | dynAuthLocalTokenIdAssignments | ltia |
| localTokenID | dynAuthLocalTokenIdAssignment | lti |
| dasInfo | dynAuthTokenReqInfo | dasi |
| dasRequest | dynAuthTokenReqInfo | daq |
| securedDasRequest | dynAuthTokenReqInfo | sdr |
| filterOperation | filterCriteria | fo |
| targetedResourceType | dynAuthDasRequest | trt |
| originatorIP | dynAuthDasRequest | oip |
| ipv4Address | dynAuthDasRequest, ipAddress | ip4 |
| ipv6Address | dynAuthDasRequest, ipAddress | ip6 |
| originatorLocation | dynAuthDasRequest | olo |
| originatorRoleIDs | dynAuthDasRequest | orid |
| requestTimestamp | dynAuthDasRequest | rts |
| targetedResourceID | dynAuthDasRequest | trid |
| proposedPrivilegesLifetime | dynAuthDasRequest | ppl |
| roleIDsFromACPs | dynAuthDasRequest | rfa |
| tokenIDs | dynAuthDasRequest | tids |
| dynamicACPInfo | dynAuthDasResponse | dai |
| grantedPrivileges | dynAuthDasResponse | gp |
| privilegesLifetime | dynAuthDasResponse | pl |
| tokens | dynAuthDasResponse | tkns |
| securityInfoType | securityInfo | sit |
| dasRequest | securityInfo | dreq |
| dasResponse | securityInfo | dres |
| esprimRandObject | securityInfo | ero |
| esprimObject | securityInfo | epo |
| escertkeMessage | securityInfo | eckm |
| resourceRef | listOfChildResourceRef | rrf |
| resourceRefList | Response Primitive Content | rrl |
| esprimRandID | originatorESPrimRandObject, receiverESPrimRandObject, | esri |
| esprimRandValue | originatorESPrimRandObject, receiverESPrimRandObject, | esrv |
| esprimRandExpiry | originatorESPrimRandObject, receiverESPrimRandObject, | esrx |
| esprimKeyGenAlgID | originatorESPrimRandObject, | esk |
| esprimKeyGenAlgIDs | receiverESPrimRandObject, | esks |
| esprimProtocolAndAlgIDs | originatorESPrimRandObject, receiverESPrimRandObject, | espa |
| supportede2ESecFeatures | e2eSecInfo | esf |
| certificates | e2eSecInfo | escert |
| sharedReceiverESPrimRandObject | e2eSecInfo | esro |
| networkAction | backOffParameters | nwa |
| initialBackoffTime | backOffParameters | ibt |
| additionalBackoffTime | backOffParameters | abt |
| maximumBackoffTime | backOffParameters | mbt |
| optionalRandomBackoffTime | backOffParameters | rbt |
| backOffParametersSet | backOffParameters | bops |
| dataLink | listOfDataLinks | dali |
| attributeName | dataLink | atn |
| dataContainerID | dataLink | dcid |
| accessControlAuthenticationFlag | accessControlRule | acaf |
| accessControlObjectDetails | accessControlRule | acod |
| dataLinkEntry | listOfDataLinks | dle |
| childResourceType | accessControlObjectDetails | chty |
| NOTE: * marked short names have been already assigned in attribute Table 8.2.3-1. |  |  |

### -----------------------End of change 8---------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?