| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | PRO#29 |
| Source:* | SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr Il-Yeup Ahn, KETI, iyahn@keti.re.kr |
| Date:* | 2017-05-18 |
| Reason for Change/s:* | To provide a URI formats for MQTT bindings |
| CR against: Release* | 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 |
| Clauses * | 6.3.3 simple data types |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

When a <subscription> resource is created, it is not clear what to put in notificationURI attribute for MQTT binding. This is because there’s a MQTT client talks to the other with MQTT topics, not directly with their IP addresses.

In this sense, m2m:poaList data type already defined a format for point of address in case of MQTT. E.g. mqtt://172.25.0.10:1883 Regarding this, we should have clearly say that the IP and the port in this example is of MQTT broker not a client.

Like this situation, to get a oneM2M notification over MQTT binding, the notificationURI attribute should able to represent 1) MQTT protocol, 2) MQTT broker address and 3) notification target(receiver) identifier that is used in MQTT topic. Then the subscription Hosting CSE can send a notification, usig MQTT binding, to a proper notification target.

NOTE: once this contribution becomes agreeable, then the mirrors will be prepared to get approvals for them all.

### -----------------------Start of change 1-------------------------------------------

### oneM2M simple data types

Table 6.3.3-1 describes oneM2M-specific simple data type definitions. XML Schema data type definitions for these data types can be found in the XSD file called CDT-commonTypes-v2_11_0.xsd.

The types in table 6.3.3-1 are either:

- Atomic data types derived from XML Schema data types by restrictions (other than enumeration) or union.

- List data types constructed from other XML Schema or oneM2M-defined atomic data types.

The oneM2M-defined enumeration data types are defined in clause Error: Reference source not found.

Table 6.3.3-1: oneM2M Simple Data Types

| XSD type name | Type Name | Examples | Description |
| --- | --- | --- | --- |
| m2m:resourceName | Resource name | myLightBulb 123Sensor | Used for resource name attribute. Can be formed by (ALPHA / DIGIT) *(ALPHA / DIGIT / "-" / "." / "_") as described in Error: Reference source not found. |
| m2m:ID | Generic ID | //globalm2m.org | Used to represent generic IDs generated and used within oneM2M (M2M-SP-ID) |
|  |  | //globalm2m.org/C190XX7T | (CSE-ID) |
|  |  | //globalm2m.org/CSE1/123A38ZZY | (AE-ID) |
| m2m:nodeID | Node ID | urn:gsma:imei:90420156-025763-0;svn=42 | Used for Node IDs. The constraints on this type are different from those on Generic IDs (IMEI as node ID) |
| m2m:deviceID | Device ID | urn:dev:ops:012345-Set%2DTop%2DBox-0123456789 | A Device ID uniquely identifies a device using a URN. The format of the URN is one of IETF RFC 4122([Error: Reference source not found]) UUID, OPS URN, OS URN, IMEI URN, ESN URN, or MEID URN.A |
| m2m:externalID | M2M-EXT-ID | urn:gsma:imei:90420156-025763-0;vers=0 | The External Identifier allows the Underlying Network to identify the M2M Device (e.g. ASN, MN) associated with the CSE-ID.In 3GPP case, the accessID is mapped to External Identifier as specified in TS 23.003 Error: Reference source not found. |
| m2m:requestID | Request ID | ab3f124a, CSE1/98821 | Used for Request IDs. This type may include the ID of the target CSE as well as a part that varies for each ID |
| m2m:nhURI | Non Hierarchical Identifier | /CSE090112/ C190XX7T | Used where a resourceID is required to be non-hierarchical |
| m2m:acpType | List of ACP Resource IDs | //IN-CSEID.m2m.myoperator.org/93405 | Used to represent a list of AccessControlPolicy identifiers. |
| m2m:labels | list of xs:token | printers networkwifi1 home_energy | A list of tokens used as keys for discovering resources (searching wifi connected printer from vendor 1) |
| m2m:triggerRecipientID | Trigger Recipient Identifier | 3010 | Used when device triggering services are requested from the Underlying Network, to identify an instance of an ASN/MN-CSE on an execution environment, to which the trigger is routed. Defined as port number in the range 0 to 65535. |
| m2m:listOfM2MID | List of M2M identifiers |  | xs:list of elements of data type m2m:ID |
| m2m:listOfMinMax | List of Time Limits | 10 2560 | xs:list of two xs:long values defining min and max limits of time intervals in units of milliseconds (value -1 representing infinite time) |
| m2m:ipv4 | IPv4 address string with optional CIDR suffix | 10.125.0.0/16,122.77.12.1 | Used in m2m:accessControlRules specified in the section Error: Reference source not found |
| m2m:ipv6 | IPv6 address string with optional CIDR suffix | ::/0, Fadf:ddd0::/32, abcd:ffff:abb0:aaaa::/64 | Used in m2m:accessControlRules specified in the section Error: Reference source not found |
| m2m:countryCode | Country Code | KR | 2-character country code as defined by ISO 3166-1 [40] |
| m2m:poaList | List of PointOfAccess strings | http://172.25.0.10:8080, coap://m2m.sp.com:5683, mqtt://172.25.0.10:1883 | list of xs:string. Each pointOfAccess entry in list is represented as a string containing the underlying transport protocol as well as the IP address and port (or an FQDN in all lowercase). In case of MQTT, IP and port of MQTT broker is specified. |
| m2m:timestamp | Time stamp string | 20141003T112032 | DateTime string using 'Basic Format' specified in ISO8601 [27]. Time zone shall be interpreted as UTC timezone. See below for more details. |
| m2m:absRelTimestamp | absolute or relative time stamp string | 20141003T112032 (absolute time),or 3600000 (relative time) | defined as xs:union of m2m:timestamp and xs:duration data types |
| m2m:typeOfContent | Type of Content | application/xml | The media type shall be an IANA registered Media Types name, or an experimental Media Type (See [26]) ':' |
| m2m:serializations | Serialization types | application/xml application/json application/cbor | A list of IANA registered media types that can be used for serialization of primitives. The permitted values are - application/xml - application/json - application/cbor |
| m2m:contentInfo | Content Information | application/xml:1 application/xml:1:0 application/xml:1:5 | A string consisting of a media type followed by a m2m:encodingType and optional m2m:contentSecurity, each separated by ':' character. If the m2m:contentSecurity value is not present, then the preceding ':' shall also be not present. If the m2m:contentSecurity value is not present then this has the same interpretation as a value of 0 for m2m:contentSecurity. See note. |
| m2m:eventCat | Event Category | 2 | Either - One of the values from m2m:stdEventCats or - A user-defined category in the range 100-999 |
| m2m:eventCatWithDef | Event Category with default | 0 | Either - A value from m2m:eventCat , or - The value 0 which has the special meaning "default" |
| m2m:listOfEventCat | List of (applicable) Event Categories | 1 101 | xs:list of elements of data type m2m:eventCat |
| m2m:listOfEventCatWithDef | List of m2m:eventCatWithDef | 0 1 101 |  |
| m2m:scheduleEntry | Schedule Entry | * 0-5 2,6,10 * * * * | The string is used to describe a duration of enablement. The string format is described in clause Error: Reference source not found |
| m2m:attributeList | List of xs:NCName | poa rr | Used for the Content parameter of Retrieve request primitives and in m2m:eventNotificationCriteria. Attributes represented with their short names. |
| m2m:roleID | Role-ID | 1234abcd@role-issuer.com | A string pattern consisting of a name (the issuerRelativeID) and an FQDN in all lowercase (the issuerID) separated by the ‘@' character, not including any whitespace characters. The issuerRelativeID shall be comprised of any combination of the Roman alphabet, numerals, '.', '_' and '-' characters. |
| m2m:descriptorRepresentation | Semantic content representation | application/rdf+xml:1 | A string consisting of a media type followed by a m2m:encoding separated by ':' character. The only permitted value is application/rdf+xml:1 |
| m2m:sparql | SPARQL content | SELECT ?functionality WHERE { ?functionality rdf:type base:Measuring. ?functionality base:refersTo ?aspect. ?aspect rdf:type instance:Temperature } | The string is used for SPARQL content, e.g. in semanticsFilter |
| m2m:missingDataList | List of absolute timestamp or list of relative timestamp | absolute time: 20141103T111832 , 20141103T112435 , 20141103T113633 or relative time: 10000 , 10005 , 10020 | Used for storing the time information of missing data points in Time Series. defined as xs:union of list of m2m:timestamp and list of xs:duration data types |
| m2m:tokenID | Token-ID | 1234abcd@token-issuer.com | A string pattern consisting of a name (the issuerRelativeID) and an FQDN in all lowercase (the issuerID) separated by the ‘@' character, not including any whitespace characters. See constraints above for the issuerRelativeID. |
| m2m:dynAuthJWT | JSON Web Token (JWT), which uses either JSON Web Encryption (JWE) Compact Serialization JSON Web Signature (JWS) Compact Serialization | See m2m:e2eCompactJWE and m2m:e2eCompactJWS | Defined as xs:union of m2m:e2eCompactJWE and m2m:e2eCompactJWS |
| m2m:e2eCompactJWS | JSON Web Signature (JWS) Compact Serialization, used in End-to-End Security Features TS-0003 [7] | eyJ0eXAiOiJK. eyJpc3MiOiJqb2UiLA0KIC. dBjftJeZ4CVP (line breaks for display purposes only) | Of the form [a].[b].[c], where components [a] and [c] are non-empty, while component [b] can be either empty or not empty. When not empty, each component is base64url encoded (IETF RFC 4648 [9]). See [i.7] |
| m2m:e2eCompactJWE | JSON Web Encryption (JWE) Compact Serialization, used in End-to-End Security Features TS-0003 [7] | eyJ0eXAiOiJK. eyJpc3MiOiJqb2UiLA0KIC. dBjftJeZ4CVP. 5eym8TW_c8SuK. SdiwkIr3a. XFBoMYUZo (line breaks for display purposes only) | Of the form [a].[b].[c].[d].[e], where components [a] and [d] are non-empty, while components [b],[c] and [e] can be empty or not empty. When not empty, each component is base64url encoded (IETF RFC 4648 [9]). See [i.8] |
| NOTE: The media type and m2m:encodingType in m2m:contentInfo describe the content data to which the End-to-End Security of Data (ESData) processing, if any, was applied as indicated by m2m:contentSecurity. The m2m:contentInfo indicates a sequence of processes to be applied to the content after being obtained from the CSE. First, the ESData processing (if any) as indicated by m2m:contentSecurity is applied. The result of this processing then has transfer decoding (if any) applied as indicated by m2m:encodingType. The result of this processing is then processed according to the media type. |  |  |  |

The m2m:timestamp datatype uses ISO8601 [27] Complete Representation using the Basic Format as described here:

- The timestamp shall be a string containing Year, Month, Day, Hours, Minutes and Seconds components using the format YYYYMMDDThhmmss as defined in [27]. In this representation the character "T" is to indicate the start of the time of day portion.

- All these components shall appear in the string; reduced representations are not permitted.

- The Seconds component may optionally contain a decimal fraction. In this case the string shall contain two integer digits, followed by a comma and then one or more fractional digits, up to a maximum of six. For example YYYYMMDDThhmmss,ssssss

- The timestamp string shall not contain Timezone information. All timestamps shall be interpreted as being in UTC.

A receiving or Hosting CSE shall accept a timestamp that contains fractional seconds, but it need not act on a timestamp with the level of precision that is implied by its fractional part. For example it is acceptable for a Hosting CSE to round up an expiration time when interpreting it.

NOTE 1:	Care should be taken when developing an application that compares timestamps. This is because AE's and CSE's are not required to have their clocks synchronized.

NOTE 2:	As the m2m:timestamp is expressed in UTC, an AE has to be aware of the Timezone in which it is operating if it is to be able to relate the timestamp to its local time.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### Resource Type <subscription>

#### 7.4.8.1	Introduction

The <subscription> resource contains subscription information for its subscribed-to resource. The subscription resource is a child of the subscribed to resource.

The detailed description can be found in clause 9.6.8 in TS-0001 [Error: Reference source not found].

Table 7.4.8.1-2: Data type definition of <subscription> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| subscription | CDT-subscription-v2_11_0.xsd |  |

Table 7.4.8.1-3: Universal/Common Attributes of <subscription> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| creator | O | NP |
| dynamicAuthorizationConsultationIDs | O | O |

Table 7.4.8.1-4: Resource Specific Attributes of <subscription> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |  |
| --- | --- | --- | --- | --- | --- |
|  | Create | Update |  |  |  |
| eventNotificationCriteria | O | O | m2m:eventNotificationCriteria | Default behaviour is notification on Update_of_Resource |  |
| expirationCounter | O | O | xs:positiveInteger | No default |  |
| notificationURI | M | O | list of xs:anyURI | No default In case of MQTT binding, this attribute contains MQTT broker and the receiver ID in SP-relative format. E.g. mqtt://172.25.0.10:1883/S123 represents the broker address is 172.25.0.10:1883 and receiver ID is S123. |  |
| groupID | O | O | xs:anyURI | No default |  |
| notificationForwardingURI | O | O | xs:anyURI | No default |  |
| batchNotify | O | O | m2m:batchNotify | No default |  |
| rateLimit | O | O | m2m:rateLimit | No default |  |
| preSubscriptionNotify | O | NP | xs:positiveInteger | No default |  |
| pendingNotification | O | O | m2m:pendingNotification | No default |  |
| notificationStoragePriority | O | O | xs:positiveInteger | No default |  |
| latestNotify | O | O | xs:boolean | No default |  |
| notificationContentType | O | O | m2m:notificationContentType | Default value is set to ‘all attributes' |  |
| notificationEventCat | O | O | m2m:eventCat | No default |  |
| subscriberURI | O | NP | xs:anyURI | No default |  |

Table 7.4.8.1-5: Reference of child resources

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <schedule> | notificationSchedule | 0..1 | Clause Error: Reference source not found |
| <notificationTargetMgmtPolicyRef> | [variable] | 0..n | Clause Error: Reference source not found |
| <notificationTargetSelfReference> | ntsr | 1 | Clause Error: Reference source not found |

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change n-------------------------------------------

<If the text is new text, turn on "Track Changes" and start typing or paste the new text>

<If the CR is changing existing text.

- Select the text from the latest baseline TS or TR.

- Turn  "Track Changes" off.

- Paste the existing text from the baseline.

- Turn "Track Changes" on.

- Make the proposed changes.>

### -----------------------End of change n---------------------------------------------

### -----------------------Start of Changes to References Section -------------

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M Drafting Rules (http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf))

## 2.2	Informative references

[i.1]	oneM2M Drafting Rules (http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf)

### -----------------------End of Changes to References  -------------

### -Start of changes to Definitions Symbols Abbreviations Acronyms -

# 3	Definitions, symbols, abbreviations  and acronyms

## 3.1	Definitions

<defined term>: <definition>

<defined term>[N]: <definition>

## 3.2	Symbols

<symbol>	<Explanation>

## 3.3	Abbreviations and Acronyms

<ABBREVIATION/ACRONYM>	<Explanation>

### ---End of changes to Definitions, Symbols, Abbreviations, Acronyms ---

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?