| CHANGE REQUEST |  |
| --- | --- |
| Group Name:* | REQ 14.1 |
| Source:* | Joerg Swetina, NEC, Joerg.swetina@neclab.eu |
| Format:* | Teleconference (joint REQ – ARC – SEC) |
| Date:* | 2015-01-08 |
| Contact:* | Joerg Swetina, NEC, Joerg.swetina@neclab.eu |
| Reason for Change/s:* | TS-0002 requires indication if requirements are fulfilled in the current release |
| CR against: Release* | Release 1 |
| CR against: TS/TR* | TS-0002 v0.6.3 |
| Clauses/Sub Clauses* | 4, 6.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change/correction to existing feature or functionality New feature or functionality |
| Post Freeze checking:* | This CR contains only essential changes and corrections YES NO |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR clarifies fulfilment of requirements of TS-0002 in the first oneM2M release. It also introduces changes to the text of some requirements where comments had been received during the review period or when requirements were considered unclear. Where possible, the original contributions that introduced the requirements were consulted to understand the author’s intention.

### -----------------------Start of change 1-------------------------------------------

# 4	Conventions

The key words “shall”, “shall not”, “should”, “should not”, “may”, “need not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

Note: according to oneM2M Drafting Rules [i.1] in order to mandate a feature in the M2M System but allow freedom to the individual deployment whether to use it or not subsequently requirements are often formulated like:
“The M2M System shall support a mechanism [function, capability...] to …”, or 
“…shall be able to …”. 
This does not mandate usage of the required feature in a M2M Solution.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

# 6	Functional Requirements

## 6.1	Overall System Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| OSR-001 | The M2M System shall allow communication between M2M Applications by using multiple communication means based on IP Access. | Implemented in Rel-1 |
| OSR-002a | The M2M System shall support communication means that can accommodate devices with constrained computing (e.g. small CPU, memory, battery) or communication capabilities (e.g. 2G wireless modem, certain WLAN node). | Implemented in Rel-1 |
| OSR-002b | The M2M System shall support communication means that can accommodate devices with rich computing capabilities (e.g. large CPU, memory) or communication (e.g. 3/4G wireless modem, wireline). | Implemented in Rel-1 |
| OSR-003 | If an M2M Applications requires it, the M2M System shall maintain M2M Sessions | Not implemented in Rel-1 |
| OSR-004 | M2M System shall support the ability to support session-less application communications for those M2M Applications that require it. | Implemented in Rel-1 |
| OSR-005 | The M2M System shall be able to expose the services offered by telecommunications networks to M2M Applications (e.g. SMS, USSD, localization, subscription configuration, authentication (e.g. Generic Bootstrapping Architecture), etc),subject to restriction based on Network Operator’s policy. | Partially implemented in Rel-1 (see note 1) |
| OSR-006 | The M2M System shall be able to reuse the services offered by Underlying Networks to M2M Applications and/or M2M Services by means of open access models (e.g. OMA, GSMA OneAPI framework).Examples of available services are: - IP Multimedia communications - Messaging - Location - Charging and billing services - Device information and profiles - Configuration and management of devices - Triggering, monitoring of devices - Small data transmission - Group management Note: The set of features or APIs to be supported depends on the M2M Common Services and access to available APIs. | Partially implemented in Rel-1 (see note 2) |
| OSR-007 | The M2M System shall provide a mechanism for M2M Applications to interact with the Applications and data/information managed by a different M2M Service Provider, subject to permissions as appropriate. | Implemented in Rel-1 |
| OSR-008 | The M2M System shall provide the capability for M2M Applications to communicate with an M2M Device (i.e. application in the device) without the need for the M2M Applications to be aware of the network technology and the specific communication protocol of the M2M Device. | Implemented in Rel-1 (see note 3) |
| OSR-009 | The M2M System shall support the ability for single or multiple M2M Applications to interact with a single or multiple M2M Devices/Gateways (application in the device/gateway). Note: The relation M2M Network Application to M2M Device/Gateway may be 1:1, 1:n, n:1 and/or n:m. | Implemented in Rel-1 |
| OSR-010 | The M2M System shall support mechanisms for confirmed delivery of a message to its addressee to those M2M Applications requesting reliable delivery to dectect failure of message within a given time interval. | Implemented in Rel-1 |
| OSR-011a | The M2M System shall be able to request different communication paths, from the Underlying Network based on Underlying Network Operator and/or M2M Service Provider policies, routing mechanisms for transmission failures. | Implemented in Rel-1 (see note 4) |
| OSR-011b | The M2M System shall be able to request different communication paths from the Underlying Network based on request from M2M Applications. | Not implemented in Rel-1 |
| OSR-012 | The M2M System shall support communications between M2M Applications and Devices supporting M2M Services by means of continuous or non-continuous connectivity. | Implemented in Rel-1 |
| OSR-013 | The M2M System shall be aware of the delay tolerance acceptable by the M2M Application and shall schedule the communication accordingly or request the Underlying Network to do it, based on policies criteria. | Implemented in Rel-1 |
| OSR-014 | The M2M System shall be able to communicate with M2M Devices, behind an M2M Gateway that supports heterogeneous M2M Area Networks. | Implemented in Rel-1 |
| OSR-015 | The M2M System shall support different communication patterns including infrequent communications, small data transfer, large file transfer, streamed communication. | Partially implemented in Rel-1 (see note 5) Support for large file transfer, streamed communication ?? |
| OSR-016 | The M2M System shall provide the capability to notify M2M Applications of the availability of, and changes to, available M2M Application/management information on the M2M Device/Gateway, including changes to the M2M Area Network. | Partially implemented in Rel-1 (see note 6) |
| OSR-017 | The M2M System shall be able to offer access to different sets of M2M Services to M2M Application Providers. The minimum set of services are: - Connectivity management - Device management (service level management) - Application Data management In order to enable different deployment scenarios, these services shall be made available by the M2M System, individually, as a subset or as a complete set of services. | Implemented in Rel-1 |
| OSR-018 | The M2M System shall be able to offer M2M Services to M2M Devices roaming across cellular Underlying Networks,subject to restriction based on Network Operator’s policy. Note:No roaming on M2M Service level is assumed by this requirement. | Implemented in Rel-1 with some limitations (see note 7) |
| OSR-019 | The M2M System shall support the capabilities for data repository (i.e. to collect/store) and for data transfer from one or more M2M Devices or M2M Gateways, for delivery to one or more M2M Gateways, M2M Services Infrastructure, or M2M Application Infrastructure, in ways requested by the M2M Application Infrastructure as listed below: - action initiated either by an M2M Device, M2M Gateway, M2M Services Infrastructure, or M2M Application Infrastructure - when triggered by schedule or event; - for specified data | Implemented in Rel-1 |
| OSR-020 | The M2M System shall be able to support policies and their management regarding the aspects of storage and retrieval of data/information. | Implemented in Rel-1 |
| OSR-021 | The M2M System shall be able to provide mechanisms to enable sharing of data among multiple M2M Applications. | Implemented in Rel-1 |
| OSR-022 | When some of the components of M2M System are not available (e.g. WAN connection lost), the M2M System shall be able to support the normal operation of components of the M2M System that are available. | Implemented in Rel-1 |
| OSR-023 | The M2M System shall be able to identify the M2M Services to be used by M2M Service Subscriptions. Note: M2M Service Subscriptions are not Application subscriptions (e.g., Home Energy Management). | Implemented in Rel-1 |
| OSR-024 | The M2M System shall be able to identify the M2M Devices used by M2M Service Subscriptions. | Implemented in Rel-1 |
| OSR-025 | The M2M System shall be able to identify the M2M Applications used by M2M Service Subscriptions. | Implemented in Rel-1 |
| OSR-026 | If provided by the Underlying Network, the M2M System shall be able to associate the M2M Device used by M2M Service Subscriptions with the device identifiers offered by the Underlying Network and the device. | Implemented in Rel-1 |
| OSR-027 | The M2M System shall provide a generic mechanism to support transparent exchange of information between the M2M Application and the Underlying Network, subject to restriction based on M2M Service Provider’s policy and/or Network Operator ‘s policy. Note: Transparent exchange of information implies information that is mainly interpreted by the M2M Application and the Underlying Network Provider. | Not implemented in Rel-1 |
| OSR-028 | The M2M System shall be able to trigger a series of commands to actuators on behalf of M2M Applications. | Not implemented in Rel-1 |
| OSR-029 | The M2M System shall be able to support sending common command(s) to each actuator or sensor via a group. | Implemented in Rel-1 |
| OSR-030 | The M2M System shall be able to support the management (i.e. addition, removal, retrieval and update) of the membership of a group. | Implemented in Rel-1 |
| OSR-031 | The M2M System shall be able to support a group as a member of another group. | Implemented in Rel-1 |
| OSR-032 | The M2M System shall be able to support Event Categories (e.g., normal, urgency) associated with data for M2M Applications when collecting, storing and reporting that data. Note: Based on the Event Categories and via interworking with Underlying Networks, the M2M System can support differentiated services (by providing Quality-of-Service) requested by M2M Applications. | Implemented in Rel-1 |
| OSR-033 | Based on the Dynamic Device/Gateway Context of the M2M Gateway and/or Device and the defined Event Categories, the M2M System shall provide the capability to dynamically adjust the scheduling of reporting and notification of the M2M Device/Gateway. Note: For example, if the battery of Gateway is remained only 10% or below, the Gateway notifies the M2M service platform of the status. The M2M Application in the Infrastructure node will adjust the scheduling of reporting and notification based on the Event Categories associated with each message. Consequently, the M2M Gateway operates longer. | Consider rephrasing. Request feedback from WG2. |
| OSR-034 | The M2M System shall support seamless replacement of M2M Devices as well as M2M Gateways (e.g., redirecting traffic, connection, recovery, etc.). | Not implemented in Rel-1 |
| OSR-035 | The M2M System shall support the exchange of non-M2M Application related relevant information (e.g. Device/Gateway classes) between M2M Device/Gateway and M2M Service Infrastructure for the purpose of efficient communication facilitation. This includes the capability for an M2M Device to report its device class to M2M Service Infrastructure and for the M2M Service Infrastructure to inform M2M Device of the M2M Service Infrastruture capabilities. | Not implemented in Rel-1 |
| OSR-036 | The M2M System should provide mechanisms to accept requests from M2M Application Service Providers for compute/analytics services. | Not implemented in Rel-1 |
| OSR-037 | The M2M System shall enable an M2M Application to request to send data, in a manner independent of the Underlying Network, to the M2M Applications of a group of M2M Devices and M2M Gateways in geographic areas that are specified by the M2M Application. | Not implemented in Rel-1 |
| OSR-038 | The M2M System shall support the inclusion of M2M Application’s QoS preference in service requests to Underlying Networks. | Not implemented in Rel-1 |
| OSR-039 | The M2M System shall be able to authorize service requests with QoS preference at service level, but shall pass M2M Application’s QoS preference in service requests to Underlying Network for authorization and granting or negotiation of the service QoS requests. | Not implemented in Rel-1 |
| OSR-040 | The M2M System shall be able to leverage multiple communication mechanisms (such as USSD or SMS) when available in the Underlying Networks. | Not implemented in Rel-1 (see note 8) |
| OSR-041 | The M2M System shall provide means to add new M2M Common Services without impact to M2M interface interoperability | Not implemented in Rel-1 |
| OSR-042 | The M2M System shall be able to support different QoS-levels specifying parameters, such as guaranteed bitrate, delay, delay variation, loss ratio and error rate, etc. | Not implemented in Rel-1 There is no interface to the underlying network (3GPP) to request IN initiated sessions with QoS. Check with WG2 if DA initiated sessions can can request QoS – are there specific interface parameters ? |
| OSR-043 | The M2M System shall be able to verify that members of a group support a common set of functions. | Implemented in Rel-1 Check with WG2 !!! |
| OSR-044 | The M2M System shall support communication with M2M Devices which are reachable based on defined time schedules (e.g. periodic) as well as M2M Devices which are reachable in an unpredictable and spontaneous manner. | Implemented in Rel-1 |
| OSR-045a | The M2M System shall be able to receive and utilize information provided by the Underlying Network about when an M2M Device can be reached. | Not implemented in Rel-1 |
| OSR-045b | The M2M System shall be able to utilize reachability schedules generated by either the M2M Device or the Infrastructure Domain. | Partially implemented in Rel-1 (see note 9) |
| OSR-046 | The M2M System shall be able to support a capability for the M2M Application to to request / disallow acknowledgement for its communication. | Not implemented in Rel-1 |
| OSR-047 | The M2M System shall be able to support mechanism for the M2M Devices and/or Gateways to report their geographical location information to M2M Applications. Note: Geographical location information can be more than simply longitude and latitude. | Implemented in Rel-1 |
| OSR-048 | The M2M System shall provide an M2M Service that allows M2M Devices and/or Gateways to share their own or other M2M Devices’ geographical location information. Note: Geographical location information can be more than simply longitude and latitude. | Implemented in Rel-1 |
| OSR-049 | The M2M System shall be able to provide the capability for an M2M Application to selectively share data (e.g. access control) among applications. | Implemented in Rel-1 |
| OSR-050 | If communication over one communication channel provided by the Underlying Network can only be triggered by one side (Infrastructure Domain or Field Domain), and alternative channel(s) is (are) available in the other direction, the M2M System shall be able to use the alternative channel(s) to trigger bidirectional communication on the first channel. | Implemented in Rel-1 |
| OSR-051 | Depending on availability of suitable interfaces provided by the Underlying Network the M2M System shall be able to request the Underlying Network to broadcast / multicast data to a group of M2M Devices in a specified area. | Implemented in Rel-1 |
| OSR-052 | The M2M System shall be able to select an appropriate Underlying Network to broadcast or multicast data depending on the network’s broadcast/multicast support and the connectivity supported by the targeted group of M2M Devices/Gateways. | Not implemented in Rel-1 |
| OSR-053 | The M2M System shall provide a means that enables backward compatibility of interfaces among different releases. Note: “means” above does not imply only technical mechanisms. | Not implemented in Rel-1 |
| OSR-054 | The M2M System shall be able to support an M2M Application, M2M Device, or M2M Gateway to obtain access to resources of another M2M Application, M2M Device, or M2M Gateway. | Implemented in Rel-1 |
| OSR-055 | The M2M System shall be able to provide the capability of M2M Applications to exchange data with one or more authorized M2M Applications which are not known. | ??? Question for WG2: can we currently find out all applications a certain application is authorized to communicate with? How can we rephrase “not known”? |
| OSR-056 | The M2M System shall enable discovery of usable M2M Applications on an M2M Gateway or at an M2M Device . | Implemented in Rel-1 |
| OSR-057 | The M2M System shall enable discovery of M2M Gateways and M2M Devices available to an M2M Application for data exchange. | Implemented in Rel-1 |
| OSR-058 | The M2M System shall be able to provide time stamps as needed by common service functions. | Implemented in Rel-1 |
| OSR-059 | The M2M System shall be able to support Role-based access control based on M2M Service Subscriptions. | Implemented in Rel-1 |
| OSR-060 | The M2M System should support time synchronization with an external clock source. | Not implemented in Rel-1 |
| OSR-061 | M2M Devices and M2M Gateways may support time synchronization within the M2M System. | Not implemented in Rel-1 |
| OSR-062 | The M2M System shall enable means of testing the connectivity towards a set of M2M Applications. | Not implemented in Rel-1 |
| OSR-063 | The M2M System shall be able to manage the scheduling of M2M Service Layer connectivity and messaging between the Infrastracture Domain and M2M Devices/Gateways. | Implemented in Rel-1 |
| OSR-064 | The M2M System shall be able to aggregate messages depending on message delay tolerance and/or category. | Implemented in Rel-1 |
| OSR-065 | The M2M System shall provide mechanisms that enable a M2M Service Provider to distribute processing functions to his M2M Devices/Gateways in the Field Domain | Not implemented in Rel-1 |
| OSR-066 | The M2M System shall be able to support the placement and operation of M2M Applications in selected M2M nodes per criteria requested by M2M Application Service Providers, subject to access rights. | Implemented in Rel-1 |
| OSR-067 | The M2M System shall be able to take operational and management action as requested by M2M Applications. | Implemented in Rel-1 |
| OSR-068 | When available from an Underlying Network, the M2M System shall be able to provide the capability to retrieve and report the information regarding whether an M2M Device is authorized to access Underlying Network services. | Not implemented in Rel-1 |
| OSR-069 | When available from the Underlying Network, the M2M System shall be able to maintain the M2M Service Operational Status of a M2M Device and update it when the Underlying Network connectivity service status changes. | Not implemented in Rel-1 |
| OSR-070 | The M2M System shall be able to provide the capability to notify an authorized M2M Application when the M2M Service Administrative State or M2M Service Operational Status of an M2M Device changes, if that M2M Application has subscribed for such notifications. | Partially implemented in Rel-1 (see note 10) |
| OSR-071 | The M2M System shall be able to enable an authorized M2M Application to set the M2M Service Administrative State of a M2M Device. | Implemented in Rel-1 |
| OSR-072 | The M2M System shall be able to initiate a set of well-defined actions (e.g. trigger upon a threshold, compare a value …) to one or more M2M Application(s) on behalf of another M2M Application. | Implemented in Rel-1 |
| Note 1: In Rel-1 only GBA and localization are available Note 2: Rel-1 covers: Location, Charging and billing services, Configuration and management of devices, Triggering Note 3: This requirement applies to M2M Devices but not to devices interworked via M2M Area Networks Note 4: Based on device triggering. Note5: No Support for communication patterns including infrequent communications, small data transfer as currently Underlying Networks don’t support it Note 6: Functionality supported through Device Management Note 7: Limitations to trigger (via Tsp interface) devices in a roamed-to network. Note 8: it is possible to deliver CoAP over SMS, but currently SMS message delivery interfaces are not expicitely defined. Note 9: Only reachability schedules generated by the Infrastructure Domain can be utilized. Note 10: Only the M2M Service Administrative State can be notified. M2M Service Operational Status is not implemented. |  |  |

Table 1 Overall System Requirements

### -----------------------End of change 2---------------------------------------------