| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | REQ 21 |
| Source:* | Sungchan Choi, KETI, csc@keti.re.kr |
| Date:* | 2016-01-11 |
| Contact:* | Sungchan Choi, Minu Ryi, Jaeho Kim, Jaeseok Yun, KETI |
| Reason for Change/s:* | This contribution intend to provide clause 5, 7, 8 for TR0013. |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <WI-0017 > MNT maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR0013 |
| Clauses/Sub Clauses* | Clause 5, 7, 8 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

### -----------------------Start of change 1-------------------------------------------

# 5		Introduction to Home Domain Enablement

## 5.1			Home Domain Overview

Home domain can be described as one of oneM2M application service domains which is capable of applying oneM2M common functionality such as registration, discovery, group, device/software mangement, subscription/notification etc. within home environment to provide convenience, comfort, security, automation and energy efficiency to its resident.

Home appliances may comprise various devices e.g. lights, refrigerator, air conditioning, home audio, television, waching machine etc. which act as providing home resident with more convenient life. In home domain environment, the home appliances are connected to gateway or server via wireless or wireline communication technology, which is then accessed and controlled by the smart phone or smart control devices in remote domain or local doamin.

Figure 5.1-1 Home Doamin Usecase Model

Figure 5,1-1 shows device and network configuration for general home domain usecases. In home domain, there might be home gateway which provides wireless (WiFi, Bluetooth, Zigbee) or wireline (Ethernet) connectivity with various home appliances. Home appliance resister its information with home gateway or home infra server and report its status or data to the home gateway or infra server. In remote domain, home controller e.g. smart phone, smart tablet enable controlling home appliances in the home doain via infra server and home gateway.

- 5.2			Overview of oneM2M Release 1 Enablement

The following table illustrates oneM2M common service functionality (CSF) and its relation to the applicable features to the home domain enablement.

| oneM2M CSF | Applicable Home Domain Features |
| --- | --- |
| Registration | In order to receive M2M services, home appliances need to register its information with home gateway or home infra server. Based on this information, home appliances can be identified and managed in oneM2M domain. |
| Discovery | Home appliances registered in oneM2M domain can be discovered via searching information about application and services which are contained oneM2M attributes and resources. |
| Data Management and Repository | Home appliances sends its data to the home gateway or remote server. The stored data enables management of data and provides information to the service appliacation regarding home appliance’s status or data information. |
| Device Management | Home appliance’s specific function can be managed by oneM2M DM(device management) CSF which may include monitoring, diagnostics and troubleshooting, etc. |
| Group Management | Home appliances and devices can be managed via group-based e.g. category1, category2, etc. Leveraging oneM2M Group CSF enables bulk operations to the group members about home appliances. |
| Subscription and Notification | Home monitoring application can subscribe to an event regarding home appliances (e.g. lights on/off) and receive notification when matching the event, which enables home automation services. |
| Security | oneM2M provides security functionalites including authentication, authorization, encrytion, secure association establishment, etc. Home domain can utilize theses features for security mechanism among home appliances and home services. |

- -----------------------End of change 1--------------------------------------------

### -----------------------Start of change 2------------------------------------------

# 7 	Overview of Potential Requirements

Potential Requimrents derived from Home Domain Usecases are summarized as follows:

- The oneM2M System shall support a semantic relation (“Is Paired To”) between two M2M Devices.

- The oneM2M System shall be able to store/update information about pairing between two M2M Devices as a relation (“Is Paired To”) between M2M Devices.

- The oneM2M System shall enable the creation and deletion of a semantic pairing relation (“Is Paired To”) between M2M Devices by M2M applications, subject to access rights.

- The oneM2M System shall be able to make information about pairing between M2M Devices available to M2M Applications.

- The M2M system shall provide mechanisms to communicate with M2M gateways for the purpose of triggering devices behind the gateway.

- The M2M system shall support the M2M device to register its communication capabilities to the M2M gateway for the purpose of device triggering.

- If the M2M device supports multiple communication channels, one of the communication channels could be used by the M2M system to trigger the setup of another communication channel in the field domain.

- The oneM2M System shall provide the capability to notify M2M devices of the availability of alternative registration points (e.g., via different underlying networks) through which the M2M device can register.

- The oneM2M System shall enable a M2M device to de-register at an old registration point and re-register at a new registration point while maintaining device state and data information.

- The oneM2M system shall support capabilities for event management, e.g. capability for arbitration of the resulting processing.

- The oneM2M System shall define a structure of an Information Model for the purpose of exchanging data. (ABR-0001)

- The oneM2M system shall support resource caching of registered M2M Devices. Resource caching is a mechanism through which the oneM2M system retains resources of a registered M2M Device in temporarily inactive state by moving the resources to a temporary storage e.g. cache bin.

- When a M2M Device registers to the oneM2M system, the M2M Device shall be able to provide information on its intention to cache resources.

- When a M2M Device becomes active again, the oneM2M system shall be able to restore all previously cached resources.

8			Conclusions

Home domain enablement is one of the most compelling service domains being able to pervade M2M services to our lives. Regarding home environement, manay standard and M2M services (HGI, OIC, Energy@Hom, BBF, WIFI, Zigbee Home Profile etc.) has been developed and utilized from network connectivity to platform technology to applications. In this document, various home domain usecases are studied and it is inspected how oneM2M system can be adapted to accommodate home domain useses.

In home domain environment, home gateway acts as a crucial role for setting up connection with home appliances but also managing home appliances. Additionally, home intelligence service utilizes home appliance’s data and manage/control home appliance automatically. To this end, the method for organizing home device’s resources can be considered as importat feature and systematically hamornizing resources are required. In order to provide interoperability between home appliances manufactured from various companies, home domain infomraiton model needs to be defined in the oneM2M scope.

- -----------------------End of change 2--------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?