| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | REQ#21 |
| Source:* | SK Telecom (TTA) |
| Date:* | 2015-01-20 |
| Contact:* | Hongbeom Ahn, SK Telecom, hb.ahn@sk.com |
| Reason for Change/s:* | Editorial modification to clearly state what ‘location information’ means. |
| CR against: Release* | 2 |
| CR against: WI* | Active <Work Item number> MNT maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0002 V2.4.0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Based on the discussion regarding a new use case, called “vehicle management based on Geo-fence”.

REQ members agreed not to put a new requirements what is suggested in REQ-2016-0003 but modify the existing note (NOTE 7 in the table 1) since the existing requirements(OSR-047, 048) can cover the use case’s potential requirements.

### -----------------------Start of change 1-------------------------------------------

Table 1: Overall System Requirements

| Requirement ID | Description | Release |
| --- | --- | --- |
| OSR-001 | The oneM2M System shall allow communication between M2M Applications by using multiple communication means based on IP access. | Implemented in Rel-1 |
| OSR-002a | The oneM2M System shall support communication means that can accommodate devices with constrained computing (e.g. small CPU, memory, battery) or communication capabilities (e.g. 2G wireless modem, certain WLAN node). | Implemented in Rel-1 |
| OSR-002b | The oneM2M System shall support communication means that can accommodate devices with rich computing capabilities (e.g. large CPU, memory) or communication (e.g. 3/4G wireless modem, wireline). | Implemented in Rel-1 |
| OSR-003 | The oneM2M System shall support the ability to maintain application-to-application communication in coordination with an application session for those M2M Applications that require it. | Targeted for Release 2 |
| OSR-004 | The oneM2M System shall support session-less application communications for those M2M Applications that require it. | Implemented in Rel-1 |
| OSR-005 | The oneM2M System shall be able to expose the services offered by telecommunications networks to M2M Applications (e.g. SMS, USSD, localization, subscription configuration, authentication (e.g. Generic Bootstrapping Architecture), etc.),subject to restriction based on Network Operator’s policy. | Partially implemented in Rel-1 (see note 9) |
| OSR-006 | The oneM2M System shall be able to reuse the services offered by Underlying Networks to M2M Applications and/or M2M Services by means of open access models (e.g. OMA, GSMA OneAPI framework).Examples of available services are: - IP Multimedia communications. - Messaging. - Location. - Charging and billing services. - Device information and profiles. - Configuration and management of devices. - Triggering, monitoring of devices. - Small data transmission. - Group management. (see note 1). | Partially implemented in Rel-1 (see note 10) |
| OSR-007 | The oneM2M System shall provide a mechanism for M2M Applications to interact with the Applications and data/information managed by a different M2M Service Provider, subject to permissions as appropriate. | Implemented in Rel-1 |
| OSR-008 | The oneM2M System shall provide the capability for M2M Applications to communicate with an M2M Device (i.e. application in the device) without the need for the M2M Applications to be aware of the network technology and the specific communication protocol of the M2M Device. | Implemented in Rel-1 (see note 11) |
| OSR-009 | The oneM2M System shall support the ability for single or multiple M2M Applications to interact with a single or multiple M2M Devices/Gateways (application in the device/gateway) (see note 2). | Implemented in Rel-1 |
| OSR-010 | The oneM2M System shall support mechanisms for confirmed delivery of a message to its addressee to those M2M Applications requesting reliable delivery to dectect failure of message within a given time interval. | Implemented in Rel-1 |
| OSR-011a | The oneM2M System shall be able to request different communication paths, from the Underlying Network based on Underlying Network Operator and/or M2M Service Provider policies, routing mechanisms for transmission failures. | Implemented in Rel-1 (see note 12) |
| OSR-011b | The oneM2M System shall be able to request different communication paths from the Underlying Network based on request from M2M Applications. | Not implemented in Rel-1 |
| OSR-012 | The oneM2M System shall support communications between M2M Applications and M2M Devices supporting M2M Services by means of continuous or non-continuous connectivity. | Implemented in Rel-1 |
| OSR-013 | The oneM2M System shall be aware of the delay tolerance acceptable by the M2M Application and shall schedule the communication accordingly or request the Underlying Network to do it, based on policies criteria. | Implemented in Rel-1 |
| OSR-014 | The oneM2M System shall be able to communicate with M2M Devices, behind an M2M Gateway that supports heterogeneous M2M Area Networks. | Implemented in Rel-1 |
| OSR-015 | The oneM2M System shall be able to assist Underlying Networks that support different communication patterns including infrequent communications, small data transfer, transfer of large file and streamed communication. | Partially implemented in Rel-1 (see note 13) |
| OSR-016 | The oneM2M System shall provide the capability to notify M2M Applications of the availability of, and changes to, available M2M Application/management information on the M2M Device/Gateway, including changes to the M2M Area Network. | Implemented in Rel-1 |
| OSR-017 | The oneM2M System shall be able to offer access to different sets of M2M Services to M2M Application Providers. The minimum set of services are: - Connectivity management. - Device management (service level management). - Application Data management. In order to enable different deployment scenarios, these services shall be made available by the oneM2M System, individually, as a subset or as a complete set of services. | Implemented in Rel-1 |
| OSR-018 | The oneM2M System shall be able to offer M2M Services to M2M Devices roaming across cellular Underlying Networks,subject to restriction based on Network Operator’s policy (see note 3). | Implemented in Rel-1 with some limitations (see note 14) |
| OSR-019 | The oneM2M System shall support the capabilities for data repository (i.e. to collect/store) and for data transfer from one or more M2M Devices or M2M Gateways, for delivery to one or more M2M Gateways, M2M Services Infrastructure, or M2M Application Infrastructure, in ways requested by the M2M Application Infrastructure as listed below: - action initiated either by an M2M Device, M2M Gateway, M2M Services Infrastructure, or M2M Application Infrastructure; - when triggered by schedule or event; - for specified data. | Implemented in Rel-1 |
| OSR-020 | The oneM2M System shall be able to support policies and their management regarding the aspects of storage and retrieval of data/information. | Implemented in Rel-1 |
| OSR-021 | The oneM2M System shall be able to provide mechanisms to enable sharing of data among multiple M2M Applications. | Implemented in Rel-1 |
| OSR-022 | When some of the components of a M2M Solution are not available (e.g. WAN connection lost), the oneM2M System shall be able to support the normal operation of components of the M2M Solution that are available. | Implemented in Rel-1 |
| OSR-023 | The oneM2M System shall be able to identify the M2M Services to be used by M2M Service Subscriptions (see note 4). | Implemented in Rel-1 |
| OSR-024 | The oneM2M System shall be able to identify the M2M Devices used by M2M Service Subscriptions. | Implemented in Rel-1 |
| OSR-025 | The oneM2M System shall be able to identify the M2M Applications used by M2M Service Subscriptions. | Implemented in Rel-1 |
| OSR-026 | If provided by the Underlying Network, the oneM2M System shall be able to associate the M2M Device used by M2M Service Subscriptions with the device identifiers offered by the Underlying Network and the device. | Implemented in Rel-1 |
| OSR-027 | The oneM2M System shall provide a generic mechanism to support transparent exchange of information between the M2M Application and the Underlying Network, subject to restriction based on M2M Service Provider’s policy and/or Network Operator’s policy (see note 5). | Not implemented in Rel-1 |
| OSR-028 | The oneM2M System shall enable an M2M Application to define trigger conditions in the oneM2M System such that the oneM2M System autonomously sends a series of commands to actuators on behalf of the M2M Application when these contitions are met. | Not implemented in Rel-1 |
| OSR-029 | The oneM2M System shall be able to support sending common command(s) to each actuator or sensor via a group. | Implemented in Rel-1 |
| OSR-030 | The oneM2M System shall be able to support the management (i.e. addition, removal, retrieval and update) of the membership of a group. | Implemented in Rel-1 |
| OSR-031 | The oneM2M System shall be able to support a group as a member of another group. | Implemented in Rel-1 |
| OSR-032 | The oneM2M System shall be able to support Event Categories (e.g. normal, urgency) associated with data for M2M Applications when collecting, storing and reporting that data (see note 6). | Implemented in Rel-1 |
| OSR-033 | Based on the Dynamic Device/Gateway Context of the M2M Gateway and/or Device and the defined Event Categories, the oneM2M System shall provide the capability to dynamically adjust the scheduling of reporting and notification of the M2M Device/Gateway (see note 17).. | Partially implemented in Rel-1 (see note 15) |
| OSR-034 | The oneM2M System shall support seamless replacement of M2M Devices as well as M2M Gateways (e.g. redirecting traffic, connection, recovery, etc.). | Not implemented in Rel-1 |
| OSR-035 | The oneM2M System shall support the exchange of non-M2M Application related relevant information (e.g. Device/Gateway classes) between M2M Device/Gateway and M2M Service Infrastructure for the purpose of efficient communication facilitation. This includes the capability for an M2M Device to report its device class to M2M Service Infrastructure and for the M2M Service Infrastructure to inform M2M Device of the M2M Service Infrastruture capabilities. | Not implemented in Rel-1 |
| OSR-036 | The oneM2M System should provide mechanisms to accept requests from M2M Application Service Providers for compute/analytics services. | Not implemented in Rel-1 |
| OSR-037 | The oneM2M System shall enable an M2M Application to request to send data, in a manner independent of the Underlying Network, to the M2M Applications of a group of M2M Devices and M2M Gateways in geographic areas that are specified by the M2M Application. | Not implemented in Rel-1 |
| OSR-038 | The oneM2M System shall support the inclusion of M2M Application’s QoS preference in service requests to Underlying Networks. | Not implemented in Rel-1 |
| OSR-039 | The oneM2M System shall be able to authorize service requests with QoS preference at service level, but shall pass M2M Application’s QoS preference in service requests to Underlying Network for authorization and granting or negotiation of the service QoS requests. | Not implemented in Rel-1 |
| OSR-040 | The oneM2M System shall be able to leverage multiple communication mechanisms (such as USSD or SMS) when available in the Underlying Networks. | Not implemented in Rel-1 (see note 16) |
| OSR-041 | The oneM2M System shall provide a mechanism, which supports the addition of new M2M Services to the oneM2M System as independent portable modules by means of the oneM2M interfaces. | Partially implemented (see note 22) |
| OSR-042 | The oneM2M System shall be able to support different QoS-levels specifying parameters, such as guaranteed bitrate, delay, delay variation, loss ratio and error rate, etc. | Not implemented in Rel-1 |
| OSR-043 | The oneM2M System shall be able to verify that members of a group support a common set of functions. | Implemented in Rel-1 |
| OSR-044 | The oneM2M System shall support communication with M2M Devices which are reachable based on defined time schedules (e.g. periodic) as well as M2M Devices which are reachable in an unpredictable and spontaneous manner. | Implemented in Rel-1 |
| OSR-045a | The oneM2M System shall be able to receive and utilize information provided by the Underlying Network about when an M2M Device can be reached. | Not implemented in Rel-1 |
| OSR-045b | The oneM2M System shall be able to utilize reachability schedules generated by either the M2M Device or the Infrastructure Domain. | Partially implemented in Rel-1 (see note 18) |
| OSR-046 | The oneM2M System shall be able to support a capability for the M2M Application to request / disallow acknowledgement for its communication. | Not implemented in Rel-1 |
| OSR-047 | The oneM2M System shall be able to support mechanism for the M2M Devices and/or Gateways to report their geographical location information to M2M Applications (see note 7). | Implemented in Rel-1 |
| OSR-048 | The oneM2M System shall provide an M2M Service that allows M2M Devices and/or Gateways to share their own or other M2M Devices’ geographical location information (see note 7). | Implemented in Rel-1 |
| OSR-049 | The oneM2M System shall be able to provide the capability for an M2M Application to selectively share data (e.g. access control) among applications. | Implemented in Rel-1 |
| OSR-050 | If communication over one communication channel provided by the Underlying Network can only be triggered by one side (Infrastructure Domain or Field Domain), and alternative channel(s) is (are) available in the other direction, the oneM2M System shall be able to use the alternative channel(s) to trigger bidirectional communication on the first channel. | Implemented in Rel-1 |
| OSR-051 | Depending on availability of suitable interfaces provided by the Underlying Network the oneM2M System shall be able to request the Underlying Network to broadcast / multicast data to a group of M2M Devices in a specified area. | Implemented in Rel-1 |
| OSR-052 | The oneM2M System shall be able to select an appropriate Underlying Network to broadcast or multicast data depending on the network’s broadcast/multicast support and the connectivity supported by the targeted group of M2M Devices/Gateways. | Not implemented in Rel-1 |
| OSR-053 | The oneM2M System shall provide a means that enables backward compatibility of interfaces among different releases (see note 8). | Not implemented in Rel-1 |
| OSR-054 | The oneM2M System shall be able to support an M2M Application, M2M Device, or M2M Gateway to obtain access to resources of another M2M Application, M2M Device, or M2M Gateway. | Implemented in Rel-1 |
| OSR-055 | The oneM2M System shall be able to provide the capability of M2M Applications to exchange data with one or more authorized M2M Applications which are not known in advance. | Implemented in Rel-1 (see note 20) |
| OSR-056 | The oneM2M System shall enable discovery of usable M2M Applications on an M2M Gateway or at an M2M Device . | Implemented in Rel-1 |
| OSR-057 | The oneM2M System shall enable discovery of M2M Gateways and M2M Devices available to an M2M Application for data exchange. | Implemented in Rel-1 |
| OSR-058 | The oneM2M System shall be able to provide time stamps as needed by Common Service Functions. | Implemented in Rel-1 |
| OSR-059 | The oneM2M System shall be able to support Role-Based Access Control based on M2M Service Subscriptions. | Implemented in Rel-1 |
| OSR-060 | The oneM2M System should support time synchronization with an external clock source. | Not implemented in Rel-1 |
| OSR-061 | M2M Devices and M2M Gateways may support time synchronization within the oneM2M System. | Not implemented in Rel-1 |
| OSR-062 | The oneM2M System shall enable means of testing the connectivity towards a set of M2M Applications. | Not implemented in Rel-1 |
| OSR-063 | The oneM2M System shall be able to manage the scheduling of M2M Service Layer connectivity and messaging between the Infrastracture Domain and M2M Devices/Gateways. | Implemented in Rel-1 |
| OSR-064 | The oneM2M System shall be able to aggregate messages depending on message delay tolerance and/or category. | Implemented in Rel-1 |
| OSR-065 | The oneM2M System shall provide mechanisms that enable a M2M Service Provider to distribute processing functions to his M2M Devices/Gateways in the Field Domain | Not implemented in Rel-1 |
| OSR-066 | The oneM2M System shall be able to support the placement and operation of M2M Applications in selected M2M Nodes per criteria requested by M2M Application Service Providers, subject to access rights. | Implemented in Rel-1 |
| OSR-067 | The oneM2M System shall be able to take operational and management action as requested by M2M Applications. | Implemented in Rel-1 |
| OSR-068 | When available from an Underlying Network, the oneM2M System shall be able to provide the capability to retrieve and report the information regarding whether an M2M Device is authorized to access Underlying Network services. | Not implemented in Rel-1 |
| OSR-069 | When available from the Underlying Network, the oneM2M System shall be able to maintain the M2M Service Operational Status of a M2M Device and update it when the Underlying Network connectivity service status changes. | Not implemented in Rel-1 |
| OSR-070 | The oneM2M System shall be able to provide the capability to notify an authorized M2M Application when the M2M Service Administrative State or M2M Service Operational Status of an M2M Device changes, if that M2M Application has subscribed for such notifications. | Partially implemented in Rel-1 (see note 19) |
| OSR-071 | The oneM2M System shall be able to enable an authorized M2M Application to set the M2M Service Administrative State of a M2M Device. | Implemented in Rel-1 |
| OSR-072 | The oneM2M System shall be able to initiate a set of well-defined actions (e.g. trigger upon a threshold, compare a value, etc.) to one or more M2M Application(s) on behalf of another M2M Application. | Implemented in Rel-1 |
| OSR-073 | The oneM2M System shall support distributed transactions to multiple devices or applications where the transaction includes the characteristics of atomicity, consistency, isolation and durability. | Targeted for Release 2 |
| OSR-074 | The oneM2M System shall support the completion of distributed transactions to multiple devices or applications while maintaining the order of the operations and performing the transaction within a given time frame. | Targeted for Release 2 |
| OSR-75 | The oneM2M System shall be able to collect, store Time Series Data. | Targeted for Release 2 |
| OSR-76 | The oneM2M System shall be able to detect and report the missing data in time series. | Targeted for Release 2 |
| OSR-077 | The oneM2M System shall be capable of collecting asynchronous responses pertaining to the broadcasted messages. | Targeted for Release 2 |
| OSR-078 | The oneM2M System shall support gateway-based capabilities for Event management, e.g. capability for arbitration of the resulting processing. | Targeted for Release 2 |
| OSR-079 | The oneM2M System shall provide the capability to notify a device hosting a group of applications when alternative registration points for that group of applications are available (e.g., via different underlying networks) based on the service requirements of each of the applications hosted. | Targeted for Release 2 |
| OSR-080 | The oneM2M System shall provide the capability to register applications in group or independently, based on their service requirements. | Targeted for Release 2 |
| OSR-081 | The oneM2M System shall be able to collect data that is broadcast (e.g. in industrial bus systems) according to data collection policies. | Targeted for Release 2 |
| OSR-082 | The oneM2M System shall allow the update, modification, or deletion of data collection policies within an M2M Application. | Targeted for Release 2 |
| OSR-083 | The oneM2M System shall be able to filter information from oneM2M Devices for a given set of parameters . | Targeted for Release 2 |
| OSR-084 | The oneM2M System shall be able to handle an event notification from an authorized M2M Application which triggers actions to be performed on the M2M Device (ex. Turn on or off the monitoring). | Targeted for Release 2 |
| OSR-085 | The oneM2M System shall support resource caching of registered M2M Devices. Resource caching is a mechanism through which the oneM2M System retains resources of a registered M2M Device in temporarily inactive state by moving the resources to a temporary storage e.g. cache bin. | Targeted for Release 2 |
| OSR-086 | The oneM2M System shall enable M2M Gateways to discover M2M Infrastructure Nodes and M2M Devices available for data exchange. | Targeted for Release 2 |
| OSR-087 | The oneM2M System shall enable M2M Infrastructure Nodes and M2M Device to discover M2M Gateways available for data exchange. | Targeted for Release 2 |
| OSR-088 | The oneM2M System shall be able to support the capabilities for data repository (i.e. to collect/store) and for data transfer among authorized M2M Devices and M2M Gateways via M2M Area Networks without involvement of the Infrastructure Domain. | Targeted for Release 2 |
| OSR-089 | The oneM2M System shall enable the cancellation of continuous data collection and/or the deletion of collected data when pre-defined conditions are met. | Targeted for Release 2 |
| OSR-090 | The oneM2M System shall be able to forward the M2M Application Data to M2M Application without storing the Data. | Targeted for Release 2 |
| OSR-091 | The oneM2M System shall be able to notify interested oneM2M entities when it detects forwarded M2M Application Data was not delivered within expected time duration. | Targeted for Release 2 |
| OSR-092 | The oneM2M System shall provide the capability for monitoring and describing data streams with associated attributes e.g. data freshness, accuracy, sampling rate, data integrity. | Targeted for Release 2 |
| OSR-093 | The oneM2M System shall support transaction management to multiple devices or applications providing policy based mechanism that should be invoked (e.g. keep status, re-schedule, rollback) depending on the outcome of the desired operation. | Targeted for Release 2 |
| OSR-094 | The oneM2M System shall provide Information Model(s) to support interoperability among different devices/applications. | Targeted for Release 2 |
| OSR-095 | The oneM2M System should provide mappings between different Information Models from non-oneM2M System(s). | Targeted for Release 2 |
| OSR-096 | The oneM2M System should be able to interwork with non-oneM2M System(s). | Targeted for Release 2 |
| OSR-097 | The oneM2M System shall be able to share data collection policies among multiple M2M Devices/Gateways within an M2M Application Service, or among different M2M Application Services. | Targeted for Release 2 |
| NOTE 1: The set of features or APIs to be supported depends on the M2M Common Services and access to available APIs. NOTE 2: The relation M2M Network Application to M2M Device/Gateway may be 1:1, 1:n, n:1 and/or n:m. NOTE 3: No roaming on M2M Service level is assumed by this requirement. NOTE 4: M2M Service Subscriptions are not Application subscriptions (e.g. Home Energy Management). NOTE 5: Transparent exchange of information implies information that is mainly interpreted by the M2M Application and the Underlying Network Provider. NOTE 6: Based on the Event Categories and via interworking with Underlying Networks, the oneM2M System can support differentiated services (by providing Quality-of-Service) requested by M2M Applications. NOTE 7: Geographical location information can be more than simply longitude, latitude and Geo-fence event. NOTE 8: “means” above does not imply only technical mechanisms. NOTE 9: In Rel-1 only GBA and localization are available NOTE 10: Rel-1 covers: Location, Charging and billing services, Configuration and management of devices, Device information and profiles, Triggering NOTE 11: This requirement applies to M2M Devices but not to devices interworked via M2M Area Networks NOTE 12: Based on device triggering. NOTE 13: No Support for streamed communication NOTE 14: Limitations to trigger (via Tsp interface) devices in a roamed-to network. NOTE 15: Detail syntax to describe Dynamic Context is not specified NOTE 16: it is possible to deliver CoAP over SMS, but currently SMS message delivery interfaces are not expicitely defined. NOTE 17: For example, if the battery of Gateway is remained only 10% or below, the Gateway notifies the M2M service platform of the status. The M2M Application in the Infrastructure node will adjust the scheduling of reporting and notification based on the Event Categories associated with each message. Consequently, the M2M Gateway operates longer. NOTE 18: Only reachability schedules generated by the Infrastructure Domain can be utilized. NOTE 19: Only the M2M Service Administrative State can be notified. M2M Service Operational Status is not implemented. NOTE 20: this can be implemented based on preconfigured access rights. NOTE 22: In Rel-1 this is supported by means of the Mca interfaces, mapping the new service module to an AE. |  |  |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?