| Input Contribution |  |
| --- | --- |
| Meeting ID* | REQ 25 |
| Title:* | TR-0026-Vehicular_Domain_Enablement_Clause_9_1_Location |
| Source:* | Hiroyuki Kubo, Hitachi, hiroyuki.kubo.do@hitachi.com Jiaxin Yin, Huawei, yinjiaxin@huawei.com Dongjoo Kim, LG Electronics, dongjoo7.kim@lge.com Nick Yamasaki, KDDI CORPORATION, nr-yamasaki@kddi.com Kenichi Yamamoto, KDDI CORPORATION, kc-yamamoto@kddi.com Ryokichi Onishi, TOYOTA ITC, onishi@jp.toyota-itc.com |
| Uploaded Date:* | 2016-10-10 |
| Document(s) Impacted* | WI 0046 oneM2M-TR-0026-Vehicular-Domain-Enablement-V0-4-0.DOC |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | Agree for inclusion in TR-0026. |
| Template Version:23 February 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

This document is the description of Clause 8 “High Level Architecture” in TR-0026

# 9	oneM2M Release 2 Enablement for Vehicular Domain

The discussions in this clause provide the guideline to utilize oneM2M Release 2 for Vehicular Domain.

- 9.1	Location

Geographical location information is important in vehicular services, because a node in vehicular domain moves dynamically. As shown in use cases in clause 6, location is the key factor of vehicular services. For instance, the procedures of vehicular services could be triggered by the change of location. In other cases, the vehicular service would provide to the vehicles locating in the dedicated area.

In oneM2M Release 2, Location CSF (LOC CSF) gives the following location functions.

- A location server in the Underlying Network

- A GPS module in an M2M device

- Information for inferring location stored in other Nodes

In the vehicular domain, each vehicle would generally equip a device to obtain location information. Through the location device, CSE in a vehicle periodically updates its location and forward it to the oneM2M platform. The accuracies of location and the applicable use cases are depending on the capability of type of the location device. Table 9-1-1 summarizes the required accuracy of location, and the methods to obtain the required accuracies.

Table 9.1-1: Required accuracy of location

| Title | AREA | ROAD | LANE |
| --- | --- | --- | --- |
| Accuracy (margin of error) | Less than dozens of meters | Less than a few meters | Less than dozens of centimeters |
| Method | An Underlying Network (e.g. Cell-ID from 3GPP) | GPS Assisted GPS Underlying Network (e. g. Cell-ID from RSUs) | RTK-GPS Recent GNSS/RNSS |
| Use cases | 6.8, 6.10 | 6.4, 6.12, 6.13 | TBD |

Figure 9.1-2: AREA

To deliver a content to vehicles locating in a dedicated area (a few hundred meters), the “AREA level” accuracy would be required. Figure 9.1-2 illustrates the concept of “AREA” use case. In this case, an acceptable error range should be less than dozens of meters. For instance, an underlying network can provide location information with the required accuracy based on Cell-ID.

Geo-Fence is also applicalbe for the AREA level accuracy. In oneM2M Release 2, Geo-Fence is utilizable as the resource type <locationPolicy>. For instance, in use case 6.8, when a traffic accident occurs, oneM2M platform sets Geo-Fence arround the accident point. Then the oneM2M platform can provide the content to the vehicles in the Geo-Fence.

Figure 9.1-3: ROAD

To deliver content to vehicles locating in a dedicated road, the “ROAD level” accuracy would be required. In this case, navigation systems or services can be utilized and its acceptable error range should be less than a few meters. Some use cases provide route navigation for vehicles based on their location. Figure 9.1-3 illustrates the reason why “ROAD” accuracy is required. As shown in the figure, the service cannot provide accurate route navigation when the location contains larger error. To obtain the location information with the required accuracy level, GPS (Global Positioning System) or Assisted GPS are applicable. Furthermore, when the underlying network consists of RSUs, if the RSUs are located every a few meters to dozens of meters, the oneM2M platform could obtain ROAD level accuracy.

To deliver content to vehicles locating in a dedicated lane, the “LANE level” accuracy would be required. At this time, no use case needs the accuracy level, but the future use cases will require the accuracy. In this case, an acceptable error range should less than dozens of centimeters. To obtain the location information with the required accuracy level, a device in a vehicle and an infrastructure should support the current posissioning systems such as RTK-GPS (Real Time Kinematic GPS) or the latest GNSS (Global Navigation Satellite System) /RNSS (Reginal Navigation Satellite System).