| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | REQ 26 |
| Source:* | Convida Wireless, KDDI, Fujitsu |
| Date:* | 2016-12-07 |
| Contact:* | Chonggang Wang, Wang.Chonggang@ConvidaWireless.com Catalina Mladin, Mladin.Catalina@ConvidaWireless.com Nick Yamasaki, nr-yamasaki@kddi.com Shingo Fujimoto, shingo_fujimoto@jp.fujitsu.com |
| Reason for Change/s:* | Introduce subscription and notification requirements for use cases in TR-0026 |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <Work Item number> MNT Maintenance / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0026 V0.3.0 |
| Clauses/Sub Clauses* | Section 6.7 - Use Case on Vehicle Data Service |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The clause 6.7 in TR-0026 V0.3.0 has described a good use case on vehicle data service, where data from M2M Devices (e.g. sensors inside a smart vehice or sensors outside of a smart vehicle) can be subscribled by an M2M Application Server via M2M Gateway (e.g. a smart vehicle). Basically, M2M Application Servers send data publication to M2M Devices via M2M Gateway, and M2M Devices send data publication to M2M Application Servers also via M2M Gateway.

This change clarifies that the subscription requests from multiple M2M Servers may be grouped or aggregated at M2M Gateway. The benefit of such subscription grouping/aggregation is in the reduced messaging overhead between M2M Gateway and M2M Devices.

It also clarifies the case when the desired notifications (i.e. data publication) involve changes which appear on multiple M2M Devices (e.g. in-vehicle sensors).

### -----------------------Start of change 1------------------------------------------

## 6.7	Use Case on Vehicle Data Service

### 6.7.1	Description

This use case introduces several services based on various data collected by sensor devices via smart vehicle (vehicle with on-board communication system) which is regarded as M2M gateway.

The sensor devices may be located in the vehicle but may also be located outside the vehicle, e.g. on the road side. Some sensor devices are equipped with M2M area network module and measure individual data. The smart vehicle connects to the sensor devices and collects data from sensor devices by using the M2M area network technology such as Wireless LAN, ZigBee, Bluetooth, etc., and sends the data to application server in infrastructure domain via mobile network.

Management server and Application Server in the M2M infrastructure domain connect to the smart vehicle via a mobile network in order to control its configurations updating software and exchanging M2M data (e.g. updating a map).

It is important to observe that the smart vehicle as M2M gateway has mobility. For instance, there are possibilities for a mobile device to simultaneously connect to many sensor devices, and to newly connect to sensor devices which have never been connected before.

This use case illustrates potential requirements from the use case of services utilizing mobile device.

### 6.7.2	Source

REQ-2014-0472R06: Use Case on Vehicle Data Services

### 6.7.3	Actors

- M2M Device: In-vehicle sensor device and outdoor sensor device. In-vehicle sensor includes on-board (built-in) sensor used for monitoring of machine health and also sensor on user carry-on device, such as smart phone and wearable device like Fitbit™, Nike Fuelband™, Sony SmartBand™ for example. In-vehicle sensor may monitor status of machine health (diagnosis), mobility, passenger’s health and environment. Outdoor sensor device may be located outdoor and monitor roadway infrastructure, agriculture, property (surveillance) and utility (telemetering), for example. It may be equipped with several kind of communication protocol.

- M2M Area Network: Area network which connects M2M Device with M2M Gateway and also provides connectivity among M2M Gateways. It may include Wireless LAN, Bluetooth, ZigBee and Ethernet.

- M2M Gateway: On-board communication system equipped on the smart vehicle, which communicates with M2M Devices and other M2M Gateways via M2M Area Network and also communicates with Application Servers and Management Servers via Mobile Network.

- Mobile Network: Network which has functions to transfer data and control messages between M2M Gateway and M2M Application Server/Management Server. It may include cellular base station.

- M2M Management Server: Server which manages M2M applications in M2M Gateway and M2M Device by installing, uninstalling and updating them.

- M2M Application Server: Server which maintains database and provides the data access services such as accepting data publication from and issuing data subscription to M2M Gateway/Device. This server also manages non-M2M applications such as navigation system as well as contents such as map data in M2M Gateway and M2M Device by installing, uninstalling and updating them.

### 6.7.4	Pre-conditions

- It is possible to establish a connection among M2M Application Server, M2M Management Server and Smart Vehicle via Mobile Network.

- It is possible to establish a connection between Smart Vehicle and M2M Device and among Smart Vehicles via M2M Area Network.

- The M2M Gateway has been configured by the M2M Management Server.

- The M2M Device has not yet been configured by the M2M Management Server.

### 6.7.5	Triggers

Subject to capabilities of the M2M Area Network, the M2M Device detects a M2M Gateway that can be associated, or vice versa. The association may require explicit operation (permission) for that association by users (e.g. owners of Gateway and/or Device) or administrator (a manufacturer of Gateway and/or Device and/or Management Server). Examples are below:

- Sensors are built into the smart vehicle at the factory (Permanent Association, no triggering is needed).

- Sensors are equipped to the smart vehicle after market, e.g. at auto dealers or auto parts stores. Triggering occurs when the smart vehicle is being equipped with the new sensor

- Users bring sensors with user carry-on devices into the smart vehicle.

- The smart vehicle detects nearby outdoor sensors.

- The smart vehicle detects other smart vehicles nearby.

### 6.7.6	Normal Flow

M2M Configuration step: Management Server configures M2M Device

Upon triggering, i.e. when the M2M Device detects a M2M Gateway that can be associated or the M2M Gateway detects a M2M Device that can be associated the following sequence is initiated:

M2M Gateway establishes a connection to M2M Device (or the M2M Device establishes the connection to the M2M Gateway) via the M2M Area Network.

M2M Device sends its attribute information (e.g. type of device, service certificates of the device, required application software, etc.) to the M2M Gateway.

M2M Gateway establishes a connection to the M2M Management Server via Mobile Network.

M2M Gateway relays the attribute information to Management Server.

M2M Management Server provides M2M Gateway with the appropriate software and configuration data for the M2M Device.

M2M Gateway relays the software and configuration data to M2M Device.

M2M Device configures itself according to the software and configuration data.

Transmission of data between M2M Device and M2M Application Server

- Data Publication of M2M Device

- M2M Gateway (re-)establishes a connection to M2M Device via M2M Area Network.

- M2M Device publishes measured data to M2M Gateway.

- M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

- M2M Gateway relays the data to M2M Application Server. M2M Gateway may distribute the data (i.e. data publication) to multiple M2M Application Servers, whose data subscriptions have been previously grouped/aggregated at M2M Gateway.

- M2M Application Server receives the data.

Data Subscription from M2M Application Server

- M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

- M2M Application Server sends M2M Gateway attribute information of data in need.

- M2M Gateway (re-)establishes a connection to M2M Device via M2M Area Network.

- M2M Gateway relays the attribute information to M2M Device. M2M Gateway may group/aggregate the attribute information (i.e. data subscription) from multiple M2M Application Servers.

- M2M Device publishes measured data that meets the attribute information to M2M Gateway.

- M2M Gateway relays the data to Application Server. M2M Gateway may distribute the data (i.e. data publication) to multiple M2M Application Servers, whose data subscriptions have been previously grouped/aggregated at M2M Gateway.

- Application Server receives the data.

Update of non-M2M Application 1: M2M Application Server and M2M Gateway

- M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

- M2M Gateway sends its attribute information to M2M Application Server.

- M2M Application Server provides M2M Gateway with the appropriate software, its configuration data and contents such as map.

- M2M Gateway configures itself according to the software, configuration data, etc.

Update of non-M2M Application 2: M2M Application Server and M2M Device

- M2M Device establishes a connection to M2M Gateway via M2M Area Network.

- M2M Device sends its attribute information to M2M Gateway.

- M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

- M2M Gateway relays the attribute information to M2M Application Server.

- M2M Application Server provides M2M Gateway with the appropriate software and configuration data.

- M2M Gateway relays the configuration software and data to M2M Device.

- M2M Device configures itself according to the software and configuration data.

### 6.7.7	Alternative Flow

Alternative Flow 1

This alternative flow may occur in the case where the M2M Gateway only occasionally connects to devices and servers – e.g. via M2M Area networks that can only occasionally be used.

- M2M Gateway Opportunistic Communication (Store and Forward)

  - M2M Gateway may store data that are destined to M2M Device, M2M Management Server and M2M Application Server.

  - M2M Gateway may send the stored data to other M2M Gateway.

  - Both, the originator and the other M2M Gateway may deliver the stored data to the destination when connected.

  - M2M Gateway may erase the stored data that has been already sent to the destination.

Alternative Flow 2

This variant flow may occur in the case where the M2M Gateway processes the data flow between M2M

Gateway and M2M Application Server.

- M2M Gateway Data Processing

  - M2M Gateway may aggregate, statistically summarize (e.g. average) and/or erase data based on criteria that are indicated in the data subscription or the configuration data from M2M Application Server.

- M2M Gateway Data Subscription

  - M2M Application Server may be interested in joint changes to multiple resources (either on the same M2M Device or multiple M2M Devices). In other words, M2M Application Server makes a data subscription where automatic notification (i.e. data publication) depends on two or more resources, not a single resource. Notificiations are then generated when the expected changes occur within each of the resources.

Alternative Flow 3

This variant flow may occur in cases where the M2M Gateway broadcasts its interest to subscribe to specific data to all M2M Devices in its vicinity.

- M2M Gateway Cross Layer Optimization

  - M2M Gateway may indicate a full or part of data subscription (attribute information of data in need) in wireless pre-association information such as beacon.

  - Smart Vehicle and Wi-Fi Hotspot may periodically broadcast beacon that contains the subscription in order to prevent sensor devices from establishing unnecessary connection and wasting radio resources as well as battery power.

Note:  M2M Application Data Security (TBD)

### 6.7.8	Post-conditions

- M2M Application Server stores data and provides data access service via API for user applications.

- M2M Devices and M2M Gateways are well maintained by M2M Management Server and M2M Application Server.

### 6.7.9	High Level Illustration

### 6.7.10	Potential requirements

- The oneM2M System shall enable discovery of M2M Application Servers, M2M Management Servers and M2M Devices available to an M2M Gateway for data exchange.

- The oneM2M System shall enable discovery of M2M Gateways available to a M2M Management Server and an M2M Device for data exchange.

- The oneM2M System shall be able to support the capabilities for data repository (i.e. to collect/store) and for data transfer from one or more M2M Devices or M2M Gateways, for delivery to one or more M2M Gateways via M2M Area Network without any assistance or instruction of M2M Management Servers and M2M Application Servers.

- Upon request from M2M Application Server, an M2M Gateway shall enable functions that pre-process (e.g. average) M2M data before providing them to the recipient.

- Upon request, an M2M Gateway shall enable functions that erase M2M data (e.g. that have been sent or could not be sent to the recipient within a certain time) based on criteria from an M2M Application Server.

- An M2M Gateway and/or an M2M Device shall be able to broadcast to all M2M Devices and/or M2M Gateways in the vicinity its need to receive/deliver specific data.

- M2M Gateway and/or M2M Device shall be able to establish a connection to each other if it is able to receive/deliver the required specific data.

- The oneM2M System shall enable M2M Gateways to set conditions used for processing jointly data subscriptions and distribute the resulting notifications according to the set conditions.

- The oneM2M System shall enable subscriptions to changes to multiple resources which aim to generate notifications if and only if the expected changes to those resources occur concurrently.

### -----------------------End of change 1------------------------------------------