| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | REQ 26 |
| Source:* | Jie Shi, Huawei, shi.jie1@huawei.com Guilin Wang, Huawei, wang.guilin@huawei.com Shelby Kiewel, iconectiv, skiewel@iconectiv.com |
| Date:* | 2016-12-09 |
| Contact:* | As above |
| Reason for Change/s:* | Add security requirements to use case for electronic toll collection service in TR-0026 |
| CR against: Release* | TR - no release applicable |
| CR against: WI* | Active WI-0046 MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | <TR-0026> and 0.4.0 |
| Clauses/Sub Clauses* | 6.5.10 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR is proposed to add two potential security requirements to the use case for Electronic Toll Collection (ETC) service and make some editorial changes in Sub Clause of “Potential Requirements”.

### -----------------------Start of change 1-------------------------------------------

## 6.5	Use cases for Electronic Toll Collection (ETC) service

### 6.5.1	Description

ETC is an important part of Intelligent Transportation System (ITS). It is vigorously promoted in many countries.

ETC aims to eliminate the delay on toll roads by collecting tolls electronically. ETC determines whether the vehicles passing are enrolled in the program, alerts enforcers for those that are not, and electronically debits the accounts of registered vehicle owners without requiring them to stop.

In the ETC Use Case, On Board Unit (OBU) is a dedicated device located in the vehicle and it can communicate with the local RSUs only. Stated differently, the communication between OBU and ETC platform is via the RSUs. With a vehicle moving, the OBU would connect to the next RSU and release connection with the previous RSU. It is necessary for the OBU to 'register' with ETC Service Platform for receiving M2M system services. The term 'registration' here refers to an OBU having its contextual information available at the ETC Service Platform.  Such registration information is for the duration of an OBU being subscribed to services from the ETC Service Platform. At the same time, OBU 'registers' with local RSUs also, as OBU connects to different RSUs. Such registration context at the RSUs is temporary, and gets released as the OBU moves out of the range of the RSU and connects to the next RSU. The OBU uses its connectivity with local RSUs for achieving communication with ETC Service Platform.

As regards the 'registration context' between the OBU and the ETC Service Platform, the contextual information needs to reflect the complete profile of the OBU at the ETC Service Platform. Such information can include OBU identity, credentials, service subscription information, payment history, account balance etc. This is referred to here as 'full registration'.

As regards the 'registration context' between the OBU and the RSU, such contextual information is a subset of the contextual information at the ETC Service Platform for the said OBU. The information in this subset needs to be sufficient for the RSU to identify the OBU to the ETC Service Platform for the OBU to receive desired services without compromising any sensitive information to the RSU. This is referred to here as 'lightweight registration'

### 6.5.2	Source

REQ-2014-0431R03 Use cases for Electronic Toll Collection (ETC) service

REQ-2014-0449R02 Use cases for Electronic Toll Collection (ETC) service

### 6.5.3	Actors

- Vehicle Owner enrolls for ETC service.

- On Board Unit (OBU) is a M2M device used to store information such as identifier of the vehicle. The OBU typically does not communicate with the ETC Service Platform directly while receiving services. Direct communication between OBU and the ETC Service Platform may however be supported, out of band, for example for subscribing for ETC services.

- Road Side Unit (RSU) is a device which is an intermediate entity and is available to connect from OBU to the ETC Service Platform. Typical role of RSU can be implemented as an M2M gateway, and it may provide other functions as well (e.g., traffic light control, barrier control etc.).

- ETC Service Platform is responsible for collecting the information regarding the OBU via the RSU, including information such as location information from the RSU.

- ETC Service Provider provides its own M2M services for the user (OBU) through the ETC Service Platform.

### 6.5.4	Pre-conditions

- Vehicle owner enrolls his/her vehicle for ETC services.

- All RSUs have their respective context (registration information) available at the ETC Service Platform.

### 6.5.5	Triggers

A vehicle equipped with OBU drives close to an electronic road charging station which equipped with RSU.

### 6.5.6	Normal Flow

Figure 6.5-1 Normal Flow for Electronic Toll Collection (ETC) service

- Vehicle owner subscribes to ETC services and the OBU is 'registered' with the ETC Service Platform. All RSUs are also registered with the ETC Service Platform.

- Note: The term 'registration' refers to communication and resulting contextual information of the registering entity at the registered entity.

- When the vehicle goes to a highway entrance (location A), OBU registers with RSU-A.

- The Vehicle (OBU) sends a request to ETC Service Platform via RSU-A asking for permission to pass through.

- RSU-A forwards the request from the OBU along with its own identifying information e.g. location information, to the ETC Service Platform.

- ETC Service Platform processes the request for the OBU, received from RSU-A, and responds to RSU-A. The processing at ETC Service Platform can include charging the 'toll' to OBU account and updating account balance etc., for the OBU. The response to RSU-A includes the recommended action for the OBU for pass through.

- RSU-A decides whether vehicle can pass through according to the response from the ETC Service Platform.

- The vehicle drives to the highway exit at location B, and OBU registers to RSU at location B (RSU-B).

- The vehicle sends a request to ETC Service Platform via RSU-B asking for permission for pass through.

- RSU-B forwards the request for the OBU along with its own information e.g. location information to the ETC Service Platform.

- ETC Service Platform processes the request for the OBU received from RSU-B. The processing at ETC Service Platform can include charging the 'toll' to OBU and updating account balance etc. for the OBU. The ETC Service Platform may communicate with the ETC Service Provider, as needed, while processing the request from the OBU.

- ETC Service Platform responds to RSU-B by including information such as the recommended action for the OBU to pass through.

- RSU-B decides whether vehicle can pass through according to the response from the ETC Service Platform.

### 6.5.7	Alternative Flow

None

### 6.5.8	Post-conditions

None

### 6.5.9	High Level Illustration

Figure 6.5-2 High Level Illustration for Electronic Toll Collection (ETC) service

### 6.5.10	Potential Requirements

- The M2M system shall provide the capability for an M2M device to maintain registration with multiple entities simultaneously.

- The registration shall be able to include information that identifies the peer entity, and other information necessary for the establishment of the respective peer relationships(e.g. management privilege, subscription).

- The M2M System shall be able to hold the complete set of information context about a peer entity for some registrations (i.e., “full registration”).

- The M2M System shall be able to hold only a subset of information context about the peer entity for some registration (i.e. “lightweight registration”).

- The M2M System shall be  able to perform "lightweight registration" at different entities pertaining to a common peer entity and hold different sets of information about the common peer entity.

- The M2M System shall be able to correlate the "full registration" and the "lightweight registration" pertaining to a common peer entity.

- The M2M System shall be possible to distinguish the "full registrations" and the "lightweight registrations" pertaining to a common peer entity.

- The M2M System shall enable each of many M2M devices to perform mutual authentication with each of many M2M gateways.

- The M2M System shall enable mutual authentication of M2M devices and M2M gateways in a timely manner.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?