| CHANGE REQUEST |  |
| --- | --- |
| Group Name:* | WG2 |
| Source:* | SEC WG4 |
| Format:* | ARC#10 |
| Date:* | 2014-03-31 |
| Contact:* | Dragan Vujcic, v.dragan@oberthur.com |
| Reason for Change/s:* | Alignment with the access control terminology |
| CR against: Release* | 1 |
| CR against: TS/TR* | TS-0001 v0.4.3 |
| Clauses/Sub Clauses* | 9.6.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change/correction to existing feature or functionality New feature or functionality |
| Post Freeze checking:* | This CR contains only essential changes and corrections YES NO |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes alighment with the definitions of the access control terminology. In addition as per minutes of TP#9  to remove “list “ by making sure the accessControlPolicy defines the set of rules and privileges  that each entity must comply with to grant access to an object. The agreed acces control terminilofy are reminded below:

Access Control Attributes: Set of parameters of the originator, target resource, and environment against which there could be rules evaluated to control access.

NOTE:	An example of Access Control Attributes of Originator is a role. Examples of Access Control Attributes of Environment are time, day and IP address. An example of Access Control Attributes of targeted resource is creation time.

Access Decision:  Authorization reached when an entity’s Privileges, as well as other Access Control Attributes, are evaluated

Privilege: Qualification given to an entity that allows a specific operation (e.g. Read/Update) on a specific resource (e.g.: an entry in ACL specifies a privilege, not an Access Decision).

NOTE:	In addition to being granted a Privilege, the entity must also satisfy any conditions of the Access Control Attributes.

## -----------------------Start of change 1-------------------------------------------

### 9.6.2	Resource Type accessControlPolicy

The <accessControlPolicy> resource is comprised of  privileges and selfPrivileges attributes which

represents a set of access control rules  defining "allowed" entities (defined as accessControlOriginator) for certain access modes (defined as accessContolOperationFlags) within a specified context (defined as accessControlContexts); these are used in making Access Decisions.

By setting one or more accessControlPolicyID attributes on a resource, the access decision for that resource are then granted by evaluating the set of access control rules by  the privileges attributes defined in the identified <accessControlPolicy> resources.

The selfPrivileges attribute represents the set of access control rules for the resource <accessControlPolicy> itself.

Access control procedures using the <accessControlPolicy> resource are specified in TS-0003 [TS0003].

{9.6.2.a} Editor's Note: There is a need for a global replacement of accessRight with accessControlPolicy, accessRightsID with accessControlPolicyID, permission with privilege.

Figure 9.6.2-1: Structure of <accessControlPolicy> resource 
(only resource specific attributes are shown)

- Table 9.6.2-1: Child resources of <accessControlPolicy> resource

| Child Resource Name of <accessControlPolicy> | Child Resource Type | Multiplicity | Description | <accessControlPolicyAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscriptionAnnc> |

{9.6.2.c} Editor's Note: Contribution ARC-2014-0089R03 does not provide the table for the <subscription> child-resource. The contributor to verify the accuracy of the proposal.

This resource shall contain the attributes according to their multiplicity in Table 9.6.2-2.

- Table 9.6.2-2: Attributes of <accessControlPolicy> resource

| Attribute Name of <accessControlPolicy> | Multiplicity | RW/ RO/ WO | Description | <accessControlPolicyAnnc> Attribute |
| --- | --- | --- | --- | --- |
| resourceType (rT) | 1 | RO | See clause 9.6.1 where this common attribute is described. | MP |
| parentID (pID) | 1 | RO | See clause 9.6.1 where this common attribute is described. | MP |
| expirationTime (eT) | 1 | RW | See clause 9.6.1 where this common attribute is described. | MP |
| labels (lBs) | 0..1 | RW | See clause 9.6.1 where this common attribute is described. | MP |
| creationTime (cT) | 1 | RO | See clause 9.6.1 where this common attribute is described. | MP |
| lastModifiedTime (lMT) | 1 | RO | See clause 9.6.1 where this common attribute is described. | MP |
| link | 1 | WO | See section 9.6.1 where this common attribute is described. This is present only for <accessControlPolicyAnnc>. | MP |
| announceTo | 1 | RW | See section 9.6.1 where this common attribute is described. | NP |
| announcedAttribute | 1 | RW | See section 9.6.1 where this common attribute is described. | NP |
| privileges (ps) | 0..1 | RW | Represents a set of access control rules applicable to resources referencing this <accessControlPolicy> resource using the accessControlPolicyID attribute. | OP |
| selfPrivileges (sP) | 0..1 | RW | Represents the set of access control rules applicable to the <accessControlPolicy> resource itself. | OP |

The self Privileges and privileges attributes are comprised of lists of access-control-rule-tuples, where each such access-control-rule-tuple is comprised of at least the (accessControlOriginators, accessControlContexts, accessControlFlags) parameters shown in Table 9.6.2-3 which are further described in the following clauses.

Table 9.6.2-3: Mandatory Parameters in access-control-rule-tuples

| Name | Description |
| --- | --- |
| accessControlOriginators | See 9.6.2.1 |
| accessControlContexts | See 9.6.2.2 |
| accessControlOperations | See 9.6.2.3 |

#### 9.6.2.1	accessControlOriginators

The accessControlOriginators in an access-control-rule-tuple represents the set of Originators that can be authorized using this access control rule. The set of Originators is described by a list of parameters, where the types of the parameter can vary within the list. Table 9.6.2.1-1 describes the supported types of parameters in accessControlOriginators.

Table 9.6.2.1-1: Types of Parameters in accessControlOriginators

| Name | Description |
| --- | --- |
| originatorIdentifier | CSE-Id or AE-ID which represent a originator identity |
| token | Access token usually provided as query parameter |
| all | All originators |
| subscriberGroup | A subscriber group name associated with the URL the an <m2mServiceSubscription> resource (clause 9.6.19) where such subscriber group(subsGroup) is defined. |
| entityGroup | An identifier for a <group> resource (clause 9.6.13) describing a group of entities. |

#### 9.6.2.2	accessControlContexts

The accessControlContexts in an access-control-rule-tuple contains a list, where each element of the list represents a request context that is permitted using this access control rule. Each request contexts is described by a set of parameters, where the types of the parameter can vary within the set. Table 9.6.2.2-1 describes the supported types of parameters in accessControlContexts.

Table 9.6.2.2-1: Types of Parameters in accessControlContexts

| Name | Description |
| --- | --- |
| accessControlTimeWindowConstraint | Represents a time window which is compared against the time that Authorization Decision is made. |
| accessControlLocationRegionConstraint | Represents a location region which is compared against the location of the Originator of the request |
| accessControlIpAddressConstraint | Represents an IP address or IP address block which is compared against the IP address of the Originator of the request |

#### 9.6.2.3	accessControlOperations

The accessControlOperations in an access-control-rule-tuple represents the set of operations that can be authorized using this access control rule. Table 9.6.2.3-1 describes the supported set of operations that can be authorized by accessControlOperations.

Table 9.6.2.3-1: Operations that can be authorized by accessControlOperations

| Name | Description |
| --- | --- |
| RETRIEVE | Privilege to retrieve the content of an addressed resource |
| CREATE | Privilege to create a child resource |
| UPDATE | Privilege to update the content of an addressed resource |
| DELETE | Privilege to delete an addressed resource |
| DISCOVER | Privilege to discover the resource |
| NOTIFY | Privilege to receive a notification |

-----------------------End of change 1-------------------------------------------