| OneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0003-Security_Solutions-V-0.4.0 |
| Document Name: | oneM2M Security Solutions |
| Date: | 2014-04-15 |
| Abstract: | Specification of oneM2M security solutions. |

oneM2M IPR and copyright statements

“Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of an agreement to be bound by all provisions of IPR policy of the admitting Partner Type 1 and permission that all communications and statements, oral or written, or other information disclosed or presented, and any translation or derivative thereof, may without compensation, and to the extent such participant or attendee may legally and freely grant such copyright rights, be distributed, published, and posted on oneM2M’s web site, in whole or in part, on a non-exclusive basis by oneM2M or oneM2M Partners Type 1 or their licensees or assignees, or as oneM2M SC directs.

# Contents

Contents	2

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	5

3	Definitions, symbols, abbreviations and acronyms	5

3.1	Definitions	5

3.2	Symbols	6

3.3	Abbreviations	6

3.4	Acronyms	7

4	Conventions	7

5	Security Architecture	8

5.1	Overview	8

5.2	Resource Structure	10

5.3	Security Layers	11

5.3.1	Security Service Layer	11

5.3.2	Secure Environment Abstraction Layer	12

5.4	Integration within overall oneM2M architecture	12

5.5	Configuration	12

6	Security Services and Interactions	13

6.1	Interaction on Reference Points	13

6.1.1	Mca-Reference Point	13

6.1.2	Mcc-Reference Point	13

6.1.3	Mcc’-Reference Point	13

6.1.4	Mcn-Reference Point	14

6.1.4.1	Interworking with 3GPP networks	14

6.2	Internal Components	14

6.2.1	Registry	14

6.3	Security Service Layer 	14

6.3.1	Access Management	14

6.3.1.1	Access Control	14

6.3.1.2	Authorization	16

6.3.1.3	Authentication	17

6.3.2	Authorization Architecture	17

6.3.3	Security Association Establishment	19

6.3.3.1	Network-based security association	19

6.3.3.1.1	GBA-based security association	19

6.3.4	Security Administration	20

6.3.4.1	Remote administration	20

6.3.4.1.1	Interworking with 3GPP and 3GPP2	20

6.3.4.1.2	tbd	20

6.3.4.2	Bootstrapping	20

6.3.4.2.1	Network-based Bootstrapping	20

6.3.4.2.1.1	GBA-based Bootstrapping	20

6.3.4.2.2	PKI-Based Bootstrapping	21

6.3.5	Identity Protection	21

6.3.6	Sensitive Data Handling	21

6.3.6.1	Sensitive Functions	22

6.3.6.2	Secure Storage	23

6.3.7	Trust Enabler security functions	24

6.4	Secure Environment AbstractionLayer Components	25

6.4.1	Secure Environment	25

6.4.2	SE Plug-in	25

6.4.3	Secure Environment Abstraction	25

7	Security API	25

7.1	API description	25

7.2	Security Transport	25

7.3	Authorization	25

7.4	Sensitive Data Handling	26

7.5	Security Association	26

7.6	Security Administration	26

7.7	Identity Protection	26

7.8	Authentication	26

8	Security Frameworks 	26

8.1	General Introductions to the Security Frameworks	26

8.1.1	General Introduction to the Direct Security Frameworks	26

8.1.1.1	General Introduction to the Pre-Provisioned Symmetric Key Framework	26

8.1.1.2	General Introduction to the Certificate-Based Security Frameworks	26

8.1.1.2.1	Credential Configuration for Certificate-Based Security Frameworks	27

8.1.1.2.2	Certificate Verification	27

8.1.2	General Introduction to the Centralized Security Frameworks	28

8.1.2.1	General Introduction to the M2M Authentication Function Framework	28

8.1.2.2	General Introduction to the GBA (Generic Bootstrapping Architecture) Framework	28

8.2	Security Association Establishment Frameworks	29

8.2.1	Overview on Security Association Establishment Frameworks	29

8.2.2	Direct Security Association Establishment Frameworks	32

8.2.2.1	Pre-Provisioned Symmetric Key Security Association Establishment Frameworks	32

8.2.2.2	Certificate-Based Security Association Establishment Frameworks	34

8.2.3	Centralized Security Association Establishment Frameworks	36

8.2.3.1	MAF-Based Symmetric Key Security Association Establishment Frameworks	36

8.2.3.2	GBA-Based Security Association Establishment Frameworks	37

8.3.1	Overview on Security Bootstrap Frameworks	37

8.3.1.1	Purpose of Security Bootstrap Frameworks	37

8.3.1.2	Overview on Security Bootstrap Frameworks	37

8.3.2	Direct Security Bootstrap Framework	40

8.3.2.1	Pre-Provisioned Symmetric Key Security Bootstrap Framework	40

8.3.2.2	Certificate-Based Security Bootstrap Framework	43

8.3.3	Centralized Security Bootstrap Framework	45

8.3.3.1	GBA-Based Security Bootstrap Framework	45

Proforma copyright release text block	45

Annexes	46

Annex <y>: Bibliography	47

History	47

# 1	Scope

The present document defines security solutions applicable within the M2M system.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	void

[2]	Open Mobile API specification V2.0.2

[3]	GP TEE Client API….

[4]	ETSI TS 133 220 "Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) (3GPP TS 33.220)".

[5]	IETF RFC 5246 "The Transport Layer Security (TLS) Protocol Version 1.2".

[6]	IETF RFC 6347 “Datagram Transport Layer Security Version 1.2”

[7]	ETSI TS 102 225 (V11.0.0) "Smart Cards; Secured packet structure for UICC based applications (Release 11)" URL:http://www.etsi.org/

[8]	ETSI TS 102 226 (V11.0.0) “Smart Cards; Remote APDU structure for UICC based applications (Release 11)” URL:http://www.etsi.org/

[9]	3GPP TS 31.115 (V10.1.0) "Remote APDU Structure for (U)SIM Toolkit applications (Release 10)"

[10]	3GPP TS 31.116 (V10.2.0) "Remote APDU Structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications (Release 10)"

[11]	3GPP2 C.S0078-0 (V1.0) "Secured packet structure for CDMA Card Application Toolkit (CCAT) applications"

[12]	3GPP2 C.S0079-0 (V1.0) "Remote APDU Structure for CDMA Card Application Toolkit (CCAT) applications"

[13]	3GPP TS 33.220 (Vx.y.z) "Generic Authentication Architecture (GAA); Generic bootstrapping architecture"

[14]	3GPP2 S.S0109-A "Generic Bootstrapping Architecture (GBA) Framework"

[15]	RFC 4279 "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS) "

[16]	RFC 5246 "The Transport Layer Security (TLS) Protocol, Version 1.2"

[17]	RFC 6347 "Datagram Transport Layer Security Version 1.2"

[18]	RFC 5705 "Keying Material Exporters for Transport Layer Security (TLS)"

Editor’s note: add version numbers to references where missing.

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

- Use the EX style, add the letter "i" (for informative) before the number (which shall be in square brackets) and separate this from the title with a tab (you may use sequence fields for automatically numbering references).

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	oneM2M-TR-0004 Definitions and abbreviations

[i.3]	3GPP TR 33.868 V0.13.0; Security aspects of Machine-Type and other Mobile Data Applications Communications Enhancements; (Release 12)

[i.4]	http://en.wikipedia.org/wiki/Public_key_certificate  retrieved 2013-10-16.

Editor’s note: a more suitable reference is required for defining a Public Key Certificate

[i.5]	http://en.wikipedia.org/wiki/Public_key_infrastructure retrieved 2013-10-16, This definition in term references an academic paper: "LPKI - A Lightweight Public Key Infrastructure for the Mobile Environments", Proceedings of the 11th IEEE International Conference on Communication Systems (IEEE ICCS'08), pp.162-166, Guangzhou, China, Nov. 2008.

Editor’s note: a more suitable reference is required for defining a Public Key Infrastructure

[i.6]	oneM2M TR-0008, Security Analysis Technical Report

[i.7]	eXtensible Access Control Markup Language (XACML) Version 3.0. 22 January 2013. OASIS Standard.

# 3	Definitions, symbols, abbreviations and acronyms

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in [i.2] and the following apply:

Certificate: See Public Key Certificate

M2M Secure Connection Key: Key shared between two CSEs of M2M Nodes (e.g. ASN/MN-CSE and IN-CSE) in order to secure the communication between those two entities. This M2M Secure Connection Key results from a successful M2M Security Association Establishment procedure.

Master Credentials: Credentials used to mutually authenticate between an ASN/MN-CSE and the MAF. This is done to secure access to the infrastructure of an M2M Service Provider. The Master Credentials are either pre-provisioned or remotely provisioned (without relying on those credentials).

Policy Access Point: The system entity that finds applicable policy or policy set.

Policy Decision Point [i.7]: The system entity that evaluates applicable policy and renders an authorization decision.

Policy Enforcement Point [i.7]: The system entity that performs access control, by making decision requests and enforcing authorization decisions.

Policy Information Point [i.7]: The system entity that acts as a source of attribute values.

Public Key Certificate [i.4]: An electronic document that uses a digital signature to bind a public key with an identity.

Public Key Infrastructure [i.5]: a set of hardware, software, people, policies, and procedures needed to create, manage, distribute, use, store, and revoke Public Key Certificates

## 3.2	Symbols

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in [x] and the following abbreviations apply:

ACL	Access Control List

API	Application Programming Interface

ASN

BSF	Bootstrapping Server Function

B-TID		Bootstrapping Transaction Identifier

DTLS

Enrolee-ID	Enrollee Identity

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

HW	Hardware

IdA

IdB

IdMAF

Kc	M2M Secure Connection Key

Ke	Enrolment Key

KeId	Enrolment Key Identifier

Km	Master Credential

KmId	Master Credential Identifier

Kpm	pre-provisioned credential for Master Credential provisioning

KpmId	pre-provisioned credential for Master Credential provisioning Identifier

Kpsa	pre-provisioned credential for M2M Security Association Establishment

KpsaId	pre-provisioned credential for M2M Security Association Establishment Identifier

Ks

Ksa

Ks_(ext/int)_NAF Derived key in GBA_U / Derived key in GBA_U which remains on UICC

MAC

MAF	M2M Authentication Function

MAF-ID	M2M Authentication Function Identifier

MEF

MIC

MN

MZF

NAF	Network Application Function

PAP	Policy Access Point

PDP	Policy Decision Point

PEP	Policy Enforcement Point

PIP	Policy Information Point

PKI	Public Key Infrastructure

RBAC	Role Based Access Control

SE	Secure Environment

SW	Software

TLS

(D)TLS-PSK

USS	User Security Settings

URI

## 3.4	Acronyms

None.

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Security Architecture

## 5.1	Overview

The following figure 5-1 provides a high level overview of the Security architecture.

The architecture consists of following layers:

- Security Functions layer
This layer contains a set of security functions that are exposed at reference point Mca and Mcc. These security functions can be classified into six categories; they are Identification, Authentication, Authorization, Security Association, Sensitive Data Handling and Security Administration.

- Security Environment Abstraction Layer
This layer implements various security capabilities such as key derivation, data encryption/decryption, signature generation/verification, security credential read/write from/to the Secure Environments, and so on. The security functions in the Security Functions Layer invoke these functions in order to do the operations related to the Secure Environments. In addition this layer also provides physical access to the Secure Environments. Implementation of this is out of scope of the present document.

- Secure Environment layer
This layer contains one or multiple secure environments that provide various security services related to sensitive data storage and sensitive function execution. The sensitive data includes SE capability, security keys, local credentials, security policies, identity information, subscription information, and so on. The sensitive functions include data encryption, data decryption, and so on. Implementation of secure environments is out of scope of the present document.

Figure 5-1: High level overview of the Security architecture

Editor’s note:  Selection of the Secure environment is FFS.

Design principles:

- The architecture is split into several components and sub-components providing a modular design. With this design, mapping of the architecture to different nodes and entities is enabled.

- Depending on the requirements of each entity, Security should consist of components relevant to fulfil the requirements of the respective node or entity and the intended use case.

- The architecture may need to be adapted to be suitable for implementation in different entities. For example, the architecture can be mapped to different device classes.

- The security administration component shall enable administration of all sensitive resources (data and functions) and shall also allow configuration and extension of Security services itself.

- The Secure Environment within the CSE is accessed via the Secure Environment Abstraction layer and shall hold all sensitive resources.

- 5.1.1	Identification and Authentication

The Identification and Authentication function is in charge of identification and mutual authentication of CSEs and AEs.

Identification is the process of checking if the identity provided for authentication is valid. How to perform an identification process will depend on the purpose of authentication. For example, in the case of resource access, the authentication function may require the identification to check if the AE or CSE has registered with the local CSE; in the case of AE or CSE registration, the authentication function may require the identification to check if the identity provided by an AE or CSE fits a certificate. Once passing this checking process, the AE or CSE is identified, and the identified identity will be supplied to authentication process.

Authentication is the process of validating if the identity supplied in the identification step is associated with a trustworthy credential. How to perform an authentication process will depend on using which mutual authentication mechanism. For example, in the case of using certificate based authentication mechanism, the authentication function may require the authentication to verify a digital signature; in the case of using symmetric key based authentication mechanism, the authentication function may require the authentication to verify a Message Authentication Code (MAC). When this validating process has been completed, the AE or CSE is authenticated.

- 5.1.2	Authorization

The Authorization function is responsible for authorizing services and data access to authenticated entities according to provisioned access control policies and assigned roles.

Access control policy is defined as sets of conditions that define whether entities should be permitted access to a protected resource. The authorization function may support different authorization mechanisms, such as Access Control List (ACL), Role Based Access Control (RBAC), etc. The Authorization function may need to evaluate multiple access control policies in an authorization process in order to get a finial access control decision.

Authorization evaluation process is based on the Service Subscription resource which specifies what M2M Services and M2M Service roles the authenticated entity has subscribed to and the access control policies associated with the protected resource. The authorization evaluation process may also need to consider contextual attributes such as time or geographic location.

Prior to authorization mutual authentication between the originator CSE or AE and hosting CSE shall be performed.

- 5.1.2	Identity Management

The Identity Management function provides oneM2M identities / identifiers to the requesting entity in case those identities are stored within the secure environment. oneM2M identifiers as defined in [x] may also be treated as sensitive data that are accessible to AEs or CSEs and used independently of Authentication or Authorization functions.

## 5.2	Resource Structure

The following figure 5-2 shows the structure of Security resources. Further elements may be added as needed. Those elements are described further in clauses below.

Figure 5-2: Security resource structure

Editor’s note: RESTful approach may only be suitable for a subset of the Security functionality. Functions that are put into an enabler function (highlighted in blue in above figure) need to be considered. Alternatively, a new resource type <Function> may be needed. Secure Environment Abstraction not classified as resource and therefore not included in the tree.

Editor’s note: Clarification on that structure required. E.g. Sensitive Data Handing and Secure Environment to be restructured? How to expose secure storage to DMR (e.g. for ownership / user name)

| Child Resource Name | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| Sensitive Data Handling |  | 1 | Indicates which Sensitive Functions and Secure Storage capabilities are available |
| Secure Environment |  | 1..n | Describes the number and types of Secure Environment available within the CSE |
| Tbd… |  |  |  |

| AttributeName | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| Access ControlPolicy | 1 | RO | Describes the access control policies |
| Security Level | 1 | RO | Indicates the supported security levels |
| Attributes tbd |  |  |  |

## 5.3	Security Layers

### 5.3.1	Security Service Layer

The security service layer provides the following services:

- Access Management

  - Authorization

  - Authentication

  - Access Control

- Sensitive Data Handling

  - Sensitive Functions protection

  - Secure Storage

- Security Association Establishment

  - Secure Connection via secure session establishment

  - Secure Connection via object security

- Security Administration (including security bootstrapping)

- Identity Protection

Each of these services provides functions and resources on the Security Service and Administration API.

### 5.3.2	Secure Environment Abstraction Layer

The Secure Environment Abstraction Layer provides access to the Secure Environment via a general Security Transport API. A Plug-in associated to the type of Secure Environment shall provide physical / logical connectivity to the secure environment. The Secure Environment Abstraction Layer shall also be accessible on the Service Layer.

## 5.4	Integration within overall oneM2M architecture

Security services may be provided within the following architectural components and interacts on the different reference points as described below:

- CSE within an Application Service Node

  - Connected via the Mca-Reference Point to an Application Dedicated Node

  - Connected via the Mcc-Reference Point to a Middle Node

  - Connected via the Mcc-Reference Point to an Infrastructure Node

  - Connected via the Mcn-Reference Point to an Underlying Network Service Entity

- CSE within a Middle Node

  - Connected via the Mca-Reference Point to an Application Dedicated Node

  - Connected via the Mcc-Reference Point to an Application Service Node

  - Connected via the Mcc-Reference Point to a Middle Node

  - Connected via the Mcc-Reference Point to an Infrastructure Node

  - Connected via the Mcn-Reference Point to an Underlying Network Service Entity

- CSE within an Infrastructure Node

  - Connected via the Mca-Reference Point to an Application Dedicated Node

  - Connected via the Mcc-Reference Point to an Application Service Node

  - Connected via the Mcc-Reference Point to a Middle Node

  - Connected via the Mcc-Reference Point to an Infrastructure Node

  - Connected via the Mcc’-Reference Point to an Infrastructure Node

  - Connected via the Mcn-Reference Point to an Underlying Network Service Entity

5.5	Configuration

Security Services are modular and configurable according to the needs of the hosting CSE, its supported reference points and its purpose. The following table indicates which security components are mandatory or optional. Each sub-function may have different capabilities and technical implementation depending on the purpose and the node it resides in. Implementation of services may rely on functions provided by the underlying network over the Mcn reference point.

NOTE: Security capabilities residing within the Application Entity are out of scope, however their implementation can rely on functions and resources exposed through the Security API.

| Resource / Component | Child resource | M/O |
| --- | --- | --- |
| Access Management | tbd | O Note: depending on use case |
| Secure Environment Abstraction | n/a | M |
| Sensitive Data Handling | Sensitive Function | O Note: depending on use case |
|  | Secure Storage | M |
| Security Administration | Tbd | M |
| Security Association Establishment | Tbd | O Note: depending on use case |
| Identity Protection | Tbd | O Note: depending on use case |

Configuration of Security Services is either done via Security Administration or by pre-provisioning of the respective entity.

The architecture shall support a plug-in concept that allows adding further security functions and secure environments.

# 6	Security Services and Interactions

## 6.1	Interaction on Reference Points

### 6.1.1	Mca-Reference Point

The Security API offers its functions to Application Entities / Application Dedicated Nodes through the Mca-Reference point. Implementation of the Security API is dependent on the operating system of the physical device it resides in.

Editor’s note: Categories of physical devices are currently discussed in WI-0006. Depending on the outcome of this WID, definitions may have to be adapted.

### 6.1.2	Mcc-Reference Point

Editor’s note: This section covers topics related end-to-end secure channel establishment between CSEs

Over this interface Security functions are exposed to other CSEs.

### 6.1.3	Mcc’-Reference Point

Editor’s note: This section covers topics end-to-end secure channel establishment in case they are different to Mcc-reference point. In addition there may be relation to topics such as M2M Subscription Management / Identifier handling.

Editor’s note: To check how this specific case of Mcc is named in the Architecture group.

### 6.1.4	Mcn-Reference Point

Editor’s note: This section covers topics such as GBA and Remote Subscription Management.

#### 6.1.4.1	Interworking with 3GPP networks

Editor’s note: Add reference to [i.3].

## 6.2	Internal Components

### 6.2.1	Registry

Editor’s note: Further internal components such as a registry are FFS. This internal component may be needed to store management related information.

## 6.3	Security Service Layer

### 6.3.1	Access Management

#### 6.3.1.1	Access Control

The access control service controls access of Application Entities and CSEs to security resources. Access Control is used by the service and Secure Environment Abstraction layer. Requesting entities access the security functions via the Mca, Mcc or Mcc’ reference point. Based on the access policies stored within the secure environment, the access control function grants or rejects access of the requesting entity to the called Security function, depending on the requested action and affected resources.

Figure 6-3-1-1: Access Control

Access Control Flow:

Editor’s note: Current Figure is not in line with decision not to specify communication within CSE: Needs to be aligned.

Step 001:	The Originator AE or CSE calls a Security Function

Step 002:	The Security Function shall forward the request to the Access Control resource together with the respective identifiers of the calling entity and related authentication material

Step 003: 	The access control component shall verify the access rights and grant access to the function requested by the Originator AE/CSE.

Step 004:	Access control provides a response on the access control verification to the Security Function

Step 005:	Depending on the response of the access control component, the called function shall be processed or rejected.

Step 006	Security function provides result / response to the originating AE/CSE

#### 6.3.1.2	Authorization

The Authorization service provides functions to authorize access of AEs or CSEs to that particular function calling the Authorization service. As indicated in the figure 6-3-1-2 below, the requesting entity shall call functions over the Mca, Mcc or Mcc’ reference point. The called function may want to verify the authorization status of the requesting entity and initiates an authorization request.

Figure: 6-3-1-2: Authorizsation

Authorization Flow:

Step 001:	The originating AE or CSE requests certain functions. If a Security function is requested the access control flow as described above shall apply.

Step 002:	The called entity shall request to check authorization of the AE/CSE and verify if access to the called function is allowed. Therefore, the called entity shall send an authorization request to the authorization function.

Step 003: 	The Authorization function shall verify that the calling entity is allowed to access the authorization function and therefore shall request an access control verification.

Step 004:	Access control shall process the verification request.

Step 005:	Depending on access control rights, the Access Control function shall send a response to the Authorization function..

Step 006	The authorization function shall process the request initiated by the originator based on the result retrieved by the access control function.

Step 007:	A response to the entity shall be sent.

Step 008:	Based on the response, the entity shall process the request of the initiating AE or CSE.

Step 009:	The result of the entity processing of the request shall be sent back to the originator.

#### 6.3.1.3	Authentication

This component provides authentication services to the Application Layer.

### 6.3.2	Authorization Architecture

The following figure 6-3-2 provides a high level overview of the authorization architecture. This architecture comprises four subcomponents that are described as follows:

- Policy Enforcement Point (PEP)

PEP intercepts resource access requests, makes access control decision requests, and enforces access control decisions. The PEP coexists with the entity that need authorization services.

- Policy Decision Point (PDP)

PDP interacts with the PAP and PIP to get applicable authorization polices and attributes needed for evaluating authorization policies respectively, and then evaluates access request using authorization policies for rendering an access control decision. The PDP is located in the Authorization service.

- Policy Access Point (PAP)

PAP obtains applicable authorization policies according to an access control decision request. These applicable policies should be combined in order to get a finial access control decision. The PAP is located in the Authorization service.

- Policy Information Point (PIP)

PIP provides attributes that are needed for evaluating authorization policies, for example the IP address of the requester, creation time of the resource, current time or location information of the requester. The PIP is located in the Authorization service.

The Authorization service may comprise any of the subcomponents: PDP, PAP and/or PIP. This means that the subcomponents PEP, PAP, PDP and PIP could be distributed across different nodes. For example the PEP is located in an ASN/MN and the PDP is located in the IN.

NOTE: Release 1 does not support separation of PAP and PIP on different CSE from PDP.

Figure 6-3-2a: Overview of the authorization architecture

The authorization procedure is shown in the figure 6-3-2b below:

Figure 6-3-3b: Authorization Procedure

Step 001:	Mutual authentication (Pre-requisite).

Step 002:	Access Requester sends an Access Request to the PEP.

Step 003:	PEP makes an Access Control Decision Request according to the requester’s Access Request, and sends the Access Control Decision Request to the PDP.

Step 004: 	PDP sends an Access Control Policy Request that is generated based on the Access Control Decision Request to the PAP.

Step 005:	PAP finds all applicable access control policies to the access request and sends them back to the PDP. When multiple access control polices are involved, the PAP also provides a policy combination algorithm for combining multiple evaluation results into one finial result.

Step 006	PDP sends Attribute Request to the PIP if any attributes are required for evaluating these access control policies.

Step 007:	PIP gets required attributes and sends them back to the PDP.

Step 008:	PDP evaluates Access Request using access control policies. When there are multiple applicable access control policies, the PEP needs to calculate a final Access Control Decision using the policy combination algorithm.

Step 009:	PDP returns the Access Control Decision back to the PEP.

Step 010:	PEP enforces the access control decision, i.e. either forwards the Access Request to the resource or denies this access.

Step 011:	PEP returns access result back to the Access Requester.

### 6.3.3	Security Association Establishment

This service shall provide functions to establish a security association between different entities.

In addition, this service shall be able to use an existing security association to perform the following functions:

- Secure Connection via secure session establishment.

- Secure Connection via object security.

The Security Association Establishment component may rely on an external M2M Authentication Function to establish security associations.

Editor’s note: to be completed.

For a secure connection on 3GPP networks [i.3] applies.

#### 6.3.3.1	Network-based security association

In case of scenario where the M2M Service Provider and the operator of the underlying network have an agreement to use the underlying network credentials as the basis for establishing a secure connection between a M2M Application Service/Middle Node and Infrastructure Node (including the case that the M2M Service Provider and the operator of an underlying network are actually the same entity), GBA procedure could be used to perform the security association establishment procedure, as specified in [4].

It is important that this feature is used only within the scope of an appropriate agreement between the M2M Service Provider and the operator of the underlying network. The normative text in clause y.1 implicitly assumes that such an agreement is already in place. Since the present document is a technical specification, it does not address the details of such an agreement.

##### 6.3.3.1.1	GBA-based security association

The GBA procedure shall be applied as specified in [4] for scenarios where the operator of an underlying network supports GBA, e.g. 3GPP or 3GPP2, and the underlying network credentials are to be used as the basis for establishing a secure connection between a M2M Application Service/Middle Node and Infrastructure Node

The Secure Connection relies on a symmetric shared short term key used for M2M Service Connection to establish a secure session between a M2M Application Service/Middle Node and Infrastructure Node.

Editor’s note: This paragraph is not GBA specific and should be moved to a more generic section when available.

Clause 8.1.3 specifies how the short term key used for M2M Service Connection is obtained using the GBA framework.

### 6.3.4	Security Administration

The Security Administration service shall provide functions to manage the Security functions, resources and attributes. This shall include management of resources provided via the secure environment. In addition it should provide functions to manage sensitive data such as identifiers and subscriptions on behalf of other entities.

Editor’s note: to be completed. Bootstrapping to be mentioned. Align with ARC related to provided functions. Align with MAS about responsibility.

#### 6.3.4.1	Remote administration

The Security Administration service shall provide mechanisms for secure remote administration of sensitive data and functions stored in secure environment of M2M Nodes, reusing mechanisms provided by existing standards where appropriate.

##### 6.3.4.1.1	Interworking with 3GPP and 3GPP2

In case the secure environment of the underlying network is used to store the sensitive data of the M2M Service Layer and in case the underlying network is a 3GPP or 3GPP2 network, OTA mechanisms as specified in [7] and [8], and its extensions [9], [10] for 3GPP underlying networks or [11] and [12] for 3GPP2 underlying networks shall be used to securely administrate the sensitive data of the M2M Service Layer.

##### 6.3.4.1.2	tbd

<Text>

Editor’s note: appropriate additional specifications and interworking mechanisms for other transport networks are FFS.

#### 6.3.4.2	Bootstrapping

Bootstrapping is the post-provisioning of the essential information for the service layer security between an Application Service/Middle Node and the Infrastructure Node of a chosen M2M Service Provider. The essential security information includes the security credentials, the identifiers and access control information. The bootstrapping component may rely on an external M2M Enrolment Function to establish appropriate credentials.

Editor’s note: Need to precise in the TS the exact content of the bootstrapped information

Editor’s note: Should Bootstrapping to Application Dedicated Node (without CSE) be addressed?

##### 6.3.4.2.1	Network-based Bootstrapping

For scenarios where the M2M Service Provider and the operator of an underlying network have an agreement to use the underlying network credentials as the basis for the bootstrapping of a root credential shared between an Application Service/Middle Node and an Infrastructure Node (including the case that the M2M Service Provider and the operator of an underlying network are actually the same entity), GBA procedure specified in [4] may be used to perform bootstrapping with that M2M Service Provider.

It is important that this feature is used only within the scope of an appropriate agreement between the M2M Service Provider and the operator of the underlying network. The normative text in clause &.3.3.2.1.1 implicitly assumes that such an agreement is already in place. Since the present document is a technical specification, it does not address the details of such an agreement.

###### 6.3.4.2.1.1	GBA-based Bootstrapping

The GBA procedure as specified in [4] shall be used for scenarios where the operator of an underlying network supports GBA infrastructure, e.g. 3GPP or 3GPP2, and the underlying network credentials are to be used as the basis for the post-provisioning of root credential shared between an Application Service/Middle Node and an Infrastructure Node.

The root credential shared between an M2M Application Service/Middle Node and an Infrastructure Node is a symmetric shared long term M2M Root Key.

Editor’s note: This paragraph is not GBA specific and should be moved to a more generic section when available.

Clause 8.1.2 specifies how the long term M2M Root Key is obtained using the GBA framework.

##### 6.3.4.2.2	PKI-Based Bootstrapping

For scenarios where post- provisioning of a root credential should be independent of underlying network credentials, a Public Key Infrastructure (PKI)-based Bootstrap may be used for the post- provisioning of a root credential shared between an Application Service/Middle Node and an Infrastructure Node.

PKI-Based bootstrap applies TLS (RFC 5246 [5]) or DTLS (RFC 6347 [6]) using both server Certificates and client Certificates to perform mutual authentication and derivation of the resulting root credential.

### 6.3.5	Identity Protection

Identity Protection provides services to the Application Layer such as pseudonyms and protecting the anonymity of transactions.

### 6.3.6	Sensitive Data Handling

The Sensitive Data Handling service provides certain Sensitive Functions to the Application Layer.

Sensitive Functions shall include following functions:

- Secure Storage.

In addition the service should support the following sensitive functions

- Cryptographic operations.

- Methods for bootstrapping initial secrets (e.g. GBA).

Editor’s note: to be completed.

Resource Structure

Editor’s note: to be completed.

| Child Resource Name | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| Sensitive Functions |  | N |  |
| Secure Storage |  | 1..N |  |

| AttributeName | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| Attributes tbd |  |  |  |

#### 6.3.6.1	Sensitive Functions

This service shall provide AEs and CSEs with access to Sensitive Functions of the SE.

Resource Structure

Editor’s note: to be completed. Sensitive Functions may include cryptographic algorithms, …

| AttributeName | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| Sensitive Function ID | 1 | RO | Unique Identifier of the sensitive function |
| Version | 1 | RO | Version number of the sensitive function |
| Supported Security Level | 1 | RO | Indicates the Security Level provided by the sensitive function |

#### 6.3.6.2	Secure Storage

This service shall provide AEs and CSEs with access to the secure storage capability of the SE. Data securely stored by the AE or CSE shall only be accessible through the Security API and by authorized entities. Secure Storage should be managed by the Secure Environment. Stored data shall be associated with the entity owning the data, i.e. the entity that requested the data to be stored within the secure storage.

Resource Structure

Editor’s note: to be completed.

### 6.3.7	Trust Enabler security functions

OneM2M Trust Enabling Architecture requires the presence of three security functionalities within the Infrastructure Domain: the M2M Authentication Function (MAF), the M2M Enrolment Function (MEF) and the M2M Authorization Function (MZF). They can be either under M2M Service Provider control or delegated to a M2M Trust Enabler.

- M2M  Enrolment Function (MEF)

The MEF supports the security bootstrap procedure enabling the provisioning of the Master Credentials to be used to mutually authenticate entities accessing the infrastructure of an M2M Service Provider. The MEF relies on an initial credential pre-provisioned in the M2M node (e.g. during manufacturing). In case of M2M Security Remote Provisioning procedure, the MEF provides the M2M Master Credential both to the MAF and the ASN/MN-CSE.

- M2M Authentication Function (MAF)

M2M Master Credentials, used to mutually authenticate CSEs/AEs before granting them access to M2M services, shall be securely stored. M2M Master Credentials shall be securely stored in a specific infrastructure functionality named M2M Authentication Function (MAF).

The MAF securely contains the set of M2M Master Credentials that are used for authenticating CSEs/AEs that have been enrolled for M2M services. The MAF stores the M2M Master Credentials and possibly the identifiers of the associated CSE/AE. The MAF is identified by  its MAF-ID.

When M2M Security Remote Provisioning procedure takes place to share a M2M Master Credential between an ASN/MN CSE and the M2M Authentication Function, the M2M Enrolment Function (MEF) communicates with the MAF through an appropriate secure interface, if not co-located.

The MAF is also in charge of all security operations involving the usage of the M2M Master Credentials.

- M2M Authorization Function (MZF)

The MZF is in charge of authorization aspects. . It provides an M2M entity (AE or CSE) with proof that it may use specific M2M services, application, or privilege (i.e. apply an existing role to perform specific operations on an M2M resource).

Editor’s note: Description and usage of MZF need further details.

## 6.4	Secure Environment AbstractionLayer Components

### 6.4.1	Secure Environment

The Secure Environment component is a logical entity that provides Sensitive Functions operating on Sensitive Data, Secure Storage and other resources / functions.

There is no assumption made on the particular implementation of the Secure Environment. A SE may be implemented as an independent HW Security Element or as an integrated SW function. Each Secure Environment shall be associated with one certain Security Level depending on the particular implementation of the SE. Different Secure Environments may provide different Security Levels.

There shall be at least one Secure Environment within or accessible via the Security API, however there may be multiple.

### 6.4.2	SE Plug-in

The SE Plug-in enables physical access to the respective Secure Environment. Depending on the type of Secure Environment, the SE Plug-in may be implemented differently for each Secure Environment.

NOTE: Specification of the SE Plug-in is out of scope of the present document.

### 6.4.3	Secure Environment Abstraction

This component shall enable entities accessing the Security Service API direct access to the Secure Environment Abstractionlayer. For this purpose the generic Secure Environment Abstractioncomponent shall expose functions provided by the Secure Environment Abstractionlayer functions to the Application layer. The Secure Environment Abstractionlayer is specific to the underlying secure environment. The Secure Environment Abstractioncomponent provides an abstraction layer exposing a Secure Environment agnostic interface to the Application layer. The Secure Environment Abstractioncomponent provides the capability to extend the SEC API depending on the use case with proprietary functions.

The generic Secure Environment Abstractioncomponent provides access to resources residing within the secure environment. Access to those functions shall be restricted to functions that are not addressable by the standardised SEC API.

# 7	Security API

## 7.1	API description

General description of the API…

## 7.2	Security Transport

The Security Transport API provides a communication framework to the secure environment.

The Security Transport API shall be implemented according to the Security Transport API as defined in [2].

Editor’s note: a more detailed description of the mapping of the architecture to the architectures defined in [2] and [3] may be added.

## 7.3	Authorization

Editor’s note: add API functions required to provide Authorization Service here.

## 7.4	Sensitive Data Handling

Editor’s note: add API functions required to provide Sensitive Data Handling here.

## 7.5	Security Association

Editor’s note: add API functions required to provide Security Association here.

## 7.6	Security Administration

Editor’s note: add API functions required to provide Security Administration here.

## 7.7	Identity Protection

Editor’s note: add API functions required to provide Identity Protection here.

## 7.8	Authentication

Editor’s note: add API functions required to provide Authentication Service here.

# 8	Security Frameworks

## 8.1	General Introductions to the Security Frameworks

- The security frameworks can be categorized into two main types

- Direct Security Frameworks: where the entities authenticate each other directly, without assistance from a Central Key Distribution Server

- Centralized Security Frameworks: where the entities authenticate each other with the assistance of a Central Key Distribution Server.

Editor’s Note: Direct Security Framework and Central security Framework, and Central Key Distribution Server should be added to the definitions.

### 8.1.1	General Introduction to the Direct Security Frameworks

#### 8.1.1.1	General Introduction to the Pre-Provisioned Symmetric Key Framework

Editor’s Note: Text is needed here.

#### 8.1.1.2	General Introduction to the Certificate-Based Security Frameworks

Editor’s Note: Further Introductory material could be appropriate here.

This clause describes the Credential Configuration and Certificate Verification used in the Certificate-Based Security Association Establishment Framework and Certificate-Based Security Bootstrap Framework.

Editor’s Note: Definitions will be required for the common terms used in this  Framework. We should determine if definitions are most appropriate in the SEC TR or SEC TS. The terms to be defined are:

Certificate Chain

Private Signing Key

Public Verification Key

Public Key Certificate (often abbreviated to Certificate).

Root Certificate

Root of Trust

##### 8.1.1.2.1	Credential Configuration for Certificate-Based Security Frameworks

If an entity is to authenticate itself using a Certificate-Based Security Framework, then the entity shall be pre-provisioned with the following information

  - The entity’s Private Signing Key and the corresponding Public Verification Key.

NOTE: an entity authenticates itself to other entities by proving that it knows the Private Signing Key corresponding to a particular Public Verification Key.

Editor’s Note: In some cases the Private Signing Key is unique to a single entity. In other cases, the Private Signing Key might be used to authenticate all entity's executing on a particular piece of hardware or used to authenticate all AEs executing a particular piece of software. This needs clarification text.

  - The  entity’s Certificate containing the entity Public Verification Key and the entity’s Certificate Name (a unique identity) provided in a name field of the Certificate (e.g. in the X.509 “Subject” or “Subject Alternative Name” attribute). Two options are available:

    - The entity is configured with a valid Certificate Chain from the entity’s Certificate to a Root Certificate. The entity’s Root of Trust is the cryptographic hash of the Public Verification Key in this Root Certificate.

    - Alternatively, the entity’s Certificate is a self-signed certificate, for which the entity’s Private Signing Key is used to generate the Certificate signature (the certificate is verified using the entity’s Public Verification Key).  In this case the cryptographic hash of entity’s Public Verification Key is both the entity’s Root of Trust and the entity’s Certificate Name. This implies that the cryptographic hash of the entity’s Public Verification Key must be present in a name field of the entity’s Certificate.

Editor’s Note: Conditions under which a self-signed certificate can be trusted or possible means to verify it (e.g. OCSP) are FFS.

NOTE: This pre-provisioning (by definition) uses mechanisms not specified by oneM2M.

Editor’s Note: Details for OCSP [RFC2560] and Certificate Revocation Lists (CRLs) are FFS.

Editor’s Note: Usage of certificates other than X.509 is FFS.

##### 8.1.1.2.2	Certificate Verification

This clause describes how an entity authenticates the other entity in the Security Handshake of a Certificate-Based Security Framework.

The other entity’s Certificate is received during the Security Handshake.

The other entity’s Certificate is verified as follows:

- If the other entity’s Certificate is a self-signed certificate, then verify each of the following:

  - The other entity’s Certificate Name and other entity’s Root of Trust (received during Association Configuration or Bootstrap Instruction Configuration) are identical.

  - The other entity’s Certificate Name (received during Association Configuration or Bootstrap Instruction Configuration) is identical to a value in a name field (e.g. in the X.509 “Subject” or “Subject Alternative Name” attribute) of other entity’s Certificate (received during the Security Handshake).

  - The other entity’s Root of Trust (received during Association Configuration or Bootstrap Instruction Configuration) is identical to the Public Verification Key (or hash thereof) in the other entity’s Certificate (received during the Security Handshake).

- If the other entity’s Certificate is not a self-signed certificate, then  verify each of the following:

  - The other entity’s Certificate Name (received during Association Configuration or Bootstrap Instruction Configuration) is identical to a value in a name field (e.g. in the X.509 “Subject” or “Subject Alternative Name” attribute) of the other entity’s Certificate (received during the Security Handshake)..

  - The other entity Certificate has a valid Certificate Chain (received during the Security Handshake) to a Certificate whose Public Verification Key (or hash thereof) is identical to the other entity' Root of Trust (received during Association Configuration or Bootstrap Instruction Configuration).

NOTE: After a successful Security Handshake in which the other entity provides a Certificate Chain, the other entity’s identity (received during Association Configuration or Bootstrap Instruction Configuration) can be associated with additional information extracted from the other entity’s Certificate Chain (e.g. the other entity Manufacturer, other entity owner, or conformance criteria). These details are not described in the present document.

Editor’s Note: Details of credentials for OCSP [RFC2560] may be required here.

### 8.1.2	General Introduction to the Centralized Security Frameworks

#### 8.1.2.1	General Introduction to the M2M Authentication Function Framework

Editor’s Note: Text is needed here.

#### 8.1.2.2	General Introduction to the GBA (Generic Bootstrapping Architecture) Framework

A general introduction to GBA is included in TR-0008 [i.6].

After a successful GBA bootstrapping, the M2M Application Service/Middle Node and the BSF share a security association which consists of a bootstrapping transaction identifier (B-TID) and key material (GBA bootstrap Ks).

This security association may be used by the M2M Application Service/Middle Node to derive NAF keys (Ks_(ext/int)_NAF) shared between a M2M Application Service/Middle Node and a M2M Infrastructure Node or an M2M Authentication Framework.

Figure 8.1.2.2-1	GBA framework. Note that the Network application Function (NAF) may be an Infrastructure Node or an M2M Authentication Function.

## 8.2	Security Association Establishment Frameworks

### 8.2.1	Overview on Security Association Establishment Frameworks

The oneM2M system supports the following Security Association Establishment Frameworks:

- Direct Security Association Establishment Frameworks

  - Pre-Provisioned M2M Secure Connection Key Security Association Establishment. A symmetric key is pre-provisioned to the entities: this is called the Pre-Provisioned M2M Secure Connection Key, and denoted Kpsa. The entities authenticate each other by verifying Message Integrity Codes (MIC) in the Security Handshake which were generated using the symmetric key.  For more details see clause 8.2.2.

  - Certificate-Based Security Association Establishment: The entities are each issued with

    - a Private Signing Key that is known only to that entity,

    - a Certificate containing the corresponding Public Verification Key, and

    - (Optionally) a Certificate Chain from the entity’s Certificate to a Root Certificate.

The entities must validate each other’s Certificate before trusting the Public Verification Keys in the Certificate. Within the Security Handshake, entity A creates a digital signature of the session parameters using its private signing key and entity B verifies the digital signature using entity A’s public verification key. Then the roles are reversed: entity B creates a digital signature and entity A verifies it. For more details see clause 8.2.5.

- Centralized Security Association Establishment Frameworks: In such schemes, entity A and a Central Key Distribution Server authenticate each other and derive a M2M Secure Connection key (Kc) that the Central Key Distribution Server delivers to entity B. The entities then authenticate each other using the M2M Secure Connection key (Kc). The oneM2M authentication Frameworks using centralized key distribution are:

  - GBA based Security Association Establishment. This Security Association Establishment Framework uses 3GPP or 3GPP2 symmetric keys to authenticate entity A and the Central Key Distribution Server. The details are specified by 3GPP [13] and 3GPP2 [14]. For more details see clause 8.2.3.

  - M2M Authentication Function (MAF)-based Security Association Establishment. This Security Association Establishment Framework uses symmetric keys to authenticate the entity A and the Central Key Distribution Server. For more details see clause 8.2.4.

For a more detailed description of the above Security Association Establishment Frameworks, it is useful to compare the following aspects of the Security Association Establishment Frameworks:

- Credential Configuration:

  - For Central Key Distribution Security Association Establishment Frameworks:

    - Entity A is configured with (or otherwise establishes) the Master Credential (Km) that the entity A will use to authenticate the entity A to the Central Key Distribution Server.

    - The Central Key Distribution Server is configured with the Master Credential (Km) that will be used to authenticate the Central Key Distribution Server to entity A.

The details for the GBA-Based Security Association Establishment Framework are out of scope.

  - For other Security Association Establishment Frameworks, each entity is pre-provisioned with the Credential that the entity will use to authenticate itself to the other entity.

- Association Configuration: Configuration of entity identifiers (that is, CSE-ID or AE-ID) for the entities to be authenticated.

Additionally, in the case of Certificate-Based Authentication Framework: each entity is configured with the Certificate Name and Root of Trust that the entity will use to verify the other entity.

- Central Key Distribution Server Handshake: When a Centralized Security Association Establishment Framework is used, entity A and the Central Key Distribution Server (MAF or GBA-BSF) perform mutual authentication and generate a M2M Secure Connection Key (Kc) which is then used in the Security Handshake for mutual authentication between entity A and entity B. This is not applicable to Direct Security Association Establishment Frameworks.

- Association Security Handshake: Identification, authentication and security context establishment between the entities.

Figure 8.2.1-1 provides a summary of the above defined four Security Association Establishment Frameworks.

Figure 8.2.1-1	: Overview of the Security Association Establishment Frameworks supported by oneM2M. See Table 8.2.1-1 for details of the abbreviations used in this figure.

Editor’s note: In the above figure, the illustrations of the MAF-based and GBA-based centralized security frameworks may need to be updated when the detailed flows have been agreed.

Table 8.2.1-1	Abbreviations used in Figures in clauses 8.2 and 8.3.

| Abbreviation | Full Term |
| --- | --- |
| Authn | Authentication |
| BSF, B-TID, Ks, USS | GBA terms (see definitions) |
| Cert[A/B/Enrolee/MEF] | Certificate for [entity A/entity B/Enrolee/ M2M Enrolment Function] |
| CertName[A/B/Enrolee/MEF] | An identifier for the Certificate for [entity A/entity B/ Enrolee/M2M Enrolment Function]that matches a unique value in a name field of the Certificate for [entity A/entity B/ Enrolee/M2M Enrolment Function ] |
| Chain[A/B/Enrolee/MEF] | Certificate chain for [entity A/entity B/ Enrolee/M2M Enrolment Function]: Certificate Chain from the Certificate for [entity A/entity B/ Enrolee/M2M Enrolment Function]to a Root Certificate for [entity A/entity B/ Enrolee/M2M Enrolment Function] |
| Enrolee-ID | Entity identifier (CSE-ID, AE-ID) for an Enrolee |
| Id[A/B ] | Entity identifier (CSE-ID, AE-ID) for [entity A/entity B] |
| Kc | M2M Secure Connection Key |
| Ke | Enrolment Key (Ke) shared by an Enrolee and an M2M Enrolment Function |
| KeId | Identifier for an Enrolment Key (Ke) |
| Km | Master Credential shared by an entity and the M2M Authentication Function |
| KmId | Identifier for an Master Credential (Km) |
| Kpsa | Pre-Provisioned M2M Secure Connection Key |
| KpsaId | Identifier for a Pre-Provisioned M2M Secure Connection Key (Ksa) |
| Ks..NAF | GBA Ks_(int/ext)_NAF |
| M-TID | MAF Transaction Identifier |
| MAF-ID | Identifier for an M2M Authentication Function |
| MIC | Message Integrity Code. This is sometimes called a “Message Authentication Code” – we have used “Message Integrity Code” since the abbreviation of “Message Authentication Code” (MAC) might be misunderstood to refer to “Media Access Control”. |
| PrivateKey[A/B/Enrolee/MEF] | Private Signing Key for [entity A/entity B] |
| RootOfTrust[A/B/Enrolee/MEF] | Root of Trust for [entity A/entity B/ Enrolee/M2M Enrolment Function]: The hash of the public key in the Certificate for [entity A/entity B/ Enrolee/M2M Enrolment Function] (if self-signed) or the Root Certificate for [entity A/entity B/ Enrolee/M2M Enrolment Function] |

### 8.2.2	Direct Security Association Establishment Frameworks

#### 8.2.2.1	Pre-Provisioned Symmetric Key Security Association Establishment Frameworks

This clause describes the Pre-Provisioned Secure Connection Key Security Association Establishment Framework.  This framework enables mutual authentication of two entities corresponding to either two CSEs or a CSE and an AE. The Credential for this framework is a long-term symmetric key that has been pre-provisioned into the entities to be authenticated. This key is called a Pre-Provisioned Secure Connection Key and is denoted Kpsa. The entities authenticate each other by verifying message authentication codes in the Security Handshake which were generated using the Pre-Provisioned Secure Connection Key.

NOTE: Long term Pre-Provisioned Secure Connection Keys can pose a security risk if not adequately secured, and for this reason Long term Pre-Provisioned Secure Connection Keys are recommended to be stored in Secure Environments.

Figure 8.2.2.1-1 illustrates the sequence of events when using the Pre-Provisioned Secure Connection Key Security Association Establishment Framework. In this description, “Entity A” and “Entity B” correspond to either two CSEs or a CSE and an AE or an AE and a CSE (respectively).

Figure 8.2.2.1-1: 	The sequence of events when using the Pre-Provisioned Secure Connection Key Security Association Establishment Framework.  See Table 8.2.1-1 for details of the abbreviations used in this table.

The following font colours differentiate the general topic that the text relates to:

- Blue italic text highlights details specific to this particular Security Association Establishment Framework.

- Purple italic text highlights technical actions that may include steps not specified by oneM2M.

- Red italic text highlights security-related properties.

Credential Configuration: The Pre-Provisioned Secure Connection Key (Kpsa) and the corresponding Pre-Provisioned Secure Connection Key Identifier, denoted KpsaId, are pre-provisioned to both entities.

NOTE:  This pre-provisioning (by definition) uses mechanisms not specified by oneM2M.

Association Configuration:  Each entity is configured with the information needed for mutual authentication and identification:

  - Entity A is commanded to initiate Security Context Establishment, and includes the following arguments:

    - The Pre-Provisioned Secure Connection Key Identifier KpsaId.

    - Entity A identity (IdA). Entity A is to assume this identity when interacting with Entity B.

    - Entity B identity (IdB). Entity A is to use this identity for Entity B authenticating using the above arguments. This identity is also used to route the (D)TLS exchange.

The mechanisms for achieving this are discussed in clause g.h.i.

NOTE: Entity A will associate Entity B’s identity with messages secured within Security Contexts established using the Pre-Provisioned Secure Connection Key Kpsa associated with the Pre-Provisioned Secure Connection Key Identifier KpsaId

Editor’s Note: Clause g.h.i referred to above is a suggested clause where the appropriate location is not yet clear.

  - Entity B is configured with the following arguments describing Entity A authorized to perform Security Context Establishment with the Registrar:

    - The Pre-Provisioned Secure Connection Key Identifier KpsaId.

    - Entity A identity (IdA). Entity B is to use this identity for Entity A authenticating using the above arguments..

    - Entity B Identity (IdB), if not already configured to Entity B. Entity B is to assume this identity when interacting with Entity A.

The mechanisms for achieving this are discussed in clause g.h.i.

NOTE: Entity B will associate the configured Entity A identity with messages secured within Security Contexts established using the Pre-Provisioned Secure Connection Key Kpsa associated with the Pre-Provisioned Secure Connection Key Identifier KpsaId.

Editor’s Note: Clause g.h.i referred to above is a suggested clause where the appropriate location is not yet clear.

NOTE: in many scenarios, the entity identities will be configured when the Pre-Provisioned Secure Connection Key Kpsa is established, so a separate configuration is not required in those scenarios.

- Centralized Key Distribution Server Handshake: There is no Centralized Key Distribution Server Handshake applied in the Pre-Provisioned Secure Connection Key Security Association Establishment Framework.

Association Security Handshake: The entities perform a (D)TLS-PSK handshake [15] to establish a secure session.

- The “psk_identity” parameter [15] is set to the value of the Pre-Provisioned Secure Connection Key Identifier KpsaId.

- The “psk” parameter [15] is set to the value of the Pre-Provisioned Secure Connection Key Kpsa.

#### 8.2.2.2	Certificate-Based Security Association Establishment Frameworks

This clause describes the  Certificate-Based Security Association Establishment Framework.

Figure 8.2.2.2-1 illustrates the sequence of events when using the Certificate-Based Security Association Establishment Framework. In this description, “Entity A” and “Entity B” correspond to either two CSEs or a CSE and an AE.

Figure 8.2.2.2-1: 	The sequence of events when using the Certificate-Based Security Association Establishment Framework.  See Table 8.2.1-1 for details of the abbreviations used in this figure.

The following font colours differentiate the general topic that the text relates to:

- Blue italic text highlights details specific to this particular Security Association Establishment Framework.

- Purple italic text highlights technical actions that may include steps not specified by oneM2M.

- Red italic text highlights security-related properties.

Credential Configuration: The master credentials for each entity are pre-provisioned as described in Clause 8.1.1.2.1 “Credential Configuration for Certificate-Based Security Frameworks”

Association Configuration: Entity A and Entity B  are configured with the information needed for the authentication and identification (during Security Handshake) of  Entity B and Entity A respectively:

  - Entity A is commanded to initiate a Security Handshake, and the command includes the following arguments:

    - The Entity B Certificate Name (Entity B Certificate must include this value in a name field).

    - The Entity B Root of Trust (Entity B Certificate must be self-signed using this root of trust, or have a Certificate Chain to this root of trust).

    - The Entity A identity (IdA) Entity A is to use this entity identity with Entity B.

    - The Entity B identity (IdB). Entity A is to use this identity for Entity B authenticating using the above arguments. This is used to route the (D)TLS exchange).

The mechanisms for achieving this are discussed in clause g.h.i.

NOTE: The Entity A will associate Entity B’s identity with messages secured within Security Contexts established in accordance with the configured Entity B Certificate Name and Entity B Root of Trust.

Editor’s Note: Clause g.h.i referred to above is a suggested clause where the appropriate location is not yet clear.

  - The Entity B is configured with the following arguments describing Entity A authorized to perform Security Handshake with Entity B:

    - The Entity A Certificate Name. The Entity A Certificate must include this value in a name field.

    - The Entity A Root of Trust. The Entity A Certificate must be self-signed using this root of trust, or have a Certificate Chain to this root of trust.

    - The Entity A identity (IdA). Entity B is to use this entity identity for Entity A authenticating using the above arguments.

    - The Entity B Identity (IdB), if not already configured to Entity B. Entity B is to assume this identity when interacting with Entity A.

The mechanisms for achieving this are discussed in clause g.h.i.

NOTE: Entity B will associate Entity A’s identity with messages secured within Security Contexts established in accordance with the configured Entity A Certificate Name and Entity A Root of Trust.

Editor’s Note: Clause g.h.i referred to above is a suggested clause where the appropriate location is not yet clear

- Centralized Key Distribution Server Handshake: There is no Centralized Key Distribution Server Handshake applied in the Certificate-Based Security Association Establishment Framework.

Association Security Handshake:

- Each entity verifies the other entity’s certificate as described in clause 8.1.1.2.2 “Certificate Verification”.

- The entities authenticate each other using the validated certificates as specified in TLS 1.2 RFC 5246 [16] and DTLS 1.2 RFC 6347 [17] specifications.

Editor’s note: the applicable (D)TLS ciphersuites are FFS.

### 8.2.3	Centralized Security Association Establishment Frameworks

#### 8.2.3.1	MAF-Based Symmetric Key Security Association Establishment Frameworks

Editor’s Note: Text is required here.

#### 8.2.3.2	GBA-Based Security Association Establishment Frameworks

To obtain a short term key used for M2M Service Connection between a M2M Application Service/Middle Node and a M2M Infrastructure Node, the M2M Application Service/Middle Node shall perform a successful GBA bootstrapping and derive a NAF key (Ks_(ext/int)_NAF). This NAF key is the M2M Secure Connection Key used for M2M Service Connection.

Editor’s Note: Further text is required here.

- 8.3	Security Bootstrap Frameworks

Editor’s note: The following terms require definitions:

Enrolee: An AE or CSE that requires remote provisioning of an Master Credential and Master Credential Identifier for an M2M Authentication Function.

Security Bootstrap Framework: a mechanism for remotely provisioning an Master Credential and Master Credential Identifier to a Enrolee and an M2M Authentication Function.

Bootstrap Credential: A pre-provisioned credential enabling mutual authentication of the Enrolee and the M2M Enrolment function.

### 8.3.1	Overview on Security Bootstrap Frameworks

#### 8.3.1.1	Purpose of Security Bootstrap Frameworks

The MAF-Based Security Association Establishment Framework uses a Master Credential (Km) and corresponding Master Credential Identifier (KmId), shared by a CSE/AE and an M2M Authentication Function, to establish security associations between the CSE/AE and other CSEs and/or AEs as described in clause 8.2.3.

The Master Credential (Km) and corresponding Master Credential Identifier (KmId) shall either be pre-provisioned or remotely provisioned to the CSE/AE and M2M Authentication Function.

The details for pre-provisioning are not described in this specification.

Clause 8.3 describes the set of remotely provisioning mechanisms; called Security Bootstrap Frameworks. An M2M Enrolment Function facilitates the remote provisioning.

#### 8.3.1.2	Overview on Security Bootstrap Frameworks

An AE or CSE that requires remote provisioning of a Master Credential and Master Credential Identifier is called an Enrolee.

The oneM2M system supports the following Security Bootstrap Frameworks:

- Direct Security Bootstrap Frameworks

  - Pre-Provisioned Symmetric Enrolee Key Security Bootstrap Framework. A symmetric key is pre-provisioned to the Enrolee and M2M Enrolment Function for the mutually authentication of those entities. For more details, see clause 8.3.2.1.

  - Certificate-Based Security Bootstrap Framework: The Enrolee and M2M Enrolment Function are each issued with

    - a Private Signing Key that is known only to that entity,

    - a Certificate containing the corresponding Public Verification Key, and

    - (Optionally) a Certificate Chain from the entity’s Certificate to a Root Certificate.

The Enrolee and M2M Enrolment Function shall validate each other’s Certificate before trusting the Public Verification Keys in the Certificate. Within the Security Handshake, the M2M Enrolment Function creates a digital signature of the session parameters using its private signing key and the Enrolee verifies the digital signature using the M2M Enrolment Function’s public verification key. Then the roles are reversed: the Enrolee creates a digital signature and the M2M Enrolment Function verifies it. For more details see clause 8.3.2.2.

- Centralized Security Bootstrap Frameworks

  - GBA-based Security Bootstrap Framework. In this case, the M2M Enrolment Function includes the functionality of a GBA Bootstrap Server Function. This framework uses 3GPP or 3GPP2 symmetric keys to authenticate the Enrolee and the M2M Enrolment Function (which is also a GBA BSF). The details are specified by 3GPP [13] and 3GPP2 [14]. For more details see clause 8.3.3.1.

For a more detailed description of the above Security Bootstrap Frameworks, it is useful to compare the following aspects of the Security Bootstrap Frameworks.

- Bootstrap Credential Configuration: The Enrolee and M2M Enrolment Function are pre-provisioned with the Bootstrap Credential that the entity will use to authenticate itself to the other entity. The mechanisms for this pre-provisioning are not described in this specification.

- Bootstrap Instruction Configuration: The Enrolee and M2M Enrolment Function are provided with

  - The M2M Authentication Function Identifier (MAF-ID) identifying the M2M Authentication Function for which the Enrolee is to be remotely provisioned.

Note: The identity of the M2M Authentication Function is assumed to have been configured prior to the Bootstrap Instruction Configuration phase.

  - The CSE-ID or AE-ID that the M2M Authentication Function is to associate with the Enrolee.

Additionally, in the case of Certificate-Based Security Bootstrap Framework:

  - The Enrolee is configured with the M2M Enrolment Function URI (for the purpose of routing the (D)TLS messages to the M2M Enrolment Function), M2M Enrolment Function Certificate Name and M2M Enrolment Function Root of Trust that the Enrolee will use to verify the M2M Enrolment Function.

  - The M2M Enrolment Function is configured with the Enrolee Certificate Name and Enrolee Root of Trust that the M2M Enrolment Function will use to verify the Enrolee.

- Bootstrap Enrolment Handshake: Identification, authentication and security context establishment between the Enrolee and M2M Enrolment Function.

- Enrolment Key Generation: generating a symmetric Enrolment Key,(Ke) and corresponding Enrolment Key Identifier (KeId) shared by the Enrolee and M2M Enrolment Function, which is used for subsequent generation of the Master Credential (Km).

- Usage in Centralized Key Distribution Server Handshake: During the Centralized Key Distribution Server Handshake of the MAF-Based Security Association Establishment, the following steps occurs

  - The Enrolee derives the Master Credential (Km) from the Enrolment Key (Ke) and M2M Authentication Function Identifier (MAF-ID). Details of the derivation are provided in clause 8.3.4.

  - The Enrolee generates the Master Credential Identifier (KmId) from Master Credential (Km) as described in clause r.s.t, and stores Km and KmId.

Editor’s Note: Clause r.s.t referred to above is a suggested clause where the appropriate location is not yet clear.

  - The Enrolee passes the Enrolment Key Identifier (KeId)  to the M2M Authentication Function (see “Centralized Key Distribution Server Handshake” in clause 8.2.3.1)

NOTE: When the Enrolee first communicates with the M2M Authentication Function, then the M2M Authentication Function has not yet retrieved the Km from the M2M Enrolment Function. Consequently, the Enrolee provides the KeId to the M2M Authentication function, which is then passed to the M2M Enrolment Function to identify the Enrolment Key. The M2M Enrolment Function then returns the Km from which the M2M Authentication Function can derive the KmId. In subsequent Security Establishments, the Enrolee may provide the KmId or the KeId, and the M2M Authentication Function will know that both identifiers indicate the retrieved Km. For more details, see “Centralized Key Distribution Server Handshake” in clause 8.2.3.1.

  - Upon receipt of the KeId, the M2M Authentication Function determines if it already has the corresponding Km and CSE-ID or AE-ID of the Enrolee

    - If the M2M Authentication Function already has the corresponding Km and CSE-ID or AE-ID of the Enrolee, then the Km is used for mutual authentication (see “Centralized Key Distribution Server Handshake” in clause 8.2.3.1)

    - If the M2M Authentication Function does not have the corresponding Master Credential (Km) and CSE-ID or AE-ID of the Enrolee, then the following steps are followed.

      - The M2M Authentication Function (securely) passes the KeId to the M2M Enrolment Function, along with the M2M Authentication Function’s URI.

      - The M2M Enrolment Function derives the Km from the Ke and MAF-ID. Details of the derivation are provided in clause 8.3.4.

      - The M2M Enrolment Function returns the Km to the M2M Authentication Function. The M2M Enrolment Function also passes the USS, including the CSE-ID or AE-ID of the Enrolee. More details are provided in clause 8.3.5.

      - The M2M Authentication Function generates the Master Credential Identifier (KmId) from Master Credential (Km) as described in Clause r.s.t, and stores Km and KmId

      - The Master Credential (Km) is used for mutual authentication (see “Centralized Key Distribution Server Handshake” in Clause 8.2.3.1)

Figure 8.3.1-1 provides a summary of the above defined Security Bootstrap Frameworks

Figure 8.3.1.2-1: 	Overview of the Security Bootstrap Frameworks supported by oneM2M. See Table 8.2.1-1 for details of the abbreviations used in this figure.

Editor’s note: In the above figure, the illustrations of the GBA-based centralized security frameworks may need to be updated when the detailed flows have been agreed. Additionally, more parameters may need to be transferred from MAF to MEF in the “Usage in Centralized Key Distribution Server Handshake”. This is FFS.

Editor’s Note: Clause r.s.t referred to in above figure is a suggested clause where the appropriate location is not yet clear.

### 8.3.2	Direct Security Bootstrap Framework

#### 8.3.2.1	Pre-Provisioned Symmetric Key Security Bootstrap Framework

This clause describes the Pre-Provisioned Symmetric Key Security Bootstrap Framework. The Bootstrap Credential for this framework is a long-term symmetric key that has been pre-provisioned into the Enrolee and M2M Enrolment Function; this key is called a Pre-Provisioned Symmetric Enrolee Key and is denoted Kpm.

NOTE: Long term Pre-Provisioned Symmetric Enrolee Keys can pose a security risk if not adequately secured, and for this reason it is recommended that Long term Pre-Provisioned Symmetric Enrolee Keys are stored in Secure Environments.

Figure 8.3.2.1-1 illustrates the sequence of events when using the Pre-Provisioned Symmetric Enrolee Key Security Bootstrap Framework.

Figure 8.3.2.1-1	: The sequence of events when using the Pre-Provisioned Symmetric Key Security Bootstrap Framework.  See Table 8.2.1-1 for details of the abbreviations used in this table.

The following font colours differentiate the general topic that the text relates to:

- Black text contains Security-Bootstrap-Framework-independent details

- Blue italic text highlights details specific to this particular Security Bootstrap Framework.

- Purple italic text highlights technical actions that may include steps not specified by oneM2M.

Editor’s Note: More parameters may need to be transferred from MAF to MEF in the “Usage in Centralized Key Distribution Server Handshake”. This is FFS.

Editor’s Note: Reference to RFC 5705 is FFS.

Editor’s Note: Clause r.s.t referred to in above figure is a suggested clause where the appropriate location is not yet clear.

Bootstrap Credential Configuration: The Pre-Provisioned Symmetric Enrolee Key (Kpm) and the corresponding Pre-Provisioned Symmetric Enrolee Key Identifier, denoted KpmId, are pre-provisioned to both entities. The Enrolee is also provisioned with the M2M Enrolment Function’s URI (MEF URI), for the purpose of routing the (D)TLS exchange.

NOTE:  This pre-provisioning (by definition) uses mechanisms not specified by oneM2M.

Bootstrap Instruction Configuration:  The Enrolee and M2M Enrolment Function are configured with the information needed for authorizing the remote provisioning:

  - The Enrolee is configured with (or otherwise obtains) the following arguments to initiate remote provisioning :

    - M2M Authentication Function identity (MAF-ID): Identifying the M2M Authentication Function for which the Enrolee is to be provisioned.

    - Enrolee identity (Enrolee-ID): the Enrolee is to use this identity with the identified M2M Authentication Function.

    - The Enrolee associates these arguments with the M2M Enrolment Function. The M2M Enrolment Function can be identified to the Enrolee using the Pre-Provisioned Symmetric Enrolee Key Identifier (KpmId) or the M2M Enrolment Function URI.

The mechanisms for achieving this are discussed in clause x.y.z.

Editor’s Note: Clause x.y.z referred to above is a suggested clause where the appropriate location is not yet clear

  - M2M Enrolment Function is configured with the following arguments to authorize the M2M Enrolment Function to remotely provision the Enrolee for a M2M Authentication Function:

    - M2M Authentication Function Identity (MAF-ID) of the M2M Authentication Function for which the Enrolee is to be provisioned.

    - Enrolee identity (Enrolee-ID). The M2M Enrolment Function is to provide this entity identity for the Enrolee with the Master Credential (Km) to the target M2M Authentication Function, when requested by the M2M Authentication Function.

    - The M2M Enrolment Function associates these arguments with an Enrolee. The Enrolee can be identified to the M2M Enrolment Function using the Pre-Provisioned Symmetric Enrolee Key Identifier (KpmId).

The mechanisms for achieving this are discussed in clause m.n.o.

Editor’s Note: Clause m.n.o referred to above is a suggested clause where the appropriate location is not yet clear.

Bootstrap Security Handshake: The Enrolee and M2M Enrolment Function perform a (D)TLS-PSK handshake [15] to establish a secure session.

- The “psk_identity” parameter [15] is set to the value of the Pre-Provisioned Symmetric Enrolee Key Identifier (KpmId).

- The “psk” parameter [15] is set to the value of the Pre-Provisioned Symmetric Enrolee Key (Kpm).

Editor’s note: the applicable (D)TLS ciphersuites are FFS.

Enrolment Key Generation:

- The Enrolment Key (Ke) is generated from the (D)TLS session secrets by the Enrolee and M2M Enrolment Function using TLS Key Export (RFC 5705) [18], as described in clause 8.3.6.

- The Enrolment Key Identifier (KeId) is generated by the Enrolee and M2M Enrolment Function, as described in clause r.s.t.

Editor’s Note: Clause r.s.t referred to above is a suggested clause where the appropriate location is not yet clear.

- The Enrolee and M2M Enrolment store the Enrolment Key (Ke) and Enrolment Key Identifier (KeId).

NOTE: The Enrolment Key Generation for the Pre-Provisioned Symmetric Enrolee Key Security Bootstrap Framework is identical to the Enrolment Key Generation for the Certificate-Based Security Bootstrap Framework.

Usage in Centralized Key Distribution Server Handshake: See “Overview of Security Frameworks” in Clause 8.3.1.

#### 8.3.2.2	Certificate-Based Security Bootstrap Framework

This clause describes the Certificate-Based Security Bootstrap Framework. The Bootstrap Credentials for this framework are Certificates.

Figure 8.3.2.2-1 illustrates the sequence of events when using the Certificate-Based Security Bootstrap Framework.

Figure 8.3.2.2-1: 	The sequence of events when using the Certificate-Based Security Bootstrap Framework.  See Table 8.2.1-1 for details of the abbreviations used in this figure.

The following font colours differentiate the general topic that the text relates to:

- Black text contains Security Bootstrap-Framework-independent details

- Blue italic text highlights details specific to this particular Security Bootstrap Framework.

- Purple italic text highlights technical actions that may include steps not specified by oneM2M.

- Red italic text highlights security-related properties.

Editor’s Note: More parameters may need to be transferred from MAF to MEF in the “Usage in Centralized Key Distribution Server Handshake”. This is FFS.

Editor’s Note: Reference to RFC 5705 is FFS.

Editor’s Note: Clause r.s.t referred to in above figure is a suggested clause where the appropriate location is not yet clear.

Bootstrap Credential Configuration: For this Security Bootstrap Framework, Enrolee and M2M Enrolment Function authenticate each other using a Public Key Certificate. The Bootstrap Credentials for the Enrolee and M2M Enrolment Function are pre-provisioned as described in clause 8.1.1.2.1 “Credential Configuration for Certificate-Based Security Frameworks”.

NOTE: The identities of the M2M Enrolment Function and M2M Authentication Function are assumed to have been configured prior to this phase.

Editor’s Note: The specification may need more details regarding configuring the identities of the M2M Enrolment Function and M2M Authentication Function.

Bootstrap Instruction Configuration: The Enrolee and M2M Enrolment Function  are configured with the information needed for authorizing the remote provisioning:

  - The Enrolee is configured with (or otherwise obtains) the following arguments to initiate remote provisioning:

    - The URI of the M2M Enrolment Function which will facilitate the remote provisioning, for the purpose of routing the (D)TLS exchange.

    - The M2M Enrolment Function Certificate Name (M2M Enrolment Function Certificate must include this value in a name field);

    - The M2M Enrolment Function Root of Trust (M2M Enrolment Function Certificate must be self-signed using this root of trust, or have a Certificate Chain to this root of trust);

    - The target M2M Authentication Function Identity (MAF-ID), with whom the Enrolee is to be provisioned using Security Bootstrap.

    - The Enrolee identity (Enrolee-ID): The Enrolee is to use this entity identity with the identified M2M Authentication Function.

The mechanisms for achieving this are discussed in clause x.y.z.

Editor’s Note: Clause x.y.z referred to above is a suggested clause where the appropriate location is not yet clear.

  - The M2M Enrolment Function is configured with the following arguments describing Enrolee authorized to perform Security Handshake with M2M Enrolment Function:

    - The Enrolee Certificate Name. The Enrolee Certificate must include this value in a name field.

    - The Enrolee Root of Trust. The Enrolee Certificate must be self-signed using this root of trust, or have a Certificate Chain to this root of trust.

    - The target M2M Authentication Function Identity (MAF-ID), with whom the Enrolee (authenticated using the above Enrolee Certificate Name and Enrolee Root of Trust) is to be provisioned using Security Bootstrap.

    - The Enrolee identity (Enrolee-ID). The M2M Enrolment Function is to provide this entity identity for the Enrolee with the Master Credential (Km) to the target M2M Authentication Function, when requested by the M2M Authentication Function..

The mechanisms for achieving this are discussed in clause m.n.o

Editor’s Note: Clause m.n.o referred to above is a suggested clause where the appropriate location is not yet clear.

Bootstrap Security Handshake: The Enrolee and M2M Enrolment Function perform a (D)TLS handshake as specified in TLS 1,2 RFC 5246 [16] and DTLS 1.2 RFC 6347 [17] specifications.to establish a secure session.

- Each entity (Enrolee and M2M Enrolment Function) verifies the other entity’s certificate as described in clause 8.1.1.2.2 “Certificate Verification”.

- The Enrolee and M2M Enrolment Function authenticate each other using the validated certificates as specified in TLS 1,2 RFC 5246 [16] and DTLS 1.2 RFC 6347 [17] specifications.

Editor’s note: the applicable (D)TLS ciphersuites are FFS.

Enrolment Key Generation:

- The Enrolment Key (Ke) is generated from the (D)TLS session secrets by the Enrolee and M2M Enrolment Function using TLS Key Export (RFC 5705) [18], as described in clause 8.3.6.

- The Enrolment Key Identifier (KeId) is generated by the Enrolee and M2M Enrolment Function, as described in clause r.s.t.

Editor’s Note: Clause r.s.t referred to above is a suggested clause where the appropriate location is not yet clear.

- The Enrolee and M2M Enrolment store the Enrolment Key (Ke) and Enrolment Key Identifier (KeId).

NOTE: The Enrolment Key Generation for the Pre-Provisioned Symmetric Enrolee Key Security Bootstrap Framework is identical to the Enrolment Key Generation for the Certificate-Based Security Bootstrap Framework.

Usage in Centralized Key Distribution Server Handshake: See “Overview of Security Frameworks” in Clause 8.3.1.

### 8.3.3	Centralized Security Bootstrap Framework

#### 8.3.3.1	GBA-Based Security Bootstrap Framework

To share a long term Master Credential between an Application Service/Middle Node and an M2M Authentication Function, the M2M Application Service/Middle Node shall perform a successful GBA bootstrapping and derive a NAF key (Ks_(ext/int)_NAF). This NAF key is the Master Credential.

Editor’s Note: Further text is required here.

The following text is to be used when appropriate:

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, OneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

## Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A> (Informative): Mapping of 3GPP GBA terminology

Following table provides a mapping of terminology and abbreviations used in GBA according to 3GPP specification [13] to corresponding oneM2M terminology and abbreviations as used within the present document.

| GBA entities, keys and processes | oneM2M Security Bootstrap entities, keys & processes |
| --- | --- |
| UE | Enrolee |
| BSF | MEF |
| NAF | MAF |
| Bootstrapping Procedure | Bootstrap Security Handshake + Temporary Enrolment Key Generation |
| Ks | Ke |
| B-TID | KeId |
| Bootstrapping Usage Procedure | Usage in Centralized Key Distribution Server Handshake |
| NAF FQDN | IdMAF |
| Ks_(ext/int)_NAF | Km (Master Credential) |

Annex <B>(Informative/Normative): Remove Informative or Normative as appropriateTitle of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

The following text is to be used when appropriate:

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

- <Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.0.0 | 07 Aug 2013 | Initial version agreed at SEC#4 in oneM2M-SEC-2013-0026R01-Skeleton_TS_Security_Solutions |
| V0.1.0 | 18 Oct 2013 | Incorporates following contributions agreed at SEC 7.0: oneM2M-SEC-2013-0041R01-SecurityCSF_Architecture oneM2M-SEC-2013-0044R04-Network-based_bootstrap oneM2M-SEC-2013-0050R03-PKI-Based_Post_Provisioning oneM2M-SEC-2013-0051R01-secure_remote_administration |
| V0.2.0 | 21 Jan 2014 | Incorporates following contributions agreed at SEC 8.0: oneM2M-SEC-2013-0066R01-Bootstrapping_Definition oneM2M-SEC-2013-0070R03-GBA_framework oneM2M-SEC-2013-0073R04-Security_CSF_Architecture_Figure oneM2M-SEC-2013-0075R01-Clean_up_Security_TS |
| V0.3.0 | 25 Mar 2014 | Incorporates following contributions agreed at SEC 9.0: SEC-2014-0044R03-SEC_CSF_Architecture_Figure_Update_for_Security_TS SEC-2014-0045R02-SEC_CSF_Authorization_Description_Update_for_Security_TS SEC-2014-0055R04-MAS_and_MTF_description |
| V0.4.0 | 15 Apr 2014 | Incorporates following contributions agreed at SEC 10.0: SEC-2014-0240R02-SEC_CSF_Authorization_Procedure_for_Security_TS SEC-2014-0241R03-SEC_CSF_Architecture_General_Description_for_Security_TS SEC-2014-0259R01-Credentials_definitions SEC-2014-0260R01-Credentials_abbreviations SEC-2014-0243R03-Overview_of_Security_Frameworks SEC-2014-0242R03-Certificate-Based_Security_Framework_Common_Details |
|  |  | SEC-2014-0247R03-Overview_of_Security_Bootstrap_Frameworks SEC-2014-0244R04-Certificate-Based_Security_Bootstrap_Framework SEC-2014-0252R03-Pre-Provisioned_Symmetric_Enrolee_Key_Security_Bootstrap_Framework SEC-2014-0224R07-Overview_of_Security_Association_Establishment_Frameworks SEC-2014-0227R05-Certificate-Based_Security_Association_Establishment_Framework SEC-2014-0226R05-Pre-Provisioned_Symmetric_Association_Key_Framework SEC-2014-0254R02-Precisions_on_M2M_Trust_Functions SEC-2014-0239R01-Clarifications_to_TS-0003 Following actions have been resolved and changes incorporated into this version of the document: A-WG4-TP10_006: extract table from document SEC-2014-0263-Temporary_Enrolment_Key_Terminology and add to TS 0003 as an informative Annex A-WG4-TP10_A02: make the necessary alignments regarding “Master Credential” when integrating the agreed contributions into the TS Several editorial corrections incorporated. |