| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | WG2 |
| Title:* | TS-0003 DM Remote Administration |
| Source:* | Tim Carey, ALU, timothy.carey@alcatel-lucent.com |
| Contact: | Tim Carey, ALU, timothy.carey@alcatel-lucent.com |
| Date:* | 2014-05-11 |
| Abstract:* | This contribution proposes clarifications to the Remote admin clause and introduces DM for administration of the SE and sensitive data. |
| Agenda Item:* | Security TS |
| Work item(s): | WI 00-7 |
| Document(s) Impacted* | Security TS (TS-0003) |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | Include in TS-0006 |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

-------------------------------Justification-------------------------------------------------------------------

In many devices in the field domain, the device doesn’t have a secure environment in the sense of a TEE or SIM card. In these scenarios, service providers many times utilize device management to administer the secure environment and sensitive data of the M2M Service Layer.

Note: TS-0003 suggests something called “sensitive data” without definition – need editors note to define sensitive data.

Note: I assumed SEC-2014-0276 would be agreed and included the TEE

### -----------------------Start of Change 1-------------

##### 6.3.4.1.1	Interworking with SE in 3GPP and 3GPP2

In case the SE of the underlying network is used and in case the underlying network is a 3GPP or 3GPP2 network, remote administration mechanisms as specified in [7] and [8], and its extensions [9], [10] for 3GPP underlying networks or [11] and [12] for 3GPP2 underlying networks shall be used to securely administrate both the SE and the sensitive data of the M2M Service Layer.

Editors Note: Sensitive data of the M2M Service Layer needs definition to what specifically is considered as sensitive data that is not part of the SE.

### ---------------------- End of Change 1-------------

### -----------------------Start of Change 2-------------

##### 6.3.4.1.x	tbdRemote Administration of SE Using Device Management

In case the where secure environmentSE of the M2M field domain nodes doesn’t utilize remote administration mechanisms in clauses 6.3.4.1.1 and clauses 6.3.4.1.2, Device Management  technologies [a], [b], [c] may be used to securely administrate both the SE and the sensitive data of the M2M Service Layer.

### ---------------------- End of Change 2-------------

### -----------------------Start of Changes to References Section -------------

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[a]	TR-069: “CPE WAN Management Protocol Issue”: 1 Amendment 5, November 2013, Broadband ForumBBF TR-069 CPE WAN Management Protocol Issue: 1 Amendment 4, July 2011

[b]	OMA-DM: “OMA Device Management Protocol”, Version 1.3, Open Mobile AllianceOMA-TS-DM_Protocol-V1_3 “OMA Device Management Protocol”

[c]	LWM2M: "OMA LightweightM2M”, Version 1.0, Open Mobile AllianceOMA-AD-LightweightM2M-V1_0 “OMA Lightweight Machine to Machine Architecture”

### -----------------------End of Changes to References  -------------