| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | WG4 (SEC) at TP#13 |
| Title:* | Group Authentication Use Case for TR-0012 |
| Source:* | CATT, China Unicom |
| Contact: | Wei Zhou, zhouwei@catt.cn Hui Xu, xuhui@catt.cn Changwei Hu, xhucw5@chinaunicom.cn |
| Date:* | 2014-09-22 |
| Abstract:* | Providing a group authentication use case and requirements for TR-0012. |
| Agenda Item:* | TBD |
| Work item(s): | SEC WI-0016 |
| Document(s) Impacted* | oneM2M-TR-0012 oneM2M End to End Security and Group Authentication |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | Agree for inclusion in TR-0012. |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

1	Introduction

This contribution Provides a group authentication use case and requirements for TR-0012.

2	Proposal

- 5.2.x	Use Case of Using Group Key

- 5.2.x.1	Description

A group key is a cryptographic key that is shared among members of a group. Group keys can be used for the purposes of authentication, data confidentiality and data integrity. Typically, group keys are distributed by sending them to individual members, either physically, or encrypted individually for each member using that member's pre-distributed private key. A common use of group keys is to allow a group of members to decrypt a broadcast message that is intended for that entire group of members, and no-one else.

In an oneM2M system the field domain devices may need to be organized into different groups for the purposes of management and operations. For example, in a smart building system the lights, video monitors, air conditioners, ventilation fans, fire sensors, automatic fire extinguishing equipments and fire doors may be managed in various groups. In a group the group members shall perform the same function. There may be a lot of devices in a group, for the reason of performance and efficiency, the control commands may be broadcasted to the group members instead of contacting them one by one. However, broadcasting control commands means there is no authentication between the devices and command issuer, and the commands can be received by any devices in the region of broadcasting. One solution for coping this issue is using group secret key that is share among the group members.

The use case in the following sections shows how the group keys can be used to improve system performance and efficiency.

- 5.2.x.2	Actors

The entities involved in this use case are shown in the Figure 5.2.x-1 and described as follows:

IN-CSE: It represents an Infrastructure Node that is responsible for creating groups, generating group keys and transferring group keys to group members.

ADN-AE: It represents an Application Dedicated Node that is responsible for accumulating data from fire sensors, controlling fire doors or fire extinguishing equipments which are attached to this ADN-AE.

MN-CSE: It represents a Middle Node that is responsible for controlling the entities in the Group-1, Group-2 and Group-3, and broadcasting control commands to these entities.

Group-1: It contains a set of ADN-AEs which are responsible for accumulating data from attached fire sensors.

Group-2: It contains a set of ADN-AEs which are responsible for controlling attached fire doors.

Group-3: It contains a set of ADN-AEs which are responsible for controlling attached fire extinguishing equipments.

Figure 5.2.x-1: Group key application use case

- 5.2.x.3	Pre-conditions

Every ADN-AE is provisioned with a group key that is used for encrypting and/or decrypting group messages.

MN-CSE is provisioned with all the group keys that are used for encrypting and/or decrypting group messages.

ADN-AEs have registered to the MN-CSE in order to interact with each other. ADN-AEs have the public key certificate of MN-CSE in order to authenticate the control commands generated by it.

ADN-AEs, MN-CSEs are all issued with a public key certificate that can be used for authentication and data integrity protection. These entities are all pre-provisioned with a CA certificate that is used to verify other public key certificates.

- 5.2.x.4	Normal Flow

Group key using procedure:

- A fire sensor detects a fire and sends this signal to an ADN-AE. The ADN-AE generates an alert message that is signed with the private key corresponding to a public key certificate, and then encrypts the signed message with the group key Key-1. This signed and encrypted message is sent to the MN-CSE.

- The MN-CSE decrypts the encrypted message with the group key Key-1 and then verifies the signed message with this AND-AE’s public key certificate.

- In the case of the message is valid, the MN-CSE generate two control commands. One command asks the ADN-AEs in the Group-2 to unlock the fire doors; another command asks the AND-AEs in the Group-3 to switch on the fire extinguishing equipments. All these commands are signed with MN-CSE’s private key. The command for the Group-2 is encrypted with the group key Key-2 and the command for the Group-3 is encrypted with the group key Key-3. The MN-CSE then broadcasts these commands.

- All the AND-AEs belonging to the Group-1, Group-2 and Group-3 can receive these messages. However, only the ADN-AEs in the Group-2 can decrypt the message encrypted with the Key-2 and then verify the signed message with MN-CSE’s public key certificate, and only the ADN-AEs in the Group-3 can decrypt the message encrypted with the Key-3 and then verify the signed message with MN-CSE’s public key certificate.

- After the received messages are decrypted and verified, the ADN-AEs will perform the operations according to the received commands, i.e. the AND-AEs in the Group-2 unlock the fire doors, and the AND-AEs in the Group-3 switch on the fire extinguishing equipments.

- 5.2.x.5	Potential requirements

- A group key shall be used by a group of members to encrypt/decrypt a broadcast message that is intended for that entire group of members, and no-one else.

- An security mechanism shall be provided to ensure that an encrypted message can be authenticated by the group members.