| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | SEC#13 |
| Title:* | Group Authentication Use Case |
| Source:* | China Mobile |
| Contact: | Ziyao Cheng, China Mobile, chengziyao@chinamobile.com Minpeng Qi, China Mobile, qiminpeng@chinamobile.com |
| Date:* | 2014-09-22 |
| Abstract:* | This contribution presents the description of group authentication use case for the security TR-0012. |
| Agenda Item:* |  |
| Work item(s): | SEC WI-0016 |
| Document(s) Impacted* | Security TR (TS-0012) |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | Incorporate text into an appropriate section of the Security TR-0012. |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

==================<Start of Change (New Text )>=================

# 5	Use Cases

## 5.2	Group Authentication Use Cases

Editor’s note: this clause includes use cases for group authentication and threat analysis of the use cases.

For some oneM2M service, it may be a large amount of entities (e.g., AEs/CSEs) deployed in the same location to execute procedures for the same purpose and own same behaviour. Therefore, these oneM2M entities can be worked together as a group. The use cases to meet the above description are concluded as follows:

Smart Meter reading: A large amount of smart meters are deployed in a block. The smart meter uploads meter report frequently through the network to Utility Data Center / AMI Headend. What is more, the Utility Data Center / AMI Headend can make a request to the smart meter that can be received via the agent (i.e., Data Aggregation Point), smart meters report at the same time, or the Utility Data Center/AMI Head End need to re-configure all smart meters at the same time.

Remote Vehicle Management: Vehicles equipped with IOT terminals which contain GPS location reporter, remote air condition control, etc. to send information such as position information, navigation, remote diagnosis, on-board communication, news and entertainment information to the M2M service provider to make better vehicle scheduling, vehicle monitoring and vehicle controlling at the same time. Meanwhile, such vehicles gather in a small place as a group like airport, train station, etc., and the communication from vehicles to M2M service provider via an agent (e.g., Remote Vehicle Gateway).

==================<End of Change >========================