| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | SEC#13 |
| Title:* | Group Authentication Use Case |
| Source:* | China Mobile |
| Contact: | Ziyao Cheng, China Mobile, chengziyao@chinamobile.com Minpeng Qi, China Mobile, qiminpeng@chinamobile.com |
| Date:* | 2014-09-24 |
| Abstract:* | This contribution presents the description of group authentication use case for the security TR-0012. |
| Agenda Item:* |  |
| Work item(s): | SEC WI-0016 |
| Document(s) Impacted* | Security TR (TS-0012) |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | Incorporate text into an appropriate section of the Security TR-0012. |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

==================<Start of Change (New Text 1)>=================

## 3.4	Acronyms

For the purposes of the present document, the abbreviations given in [i.2] and the following apply:

AMI	Advanced Metering Infrastructure

DAP	Data Aggregation Point

==================<End of Change 1>========================

==================<Start of Change (New Text 2)>=================

# 5	Use Cases

## 5.2	Group Authentication Use Cases

Editor’s note: this clause includes use cases for group authentication and threat analysis of the use cases.

For some oneM2M service, it can be a large amount of entities (e.g., AEs/CSEs) deployed in the same location to execute procedures for the same purpose and own same behaviour. Therefore, these oneM2M entities can be worked together as a group. The use cases to meet the above description are concluded as follows:

- 5.2.1	Smart Meter Reading

- 5.2.1.1	Description

A large amount of smart meters are deployed in a block. The smart meter uploads meter report frequently through the network to Utility Data Center / AMI Headend. What is more, the Utility Data Center / AMI Headend can make a request to the smart meter that can be received via the agent (e.g., Data Aggregation Point), smart meters report at the same time, or the Utility Data Center/AMI Head End need to re-configure all smart meters at the same time.

- 5.2.1.2	Actors

The entities involved in this use case are shown in the Figure 5.2.1.2-1 and described as follows:

Figure 5.2.1.2-1: Entities Involved for Smart Metering Reading

Smart meter: it (e.g., ASN or ADN) plays role of specific group member to forward meter report via the group agent to Utility Data Center/AMI Hended.

DAP (Data Aggregation Point): it (e.g., MN) plays role of group agent which is responsible to delegate all members to make mutual authentication with Utility Data Center/AMI Hended.

Utility Data Center/AMI Hended: it (e.g., IN) belongs to the service provider to obtain meter reports from the smart meters.

- 5.2.1.3	Pre-conditions

The smart meters are designed in the group as a whole initially.

- 5.2.1.4	Normal flow

- Smart meters make a request to Utility Data Center / AMI HeadEnd for uploading information in a group via the DAP.

- DAP needs to confirm smart meters have rights to upload information before it forward request to the Utility Data Center/ AMI HeadEnd.

- Smart meters in a group complete the requests and send their information to the group agent. Moreover, most smart meters would send information at same time by using default configuration, or be configured at the same time based on end user employee preference.

- DAP can subsequently buffer the information for some time, or may serve as a router to directly forward the smart meters information to Utility Data Center / AMI HeadEnd.

- Utility Data Center / AMI HeadEnd deal with the information after it confirms smart meters rights.

- 5.2.1.5	Potential requirements

- Each smart meter in the group and Utility Data Center/AMI Hended (i.e., Infrastructure Node) can make implicit authentication with each other.

- Each smart meter in the group negotiates with the Utility Data Center/AMI Hended (i.e., Infrastructure Node) to generate a specific session key for data integrity protection and data confidentiality protection.

- 5.2.2	Remote Vehicle Management

- 5.2.2.1	Description

Vehicles equipped with IOT terminals which contain GPS location reporter, remote air condition control, etc. to send information such as position information, navigation, remote diagnosis, on-board communication, news and entertainment information to the vehicle service centre to make better vehicle scheduling, vehicle monitoring and vehicle controlling at the same time. Meanwhile, such vehicles gather in a small place as a group which either be static or dynamic one like airport, train station, etc., and the communication from vehicles to Vehicle Service Centre via an agent (e.g., Remote Vehicle Gateway).

5.2.2.2	Actors

The entities involved in this use case are shown in the Figure 5.2.2.3-1 and described as follows:

Figure 5.2.2.3-1: Entities Involved for Remote Vehicle Management

Vehicle: it (e.g., ASN or ADN) plays role of specific group member to forward meter report via the group agent to Vehicle Service Centre.

Remote Vehicle Gateway: it (e.g., MN) plays role of group agent which is responsible to delegate all members to make mutual authentication with Vehicle Service Centre.

Vehicle Service Centre: it (e.g., IN) belongs to the service provider to obtain the collected information from vehicles for controlling.

- 5.2.2.3	Pre-conditions

The vehicles can either be designed as a group in advance or dynamically changed before the group authentication execution.

      - Normal flow

- The vehicles in a group collect the information (e.g., position information, navigation, remote diagnosis, etc. ) from IOT terminals and sends it to the Remote Vehicle Gateway at the same time.

- The Remote Vehicle Gateway sends the received information to the Vehicle Service Centre after it verifies that vehicles have the right to do so.

- Vehicle Service Centre firstly verifies vehicles identities, then analyses the information and send the response (e.g., management of vehicle scheduling, vehicle monitoring and vehicle controlling, etc.) to the vehicles.

- 5.2.2.5	Potential requirements

- Each vehicle in the group and Vehicle Service Centre (i.e., Infrastructure Node) can make implicit authentication with each other.

- Each vehicle in the group negotiates with the Vehicle Service Centre (i.e., Infrastructure Node) to generate a specific session key for data integrity protection and data confidentiality protection.

==================<End of Change 2>========================