| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | ARC/SEC |
| Title:* | AE-ID and AE-Registration |
| Source:* | Qualcomm Inc. |
| Contact: | Josef Blanz (jblanz@qti.qualcomm.com ) |
| Date:* | 2014-10-29 |
| Abstract:* | Discussion contribution to clarify flows when AEs are registering and how AE-ID are assigned. |
| Agenda Item:* |  |
| Work item(s): |  |
| Document(s) Impacted* |  |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

## Scenarios where a specific AE (Application Instance) is authenticated in SAEF

| Phase/Step/Process |  |  |  | Security Association Establishment Framework Type |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Certificate Based |  | Provisioned PSK (PSK[C] shared by AE and Registrar) | MAF-Based (PSK[D] shared by AE and MAF) |
|  |  |  |  | Certificate includes Sxxx AE-ID and App-ID attributes[A] | Certificate doesn’t include AE-ID[B] |  |  |
| 1. Provisioning |  | Information provisioned[1] to Entities |  | AE: Cert Cert with AE-ID is associated with SS Profile and | AE: Cert | AE+Registrar: PSK + PSK-ID | AE+MAF: PSK + PSK-ID |
|  |  | Subscriber[2] knows[3] |  | AE ↔ Sxxx AE-ID | AE ↔ Cert | AE ↔ PSK-ID |  |
| 2. Information[6] to be placed in M2M SS Profile |  |  |  | AE-ID not looked up during AE registration but AE-ID would need to be associated with a specific service subscription profile | Cert-ID & (AE-ID Type or specific AE-ID) | PSK-ID & (AE-ID Type or specific AE-ID) |  |
| 3. Security Association Establishment. Registrar authenticates this credential identifier |  |  |  | Sxxx AE-ID in Cert | Cert-ID | PSK-ID |  |
| 4. AE sends AE Registration Request |  |  |  | - If AE has never registered before, then it does not provide an AE-ID - If AE has registered before, then AE may provide a previously issued AE-ID |  |  |  |
| 5. Registrar obtains AE-related attributes and optionally other info |  |  |  | Retrieve AE-related attributes from Certificate | Retrieve AE-related attributes from M2M SS Profile[8] using credential identifier (Cert-ID or PSK-ID plus optionally AE-ID) |  |  |
|  |  |  |  | Registrar verifies that <AE> resource in AE Registration Request agrees with retrieved AE-related attributes |  |  |  |
| 6.a If AE-ID Type == Sxxx | If AE provided Sxxx AE-ID |  | Registrar | Sends UPDATE <AEAnnc> Request to IN-CSE including Cert-ID or PSK-ID in labels attribute. |  |  |  |
|  |  |  | IN-CSE | Updates <AEAnnc> |  |  |  |
|  | If AE did not provide Sxxx AE-ID |  | Registrar | Sends CREATE <AEAnnc> Request to IN-CSE including Cert-ID or PSK-ID in labels attribute. |  |  |  |
|  |  |  | IN-CSE | - Selects Sxxx AE-ID and creates <AEAnnc> resource - Returns created <AEAnnc> Response, including Sxxx AE-ID |  |  |  |
| 6.b. If AE-ID Type == Cxxx | If AE provided Cxxx AE-ID |  |  | Not Applicable | If this Cxxx AE-ID is still available, then Registrar re-assigns this Cxxx AE-ID to AE, otherwise returns fail. |  |  |
|  | If AE did not provide Cxxx AE-ID |  |  |  | Registrar selects an available Cxxx AE-ID |  |  |
| 7. Registrar sends AE Registration Response |  |  |  | - If successful, response includes assigned Cxxx AE-ID or Sxxx AE-ID - If not successful, AE returns to Step 4 “AE sends AE Registration Request” |  |  |  |

## Scenarios where SAEF authenticates the Application Software but does not authenticate a specific AE (Application Instance).

The authentication may or may not allow determining the specific Device/Node executing the Application.

| Phase/Step/Process |  |  |  | Security Association Establishment Framework Type |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Certificate Based |  | Provisioned PSK (PSK[C] shared by App and Registrar) | MAF-Based (PSK[D] shared by App and MAF) |
|  |  |  |  | Certificate indicates AE-ID Type[B] | Certificate doesn’t indicate AE-ID-Type[B] |  |  |
| 1. Provisioning |  | Information provisioned[1] to Entities |  | App: Cert | App: Cert | App+Registrar: PSK + PSK-ID | App+MAF: PSK + PSK-ID |
|  |  | Subscriber[2] knows[3] |  | App ↔ Cert | App ↔ Cert | App ↔ PSK-ID |  |
| 2. Information[6] to be placed in M2M SS Profile |  |  |  | Cert-ID & (AE-ID Type or AE-ID list) | Cert-ID & (AE-ID Type or AE-ID list) | PSK-ID & (AE-ID Type or AE-ID list) |  |
| 3. Security Association Establishment. Registrar authenticates this credential identifier |  |  |  | Cert-ID, AE-ID Type[B] | Cert-ID[B] | PSK-ID |  |
| 4. AE sends AE Registration Request |  |  |  | - If AE has never registered before, then it does not provide an AE-ID - If AE has registered before, then AE provides previously issued AE-ID |  |  |  |
| 5. Registrar obtains App-related attributes and optionally other info |  |  |  | Retrieve App-related attributes from Certificate | Retrieve App-related attributes from M2M SS Profile[8] using credential identifier (Cert-ID or PSK-ID) |  |  |
|  |  |  |  | Registrar verifies that <AE> resource in AE Registration Request agrees with retrieved App-related attributes |  |  |  |
| 6.a If AE-ID Type == Sxxx | If AE provided Sxxx AE-ID |  | Registrar | Sends UPDATE <AEAnnc> Request to IN-CSE including credential identifier |  |  |  |
|  |  |  | IN-CSE | - Checks if <AEAnnc> exists and mandatory attributes match M2M SS Profile - Returns updated <AEAnnc> |  |  |  |
|  | If AE did not provide Sxxx AE-ID |  | Registrar | Sends CREATE <AEAnnc> Request to IN-CSE including credential identifier |  |  |  |
|  |  |  | IN-CSE | - Checks mandatory attributes match M2M SS Profile - Selects Sxxx AE-ID and creates <AEAnnc> resource - Returns created <AEAnnc> Response, including Sxxx AE-ID |  |  |  |
| 6.b. If AE-ID Type == Cxxx | If AE provided Cxxx AE-ID |  |  | If this Cxxx AE-ID is still available, then Registrar re-assigns this Cxxx AE-ID to the AE, otherwise returns fail. |  |  |  |
|  | If AE did not provide Cxxx AE-ID |  |  | Registrar selects an available Cxxx AE-ID |  |  |  |
| 7. Registrar sends AE Registration Response |  |  |  | - If successful, response includes assigned Cxxx AE-ID or Sxxx AE-ID - If not successful, AE can returns to Step 4 “AE sends AE Registration Request” |  |  |  |

## Scenarios where a specific Node/Device (but not a specific Application nor AE) is authenticated in SAEF

| Phase/Step/Process |  |  |  | Security Association Establishment Framework Type |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Certificate Based |  | Provisioned PSK (PSK[C] shared by Device/Node and Registrar) |  | MAF-Based (PSK[D] shared by Device/Node and MAF) |
|  |  |  |  | Certificate includes list of (App-ID, AE-ID Type) [B] | Certificate doesn’t include list of (App-ID, AE-ID Type) [B] |  |  |  |
| 1. Provisioning |  | Information provisioned[1] to Entities |  | Device/Node: Cert | Device/Node: Cert | Device/Node +Registrar: PSK + PSK-ID |  | Device/Node +MAF: PSK + PSK-ID |
|  |  | Subscriber[2] knows[3] |  | Device/Node ↔ Cert |  | Device/Node ↔ PSK-ID |  |  |
| 2. Information[6] for M2M SS Profile |  |  |  | Cert-ID & List of (App-ID & [AE-ID Type or AE-ID list]) | Cert-ID & list of (App-ID, [AE-ID Type] or [AE-ID list]) | PSK-ID & list of (App-ID, [AE-ID Type] or [AE-ID list]) |  |  |
| 3. Security Association Establishment. Registrar authenticates this identifier |  |  |  | (App-ID, AE-ID Type) list [B] | Cert-ID[B] | PSK-ID | PSK-ID |  |
| 4. AE sends AE Registration Request |  |  |  | - If AE has never registered before, then it does not provide an AE-ID - If AE has registered before, then AE provides previously issued AE-ID |  |  |  |  |
| 5. Registrar obtains Device/Node-related attributes and optionally other info |  |  |  | Retrieve Device/Node-related attributes from Certificate | Retrieve Device/Node-related attributes from M2M SS Profile[8] using credential identifier (Cert-ID or PSK-ID) |  |  |  |
|  |  |  |  | Registrar verifies that <AE> resource in AE Registration Request agrees with retrieved Device/Node-related attributes |  |  |  |  |
| 6.a If AE-ID Type == Sxxx | If AE provided Sxxx AE-ID |  | Registrar | Sends UPDATE <AEAnnc> Request to IN-CSE including credential identifier |  |  |  |  |
|  |  |  | IN-CSE | - Checks if <AEAnnc> exists and critical parameters match M2M SSP - Returns updated <AEAnnc> |  |  |  |  |
|  | If AE did not provide Sxxx AE-ID |  | Registrar | Sends CREATE <AEAnnc> Request to IN-CSE including credential identifier |  |  |  |  |
|  |  |  | IN-CSE | - Selects Sxxx AE-ID and creates <AEAnnc> resource - Returns created <AEAnnc> Response, including Sxxx AE-ID |  |  |  |  |
| 6.b. If AE-ID Type == Cxxx | If AE provided Cxxx AE-ID |  |  | If this Cxxx AE-ID is still available, then Registrar re-assigns this Cxxx AE-ID to the AE, otherwise returns fail. |  |  |  |  |
|  | If AE did not provide Cxxx AE-ID |  |  | Registrar selects an available Cxxx AE-ID |  |  |  |  |
| 7. Registrar sends AE Registration Response |  |  |  | - If successful, response includes assigned Cxxx AE-ID or Sxxx AE-ID - If not successful, AE can returns to Step 4 “AE sends AE Registration Request” |  |  |  |  |

Authentication-Mechanism-Independent notes

- Provisioning may constitute pre-provisioning or remote provisioning. This row includes information

- Subscriber is an abbreviation for “M2M Service Subscriber”.

- “[Thing] ↔ [Credential identifier]” should be interpreted as “The M2M Service Subscriber can be aware which [Thing] is authenticated by the credential with this [Credential identifier]”, where [Thing] may be a device, node, application or AE.  This has two purposes

  - The M2M Service Subscriber can subsequently use this information to enroll the [Thing] in the M2M Service Subscriber’s M2M Service Subscription Profile, and

  - The M2M Service Subscriber can subsequently use this information to identify registered AEs as follows:

    - If [Thing] is a Device/Node: Which registered AE(s) are hosted on that Device/Node associated with that [Credential Identifier].

    - If [Thing] is an Application: Which registered AE(s) is/are an instance of that Application associated with that [Credential Identifier].

    - If [Thing} is an AE: Which registered AE corresponds to the AE associated with that [Credential Identifier].

- VOID.

- VOID.

- The M2M Service Subscriber may provide other information (in addition to the information shown here) to the M2M SP for inclusion in the M2M Service Subscription Profile.

- “AE-ID Type” indicates whether the AE(s) should be use an M2M SP assigned Sxxx AE-ID or a M2M SP assigned Cxxx AE-ID.

- The information in a M2M Service Subscription Profile resource may arrive at the Registrar in the following way

  - Configuring the M2M Service Subscription Profile to the AE or App or Node/Device.

    - The M2M Service Subscription Profile resource is signed by the M2M SP

    - The signed M2M Service Subscription Profile resource is configured to the AE or AE Host (e.g. pre-configured or configured via remote entity management procedures).

    - The AE then provides the signed M2M Service Subscription Profile resource to the Registrar in the AE Registration Request.

    - The Registrar verifies the M2M SP’s signature. The Registrar now has a trusted copy of the M2M Service Subscription Profile.

  - Retrieving the M2M Service Subscription Profile from IN-CSE

    - The Registrar sends filtered RETRIEVE <m2mServiceSubscriptionProfile> request to the IN-CSE using the Credential Identifier as the filter criteria.

    - The IN-CSE determines the <m2mServiceSubscriptionProfile> resource including the Credential Identifier.

    - The IN-CSE returns the  <m2mServiceSubscriptionProfile> resource Content

    - The Registrar may subscribe to this M2M Service Subscription Profile resource to be notified of subsequent changes.

  - Configuring the M2M Service Subscription Profile to the Registrar. The M2M Service Subscription Profile resource is configured to the Registrar (e.g. pre-configured or configured via remote entity management procedures).

  - Caching. The Registrar may cache the M2M Service Subscription Profile resource when it receives it via one of the above mechanisms, for subsequent reference. The details for caching of the M2M Service Subscription Profile resource are implementation specific.

Authentication-Mechanism-specific notes

- This certificate contains the Sxxx AE-ID and all other information from the M2M Service Subscription Profile relevant to the AE. This case assumes that the M2M SP and M2M Service Subscriber agree on this information before the provisioning of the Certificate including an Sxxx AE-ID. In this case, the Registrar does not reference the M2M Service Subscription Profile.

- The certificate may also include other attributes of the thing (AE, App, Device/Node) to which the certificate is issued.

- The PSK and corresponding PSK-ID in the case of the Provisioned PSK SAEF are called Kpsa and KpsaId correspondingly. We have left these details out of the table to avoid confusion.

- The PSK and corresponding PSK-ID in the case of the MAF-Based SAEF are called Km and KmId correspondingly. We have left these details out of the Below are samples of the styles to be used in this document