| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | Technical Plenary (TP) |
| Title:* | Smart Grid Activities in North America |
| Source:* | Qualcomm Incorporated (TIA) |
| Contact: | Farrokh Khatibi, Qualcomm, fkhatibi@qti.qualcomm.com Dilip Krishnaswamy, Qualcomm, dilipk@qti.qualcomm.com Yenamandra Rao, Qualcomm, ysrao@qti.qualcomm.com Josef Blanz, Qualcomm, jblanz@qti.qualcomm.com Mark Klerer, Qualcomm, mklerer@qualcomm.com |
| Date:* | 2013-02-15 |
| Abstract:* | This contribution provides selected Smart Grid use cases extracted from the work done in Smart Grid Interoperability Panel (SGIP) and OpenSG users group. The purpose of this contribution is to highlight the vast amount of work that is readily available to characterize SG use cases. |
| Agenda Item:* | 7 |
| Work item(s): | N/A |
| Document(s) Impacted* | N/A |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | It is recommended that oneM2M and SGIP establish a liaison relationship to enable the exchange of information between the two organizations. |

oneM2M IPR STATEMENT

Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by all provisions of IPR policy of the admitting Partner Type 1 and permission that all communications and statements, oral or written, or other information disclosed or presented, and any translation or derivative thereof, may without compensation, and to the extent such participant or attendee may legally and freely grant such copyright rights, be distributed, published, and posted on oneM2M’s web site, in whole or in part, on a non-exclusive basis by oneM2M or oneM2M Partners Type 1 or their licensees or assignees, or as oneM2M SC directs.

Smart Grid Activities in North America

A number of organizations have been working to define use cases for Smart Grid (SG). Smart Grid Interoperability Panel (SGIP) and OpenSG users group have been leading the effort in North America.

In this contribution, we will introduce the work being done in these groups, and then focus our attention on a smaller subset of use cases that we see relevant to oneM2M.

The SG-NET Diagram

Figure 1 shows the conceptual actors/data flow diagram developed by SG-Net. In this figure, each element is an “actor” that is communicating with another actor using the shown data flows. As an example, consider “Smart Meter” in the “Customer” quadrant (lower right). Smart Meter (SM) communicates with a number of other actors, such as a Data Aggregation Point (DAP) located in the AMI Network. The DAP can then transmit the aggregated data to the Service Provider using the Wide Area Network.

Figure 1- Conceptual Actors/Data Flow Diagram

## The SGIP/OpenSG Requirement Specification

A requirement specification is being developed in SGIP/OpenSG that describes the characteristics of the communication between all the relevant actors in Figure  1. This tool is in the form of a spreadsheet that currently has 7855 rows (actor-actor flows). Figure 2 shows a few rows of this table. Please note that some information has been hidden to make it possible to view the table. The complete table can be found here:

http://osgug.ucaiug.org/UtiliComm/Shared%20Documents/Latest_Release_Deliverables/SG%20Network%20System%20Requirements%20Specification%20v5.1.xls

As one can see from the heading row of the table shown in Figure 2, the tool provides some relevant data such as the hours of operation, the frequency of data transmission, the reliability and latency requirement for the transmission, security, confidentiality, and availability of the data, and the size of the packets.

To take advantage of this table, one can use the headers to filter for the appropriate values. For example, let us consider the use case of Meter Reading from a 2-way electric meter to a Data Aggregation Point (DAP). For this example, one can select the following setting:

- Data Flow from Actor =  2-Way Meter Electr

- Data Flow to Actor = DAP

- Use Case Ref = Meter Reading

Figure 2 - System Characteristics Requirement

Figure 3 - Primary Focus of our Use Cases

## Use Case Analysis

In this section we will consider only a few use cases that are potentially relevant to oneM2M. In specific, we will consider use cases that are from SM focused on key actors in the Consumer Domain as shown in Figure 3. These include:

Meter Reading flows

There are over 454 flows in the Meter Reading use case involving a large number of actors. However, only 7 flows are between the 2-Way Electric Meter and DAP. There are also 7 flows between 2-way Electric Meter and actors other than DAP. See tables below.

Meter Event flows

There are over 123 meter event flows.  However, there are only 7 flows that terminate in DAP.  See table below.

EV/PHEV flows

There are 120 flows in the PHEV use case; however, there are only 7 flows between the AMI Head-End and DAP or DMS. See table below.

Note that the EIS Alliance has a comprehensive summary of PEV charging station (EVSE and CEMS) flows in a single summary figure.

Pre-Pay flows

There are 650 flows related to the Pre-Pay use case. Only 20 of them are initiated by 2-Way Elect Meters. Of those 10 of them terminate in DAP. See table below.

DR/DLC flows

There are 24 flows for the DR/DLC use case. Only 8 of them terminate at DAP. See table below.

Premise Network Administration

There are 544 flows associated with the Premise Network Administration use case. Only 10 of them are between 2-Way Electric Meter and DAP. See table below.

Firmware / Program Update

There are 1037 flows associated with the Firmware/program update use case. 8 of these flows are between 2-Way Electric Meter and DAP.  See table below.

Use Case Meter Reading. Data Flow from “2-Way Meter – Electr *” to “DAP”.

Use Case Meter Reading. Data Flow from “2-Way Meter – Electr *” to “all BUT DAP”.

Use Case Meter Event. Data Flow from “2-Way Meter – Electr *” to “DAP”

Use Case PHEV. Data Flow from “AMI Head-End” to “DAP” or “DMS”.

Use Case Pre-Pay. Data Flow from “2-Way Meter – Electr” to “DAP”.

Use Case DR/DLC. Data Flow from “2-Way Meter – Electr” to “DAP”.

Use Case Premise Network Admin. Data Flow from “2-Way Meter – Electr” to “DAP”.

Use Case = Firmware/Program update. Data Flow from “2-Way Meter – Electr” to “DAP”.

## Conclusion

The purpose of this contribution is to highlight the vast amount of work that is readily available to characterize SG use cases. This contribution is not meant to provide an exhaustive list of all the use cases and their related flows; the details of each use case and the associated requirements will be submitted in separate contribution(s).

Based on the information provided, it is recommended that oneM2M and SGIP establish a liaison relationship to enable the exchange of information between the two organizations.

Source: OpenSG users group http://osgug.ucaiug.org/UtiliComm/Shared%20Documents/Latest_Release_Deliverables/Diagrams/