| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | TP#4 |
| Title:* | M2M Device Classification Discussion |
| Source:* | Phil Jacobs; Cisco Systems; phjacobs@cisco.com Gale Lightfoot; Cisco Systems; lightfg@cisco.com Ming-Yee Lai, Applied Communication Sciences; mlai@appcomsci.com Hongbeom Ahn, LGE, hongbeom.ahn@lge.com |
| Contact: | Same as Above |
| Date:* | 2013-04-08 |
| Abstract:* | Our contribution is meant to motivate discussions relative to M2M Device Classification. Additionally, we recommend that Device Classification be based on functionality and a coarse set of the technology elements currently leveraged in M2M services around the world |
| Agenda Item:* | Architecture Discussion |
| Work item(s): | Relative to New Work Item: Device/Gateway Classification |
| Document(s) Impacted* | New Work Item: Device/Gateway Classification |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | There is a New Work Item being suggested by the incoming contribution to TP#4 (“New Work Item: Device/Gateway Classification”) that this contribution is addressing. |

oneM2M IPR STATEMENT

Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by all provisions of IPR policy of the admitting Partner Type 1 and permission that all communications and statements, oral or written, or other information disclosed or presented, and any translation or derivative thereof, may without compensation, and to the extent such participant or attendee may legally and freely grant such copyright rights, be distributed, published, and posted on oneM2M’s web site, in whole or in part, on a non-exclusive basis by oneM2M or oneM2M Partners Type 1 or their licensees or assignees, or as oneM2M SC directs.

1.0	Introduction

Since M2M devices will have significantly different available resources and capabilities, it is clear that not all devices will be able to meet all oneM2M requirements or support all oneM2M services. A framework and method to classify devices would be helpful to describe device constraints, clarify and simplify the M2M system, and optimize M2M communications. Furthermore, specific requirements driven by device constraints will be needed in order for an M2M system to support them.

2.0 	Candidate profile elements

As a first step, this contribution presents 16 candidate profile elements (not intended to be all inclusive list) by which an M2M device could be classified. An initial examination of their impact on an M2M system are included.

| Candidate profile elements | Example Impact on M2M system | Notes |
| --- | --- | --- |
| Wireless wide area network interface | Might indicate gateway candidate. Might indicate mobility. | e.g. 3G, 4G, WiMAX, GPRS, satellite OMA[1] |
| Wireless Local Area Network Interface | Might indicate gateway candidate. Might indicate mobility. | e.g. Wi-Fi, Bluetooth, Zigbee, DSRC OMA[1] |
| Fixed Wide Area Network Interface | Might indicate gateway candidate | e.g. ADSL, FTTH, PLC OMA[1] |
| Fixed Local Area Network Interface | Might indicate gateway candidate | e.g. Ethernet, USB, RS232, TIA485 OMA[1] |
| IP Stack | Input information for transport protocol choice | No IP, IP-UDP, IP-UDP+TCP, OMA[1] |
| User Input/Output | Might be useful for device replacement | e.g. keypad, touch screen, speaker OMA[1] |
| Persistent Configuration Storage | Can hold programs and state | e.g. SIM card, eSIM, flash, micro SD card OMA[1] |
| Intermittently connected | A command might be necessary to repeat after a longer timeout or the timeout value with which one endpoint waits on a response needs to be sufficiently high. | e.g. out of radio range, interference impacted, powered-down, resources running other application IETF[1] |
| Bandwidth constrained | If the available bandwidth is insufficient, packets will be buffered and eventually dropped, applications using certain protocols will not run. The client and server of a management protocol need to gracefully ignore incomplete commands or repeat commands as necessary. Compression may be needed. | e.g. poor radio connection, shared BW with many other devices or applications IETF[1] |
| Power constrained | Might only be able to support a limited operating time (e.g. Based on the available battery). The devices need to economize their energy usage with suitable mechanisms and the managing entity needs to monitor and control the energy status of the constrained devices it manages. | Power-Affluent nodes – mains-powered or guaranteed regularly recharged battery. Power-constrained nodes – constrained by non-rechargeable battery, or energy harvesting. IETF [3] e.g. solar or wave powered, long required lifecycle IETF[1] |
| Memory constrained | Unlikely to be able to communicate directly and securely with Internet. Likely to use larger proxy devices. Pre-configuration likely. Might only be able to support one simple communication protocol. Might only be able to parse small data models. Can only hold minimal state. No IP | C0 data size e.g. RAM <<10Kb Code size e.g. flash << 100Kb. IETF[2] |
|  | Cannot easily talk securely using full protocol stack with multiple protocols. Can talk without gateway. Must limit functions to niche. Might only be able to communicate with a limited number of entities. IP-UDP (+CoAP) | C1 data size e.g. RAM ~10Kb Code size e.g. flash ~100Kb. IETF[2] |
|  | Benefit from lightweight protocols to allow more resource for applications. IP-UDP/TCP (with lightweight and/or energy-efficient) | C2 data size e.g. RAM ~50Kb Code size e.g. flash ~ 250Kb. IETF[2] |
|  | Rich devices with sufficient memory. Can use any protocols. IP-UDP/TCP (w/out lightweight and/or energy efficient) | C3 (Beyond C2) data size e.g. RAM over 50KKb. Code size e.g. flash over 250 Kb. |
| CPU constrained | Might not be able to support compression/decompression of exchanged data, and need an proxy to handle these types of functions | e.g. due to cost, age, power, heat dissipation, shared applications IETF[1] |
| Mobile | Unreliable topological-based devices associations. Optimise mobile functions when fixed. | e.g. not mobile, geo region limited, unlimited |
| Risk to humans | Stronger security and reliability | Risks of death, injury, fire, financial, embarrassment |
| Usage frequency | One time IP address, reliable communications, device management | Single use device, hourly, daily, yearly, etc. |
| Physically accessible | No reset or power-cycle. Remote access only. | e.g. mountain-top, sea-bottom, human implant |

Due to correlation among some device elements, certain elements may be predictive of others e.g. a wired network connection may infer a good power supply.

- OMA[1] refers to OMA-WP-M2M_Device_Classification-20121030-A

- IETF[1] refers to  Management of Networks with Constrained Devices: Problem Statement, Use Cases and Requirements, informational draft http://tools.ietf.org/html/draft-ersue-constrained-mgmt-03

- IETF[2] refers to Terminology for Constrained Node Networks, informational draft http://tools.ietf.org/html/draft-bormann-lwig-terms-00#section-3

- IETF[3] refers to Problem Statement and Requirements for IPv6 over Low-Power Wireless Personal Area Network (6LoWPAN) Routing, informational draft http://tools.ietf.org/html/rfc6606

3.0 	Example device classifications

Next we pick 2 aspects from the candidate profile elements identified above, and consider how 4 device classifications might map to applications and how they might impact an M2M system.  For the basis of discussion, we have used the 2 WAN profiles, and the 2 LAN profiles from which to base a coarse Device Classification, with associations of the likely applications for each. We suggest that the four quadrants in the figure below could represent 4 major Device Classifications for M2M services/application enablement.

Fixed

Mobile

1

2

Applications

Car automation

eHealth

Logistics

Portable Consumer Electronics

| WAN Applications Smart Grid, Meter, City Remote Monitoring Device Capabilities PSTN Broadband 3 | WAN 4 Device Capabilities 2G/3G/4G Power line communication 2G/3G/4G Satellite |
| --- | --- |
| Fixed LAN Device Capabilities Wireless Personal Area Networks Wired networks Indoor electrical wiring Applications Smart Home Factory Automation eHealth | Mobile LAN Applications On-site logistics Fixed Mobile Device Capabilities Wi-Fi, WPAN, Fixed Mobile |

In the diagram above we suggest 4 Device Classification from which to initiate a discussion (1, 2, 3, and 4).

Once such a coarse classification of devices is made, then we suggest further discussion to include a minimal set of elements from the table is Section 2.0 that would be key to the chosen classification of devices.  The examples we suggest in the 4 quadrants are a basis for discussion.  Others may be chosen.  We are only trying to make a point of how a coarse set of device classifications could be made.

It is important to identify Device Classifications based on functional and technical parameters.  This is a very difficult task, but one that is necessary for most efficient means to develop/manufacture the many M2M enabling devices, and needed for the successful deployment of M2M services/applications across the plethora of these devices.

Our contribution is meant to motivate discussions relative to M2M Device Classification.  Additionally, we recommend that Device Classification be based on functionality and a coarse set of the technology elements currently leveraged in M2M services around the world.

We suggest that oneM2M consider the classification of devices at a level of granularity that will turn an overabundance of data into intelligible bits of useful information for the overall M2M system.