| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #66 |
| Source:* | Mohd Uvaish Siddiqui, C-DOT, uvaish@cdot.in Prateek Varshney, C-DOT, prateekv@cdot.in Poornima Shandilya, C-DOT, poornima@cdot.in Anupama Chopra, C-DOT, anupama@cdot.in |
| Date:* | 2024-09-12 |
| Reason for Change/s:* | TS-0001 – accessControlObjectDetails handling in <accessControlPolicy> resource |
| CR against: Release* | Release 5 |
| CR against: WI* | Active WI-xxxx MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v.5.6.0 |
| Clauses * | 9.6.2.4 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0003 and TS-0004 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

Introduction

This CR proposes changes to accessControlObjectDetails attribute in access control rule of <accessControlPolicy> resource for handling of Create, Retrieve, Update, Delete and Discovery operations on a target resource. This is in continuation to discussion of SDS-2024-0078R02-oneM2MResourceTypeACR presented in SDS session of TP-65 on 24th June 2024. Two proposals were discussed for handling of resourceType. Proposal no. 1 was agreed and the changes for the same are introduced in this CR.

### **********************  Start of Change 1   *****************************

#### 9.6.2.4	accessControlObjectDetails

The accessControlObjectDetails is an optional parameter of an access control rule. It may specify the resource type of targeted resource and a subset of child resource types of the targeted resource to which the access control rule applies. The accessControlObjectDetails parameter shall consist of the elements listed in table 9.6.2.4-1.The resourceType or specialization element of accessControlObjectDetails, if present, shall be checked for all operations. The CSE shall match the type of resource or specialization, whichever applicable, of the targeted resource with the value specified in the respective element.

For Create operations, If childResourceType is present, the CSE shall also match one of the elements present in the childResourceType with the type of resource that is to be created. Further, if childSpecializationType is present, the CSE shall match one of the elements present in the childSpecializationType with the specialization type of resource that is to be created.

Table 9.6.2.4-1: Types of Parameters in accessControlObjectDetails

| Name | Description |
| --- | --- |
| resourceType | Identifier of the resource type to which this access control rule applies. |
| specializationType | When the resourceType is mgmtObj or flexContainer, the identifier of the specialization as defined by mgmtDefinition or containerDefinition attribute, respectively, shall be specified. |
| childResourceType | List of child resource types. |
| childSpecializationType | List of child specialization types of resourceType mgmtObj or flexContainer. |

### ********************* End of Change 1 *********************************