| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | RDM#69 |
| Source:* | Andreas Kraft |
| Date:* | 2025-03-23 |
| Reason for Change/s:* | Add_missing_short_names_for_common_domain_devices |
| CR against: Release* | R5 |
| CR against: WI* | Active <#WI#> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0023 v5.7.0 |
| Clauses * | 5.5.1.1, 5.5.1.2, 5.5.1.3, 5.5.1.4, 5.5.1.5, 5.5.1.6, 5.5.1.7, 5.5.1.8, 5.5.1.9, 5.5.1.10, 5.5.1.11, 5.5.1.12, 5.5.1.13, 5.5.1.14, 5.5.1.15, 5.5.1.16, 5.5.1.17, 5.5.1.18, 5.5.1.19, 6.3.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Add missing short names for common domain devices

https://git.onem2m.org/specifications/ts/ts-0023/-/merge_requests/55

https://git.onem2m.org/specifications/ts/ts-0023/-/merge_requests/55/diffs?commit_id=abe5eff3f454941a34797b935f7cc8ec50a1f529

----------------------- Start of change 1 -----------------------

---a/TS-0023-SDT_based_Information_Model_and_Mapping_for_Vertical_Industries.md
+++b/TS-0023-SDT_based_Information_Model_and_Mapping_for_Vertical_Industries.md

@@ -2616,8 +2616,6 @@ A 3D printer is a smart home appliance to provide 3D printing capabilities.

|printQueue |[printQueue](#53167-printqueue) |1 |See clause [5.3.1.67](#53167-printqueue). |

#### 5.5.1.2 deviceAirQualityMonitor

An air quality monitor is an environmental monitoring device used to monitor the air quality.

@@ -2631,12 +2629,9 @@ An air quality monitor is an environmental monitoring device used to monitor the

|periodicalReportConfig |[periodicalReportConfig](#53163-periodicalreportconfig) |0..1 |See clause [5.3.1.63](#53163-periodicalreportconfig). |

#### 5.5.1.3 deviceAudioReceiver

An audio receiver is a device that receives audio signals from a number of sources, processing them to drive speakers.

**Table 5.5.3.1-1: Modules of deviceAudioReceiver model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2647,12 +2642,9 @@ An audio receiver is a device that receives audio signals from a number of sourc

|mediaOutput |[mediaSelect](#53153-mediaselect) |0..1 |See clause [5.3.1.53](#53153-mediaselect). |

#### 5.5.1.4 deviceCamera

A camera is an optical instrument for recording or capturing images, which may be stored locally or transmitted to another locations.

**Table 5.5.1.4-1: Modules of deviceCamera Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2662,12 +2654,9 @@ A camera is an optical instrument for recording or capturing images, which may b

|motionSensor |[motionSensor](#53154-motionsensor) |0..1 |See clause [5.3.1.54](#53154-motionsensor). |

#### 5.5.1.5 deviceDoor

A door is a device that is used to open and close a door.

**Table 5.5.1.5-1: Modules of deviceDoor model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2677,12 +2666,9 @@ A door is a device that is used to open and close a door.

|doorStatus |[doorStatus](#53130-doorstatus) |0..1 |See clause [5.3.1.30](#53130-doorstatus). |

#### 5.5.1.6 deviceDoorLock

A door lock is a device that can be used to lock, for example, a door.

**Table 5.5.1.6-1: Modules of deviceDoorLock Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2692,12 +2678,9 @@ A door lock is a device that can be used to lock, for example, a door.

|battery |[battery](#53110-battery) |0..1 |See clause [5.3.1.10](#53110-battery). |

#### 5.5.1.7 deviceLight

A light is a device that is used to control the state of an illumination appliance.

**Table 5.5.1.7-1: Modules of deviceLight Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2711,12 +2694,9 @@ A light is a device that is used to control the state of an illumination applian

|timer |[timer](#53190-timer) |0..1 |See clause [5.3.1.90](#53190-timer). |

#### 5.5.1.8 deviceMultiFunctionPrinter

A Multi Function Printer (MFP) is an office machine which incorporates the functionality of multiple devices in one, so as to have a smaller footprint in home or office. A typical MFP may act as a combination of printer, scanner and more. This MFP information model provides capabilities to control and monitor MFP specific functions and resources.

**Table 5.5.1.8-1: Modules of deviceMultiFunctionPrinter Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2728,12 +2708,9 @@ A Multi Function Printer (MFP) is an office machine which incorporates the funct

|printQueue |[printQueue](#53167-printqueue) |0..1 |See clause [5.3.1.67](#53167-printqueue). |

#### 5.5.1.9 devicePrinter

A printer is a device that is used to monitor or control the state of a printing appliance.

**Table 5.5.1.9-1: Modules of devicePrinter Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2743,12 +2720,9 @@ A printer is a device that is used to monitor or control the state of a printing

|printQueue |[printQueue](#53167-printqueue) |0..1 |See clause [5.3.1.67](#53167-printqueue). |

#### 5.5.1.10 deviceScanner

A scanner is a device that optically scans images, printed text, handwriting or an object, and converts it to a digital image.

**Table 5.5.1.10-1: Modules of deviceScanner model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2758,12 +2732,9 @@ A scanner is a device that optically scans images, printed text, handwriting or

|autoDocumentFeeder |[autoDocumentFeeder](#5319-autodocumentfeeder) |0..1 |See clause [5.3.1.9](#5319-autodocumentfeeder). |

#### 5.5.1.11 deviceSmartPlug

A smart plug is a device that can turn on and off a connected appliance.

**Table 5.5.1.11-1: Modules of deviceSmartPlug model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2773,8 +2744,6 @@ A smart plug is a device that can turn on and off a connected appliance.

|remoteControlEnable |[remoteControlEnable](#53173-remotecontrolenable) |0..1 |See clause [5.3.1.73](#53173-remotecontrolenable). |

**Table 5.5.1.11-2: SubDevice of deviceSmartPlug Device model**

|SubDevice Instance Name |SubDevice Name |Multiplicity |Description |

@@ -2782,7 +2751,6 @@ A smart plug is a device that can turn on and off a connected appliance.

|powerOutlet0<br/><mark>Discuss: This should be "powerOutlet"</mark> |[subDevicePowerOutlet](#5412-subdevicepoweroutlet) |1..N |See clause [5.4.1.2](#5412-subdevicepoweroutlet). |

#### 5.5.1.12 deviceSwitch

A switch is a device that is used to control and monitor the state of power.

@@ -2793,12 +2761,9 @@ A switch is a device that is used to control and monitor the state of power.

|binarySwitch |[binarySwitch](#53112-binaryswitch) |1 |See clause [5.3.1.12](#53112-binaryswitch). |

#### 5.5.1.13 deviceThermometer

A thermometer is a device that can be used to check, for example, the body or other temperatures.

**Table 5.5.1.13-1: Modules of deviceThermoMeter Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2808,8 +2773,6 @@ A thermometer is a device that can be used to check, for example, the body or ot

|temperatureAlarm |[temperatureAlarm](#53188-temperaturealarm) |0..1 |See clause [5.3.1.88](#53188-temperaturealarm). |

#### 5.5.1.14 deviceThermostat

A thermostat is used to control the ambient temperature of rooms within, for example, a house. This information model provides capabilities to interact with specific functions of thermostats.

@@ -2822,12 +2785,9 @@ A thermostat is used to control the ambient temperature of rooms within, for exa

|temperature |[temperature](#53187-temperature) |1 |See clause [5.3.1.87](#53187-temperature). |

#### 5.5.1.15 deviceWaterValve

A water valve is a device that is used to turn the water supply ON or OFF remotely.

**Table 5.5.1.15-1: Modules of waterValve Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2835,16 +2795,9 @@ A water valve is a device that is used to turn the water supply ON or OFF remote

|openLevel |[openLevel](#53156-openlevel) |1 |See clause [5.3.1.56](#53156-openlevel). |

#### 5.5.1.16 deviceServiceButton

A service button is a device that initiates and controls technical or business processes, such as ordering consumer and industrial goods over the Internet. It may support optional ModuleClasses to present information to a user, such as presenting textual and graphical information.

<mark>Editor note: the device is missing from the short name tables in 6.3</mark>

**Table 5.5.1.16-1: Modules of deviceServiceButton Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2859,15 +2812,9 @@ A service button is a device that initiates and controls technical or business p

|geoLocation |[geoLocation](#53141-geolocation) |0..1 |See clause [5.3.1.41](#53141-geolocation). This ModuleClass may be used to detect and report the geo-location of a deviceServiceButton device. |

#### 5.5.1.17 deviceGenericSensor

A device that is composed of one or more basic sensors. This generic model is proposed to represent very simple appliances that feature one or more sensing behaviours (mono/multi sensors).

<mark>Editor note: the device is missing from the short name tables in 6.3</mark>

**Table 5.5.1.17-1: Modules of deviceGenericSensor Device model**

|Module Instance Name |Module Class Name |Optional |Description |

@@ -2881,16 +2828,9 @@ A device that is composed of one or more basic sensors. This generic model is pr

|waterSensor |[waterSensor](#53197-watersensor) |0..1 |See clause [5.3.1.97](#53197-watersensor). |

#### 5.5.1.18 device3DDisplay

A 3D display is a device to display 3D contents such as 3D character or holographic image.

<mark>Editor note: the device is missing from the short name tables in 6.3</mark>

**Table 5.5.1.18-1: Modules of device3DDisplay Device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2904,14 +2844,9 @@ A 3D display is a device to display 3D contents such as 3D character or holograp

|lock |[lock](#53152-lock) |0..1 |See clause [5.3.1.52](#53152-lock). |

#### 5.5.1.19 device3DScanner

A 3D scanner is a device to scan 3D objects such as a statue of a character.

<mark>Editor note: the device is missing from the short name tables in 6.3</mark>

**Table 5.5.1.19-1: Modules of device3DScanner device model**

|Module Instance Name |Module Class Name |Multiplicity |Description |

@@ -2923,8 +2858,6 @@ A 3D scanner is a device to scan 3D objects such as a statue of a character.

|connectivity |[connectivity](#53125-connectivity) |1 |See clause [5.3.1.25](#53125-connectivity). |

### 5.5.2 City Domain

#### 5.5.2.1 deviceOutdoorLamp

@@ -5700,7 +5633,9 @@ In protocol bindings resource type names for device models shall be translated i

|Resource Type Name |Short Name |

|-|-|

|[device3DDisplay](#55118-device3ddisplay) |dTDDy |

|[device3DPrinter](#5511-device3dprinter) |dTDPr |

|[device3DScanner](#55119-device3dscanner)| dTDSr |

|[deviceAirConditioner](#5541-deviceairconditioner) |deACr |

|[deviceAirHeater](#55433-deviceairheater) |deAHr |

|[deviceAirPurifier](#5542-deviceairpurifier) |deAPr |

@@ -5723,6 +5658,7 @@ In protocol bindings resource type names for device models shall be translated i

|[deviceFan](#55412-devicefan) |devFn |

|[deviceFoodProbe](#55413-devicefoodprobe) |deFPe |

|[deviceFreezer](#55414-devicefreezer) |devFr |

|[deviceGenericSensor](#55117-devicegenericsensor) |deGSr |

|[deviceGlucosemeter](#5532-deviceglucosemeter) |devGr |

|[deviceHeartRateMonitor](#5533-deviceheartratemonitor) |dHRMr |

|[deviceHomeCCTV](#55415-devicehomecctv) |dHCCT |

@@ -5740,6 +5676,7 @@ In protocol bindings resource type names for device models shall be translated i

|[deviceRobotCleaner](#55422-devicerobotcleaner) |deRCr |

|[deviceScanner](#55110-devicescanner) |devSr |

|[deviceSecurityPanel](#55423-devicesecuritypanel) |deSPl |

|[deviceServiceButton](#55116-deviceservicebutton) |deSBn |

|[deviceSetTopBox](#55424-devicesettopbox) |dSTBx |

|[deviceShoesWasher](#55435-deviceshoeswasher) |deSWr |

|[deviceSmartElectricMeter](#5523-devicesmartelectricmeter) |dSEMr |

@@ -5760,7 +5697,6 @@ In protocol bindings resource type names for device models shall be translated i

|[flexNode](#581-flexnode) |fleNe |

In protocol bindings resource type names for SubDevice model shall be translated into short names of Table 6.3.2-2.

**Table 6.3.2-2: Specialization type short names (SubDevice models)**

----------------------- End of change 1 -----------------------