| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #58 |
| Source:* | Andreas Kraft, DT, Andreas.Kraft@t-systems.com Andreas Neubacher, DT, Andreas.Neubacher@magenta.at Miguel Angel Reina Ortega, ETSI, MiguelAngel.ReinaOrtega@etsi.org |
| Date:* | 2023-02-23 |
| Reason for Change/s:* | TS-0001 - Another correction for the notificationStatsInfo and notificationStatsEnable (R5) |
| CR against: Release* | Release 5 |
| CR against: WI* | Active WI-xxxx MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v.5.1.0 |
| Clauses * | 9.6.8, 9.6.58, 10.2.10.22, 10.2.10.27 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

Introduction

The notificationStatsInfo attribute that is present in the <crossResourceSubscription> and <subscription> resource types is an optional attribute. It is filled, depending on the state of the notificationStatsEnable attribute (of the same resource type), with notification statistics.

There is, however, a discrepancy in the definitions of these attributes: The notificationStatsEnable attribute is defined as mandatory and, if not present in a CREATE request, is provided with a default value by the CSE. This implies that the notificationStatsInfo attribute is always present as well. In TS-0001, though, it is defined as "0..1(L)", meaning optional, which in this case is not necessary.

There is another problem that is been solved by change 4: If the notificationStatsEnable attribute is updated to true (independent from its current value), then the notificationsStatsInfo attribute is only emptied. But empty lists are usually not allowed for optional list attributes. The changes correct this by removing the notificationsStatsInfo attribute instead of emptying it.

The proposed changes in this CR will address the problems described above in TS-0001. SDS-2022-0184 proposes the necessary changes for TS-0004.

Change 1 provides the necessary changes for the <subscription> resource type.

Change 2 provides the necessary changes for the <crossResourceSubscription> resource type.

Change 3 removes a superfluous clarification in clause 10.2.10.22 that describes what NOT to do and that is confusing in this part of the specification. It is better specified in the procedures in TS-0004.

Change 4 provides the necessary changes for the Notification Recording procedure in clause 10.2.10.27.

This is a mirror for SDS-2022-0183R01 for R5.

### **********************  Start of Change 1   *****************************

### 9.6.8	Resource Type subscription

The <subscription> resource contains subscription information for its subscribed-to resource.

A subscription to a resource allows an entity in the oneM2M architecture to be notified about changes of the subscribed-to resource. The <subscription> resource shall represent a subscription to a subscribed-to resource. In order to establish a subscription, a <subscription> resource shall be created as a child resource of the subscribed-to resource. The <subscription> child resource contains information about the exact scope of the subscription and targets to be notified. For example, a <container> resource having a <subscription> resource as a child resource (see clause 9.6.6) shall result in notification(s) of target(s) configured in the <subscription> child resource when changes to the parent <container> resource matching with notification event criteria described by the child <subscription> resource occur. A <subscription> resource shall be deleted when the parent subscribed-to resource is deleted.

In general, an Originator shall be able to create a resource of <subscription> resource type when the Originator has RETRIEVE privilege to the subscribed-to resource. The Originator which creates a <subscription> resource becomes the resource subscriber.

A <subscription> resource can be configured to implement a blocking "UPDATE" to a resource or attributes of a resource whereby a notification is sent to the notification target to respond with the result of the "UPDATE" request.

Each <subscription> may include notification policies that specify which, when, and how notifications are sent. These notification policies may work in conjunction with CMDH policies.

When a <subscription> resource is deleted, a Notify request shall be sent to the target indicated by the attribute subscriberURI if it is provided by the Subscriber.

The <subscription> resource shall contain the child resources specified in table 9.6.8-1.

Table 9.6.8-1: Child resources of <subscription> resource

| Child Resources of <subscription> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| notificationSchedule | <schedule> | 0..1 | In the context of the <subscription> resource, the notificationSchedule specifies when notifications may be sent by the Hosting CSE to the notificationURI(s). See clause 9.6.9. |
| [variable] | <notificationTargetMgmtPolicyRef> | 0..n | See 9.6.31 for this type of resource. |
| nstr | <notificationTargetSelfReference> | 1 | See 9.6.34 for this type of resource. |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |

The <subscription> resource shall contain the attributes specified in table 9.6.8-2.

Table 9.6.8-2: Attributes of <subscription> resource

| Attributes of <subscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creator | 0..1 | RO | See clause 9.6.1.3. |
| custodian | 0..1 | RW | See clause 9.6.1.3 |
| eventNotificationCriteria | 0..1 | RW | This attribute (notification policy) indicates the event criteria for which a notification is to be generated. When no eventNotificationCriteria attribute is present in a <subscription> resource, the Hosting CSE shall trigger notifications for this subscription when any of the attributes of the subscribed-to resource is modified. |
| expirationCounter | 0..1 | RW | This attribute (notification policy) indicates that the subscriber wants to set the life of this subscription to a limit of a maximum number of notifications. When the number of notifications sent reaches the count of this counter, the <subscription> resource shall be deleted, regardless of any other policy. |
| notificationURI | 1 (L) | RW | This attribute shall be configured as a list consisting of one or more targets that the Hosting CSE shall send notifications to. A target shall be formatted as a oneM2M compliant Resource-ID as defined in clause 7.2 or as an identifier compliant with a oneM2M supported protocol binding (e.g. http, coap, mqtt). If a target is formatted as a oneM2M compliant Resource-ID, then the target shall be formatted as a structured or unstructured CSE-Relative-Resource-ID, SP-Relative-Resource-ID, and/or Absolute-Resource-ID of an <AE> or <CSEBase> resource. A Hosting CSE shall use this information to determine proper pointOfAccess, requestReachability and/or pollingChannel information needed to send a notification to the target. The following is an example. /CSE0001/AE0001 For a target that is formatted as an identifier compliant with a oneM2M supported protocol binding, the details of this format are defined by the respective oneM2M protocol specification. The following is an example of an HTTP URI compliant with oneM2M HTTP protocol binding. https://172.25.30.25:7000/notification/handler For a subscription to a <fanoutpoint> resource, if <subscription> resource in request contains a notificationForwardingURI, then the group hosting CSE shall configure the notificationURI of the fanout subscription request with an address specified by the Group Hosting CSE that can be used by the Group Hosting CSE to receive aggregated notifications. A notification serialization type may be appended to each notification target configured in this list. The Hosting CSE shall serialize notifications and send it to a notification target based on this serialization type indicator. Possible serialization types are defined in oneM2M TS-0004 [3] (e.g. XML, JSON or CBOR). If a notification serialization type is not appended to a notification target, a default shall apply based on the Hosting CSE local policy. The syntax for appending a serialization type to a notification target shall use the "?" delimiter character as shown in the below examples. http://mydomain/notificationHandler?ct=json CSE02/base/ae2?ct=xml |
| groupID | 0..1 | RW | The ID of a <group> resource in case the subscription is made through a group. This attribute may be used in the Filter Criteria to discover all subscription resources created via a <fanOutPoint> resource to a specific groupID. |
| notificationForwardingURI | 0..1(L) | RW | The attribute shall be present only for group related subscriptions. If the subscriber intends the Group Hosting CSE to aggregate the notifications, the attribute shall be set identical to the notificationURI attribute. It shall be used by Group Hosting CSE for forwarding aggregated notifications. See clauses 10.2.7.10 and 10.2.7.11. |
| batchNotify | 0..1 | RW | This attribute (notification policy) indicates that the subscription originator wants to receive batches of notifications rather than receiving them one at a time. This attribute includes: the number of notifications to be batched for delivery and the duration. When only the number is specified by the subscription originator, the Hosting CSE shall set the default duration given by M2M Service Provider. If batchNotify is used simultaneously with latestNotify, only the latest notification shall be sent and have the Event Category set to "latest". |
| rateLimit | 0..1 | RW | This attribute (notification policy) indicates that the subscriber wants to limit the rate at which it receives notifications. This attribute expresses the subscriber's notification policy and includes two values: a maximum number of events that may be sent within some duration, and the rateLimit window duration. When the number of generated notifications within the rateLimit window duration exceeds the maximum number, notification events are temporarily stored, until the end of the window duration, when the sending of notification events restarts in the next window duration. The sending of notification events continues as long as the maximum number of notification events is not exceeded during the window duration. The rateLimit policy may be used simultaneously with other notification policies. |
| preSubscriptionNotify | 0..1 | WO | This attribute (notification policy) indicates that the subscriber wants to be sent notifications for events that were generated prior to the creation of this subscription. This attribute has a value of the number of prior notification events requested. If up-to-date caching of retained events is supported on the Hosting CSE and contains the subscribed events, then prior notification events will be sent up to the number requested. The preSubscriptionNotify policy may be used simultaneously with any other notification policy. |
| pendingNotification | 0..1 | RW | This attribute (notification policy), if set, indicates how missed notifications due to a period of no connectivity are handled (according to the reachability and notification schedules). The possible values for pendingNotification are: "sendLatest"; "sendAllPending". This policy depends upon caching of retained notifications on the hosted CSE. When this attribute is set to "sendLatest", only the last notification shall be sent and it shall have the Event Category set to "latest". If this attribute is not present, the Hosting CSE sends no missed notifications. This policy applies to all notifications regardless of the selected delivery policy (batchNotify, latestNotify, etc.) Note that unreachability due to reasons other than scheduling is not covered by this policy. |
| notificationStoragePriority | 0..1 | RW | Indicates that the subscriber wants to set a priority for this subscription relative to other subscriptions belonging to this same subscriber. This attribute sets a number within the priority range. When storage of notifications exceeds the allocated size, this policy is used as an input with the storage congestion policy (notificationCongestionPolicy) specified in clause 9.6.3 to determine which stored and generated notifications to drop and which ones to retain. |
| latestNotify | 0..1 | RW | This attribute (notification policy) indicates if the subscriber wants only the latest notification. If multiple notifications of this subscription are buffered, and if the value of this attribute is set to true, then only the last notification shall be sent and it shall have the Event Category value set to "latest". |
| notificationContentType | 1 | RW | Indicates a notification content type that shall be contained in notifications. For example, "modified attributes" or "all attributes". For a list of the default and allowed values of notificationContentType for each of the supported values of notificationEventType refer to table 9.6.8-4. |
| notificationEventCat | 0..1 | RW | This attribute (notification policy) indicates the subscriber's requested Event Category to be used for notification messages generated by this subscription. |
| subscriberURI | 0..1 | WO | This attribute shall be configured with the target of the subscriber. The target is used by the Hosting CSE to determine where to send a notification when the subscription is deleted. A target shall be formatted as a oneM2M compliant Resource-ID as defined in clause 7.2 or as an identifier compliant with one of the oneM2M supported protocol bindings (the detailed format of which are defined by each respective oneM2M protocol binding specification). |
| associatedCrossResourceSub | 0..1 | RW | This attribute lists the identifier of <crossResourceSubscription> resources where this <subscription> is involved in. |
| primitiveProfileID | 0..1 | RW | This attribute lists the identifier of a <primitiveProfile> resource that specifies attributes and parameters to be added, removed, or modified in the notifications for this subscription. |
| notificationStatsEnable | 0..1 | RW | When set to "TRUE", the Hosting CSE shall clear any statistics that were previously stored in the notificationStatsInfo attribute and start recording notification statistics for each notification generated for this resource. When set to "FALSE", the Hosting CSE shall not record notification statistics for this resource and maintain the current value of the notificationStatsInfo attribute. Default is "FALSE". |
| notificationStatsInfo | 0..1(L) | RO | A list containing notification statistics recorded by the Hosting CSE for each notification target specified by the notificationURI attribute of this resource. The Hosting CSE shall maintain a separate set of notification statistics that include: - Total number of notification requests sent to a notification target - Total number of notification responses received from a notification target Refer to oneM2M TS 0004 [3] for further details regarding the format of this attribute. |

Table 9.6.8-3 describes the eventNotificationCriteria conditions.

Table 9.6.8-3: eventNotificationCriteria conditions

| Condition tag | Multiplicity | Matching condition |
| --- | --- | --- |
| createdBefore | 0..1 | The creationTime attribute of the resource is chronologically before the specified value. |
| createdAfter | 0..1 | The creationTime attribute of the resource is chronologically after the specified value. |
| modifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically after the specified value. |
| unmodifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically before the specified value. |
| stateTagSmaller | 0..1 | The stateTag attribute of the resource is smaller than the specified value. |
| stateTagBigger | 0..1 | The stateTag attribute of the resource is bigger than the specified value. |
| expireBefore | 0..1 | The expirationTime attribute of the resource is chronologically before the specified value. |
| expireAfter | 0..1 | The expirationTime attribute of the resource is chronologically after the specified value. |
| sizeAbove | 0..1 | The contentSize attribute of the <contentInstance> resource is equal to or greater than the specified value. |
| sizeBelow | 0..1 | The contentSize attribute of the <contentInstance> resource is smaller than the specified value. |
| notificationEventType | 0..5 | The type of event that shall trigger a notification. If multiple notificationEventType tags are present, a notification shall be triggered if any of the configured events occur. Note that not all combinations of event type are meaningful. Possible notification event type values are: - Update to attributes of the subscribed-to resource - Deletion of the subscribed-to resource, - Creation of a direct child of the subscribed-to resource, - Deletion of a direct child of the subscribed-to resource - An attempt to retrieve a <contentInstance> direct-child-resource of a subscribed-to <container> resource is performed while this <contentInstance> child resource is an obsolete resource or the reference used for retrieving this resource is not assigned. This retrieval is performed by a RETRIEVE request targeting the subscribed-to resource with the Result Content parameter set to either "child-resources" or "attributes+child-resources". This value for the notificationEventType tag implies that the subscribed-to resource shall be an <container> resource. Otherwise this setting is not valid. - Trigger Received targeting the MN/ASN-AE associated with the <AE> parent resource. This implies that the subscribed-to resource shall be an <AE> resource instance. Otherwise this setting is not valid. - Update to attributes of the subscribed-to resource with blocking of the triggering UPDATE operation. For this notificationEventType value setting, only one single Notification Target shall be present in the notificationURI attribute – see notificationURI attribute definition. This value for the notificationEventType tag shall not be combined with any other notificationEventType tag value. This value for notificationEventType establishes a subscription that is triggered for the same events as for the value "Update to attributes of the subscribed-to resource". However, upon occurrence of a triggering UPDATE operation that has been validated and results in an authorized UPDATE operation, the triggering UPDATE operation shall be blocked by the Hosting CSE until a notification request was sent out and a corresponding response message was received or a timeout happens. When the response status code of the notification response message indicates a successful notification reception in combination with a successful notification action taken by the Notification Target entity, the triggering UPDATE operation shall be completed with a successful update of the targeted attribute(s). If the notification response message indicates an unsuccessful notification reception or a successful notification reception with unsuccessful notification action by the targeted entity or times out, the blocked UPDATE operation shall be completed with no success and no change of the targeted attribute(s). For any subscribed-to resource there shall exist a maximum of one subscription with this setting of notificationEventType. All other notification policies shall not be allowed when this setting of notificationEventType is used. The notificationContentType shall be "modified attributes". When an UPDATE operation has been blocked due to triggering this type of notification, any other occurring UPDATE or DELETE requests to the same resource shall be handled only after the blocked UPDATE operation has been completed. - Report on missing data points. This notificationEventType value shall not be combined with any other notificationEventType value. The other conditions in eventNotificationCriteria conditions apply within the scope of the selected notificationEventType. For example, if notificationEventType is "Creation of a direct child of the subscribed-to resource" then other eventNotificationCriteria conditions is applied to the direct child resources of the subscribed-to resource. If this condition is not specified, the default value is "Update to attributes of the subscribed-to resource". This default value shall apply only if operationMonitor is not present in the resource. The notion of "obsolete resource" is defined in clause 9.6.1.3.2 (Common attributes). For a list of the default and allowed values of notificationContentType for each of the supported values of notificationEventType refer to Table 9.6.8-4. |
| operationMonitor | 0..n | The operations and/or the Originators accessing the subscribed-to resource matches with the specified value. It allows monitoring which operation and/or which Originator is attempting to the access subscribed-to resource regardless of whether the operation is performed. This feature is useful to detect AEs that send requests to a subscribed-to resource and that result in a successful or failure response. Possible arguments are operation(s) (e.g.: CREATE, RETRIEVE, UPDATE, DELETE, NOTIFY) and/or Originator identifier(s). If a set of Originator identifier(s) is included in this tag and no operations are listed, any operations initiated from any of the indicated Originator(s) shall trigger a notification. If a set of operation(s) is included in this tag and no Originator identifier, any of the listed operations shall trigger a notification. If both, a set of Originator identifiers and a set of operations are listed, then any of the listed operations initiated from any of the listed Originators shall trigger the notification. |
| attribute | 0..n | A list of attribute names of a subscribed-to-resource. This list is only applicable when notificationEventType has a value of "Update to attributes of the subscribed-to resource". or "Update to attributes of the subscribed-to resource with blocking of the triggering UPDATE operation". If this list is present, then it is used to specify a subset of a subscribed-to resource's attributes for which updates shall result in a notification. If ANY attribute specified on this list is updated, then a notification shall be generated. If an attribute that is not specified in this list is updated, then a notification shall not be generated. If this list is not presented, then the default attribute list is the full set of a subscribed-to resource's attributes. If ANY attribute of a subscribed-to resource is updated, then a notification shall be generated. |
| childResourceType | 0.. 1 (L) | A list of resource types. This list is only applicable when notificationEventType has a value of "Creation of a direct child of the subscribed-to resource" or "Deletion of a direct child of the subscribed-to resource". If this list is present, then it is used to specify a subset of resource type for direct child resource of which creation or deletion shall result in a notification. If ANY resource type specified on this list is created or deleted, then a notification shall be generated. If a resource type that is not specified in this list is created or deleted, then a notification shall not be generated. If this list is not present, then the default resource type list is the full set of a direct child resource. |
| missingData | 0..1 | The missingData includes two values: a minimum specified missing number of the Time Series Data within the specified window duration, and the window duration. The condition only applies to subscribed-to resources of type <timeSeries>. This condition is ignored unless notificationEventType has a value of "Report on missing data points". If this attribute is modified by an UPDATE the associated timer/counter are stopped and restarted with the new values. The first detected missing data point starts the timer associated with the window duration. The window duration is restarted upon its expiry until such time as the entire subscription is terminated or not refreshed. More details about NOTIFICATIONS related to data reporting is found in clause 10.2.4.29 |
| filterOperation | 0..1 | Indicates the logical operation (AND/OR/XOR) to be used for the condition tags createdBefore, createdAfter, modifiedSince, unmodifiedSince, stateTagSmaller, stateTagBigger, expireBefore, expireAfter, sizeAbove, sizeBelow. The default value is logical AND. |

The rules when multiple conditions are used together shall be as follows:

Different condition tags shall use the "AND/OR/XOR" logical operation based on the filterOperation specified;

Same condition tags shall use the "OR" logical operation.

The XOR operation evaluates to true if and only if an odd number of its inputs are true.

No mixed AND/OR/XOR filter operation will be supported.

Table 9.6.8-4 defines the default and allowed values of notificationContentType for each of the supported values of notificationEventType.

Table 9.6.8-4: Default and allowed values of notificationContentType

| notificationEventType notificationContentType | A | B | C | D | E | F | G | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| "modified attributes"; | valid | n/a | n/a | n/a | n/a | n/a | valid (default) | n/a |
| "all attributes"; | valid (default) | valid (default) | valid (default) | valid (default) | valid (default) | n/a | n/a | n/a |
| "ID" of the resource indicated in the notificationEventType condition. | valid | valid | valid | valid | valid | n/a | n/a | n/a |
| "Trigger Payload" | n/a | n/a | n/a | n/a | n/a | valid (default) | n/a | n/a |
| "TimeSeries notification" | n/a | n/a | n/a | n/a | n/a | n/a | n/a | valid (default) |

### ********************* End of Change 1 *********************************

### **********************  Start of Change 2   *****************************

### 9.6.58	Resource Type crossResourceSubscription

The <crossResourceSubscription> resource represents a cross-resource subscription over a set of target resources which could be existing <subscription> and/or other subscribable oneM2M resources. The Hosting CSE shall generate a cross-resource notifications only when expected changes occur on a designated number of target resources concurrently within a time window.  The <crossResourceSubscription> resource shall specify the involved target resources in order to generate cross-resource notification.

The <crossResourceSubscription> resource shall contain the child resources specified in table 9.6.58-1.

Table 9.6.58-1: Child resources of <crossResourceSubscription> resource

| Child Resources of <crossResourceSubscription> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| notificationSchedule | <schedule> | 0..1 | See clause 9.6.9. |
| [variable] | <notificationTargetMgmtPolicyRef> | 0..n | See clause 9.6.31. |
| nstr | <notificationTargetSelfReference> | 1 | See clause 9.6.34. |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |

The <crossResourceSubscription> resource shall contain the attributes specified in table 9.6.58-2.

Table 9.6.58-2: Attributes of <crossResourceSubscription> resource

| Attributes of <crossResourceSubscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creator | 1 | RO | See clause 9.6.1.3. |
| custodian | 0..1 | RW | See clause 9.6.1.3. |
| expirationCounter | 0..1 | RW | See clause 9.6.8. |
| notificationURI | 1 (L) | RW | See clause 9.6.8. |
| notificationEventCat | 0..1 | RW | See clause 9.6.8. |
| subscriberURI | 0..1 | WO | See clause 9.6.8. |
| regularResourcesAsTarget | 0..1 | RW | This attribute indicates a list of regular resources (i.e. normal resources rather than <subscription> resources), which shall be used as the target resource for this cross-resource subscription. Here, the regular resource is referred to as any subscribable oneM2M resources. |
| subscriptionResourcesAsTarget | 0..1 | RW | This attribute indicates a list of existing <subscription> resources, which shall be used as the target resource for this cross-resource subscription. |
| regularResourcesAsTargetSubscriptions | 0..1 | RO | This attribute indicates a list of subscription resources which correspond to the created <subscription> resources created for each entry in regularResourcesAsTarget attribute. It is mandatory if regularResourcesAsTarget is present. |
| timeWindowType | 1 | RW | This attribute indicates the type of time window mechanisms (timeWindowType can stand for periodic time window without any overlapping or for sliding time window where the current time window will be slid to become the next time window when a cross-resource notification is generated) which will be used to determine the generation of a cross-resource notification. |
| timeWindowSize | 1 | RW | This attribute indicates the size or time duration of the time window, based on which cross-resource notifications shall be generated. Note that the maximum window size (e.g. 60 seconds) may be enforced by the Hosting CSE for a subscriber; if the timeWindowSize indicated or requested by a subscriber is larger than the maximum window size, the Hosting CSE may reject the subscriber's request for cross-resource subscription. |
| eventNotificationCriteriaSet | 0..1(L) | RW | This attribute lists eventNotificationCriteria for each regular target resource as indicated in regularResourcesAsTarget attribute and involved in a cross-resource subscription. If there is only one eventNotificationCriteria contained in this attribute, it shall be applied to all target resources as indicated by regularResourcesAsTarget attribute. If only subscriptionResourcesAsTarget attribute appears (i.e. no regularResourcesAsTarget attribute), eventNotificationCriteriaSet shall not be needed. See clause 9.6.8 for the description of eventNotificationCriteria. |
| notificationStatsEnable | 0..1 | RW | When set to "TRUE", the Hosting CSE shall clear any statistics that were previously stored in the notificationStatsInfo attribute and start recording notification statistics for each notification generated for this resource. When set to "FALSE", the Hosting CSE shall not record notification statistics for this resource and maintain the current value of the notificationStatsInfo attribute. Default is "FALSE". |
| notificationStatsInfo | 0..1(L) | RO | A list containing notification statistics recorded by the Hosting CSE for each notification target specified by the notificationURI attribute of this resource. The Hosting CSE shall maintain a separate set of notification statistics that include: - Total number of notification requests sent to a notification target - Total number of notification responses received from a notification target Refer to oneM2M TS 0004 [3] for further details regarding the format of this attribute. |

### ********************* End of Change 2 *********************************

### **********************  Start of Change 3   *****************************

#### 10.2.10.22	Create <crossResourceSubscription>

This procedure shall be used to request the creation of a new <crossResourceSubscription> resource to be notified for the modifications of multiple subscribed-to target resources. The generic create procedure is described in clause 10.1.2.

Table 10.2.10.22-1: <crossResourceSubscription> CREATE

| <crossResourceSubscription> CREATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: Content: The resource content shall provide the information as defined in clause 9.6.58 |
| Processing at Originator before sending Request | According to clause 10.1.2 with the following additions: The Request shall include at least one of the following attributes: regularResourcesAsTarget, subscriptionResourcesAsTarget. The Request shall include timeWindowType and timeWindowSize. The Request shall include notificationURI(s). The Request shall include notificationContentType. The Request shall include eventNotificationCriteriaSet if regularResourcesAsTarget is included in the Request. If the request includes notificationURI(s) which is not the Originator, the Originator should send the request as non-blocking request (see clauses 8.2.2 and 9.6.12) |
| Processing at Receiver | According to clause 10.1.2 with the following additions: The Hosting CSE shall validate the followings: - Check if the Originator has privileges for creating a child resource in the To parameter in the Request. - Check if each target resource in regularResourcesAsTarget is a subscribable resource. - Check if the Originator has privileges for retrieving the subscribed-to resource - If a notificationURI is not the Originator, the Hosting CSE may send a Notify request to the notificationURI to verify this <crossResourceSubscription> creation request. If the Hosting CSE initiates the verification, it shall check if the verification result in the Notify response is successful or not. If any notificationURI contained in a list fails verification then the <crossResourceSubscription> create process fails If any of the checks above fails, the Hosting CSE shall send an unsuccessful response to the Originator with corresponding error information. Otherwise, the Hosting CSE shall use the following procedure to create the <crossResourceSubscription> resource and send a successful or an unsuccessful response to the Originator. - If regularResourcesAsTarget is included, the Hosting CSE shall send a CREATE request message to each target resource host to create a <subscription> child resource under each target resource indicated by regularResourcesAsTarget using corresponding event notification criteria as included in eventNotificationCriteriaSet. The notificationURI attribute for each <subscription> resource shall be configured to the resource identifier of this <crossResourceSubscription> resource being created. The associatedCrossResourceSub attribute of each <subscription> resource shall be configured with the resource identifier of this <crossResourceSubscription> resource. In the From parameter of each <subscription> CREATE request, the Hosting CSE shall include the identifier of the Originator of the <crossResourceSubscription> resource CREATE request, which shall be leveraged by the <subscription> resource host to verify if the Originator has the privilege to create a <subscription> resource; if the Originator has no privilege to create this <subscription> resource, this step shall be regarded as a failure. If any <subscription> resource cannot be successfully created, the Hosting CSE shall send an unsuccessful response to the Originator and shall delete any <subscription> resources that were created during the processing of this <crossResourceSubscription> resource CREATE request. - If subscriptionResourcesAsTarget is included, the Hosting CSE shall add the resource identifier of this <crossResourceSubscription> resource to the associatedCrossResourceSub and notificationURI attributes of each <subscription> resource as indicated in subscriptionResourcesAsTarget by issuing an UPDATE request to each <subscription> resource. In the From parameter of each UPDATE request, the Hosting CSE shall include the identifier of the Originator of the <crossResourceSubscription> resource CREATE request, which shall be leveraged by the <subscription> resource host to verify if the Originator has the privilege to retrieve the <subscription> resource and update the associatedCrossResourceSub attribute; if the Originator has no privilege to retrieve the <subscription> resource and update the associatedCrossResourceSub attribute, this step shall be regarded as a failure. If this step is not successfully performed, the Hosting CSE shall send an unsuccessful response to the Originator and shall also delete the resource identifier of this <crossResourceSubscription> resource from the associatedCrossResourceSub and notificationURI attributes of each <subscription> resource that was updated during the processing of this <crossResourceSubscription> resource CREATE request. - Otherwise, the Hosting CSE shall send a successful response to the Originator. Once the <crossResourceSubscription> resource is created, the Hosting CSE shall start the time window. If the notificationStatsEnable attribute is set to TRUE, the Hosting CSE shall collect and record notification statistics for the <crossResourceSubscription> resource as defined in clause 10.2.10.27. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: Content: address of the created <crossResourceSubscription> resource, according to clause 10.1.2. |
| Processing at Originator after receiving Response | According to clause 10.1.2 |
| Exceptions | According to clause 10.1.2 |

### ********************* End of Change 3 *********************************

### **********************  Start of Change 4   *****************************

#### 10.2.10.27	Notification Recording Procedure

For each <subscription> and <crossResourceSubsription> resource, the Hosting CSE shall use the following procedure to record notification statistics.

- If the notificationStatsEnable attribute of a <subscription> or <crossResourceSubsription> resource is set to TRUE, either upon the successful creation or update of the <subscription> or <crossResourceSubsription> resource, the Hosting CSE shall clear any statistics that were stored previously in the notificationStatsInfo attribute by removing the notificationStatsInfo attribute and begin collecting notification statistics for each notification the Hosting CSE generates and sends to each notification target defined by the notificationURI attribute.

- While the notificationStatsEnable attribute is set to TRUE, the Hosting CSE shall collect and store the following types of statistics within the notificationStatsInfo attribute for each notification target defined by the notificationURI attribute.

  - Total number of notification requests sent to a notification target

  - Total number of notification responses received from a notification target

- When the notificationStatsEnable attribute of a <subscription> or <crossResourceSubsription> resource is set to FALSE, the Hosting CSE shall stop collecting notification statistics for the <subscription> or <crossResourceSubsription> resource. The Hosting CSE shall maintain the current value of the notificationStatsInfo attribute until a subsequent update of the notificationStatsEnable attribute to a value of TRUE.

- When the notificationStatsEnable attribute of a <subscription> or <crossResourceSubsription> resource is removed from a resource, the Hosting CSE shall stop collecting notification statistics for the <subscription> or <crossResourceSubsription> resource. The Hosting CSE shall also remove the notificationStatsInfo attribute.

### ********************* End of Change 4 *********************************