| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #72 |
| Source:* | Peter Niblett, Exacta GSS, peter.niblett@exactagss.com Andreas Kraft, Exacta GSS |
| Date:* | 2025-10-27 |
| Reason for Change/s:* | TS-0010: Clarify support for MQTT over WebSocket |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-xxxx MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0010 v.4.0.0 |
| Clauses * | 6.6 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

Introduction

- Clause 6.2 of TS-0010 states that the oneM2M MQTT binding supports the user of MQTT running over “WebSocket - either with or without the use of TLS 1.2.”

- However the URI schemes defined in clause 6.6 do not include MQTT over WebSocket.  This Change Request adds the missing schemes.

- R01: Miscellaneous improvements.

- R02:

- Changed Clause 6.6 to talk about URL’s rather than URI’s and generalised the description so that an MQTT URL can be used anywhere where a URL is permitted by oneM2M.  Included a mention of the notificationURI attribute as this is likely to be the most widely-used case.

- Changed wording of the scheme descriptions from “shall be used to show that the server requires the use of … when this particular point of access is being used to “indicate that … shall be used when connecting to the MQTT server.” This is because the URL can be used for things other than point of access.

- Reworded the text about the / that precedes the path component. We need to make it clear that we are using / as the separator between <authority> and <path> whereas RFC3986 describes the / as part of the path. The reason for the difference is that we want to allow the use of MQTT topics that don’t start with a /.

- R03.  Add better examples of how the URL is ised

- **********************  Start of Change 1   *****************************

## 6.6	URL format

### 6.6.1	Introduction

oneM2M defines an MQTT URL format to be used in the pointOfAccess attributes in several entity resource types (e.g. <CSEBase>, <remoteCSE>, <AE>) and in other places where oneM2M allows the use of a URL, for example in the notificationURI attribute of the <subscription> resource type.

### 6.6.2	MQTT URL formats

oneM2M supports the following MQTT URL formats:

mqtt://<authority>

mqtts://<authority>

mqtt+ws://<authority>

mqtt+wss://<authority>

mqtt://<authority>/<path>

mqtts://<authority>/<path>

mqtt+ws://<authority>/<path>

mqtt+wss://<authority>/<path>

The <authority> component is defined in clause 3.2 of IETF RFC 3986 [8] and includes the host and optionally the port of the MQTT Server that is to be used.

The formats with the scheme mqtt: indicate that TLS shall not be used when connecting to the MQTT server.

The formats with the scheme mqtts: indicate that TLS shall be used when connecting to the MQTT server.

The formats with the scheme mqtt+ws: indicate that MQTT over WebSocket (no TLS) shall be used when connecting to the MQTT server.

The formats with the scheme mqtt+wss: indicate that MQTT over WebSocket with TLS shall be used when connecting to the MQTT server.

If the <authority> does not contain a port component, then the IANA-registered MQTT ports shown in Table 6.6-1 shall be used.

Table 6.6-1: Default port numbers

| Scheme | Port Number |
| --- | --- |
| mqtt: | 1883 |
| mqtts: | 8883 |
| mqtt+ws: | 80 |
| mqtt+wss: | 443 |

The <path>, if present, contains one or more MQTT topic levels (separated by a "/" if there are multiple levels). If the path is present it is preceded by a "/" character to separate it from the authority component. This "/" character is not treated as being part of the path. If the path itself starts with a "/" character this results in the authority component being followed by two "/" characters. A "/" character inside the path may be repeated.

### 6.6.3	Using an MQTT URL in a pointOfAccess attribute

A pointOfAccess attribute contains a list of one or more strings, each of which indicates a way in which that entity can be addressed. An entity can indicate support for MQTT by including one or more strings that use the formats defined in clause 6.6.2.

If there is a path component in a pointOfAccess string, the path is prepended to the topic strings defined in the present document.

EXAMPLES:

An AE with AE-ID of "CAE01" would subscribe to MQTT topic strings as follows:

- If it has a pointOfAccess string mqtt://myserver.example it would subscribe to oneM2M/req/+/CAE01/# (see clause 6.5.4).

- If it has a pointOfAccess string mqtt://myserver.example/abc/def it would subscribe to

- abc/def/oneM2M/req/+/CAE01/#.

- If it has a pointOfAccess string mqtt://myserver.example//def/ghi it would subscribe to /def/ghi/oneM2M/req/+/CAE01/#.

- If it has a pointOfAccess string mqtt://myserver.example///uvw//xyz it would subscribe to //uvw//xyz/oneM2M/req/+/CAE01/#.

### 6.6.4	Using an MQTT URL other than in a pointOfAccess attribute

An MQTT URI that is used anywhere other than in a pointOfAccess attribute shall contain a path component. That path gives the entire MQTT topic string that is to be used (note that the "/" character preceding the path does not form part of the topic string). The structure of that topic string does not have to conform to any of the topic string patterns defined in the present document.

EXAMPLES:

- The URL mqtt://myserver.example/abc/def refers to the Topic abc/def on the MQTT server hosted on myserver.example.

- The URL mqtt://myserver.example//def/ghi refers to the Topic /def/ghi on the MQTT server hosted on myserver.example.

- The URL mqtt://myserver.example///uvw//xyz refers to the Topic //uvw//xyz on the MQTT server hosted on myserver.example.

### ********************* End of Change 1 *********************************