| Input Contribution |  |
| --- | --- |
| Meeting ID* | TST#26 |
| Title:* | TS-0018 Test Purpose for ADN |
| Source:* | Keebum Kim, TTA, keebum.kim@tta.or.kr Sookhyun Jeon, TTA, shjeon@tta.or.kr |
| Uploaded Date:* | 2016-12-04 |
| Document(s) Impacted* | TS-0018 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* |  |
| Template Version:23 February 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

Based on TST-2016-discussion on TP development, below is the list of TTA tasks to develop TP of ADN profile.

| REG (F) | AE registration (F1) | AE registration operation in originator role, only with mandatory attributes. | TP/oneM2M/AE/REG/BV/001 is updated |
| --- | --- | --- | --- |
|  | <AE> resource management | Retrieve operation in originator role. | These features’ category shall be changed to DMR |
|  |  | Update operation in originator role, only with mandatory attributes |  |
|  | AE deregistration | AE deregistration operation in originator. | New Contribution TP/oneM2M/AE/REG/BV/00X(TBD) |
| DMR (F) | <CSEBase> resource management | Retrieve operation in originator role. | New Contribution TP/oneM2M/AE/DMR/REV/BV/00X(TBD) - TP/oneM2M/AE/DMR/RET/BV/00X_01 |
|  | <latest> resource management | Retrieve operation in originator role. | TP/oneM2M/AE/DMR/RET/BV/00X(TBD-latest) |
|  |  | Delete operation in originator role. | TP/oneM2M/AE/DMR/DEL/BV/00X(TBD-latest) |
|  | <oldest> resource management | Retrieve operation in originator role. | TP/oneM2M/AE/DMR/RET/BV/00X(TBD-oldest) |
|  |  | Delete operation in originator role. | TP/oneM2M/AE/DMR/DEL/BV/00X(TBD-oldest) |
|  | <AE> resource management | Retrieve operation in originator role. | New Contribution TP/oneM2M/AE/DMR/REV/BV/00X(TBD) - TP/oneM2M/AE/DMR/RET/BV/00X_02 |
|  |  | Update operation in originator role, only with mandatory attributes | New Contribution TP/oneM2M/AE/DMR/UPD/BV/00X(TBD) -TP/oneM2M/AE/DMR/UPD/BV/00X_01 |

# Introduction

This contribution proposes a test purpose for REG and DMR for AND-AE

## -----------------------Start of change 1--------------------------------------------

## Group AE

## Registration (REG)

Editor’s Note: Based on TST-2016-0163R03-TS-0018 Test Purposes for Group AE, TTA proposes TPs of REG.

| TP Id |  | TP/oneM2M/AE/REG/BV/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends an AE initial registration request including attribute App-ID set to APP-ID but no AE-ID-STEM provided when it is started |  |
| Reference |  | TS-0001 10.1.1.2.2 - case C, 9.6.19 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT never being registered and the IUT being switched off and the IUT having got a valid APP-ID } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT is started } |  | NA |
|  | then { the IUT sends a valid CREATE request to CSE containing Resource-Type set to 2 (AE) and From set to empty and Content <AE> containing App-ID attribute set to APP-ID requestReachability attribute set to value REQUEST_REACHABILITY } |  | CSE  IUT |

| TP Id |  | TP/oneM2M/AE/REG/BV/00X |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends AE deregistration request to CSE. |  |
| Reference |  | TS-0001 10.1.4.2.2 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being in the "initial state" the IUT having registered to CSE and the IUT having privileges to perform DELETE operation on the resource AE to CSE } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid DELETE request message containing To set to 2 (AE) and From set to AE_ID } |  | IUT  CSE |
|  | then { the IUT receives a valid response message from CSE containing Response Status Code set to 2002 (DELETED) } |  | IUT  CSE |

## -----------------------End of change 1--------------------------------------------

## -----------------------Start of change 2--------------------------------------------

## Data Management and Repository (DMR)

Editor’s Note: Based on TST-2016-0218R01-TS-0018 Test Purposes for Group AE, TTA proposes TPs of DMR.

| TP Id |  | TP/oneM2M/AE/DMR/REV/BV/00X(TBD) |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Retrieve request on the TARGET_RESOURCE_ADDRESS to CSE |  |
| Reference |  | TS-0001 10.1.2, TS-0004 7.2.2.1 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE. |  |
| Initial conditions |  | with { the IUT being registered and the IUT being switched on and the CSE having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the IUT having privileges to perform RETRIEVE operation on resource TARGET_RESOURCE_ADDRESS and the IUT is triggered to send a retrieve request on the TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid RETRIEVE request message containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  CSE |
|  | then { the IUT receives a valid Response message from CSE containing Response Status Code set to 2000 (OK) and Content attribute containing the resource TARGET_RESOURCE_ADDRESS } |  | IUT  CSE |

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/RET/BV/00X_01 | TS-0001 10.2.3.2 | 5 (CSEBase) |
| TP/oneM2M/AE/DMR/RET/BV/00X_02 | TS-0001 10.2.1.2 | 2 (AE) |

| TP Id |  | TP/oneM2M/AE/DMR/UPD/BV/00X(TBD) |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a Update request with the value of the attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 10.1.3 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW attribute ATTRIBUTE_NAME set to VALUE_1 and the IUT having privileges to perform UPDATE operation on resource TARGET_RESOURCE_ADDRESS and the IUT is triggered to send a update request on the TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid RETRIEVE request message containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and no Content attribute } |  | IUT  CSE |
|  | then { the IUT receives a Response message containing Response Status Code set to 2004 (UPDATED) and (optional) Content attribute containing RESOURCE_TYPE resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  CSE |

| TP Id | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME | VALUE_1 | VALUE_2 |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/AE/DMR/UPD/BV/00X_01 | TS-0001 10.2.1.3 | 2 (AE) | labels | “VALUE_1” | “VALUE_2” |

| TP Id |  | TP/oneM2M/AE/DMR/RET/BV/00X(TBD-oldest) |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a RETRIEVE request of <oldest> virtual resource. |  |
| Reference |  | TS-0001 10.2.23.1, TS-0004 7.4.29.2.3 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a <container> resource and the CSE having created a <contentInstance> resource under <container> resource and the IUT having privileges to perform RETRIEVE operation on <contentInstance> resource and the IUT is triggered to send a retrieve request on the <oldest> virtual resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid RETRIEVE request message containing To set to <oldest> virtual resource and From set to AE_ID and no Content attribute } |  | IUT  CSE |
|  | then { the IUT receives a valid Response message from CSE containing Response Status Code set to 2000 (OK) and Content attribute containing the resource <contentInstance> } |  | IUT  CSE |

| TP Id |  | TP/oneM2M/AE/DMR/RET/BV/00X(TBD-latest) |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a RETRIEVE request of <latest> virtual resource. |  |
| Reference |  | TS-0001 10.2.22.1, TS-0004 7.4.28.2.3 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a <container> resource and the CSE having created a <contentInstance> resource under <container> resource and the IUT having privileges to perform RETRIEVE operation on <contentInstance> resource and the IUT is triggered to send a retrieve request on the <latest> virtual resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid RETRIEVE request message containing To set to <latest> virtual resource and From set to AE_ID and no Content attribute } |  | IUT  CSE |
|  | then { the IUT receives a valid Response message from CSE containing Response Status Code set to 2000 (OK) and Content attribute containing the resource <contentInstance> } |  | IUT  CSE |

| TP Id |  | TP/oneM2M/AE/DMR/DEL/BV/00X(TBD-oldest) |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a DELETE request of <oldest> virtual resource. |  |
| Reference |  | TS-0001 10.2.23.2, TS-0004 7.4.29.2.5 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a <container> resource and the CSE having created a <contentInstance> resource under <container> resource and the IUT having privileges to perform RETRIEVE operation on <contentInstance> resource and the IUT is triggered to send a delete request on the <oldest> virtual resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid DELETE request message containing To set to <oldest> virtual resource and From set to AE_ID and no Content attribute } |  | IUT  CSE |
|  | then { the IUT receives a valid response message from CSE containing Response Status Code set to 2002 (DELETED) } |  | IUT  CSE |

| TP Id |  | TP/oneM2M/AE/DMR/DEL/BV/00X(TBD-latest) |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a DELETE request of <latest> virtual resource. |  |
| Reference |  | TS-0001 10.2.22.2, TS-0004 7.4.28.2.5 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a <container> resource and the CSE having created a <contentInstance> resource under <container> resource and the IUT having privileges to perform RETRIEVE operation on <contentInstance> resource and the IUT is triggered to send a delete request on the <latest> virtual resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT sends a valid DELETE request message containing To set to <latest> virtual resource and From set to AE_ID and no Content attribute } |  | IUT  CSE |
|  | then { the IUT receives a valid response message from CSE containing Response Status Code set to 2002 (DELETED) } |  | IUT  CSE |

## -----------------------End of change 2--------------------------------------------