| Input Contribution |  |
| --- | --- |
| Meeting ID* | TST35.1 |
| Title:* | New Configuration and Notification Test |
| Source:* | In Song Lee, KETI, insong@keti.re.kr Il-Yeop Ahn, KETI, iyahn@keti.re.kr |
| Date:* | 2018-06-18 |
| -Input related to* | TS-0013 v.2.4.0 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Decision requested or recommendation:* | <A concise statement of the decision required or the recommended action to be taken> |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

## Introduction

This contribution is written to fix minor error in notification Interop test cases.

As discussed in TST 35, notification test cases have two AE but the configuration was not fit to the test cases.

New text1 contains new configuration which has two AE and CSE.

Change 1 contains changes to follow new configuration.

Change 2 contains update information of test lists.

New text2 is about notify aggregation test case. This had removed from the last contribution(TST-2018-0078R03) because notifyAggregation was not specified in TS-0004.

As per TS-0001:

#### 10.2.7.11	Subscribe and Un-Subscribe <fanOutPoint> of a group

This procedure shall be used for receiving information about modifications of all member resources belonging to an existing <group> resource.

Table 10.2.7.11-1: <fanOutPoint> Subscribe/Un-subscribe

| <fanOutPoint> Subscribe/Un-subscribe |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or CSE that initiates the request To: The address of the <fanOutPoint> resource appended with the ID of the <subscription> resource to be created Group Request Identifier: The group request identifier |
| Processing at Originator before sending Request | The Originator shall request to create a subscription resource under all member resources belonging to an existing <group> resource by using a CREATE operation. The request may address the virtual child resource <fanOutPoint> of the specific <group> resource of a group Hosting CSE. The request may also address the address that results from appending a relative address to the <fanOutPoint> in order to create the corresponding subscription to the resource represented by the relative address with respect to all member resources. In both cases the targeted resource shall the parent of the newly created <subscription> resource(s). The request shall include notificationForwardingURI attribute if the Originator wants the group Hosting CSE to aggregate the notifications. The request shall include the required information and may include the optional information as described in subscription management clause 10.2.11. The Originator may be an AE or a CSE |
| Processing at Group Hosting CSE | The <group> Hosting CSE shall: - Check if the Originator has CREATE privilege in the <accessControlPolicy> resource referenced by the membersAccessControlPolicyIDs in the group resource. In the case membersAccessControlPolicyIDs is not provided the access control policy defined for the group resource shall be used - If the subscription resource in the request contains an notificationForwardingURI attribute, assign a URI to replace the notificationURI of the subscription resource which will be used to receive notifications from member hosting CSEs. The ID of the <group> resource shall be set to the groupID attribute of the <subscription> resource. The group Hosting CSE shall maintain the mapping of the generated notificationURI and the former notificationURI - Upon successful validation, obtain the IDs of all member resources from the attribute membersIDs of the addressed <group> resource Generate fan out requests addressing the obtained address (appended with the relative address if any) to the member hosting CSEs as indicated in figure 10.2.7.6-1. From parameter in the request is set to ID of the Originator from the request from the original Originator - If the group Hosting CSE determines that multiple members resources belong to one CSE according to the IDs of the member resources, it may converge the requests accordingly before sending out. This may be accomplished by the <group> Hosting CSE creating a <group> resource on the members Hosting CSE to collect all the members on that members Hosting CSE - After receiving the responses from the member hosting CSEs, respond to the Originator with the aggregated results and the associated members list |
| Processing at Member Hosting CSE | For the subscribe/un-subscribe procedure, the Members Hosting CSE shall treat the request received from the group Hosting CSE as a normal SUBSCRIBE request on the addressed member resource as if it comes from the original Originator. Therefore the members Hosting CSE shall: - Check if the request has a group request identifier. Check if the group request - identifier is contained in the requested identifier stored locally. If match is - found, ignore the current request and respond an error. If no match is found, - locally store the group request identifier until the expiration of the request expiration time or local policy - Check if the original Originator has the READ permission on the members resource - Upon successful validation, perform the subscribe procedures for the corresponding type of member resource as described in clause 10.2.12 - Send the corresponding response to the group Hosting CSE |
| Information in Response message | Converged responses from member hosting CSEs |
| Processing at Originator after receiving Response | None |
| Exceptions | - Same request with identical request identifier received - Originator does not have the access control privilege to access the <fanOutPoint> resource |

### -----------------------Start of new text 1-------------------------------------------

#### 7.1.1.3	M2M_CFG_10

oneM2M entities model

### -----------------------End of new text 1---------------------------------------------

### -----------------------Start of change 1-------------------------------------------

#### 8.1.13.2	Update Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_80 |
| Objective: |  |  | AE2 sends maxNrOfInstances UPDATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has specific setting in eventNotificationCriteria, Hosting CSE send notification to AE1 |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 9.6.8 oneM2M TS-0004 [Error: Reference source not found], clause 6.3.4.2.19 |
| Pre-test conditions: |  |  | - CSEBase resource has been created in registrar CSE with name {CSEBaseName} - AE1 has created a <container> resource on registrar CSE - AE1 has created <subscription> resources under the <container> resource. AE1 has set attribute of eventNotificationCriteria to a specific condition (e.g. maxNrOfInstances in this scenario) and notificationEventType set to 1(Update of Resource) - AE2 has permissions to UPDATE the <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Update Request to the <container>. This triggers or causes the Hosting CSE to send a notification to AE1. |
| 2 | Check Mca | PRO Check Primitive | - op = 3 (Update) - to = ID of <container> resource - from = AE2-ID - rqi = (token-string) - pc = Serialized representation of maxNrOfInstances update |
| 3 |  | IOP Check | Hosting CSE successfully updated mxNrOfInsatnce of the <container> resource |
| 4 | Check Mca | PRO Check Primitive | - rsc = 2004 (Updated) - rqi = (token-string) same as received in request message - pc = Serialized representation of <container> resource |
| 5 |  | IOP Check | AE2 successfully received response of Update request |
| 6 | Check Mca | PRO Check Primitive | - op = 5 (Notify) - to = ID of <subscription> resource - from = Registrar CSE-ID - rqi = (token-string) - pc = Serialized representation of Notify request which contain notificationEventType equal to 1(Update of Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Check Mca | PRO Check Primitive | - rsc = 2000 (OK) - rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| 10 | Check Mca | PRO Check Primitive | - op = 3 (Update) - to = ID of <container> resource - from = AE2-ID - rqi = (token-string) - pc = Serialized representation of labels update(can be any other attribute) |
| 11 |  | IOP Check | Hosting CSE successfully updated labels of the <container> resource |
| 12 | Check Mca | PRO Check Primitive | - rsc = 2004 (Updated) - rqi = (token-string) same as received in request message - pc = Serialized representation of <container> resource |
| 13 |  | IOP Check | AE2 successfully received response of Update request, notify request will not be sent |
| IOP Verdict |  | Check that the notification is only sent when there is update of maxNrOfInstances due to setting of the eventNotificationCriteria |  |
| PRO Verdict |  |  |  |

#### 8.1.13.3	Delete Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_81 |
| Objective: |  |  | AE2 sends DELETE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Resource’, Hosting CSE send notification to AE1 |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 9.6.8 oneM2M TS-0004 [Error: Reference source not found], clause 6.3.4.2.19 |
| Pre-test conditions: |  |  | - CSEBase resource has been created in registrar CSE with name {CSEBaseName} - AE1 has created a <container> resource on registrar CSE - AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 2(Delete of Resource) - AE2 has permissions to DELETE the <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Delete Request to the <container>. This triggers or causes the Hosting CSE to send a notification to AE1. |
| 2 | Check Mca | PRO Check Primitive | - op = 4 (Delete) - to = ID of <container> resource - fr = AE2-ID - rqi = (token-string) - pc = empty |
| 3 |  | IOP Check | Hosting CSE successfully deleted the <container> resource |
| 4 | Check Mca | PRO Check Primitive | - rsc = 2002 (DELETED) - rqi = (token-string) same as received in request message - pc = empty |
| 5 |  | IOP Check | AE2 successfully received response of Delete request |
| 6 | Check Mca | PRO Check Primitive | - op = 5 (Notify) - to = ID of <subscription> resource - from = Registrar CSE-ID - rqi = (token-string) - pc = Serialized representation of Notify request which contain notificationEventType equal to 2(Delete of Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Check Mca | PRO Check Primitive | - rsc = 2000 (OK) - rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.13.4	Creation of Direct Child Resource Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_82 |
| Objective: |  |  | AE2 sends <contentInstance> CREATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Create of Direct Child Resource’, Hosting CSE send notification to AE1 |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 9.6.8 oneM2M TS-0004 [Error: Reference source not found], clause 6.3.4.2.19 |
| Pre-test conditions: |  |  | - CSEBase resource has been created in registrar CSE with name {CSEBaseName} - AE1 has created a <container> resource on registrar CSE - AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 3(Create of Direct Child Resource) - AE2 has permissions to CREATE child resource under the <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Create Request of <contentInstance> resource to the <container> resource. This triggers or causes the Hosting CSE to send a notification to AE1 |
| 2 | Check Mca | PRO Check Primitive | - op = 1 (Create) - to = ID of <container> resource - fr = AE2-ID - rqi = (token-string) - ty = 4 (contentInstance) - pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <contentInstance> resource under the <container> resource |
| 4 | Check Mca | PRO Check Primitive | - rsc = 2001 (CREATED) - rqi = (token-string) same as received in request message - pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | AE2 successfully received response of Create request |
| 6 | Check Mca | PRO Check Primitive | - op = 5 (Notify) - to = ID of <subscription> resource - from = Registrar CSE-ID - rqi = (token-string) - pc = Serialized representation of Notify request which contain notificationEventType equal to 3(Create of Direct Child Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Check Mca | PRO Check Primitive | Sent response contains - rsc = 2000 (OK) - rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.13.5	Deletion of Direct Child Resource Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_83 |
| Objective: |  |  | AE2 sends DELETE request to the <contentInstance> which is located under the subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Direct Child Resource’, Hosting CSE send notification to AE1 |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 9.6.8 oneM2M TS-0004 [Error: Reference source not found], clause 6.3.4.2.19 |
| Pre-test conditions: |  |  | - CSEBase resource has been created in registrar CSE with name {CSEBaseName} - AE1 has created a <container> resource on registrar CSE - AE1 has created a <contentInstance> as a child resource of <container> created by AE1 - AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 4(Delete of Direct Child Resource) - AE2 has permissions to DELETE the <contentInstance> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Delete Request to the <contentinstance>. This triggers or causes the Hosting CSE to send a notification to AE1. |
| 2 | Check Mca | PRO Check Primitive | - op = 4 (Delete) - to = ID of <contentInstance> resource - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 |  | IOP Check | Hosting CSE successfully deleted the <contentInstance> resource |
| 4 | Check Mca | PRO Check Primitive | - rsc = 2002 (DELETED) - rqi = (token-string) same as received in request message - pc = empty |
| 5 |  | IOP Check | AE2 successfully received response of Delete request |
| 6 | Check Mca | PRO Check Primitive | - op = 5 (Notify) - to = ID of <subscription> resource - from = Registrar CSE-ID - rqi = (token-string) - pc = Serialized representation of Notify request which contain notificationEventType equal to 4(Delete of Direct Child Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Check Mca | PRO Check Primitive | - rsc = 2000 (OK) - rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

## Tests list

| Nb | Procedure/Resource | TD ID | TD Description |
| --- | --- | --- | --- |
| 1 | CSEBase Management | TD_M2M_NH_01 | AE retrieves the CSEBase resource |
| 2 | RemoteCSE | TD_M2M_NH_02 | Registree CSE registers to Registrar CSE |
| 3 |  | TD_M2M_NH_03 | Registree CSE retrieves RemoteCSE from Registrar CSE |
| 4 |  | TD_M2M_NH_04 | Registree CSE updates RemoteCSE from Registrar CSE |
| 5 |  | TD_M2M_NH_05 | Registree CSE deletes RemoteCSE from Registrar CSE |
| 6 | Application Entity | TD_M2M_NH_06 | AE registers to its registrar CSE via an AE Create Request |
| 7 |  | TD_M2M_NH_07 | AE retrieves <AE> resource via an AE Retrieve Request |
| 8 |  | TD_M2M_NH_08 | AE updates attribute in <AE> resource via an AE Update Request |
| 9 |  | TD_M2M_NH_09 | AE de-registers by deleting <AE> resource via an AE Delete Request |
| 10 | Container | TD_M2M_NH_10 | AE creates a container resource in registrar CSE via a container Create Request |
| 11 |  | TD_M2M_NH_11 | AE retrieves information of a container resource via a container Retrieve Request |
| 12 |  | TD_M2M_NH_12 | AE updates attribute in application resource via a container Update Request |
| 13 |  | TD_M2M_NH_13 | AE deletes a specific container resource via a container Delete Request |
| 14 | ContentInstance | TD_M2M_NH_14 | AE adds a contentInstance resource <contentInstance> to a specific container in Registrar CSE via a contentInstance Create Request and the registrar CSE updates the parent <container> resource with stateTag, and currentNrOfInstances, CurrentByteSize attributes correspondingly |
| 15 |  | TD_M2M_NH_15 | AE retrieves information of a contentInstance resource via a contentInstance Retrieve Request |
| 16 |  | TD_M2M_NH_17 | AE deletes contentInstance resource via a Delete Request and the registrar CSE updates the parent <container> resource with currentNrOfInstances, and CurrentByteSize attribute correspondingly |
| 17 |  | TD_M2M_NH_49 | AE deletes a <latest> resource in a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| 18 |  | TD_M2M_NH_50 | AE deletes a <oldest> resource in a <container> resource and the Registrar CSE points an oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| 19 |  | TD_M2M_NH_51 | AE sends a <contentInstance> CREATE request to a <container> which contains attribute currentNrOfInstances whose value equals to that of maxNrOfInstances and Registrar CSE deletes the oldest <contentInstance> from the parent <container> and then creates the requested <contentInstance> resource |
| 20 |  | TD_M2M_NH_71 | AE retrieves a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| 21 |  | TD_M2M_NH_72 | AE retrieves a <oldest> resource of a <container> and the Registrar CSE points a oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| 22 | Discovery | TD_M2M_NH_18 | AE discovers resources residing in Registrar CSE |
| 23 |  | TD_M2M_NH_19 | AE discovers accessible resources residing in Registrar CSE using the label filter criteria |
| 24 |  | TD_M2M_NH_20 | AE discovers accessible resources residing in Registrar CSE limiting the number of matching resources to the specified value. |
| 25 |  | TD_M2M_NH_21 | AE discovers accessible resources residing in Registrar CSE using multiple Filter Criteria |
| 26 |  | TD_M2M_NH_58 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 1 |
| 27 |  | TD_M2M_NH_59 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 2 |
| 28 |  | TD_M2M_NH_60 | AE1 discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 3 |
| 29 |  | TD_M2M_NH_61 | AE discovers accessible resources residing in Registrar CSE using the offset filter criteria value set to 3 |
| 30 |  | TD_M2M_NH_62 | AE discovers all the accessible resources residing in Registrar CSE using the offset filter criteria |
| 31 | Subscription | TD_M2M_NH_22 | AE creates a subscription to Application Entity resource via subscription Create Request |
| 32 |  | TD_M2M_NH_23 | AE retrieves information about a subscription via subscription Retrieve Request such as expirationTime, labels, etc. |
| 33 |  | TD_M2M_NH_24 | AE updates information about a subscription via subscription Retrieve Request |
| 34 |  | TD_M2M_NH_25 | AE cancels subscription via an subscription Delete Request |
| 35 | AccessControlPolicy | TD_M2M_NH_26 | AE creates an accessControlPolicy resource |
| 36 |  | TD_M2M_NH_27 | AE retrieves accessControlPolicy resource |
| 37 |  | TD_M2M_NH_28 | AE updates attribute in accessControlPolicy resource |
| 38 |  | TD_M2M_NH_29 | AE deletes accessControlPolicy resource |
| 39 |  | TD_M2M_NH_30 | AE delete request is rejected due to accessControlPolicy |
| 40 |  | TD_M2M_NH_73 | AE delete request is rejected due to accessControlPolicy (accessControlOriginators) |
| 41 |  | TD_M2M_NH_74 | AE delete request is allowed due to accessControlPolicy |
| 42 | Group | TD_M2M_NH_31 | AE creates a group resource |
| 43 |  | TD_M2M_NH_32 | AE retrieves group resource |
| 44 |  | TD_M2M_NH_33 | AE updates attribute in group resource |
| 45 |  | TD_M2M_NH_34 | AE deletes group resource |
| 46 | Node | TD_M2M_NH_35 | AE creates a node resource |
| 47 |  | TD_M2M_NH_36 | AE retrieves node resource |
| 48 |  | TD_M2M_NH_37 | AE updates attribute in node resource |
| 49 |  | TD_M2M_NH_38 | AE deletes node resource |
| 50 | PollingChannel | TD_M2M_NH_39 | AE creates a <pollingChannel> resource in registrar CSE via a Create Request |
| 51 |  | TD_M2M_NH_40 | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| 52 |  | TD_M2M_NH_41 | AE updates attribute in pollingChannel resource via a Update Request |
| 53 |  | TD_M2M_NH_42 | AE deletes a pollingChannel resource via a Delete Request |
| 54 |  | TD_M2M_NH_43 | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| 55 | FanoutPoint | TD_M2M_NH_44 | AE creates a <contentInstance> resource in each group member |
| 56 |  | TD_M2M_NH_45 | AE retrieves the <container> resource from in each group member |
| 57 |  | TD_M2M_NH_46 | AE updates an <container> resource of each member resource |
| 58 |  | TD_M2M_NH_47 | AE deletes a <container> of each member |
| 59 | Notification | TD_M2M_NH_48 | AE receives a notification request from the HOST CSE |
| 60 |  | TD_M2M_NH_80 | AE2 sends maxNrOfInstances UPDATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has specific setting in eventNotificationCriteria, Hosting CSE send notification to AE1 |
| 61 |  | TD_M2M_NH_81 | AE2 sends DELETE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Resource’, Hosting CSE send notification to AE1 |
| 62 |  | TD_M2M_NH_82 | AE2 sends <contentInstance> CREATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Create of Direct Child Resource’, Hosting CSE send notification to AE1 |
| 63 |  | TD_M2M_NH_83 | AE2 sends DELETE request to the <contentInstance> which located under the subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Direct Child Resource’, Hosting CSE send notification to AE1 |
| 64 |  | TD_M2M_NH_89 | AE creates <subscription> resources by sending Create Request to the fanOutPoint. Since AE has set notifyAggregation to 2, Hosting CSE aggregate notification and send aggregated notification to AE |
| 65 | FlexContainer | TD_M2M_NH_52 | AE creates a flexcontainer resource in Registrar CSE via a flexcontainer Create Request |
| 66 |  | TD_M2M_NH_53 | AE retrieves information of a flexContainer resource via a flexContainer Retrieve Request |
| 67 |  | TD_M2M_NH_54 | AE updates attribute in application resource via a flexContainer Update Request |
| 68 |  | TD_M2M_NH_55 | AE deletes a specific container resource via a container Delete Request |
| 69 |  | TD_M2M_NH_56 | AE receives a notification request on flexContainer update from the HOST CSE |
| 70 |  | TD_M2M_NH_57 | AE discovers accessible resources residing in Registrar CSE using attribute filter criteria which has a customAttribute name and value assigned to it. |
| 71 | External Management Operations | TD_M2M_NH_63 | AE creates a mgmtCmd resource |
| 72 |  | TD_M2M_NH_64 | AE retrieves mgmtCmd resource |
| 73 |  | TD_M2M_NH_65 | AE updates attribute (not with 'true' in execEnable attribute) in mgmtCmd resource |
| 74 |  | TD_M2M_NH_66 | AE updates attribute (with 'true' in execEnable attribute) in mgmtCmd resource |
| 75 |  | TD_M2M_NH_67 | AE deletes mgmtCmd resource |
| 76 |  | TD_M2M_NH_68 | AE retrieves execInstance resource |
| 77 |  | TD_M2M_NH_69 | AE updates attribute 'execDisable' to true in execInstance resource to cancel pending management command. |
| 78 |  | TD_M2M_NH_70 | AE deletes execInstance resource |
| 79 | SemanticDescriptor Management | TD_M2M_NH_75 | AE creates a SemanticDescriptor resource in Registrar CSE via a SemanticDescriptor Create Request |
| 80 |  | TD_M2M_NH_76 | AE retrieves information of a semanticDescriptor resource via a semanticDescriptor Retrieve Request |
| 81 |  | TD_M2M_NH_77 | AE updates attribute in <semanticDescriptor> resource via a semanticDescriptor Update Request |
| 82 |  | TD_M2M_NH_78 | AE deletes SemanticDescriptor resource via a SemanticDescriptor Delete Request |
| 83 | Semantic Resource Discovery | TD_M2M_NH_79 | AE discovers accessible resources residing in Registrar CSE using the semanticFilter filter criteria |
| 84 | ResultContent | TD_M2M_NH_84 | Check creation of <container> resource with result content set to 0(nothing) |
| 85 |  | TD_M2M_NH_85 | Check creation of <container> resource with result content set to 1(attributes) |
| 86 |  | TD_M2M_NH_86 | Check creation of <container> resource with result content set to 2(hierarchical address) |
| 87 |  | TD_M2M_NH_87 | Check creation of <container> resource with result content set to 3(hierarchical address and attributes) |
| 88 |  | TD_M2M_NH_88 | Check retrievability of <container> resource with result content set to 4(attributes and child resources) |
| 89 | Synchronous request | TD_M2M_NB_01 | AE creates a container resource using non-blocking synchronous request in registrar CSE |
| 90 |  | TD_M2M_NB_02 | AE retrieves a Container resource using non-blocking synchronous request in registrar CSE |
| 91 |  | TD_M2M_NB_03 | AE updates a Container resource using non-blocking synchronous request in registrar CSE |
| 92 |  | TD_M2M_NB_04 | AE deletes a Container resource using non-blocking synchronous request |
| 93 | Asynchronous request | TD_M2M_NB_05 | AE creates a container resource using non-blocking asynchronous request |
| 94 |  | TD_M2M_NB_06 | AE retrieves a Container resource using non-blocking asynchronous request |
| 95 |  | TD_M2M_NB_07 | AE updates a Container resource using non-blocking asynchronous request |
| 96 |  | TD_M2M_NB_08 | AE deletes a Container resource using non-blocking asynchronous request |
| 97 | Retargeting | TD_M2M_SH_01 | AE creates a remote <Resource> resource |
| 98 |  | TD_M2M_SH_02 | AE retrieves a remote <Resource> resource |
| 99 |  | TD_M2M_SH_03 | AE updates a remote <Resource> resource |
| 100 |  | TD_M2M_SH_04 | AE delete a remote <Resource> resource |
| 101 | Discovery | TD_M2M_SH_09 | AE discovers accessible resources residing in the remote Hosting CSE using multiple Filter Criteria |
| 102 | Unauthorized operation | TD_M2M_SH_10 | AE delete request is rejected after access rights verification using retargeting. |
| 103 | Notification | TD_M2M_SH_11 | AE receives a notification request from the remote hosting CSE |
| 104 | mgmtObj | TD_M2M_SH_05 | AE creates a <mgmtObj> resource |
| 105 |  | TD_M2M_SH_06 | AE updates a <mgmtObj> resource |
| 106 |  | TD_M2M_SH_07 | AE retrieves a <mgmtObj> resource |
| 107 |  | TD_M2M_SH_08 | AE deletes a <mgmtObj> resource |
| 108 | Announcement | TD_M2M_SH_12 | AE1 announces itself to CSE2 |
| 109 |  | TD_M2M_SH_13 | AE1 announces a child container to CSE2 |
| 110 |  | TD_M2M_SH_14 | AE1 announces an Optional Announce attribute to CSE2 |
| 111 |  | TD_M2M_SH_15 | AE2 retrieves an Announced Resource |
| 112 |  | TD_M2M_SH_16 | AE2 retrieves the original resource representation of an announced resource |
| 113 | fanOut | TD_M2M_SH_17 | AE creates a <contentInstance> resource in each group member, where some memberIDs are on a remoteCSE |
| 114 |  | TD_M2M_SH_18 | AE retrieves a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| 115 |  | TD_M2M_SH_19 | AE updates a <container> resource in each group member, where some memberIDs are on a remoteCSE |
| 116 |  | TD_M2M_SH_20 | AE deletes a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| 117 | Secure AE Registration | TD_M2M_SE_01 | AE uses Provisioned Symmetric Key Security Association Establishment Framework to enable mutual authentication with the Registrar CSE. Registrar CSE performs AE authorization check on incoming AE registration request. |

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of new text 2-------------------------------------------

#### 8.1.13.6	Notifcation Aggregation

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_89 |
| Objective: |  |  | AE creates <subscription> resources by sending Create Request to the fanOutPoint. Since AE has set notifyAggregation to 2, Hosting CSE aggregate notification and send aggregated notification to AE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.7.11 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.14.2.4 |
| Pre-test conditions: |  |  | - CSEBase resource has been created in registrar CSE with name {CSEBaseName} - AE has created two <container> resources on registrar CSE - AE has created a <group> resource with memberIDs set to two <container> resources. AE has set number in notifyAggregation to 2 - AE has created <subscription> resources under the members of <group> resources by sending Create Request to the fanOutPoint. AE has set notificationForwardingURI and notificationEventType set to 3(Create of Direct Child Resource) |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request of <contentInstance> resource to the fanOutPoint |
| 2 | Check Mca | PRO Check Primitive | - op = 1 (Create) - to = {CSEBaseName}/{group}/fopt - fr = AE-ID - rqi = (token-string) - ty = 4 (contentInstance) - pc = Serialized representation of <subscription> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <contentInstance> resources in each member |
| 4 | Check Mca | PRO Check Primitive | - rsc = 2000 (OK) - rqi = (token-string) same as received in request message - pc = aggregated response |
| 5 |  | IOP Check | AE successfully received converged response |
| 6 | Check Mca | PRO Check Primitive | - op = 5 (Notify) - to = ID of <subscription> resource - from = Registrar CSE-ID - rqi = (token-string) - pc = aggregated Notify request which contain the occurrence of child resource creation |
| 7 |  | IOP Check | AE successfully received aggregated Notify request |
| 8 | Check Mca | PRO Check Primitive | - rsc = 2000 (OK) - rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Group Hosting CSE successfully received response of Notify request |
| IOP Verdict |  | Check that the response is aggregated by the group Hosting CSE and successfully parse to AE |  |
| PRO Verdict |  |  |  |

### -----------------------End of new text 2---------------------------------------------