| Input Contribution |  |
| --- | --- |
| Meeting ID* | TST35.1 |
| Title:* | Interop Test Cases for timeSeries and timeSeriesInstance |
| Source:* | In Song Lee, KETI, insong@keti.re.kr NakMyoung Sung, KETI, nmsung@keti.re.kr |
| Date:* | 2018-06-18 |
| -Input related to* | TS-0013 v.2.4.0 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Decision requested or recommendation:* | <A concise statement of the decision required or the recommended action to be taken> |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution is written for <timeSeries> resource and <timeSeriesInstance> resource Management test cases.

Test cases are made based on the CRUD function.

8.1.19.1 timeSeries Create

8.1.19.2 timeSeries Retrieve

8.1.19.3 timeSeries Update

8.1.19.4 timeSeries Delete

8.1.20.1 timeSeriesInstance Create

8.1.20.2 timeSeriesInstance Retrieve

8.1.20.3 timeSeriesInstance Delete

8.1.20.4 timeSeriesInstance Create when currentNrOfInstance equals to maxNrOfInstances in parent <timeSeries> resource

New test cases which focus on the function of the resources may be proposed before the Interop#6.

As per TS-0004:

#### <timeSeriesInstance> resource specific procedure on CRUD operations

##### Introduction

This clause describes <timeSeriesInstance> resource specific behaviour for CRUD operations.

##### 7.4.39.2.1 Create

Originator:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

The Originator shall maintain an internal counter to generate sequenceNr which is increased by one. When the sequenceNr reaches to the maxNrOfInstances of the direct parent <timeSeries> resource, it shall be set to one.

Receiver:

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

- Steps for Create procedure of <timeSeriesInstance> shall be same as that steps of <contentInstance> resource described in clause , except <container> resource in that procedure would correspond to <timeSeries> resource and <contentInstance> resource would correspond to <timeSeriesInstance> resource.

#### <contentInstance> resource specific procedure on CRUD operations

##### 7.4.7.2.1 Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" with the following additional operations:

- The Hosting CSE shall check whether the size in bytes of the content attribute of the <contentInstance> resource is greater than maxByteSize of the targeted parent <container> resource:

a)	If true, the Hosting CSE shall return the response primitive with a Response Status Code indicating "NOT_ACCEPTABLE" error. Skip steps 2 and 3 below.

b)	If false, the Hosting CSE shall set the contentSize attribute of the <contentInstance> resource to the size in bytes of the content attribute.

- The Hosting CSE shall check the currentNrOfInstances and currentByteSize of the targeted parent <container> resource:

a)	If maxNrOfInstances of the targeted parent <container> resource is specified then if the currentNrOfInstances when modified to reflect the addition of the new <contentInstance> exceeds maxNrOfInstances, the Hosting CSE shall remove the oldest <contentInstance> resource from the targeted <container> resource.

b)	If maxByteSize of the targeted parent <container> resource is specified then if the currentByteSize when modified to reflect the addition of the new <contentInstance> exceeds maxByteSize the Hosting CSE shall remove the oldest <contentInstance> resources from the targeted <container> resource until maxByteSize conditions are met.

c)	The Hosting CSE shall update the currentNrOfInstances of the targeted parent <container> resource with the count of <contentInstance> resources in the targeted parent <container> resource. The Hosting CSE shall update the currentByteSize of the targeted parent <container> resource with the sum of the contentSize attributes of the <contentInstance> resources in the targeted parent <container> resource.

d)	When removing oldest <contentInstance> resources, the Hosting CSE shall not generate notifications even if there exists a <subscription> to the targeted <container> resource and this <subscription> is configured to generate a notification on "Delete_of_Direct_Child_Resource".

e)	If maxInstanceAge attribute is present in targeted parent <container> resource, then the Hosting CSE shall set the expirationTime attribute in <contentInstance> resource such that time difference between expirationTime and creationTime of <contentInstance> resource shall not exceed maxInstanceAge of targeted parent <container> resource.

- The Hosting CSE shall increment the stateTag attribute of the targeted parent <container> resource and copy the value into the stateTag attribute of the <contentInstance> resource.

No other changes from the generic procedures in clause Error: Reference source not found.

##### Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.39.2.3 Update

Originator:

The <timeSeriesInstance> resource shall not be Updated via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message".

If the request is received, the Receiver CSE shall execute the following steps in order:

- "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

##### 7.4.39.2.4 Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

- currentNrOfInstances and currentByteSize of direct parent <timeSeries> resource shall be updated.

As per TS-0001:

Table 10.2.31.1-1: <timeSeriesInstance> CREATE

| <timeSereisInstance> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in table 8.1.2-2 apply with the specific details for: - Content: The resource content shall provide the information as defined in clause 9.6.37. |
| Processing at Originator before sending Request | According to clause 10.1.1.1. |
| Processing at Receiver | According to clause 10.1.1.1. If the newly created <timeSeriesInstance> resource violates any of the policies defined in the parent <timeSeries> resource (i.e. maxInstanceAge,maxNrOfInstances or maxByteSize), then the <timeSeriesInstance> resource with the oldest dataGenerationTime attribute shall be removed to enable the creation of the new <timeSeriesInstance> resource. The Create Request of the other entities except the creator, shall be rejected. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: Content: Address of the created <timeSeriesInstance> resource, according to clause 10.1.1.1. |
| Processing at Originator after receiving Response | According to clause 10.1.1.1. |
| Exceptions | According to clause 10.1.1.1. |

### -----------------------Start of change 1-------------------------------------------

## Tests list

| Nb | Procedure/Resource | TD ID | TD Description |
| --- | --- | --- | --- |
| 1 | CSEBase Management | TD_M2M_NH_01 | AE retrieves the CSEBase resource |
| 2 | RemoteCSE | TD_M2M_NH_02 | Registree CSE registers to Registrar CSE |
| 3 |  | TD_M2M_NH_03 | Registree CSE retrieves RemoteCSE from Registrar CSE |
| 4 |  | TD_M2M_NH_04 | Registree CSE updates RemoteCSE from Registrar CSE |
| 5 |  | TD_M2M_NH_05 | Registree CSE deletes RemoteCSE from Registrar CSE |
| 6 | Application Entity | TD_M2M_NH_06 | AE registers to its registrar CSE via an AE Create Request |
| 7 |  | TD_M2M_NH_07 | AE retrieves <AE> resource via an AE Retrieve Request |
| 8 |  | TD_M2M_NH_08 | AE updates attribute in <AE> resource via an AE Update Request |
| 9 |  | TD_M2M_NH_09 | AE de-registers by deleting <AE> resource via an AE Delete Request |
| 10 | Container | TD_M2M_NH_10 | AE creates a container resource in registrar CSE via a container Create Request |
| 11 |  | TD_M2M_NH_11 | AE retrieves information of a container resource via a container Retrieve Request |
| 12 |  | TD_M2M_NH_12 | AE updates attribute in application resource via a container Update Request |
| 13 |  | TD_M2M_NH_13 | AE deletes a specific container resource via a container Delete Request |
| 14 | ContentInstance | TD_M2M_NH_14 | AE adds a contentInstance resource <contentInstance> to a specific container in Registrar CSE via a contentInstance Create Request and the registrar CSE updates the parent <container> resource with stateTag, and currentNrOfInstances, CurrentByteSize attributes correspondingly |
| 15 |  | TD_M2M_NH_15 | AE retrieves information of a contentInstance resource via a contentInstance Retrieve Request |
| 16 |  | TD_M2M_NH_17 | AE deletes contentInstance resource via a Delete Request and the registrar CSE updates the parent <container> resource with currentNrOfInstances, and CurrentByteSize attribute correspondingly |
| 17 |  | TD_M2M_NH_49 | AE deletes a <latest> resource in a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| 18 |  | TD_M2M_NH_50 | AE deletes a <oldest> resource in a <container> resource and the Registrar CSE points an oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| 19 |  | TD_M2M_NH_51 | AE sends a <contentInstance> CREATE request to a <container> which contains attribute currentNrOfInstances whose value equals to that of maxNrOfInstances and Registrar CSE deletes the oldest <contentInstance> from the parent <container> and then creates the requested <contentInstance> resource |
| 20 |  | TD_M2M_NH_71 | AE retrieves a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| 21 |  | TD_M2M_NH_72 | AE retrieves a <oldest> resource of a <container> and the Registrar CSE points a oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| 22 | Discovery | TD_M2M_NH_18 | AE discovers resources residing in Registrar CSE |
| 23 |  | TD_M2M_NH_19 | AE discovers accessible resources residing in Registrar CSE using the label filter criteria |
| 24 |  | TD_M2M_NH_20 | AE discovers accessible resources residing in Registrar CSE limiting the number of matching resources to the specified value. |
| 25 |  | TD_M2M_NH_21 | AE discovers accessible resources residing in Registrar CSE using multiple Filter Criteria |
| 26 |  | TD_M2M_NH_58 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 1 |
| 27 |  | TD_M2M_NH_59 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 2 |
| 28 |  | TD_M2M_NH_60 | AE1 discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 3 |
| 29 |  | TD_M2M_NH_61 | AE discovers accessible resources residing in Registrar CSE using the offset filter criteria value set to 3 |
| 30 |  | TD_M2M_NH_62 | AE discovers all the accessible resources residing in Registrar CSE using the offset filter criteria |
| 31 | Subscription | TD_M2M_NH_22 | AE creates a subscription to Application Entity resource via subscription Create Request |
| 32 |  | TD_M2M_NH_23 | AE retrieves information about a subscription via subscription Retrieve Request such as expirationTime, labels, etc. |
| 33 |  | TD_M2M_NH_24 | AE updates information about a subscription via subscription Retrieve Request |
| 34 |  | TD_M2M_NH_25 | AE cancels subscription via an subscription Delete Request |
| 35 | AccessControlPolicy | TD_M2M_NH_26 | AE creates an accessControlPolicy resource |
| 36 |  | TD_M2M_NH_27 | AE retrieves accessControlPolicy resource |
| 37 |  | TD_M2M_NH_28 | AE updates attribute in accessControlPolicy resource |
| 38 |  | TD_M2M_NH_29 | AE deletes accessControlPolicy resource |
| 39 |  | TD_M2M_NH_30 | AE delete request is rejected due to accessControlPolicy |
| 40 |  | TD_M2M_NH_73 | AE delete request is rejected due to accessControlPolicy (accessControlOriginators) |
| 41 |  | TD_M2M_NH_74 | AE delete request is allowed due to accessControlPolicy |
| 42 | Group | TD_M2M_NH_31 | AE creates a group resource |
| 43 |  | TD_M2M_NH_32 | AE retrieves group resource |
| 44 |  | TD_M2M_NH_33 | AE updates attribute in group resource |
| 45 |  | TD_M2M_NH_34 | AE deletes group resource |
| 46 | Node | TD_M2M_NH_35 | AE creates a node resource |
| 47 |  | TD_M2M_NH_36 | AE retrieves node resource |
| 48 |  | TD_M2M_NH_37 | AE updates attribute in node resource |
| 49 |  | TD_M2M_NH_38 | AE deletes node resource |
| 50 | PollingChannel | TD_M2M_NH_39 | AE creates a <pollingChannel> resource in registrar CSE via a Create Request |
| 51 |  | TD_M2M_NH_40 | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| 52 |  | TD_M2M_NH_41 | AE updates attribute in pollingChannel resource via a Update Request |
| 53 |  | TD_M2M_NH_42 | AE deletes a pollingChannel resource via a Delete Request |
| 54 |  | TD_M2M_NH_43 | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| 55 | FanoutPoint | TD_M2M_NH_44 | AE creates a <contentInstance> resource in each group member |
| 56 |  | TD_M2M_NH_45 | AE retrieves the <container> resource from in each group member |
| 57 |  | TD_M2M_NH_46 | AE updates an <container> resource of each member resource |
| 58 |  | TD_M2M_NH_47 | AE deletes a <container> of each member |
| 59 | Notification | TD_M2M_NH_48 | AE receives a notification request from the HOST CSE |
| 60 |  | TD_M2M_NH_80 | AE2 sends maxNrOfInstances UPDATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has specific setting in eventNotificationCriteria, Hosting CSE send notification to AE1 |
| 61 |  | TD_M2M_NH_81 | AE2 sends DELETE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Resource’, Hosting CSE send notification to AE1 |
| 62 |  | TD_M2M_NH_82 | AE2 sends <contentInstance> CREATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Create of Direct Child Resource’, Hosting CSE send notification to AE1 |
| 63 |  | TD_M2M_NH_83 | AE2 sends DELETE request to the <contentInstance> which located under the subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Direct Child Resource’, Hosting CSE send notification to AE1 |
| 64 |  | TD_M2M_NH_89 | AE creates <subscription> resources by sending Create Request to the fanOutPoint. Since AE has set notifyAggregation to 2, Hosting CSE aggregate notification and send aggregated notification to AE |
| 65 | FlexContainer | TD_M2M_NH_52 | AE creates a flexcontainer resource in Registrar CSE via a flexcontainer Create Request |
| 66 |  | TD_M2M_NH_53 | AE retrieves information of a flexContainer resource via a flexContainer Retrieve Request |
| 67 |  | TD_M2M_NH_54 | AE updates attribute in application resource via a flexContainer Update Request |
| 68 |  | TD_M2M_NH_55 | AE deletes a specific container resource via a container Delete Request |
| 69 |  | TD_M2M_NH_56 | AE receives a notification request on flexContainer update from the HOST CSE |
| 70 |  | TD_M2M_NH_57 | AE discovers accessible resources residing in Registrar CSE using attribute filter criteria which has a customAttribute name and value assigned to it. |
| 71 | External Management Operations | TD_M2M_NH_63 | AE creates a mgmtCmd resource |
| 72 |  | TD_M2M_NH_64 | AE retrieves mgmtCmd resource |
| 73 |  | TD_M2M_NH_65 | AE updates attribute (not with 'true' in execEnable attribute) in mgmtCmd resource |
| 74 |  | TD_M2M_NH_66 | AE updates attribute (with 'true' in execEnable attribute) in mgmtCmd resource |
| 75 |  | TD_M2M_NH_67 | AE deletes mgmtCmd resource |
| 76 |  | TD_M2M_NH_68 | AE retrieves execInstance resource |
| 77 |  | TD_M2M_NH_69 | AE updates attribute 'execDisable' to true in execInstance resource to cancel pending management command. |
| 78 |  | TD_M2M_NH_70 | AE deletes execInstance resource |
| 79 | SemanticDescriptor Management | TD_M2M_NH_75 | AE creates a SemanticDescriptor resource in Registrar CSE via a SemanticDescriptor Create Request |
| 80 |  | TD_M2M_NH_76 | AE retrieves information of a semanticDescriptor resource via a semanticDescriptor Retrieve Request |
| 81 |  | TD_M2M_NH_77 | AE updates attribute in <semanticDescriptor> resource via a semanticDescriptor Update Request |
| 82 |  | TD_M2M_NH_78 | AE deletes SemanticDescriptor resource via a SemanticDescriptor Delete Request |
| 83 | Semantic Resource Discovery | TD_M2M_NH_79 | AE discovers accessible resources residing in Registrar CSE using the semanticFilter filter criteria |
| 84 | ResultContent | TD_M2M_NH_84 | Check creation of <container> resource with result content set to 0(nothing) |
| 85 |  | TD_M2M_NH_85 | Check creation of <container> resource with result content set to 1(attributes) |
| 86 |  | TD_M2M_NH_86 | Check creation of <container> resource with result content set to 2(hierarchical address) |
| 87 |  | TD_M2M_NH_87 | Check creation of <container> resource with result content set to 3(hierarchical address and attributes) |
| 88 |  | TD_M2M_NH_88 | Check retrievability of <container> resource with result content set to 4(attributes and child resources) |
| 89 | timeSeries | TD_M2M_NH_90 | AE creates a <timeSeries> resource in registrar CSE via a Create Request |
| 90 |  | TD_M2M_NH_91 | AE retrieves information of a <timeSeries> resource via a Retrieve Request |
| 91 |  | TD_M2M_NH_92 | AE updates attribute in <timeSeries> resource via a Update Request |
| 92 |  | TD_M2M_NH_93 | AE deletes a <timeSeries> resource via a Delete Request |
| 93 | timeSeriesInstance | TD_M2M_NH_94 | AE sends Create Request of a <timeSeriesInstance> resource to a <timeSeries> resource in Registrar CSE. Registrar CSE creates the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| 94 |  | TD_M2M_NH_95 | AE retrieves information of a <timeSeriesInstance> resource via a Retrieve Request |
| 95 |  | TD_M2M_NH_96 | AE sends Delete Request of a <timeSeriesInstance> resource in Registrar CSE. Registrar CSE delete the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| 96 |  | TD_M2M_NH_97 | AE sends a <timeSeriesInstance> resource Create Request to a <timeSeries> resource which contains currentNrOfInstances value equals to that of maxNrOfInstances and Registrar CSE delets the oldest <timeSeriesInstance> resource from the <timeSeries> resource and then creates the requested <timeSeriesInstance> resource |
| 97 | Synchronous request | TD_M2M_NB_01 | AE creates a container resource using non-blocking synchronous request in registrar CSE |
| 98 |  | TD_M2M_NB_02 | AE retrieves a Container resource using non-blocking synchronous request in registrar CSE |
| 99 |  | TD_M2M_NB_03 | AE updates a Container resource using non-blocking synchronous request in registrar CSE |
| 100 |  | TD_M2M_NB_04 | AE deletes a Container resource using non-blocking synchronous request |
| 101 | Asynchronous request | TD_M2M_NB_05 | AE creates a container resource using non-blocking asynchronous request |
| 102 |  | TD_M2M_NB_06 | AE retrieves a Container resource using non-blocking asynchronous request |
| 103 |  | TD_M2M_NB_07 | AE updates a Container resource using non-blocking asynchronous request |
| 104 |  | TD_M2M_NB_08 | AE deletes a Container resource using non-blocking asynchronous request |
| 105 | Retargeting | TD_M2M_SH_01 | AE creates a remote <Resource> resource |
| 106 |  | TD_M2M_SH_02 | AE retrieves a remote <Resource> resource |
| 107 |  | TD_M2M_SH_03 | AE updates a remote <Resource> resource |
| 108 |  | TD_M2M_SH_04 | AE delete a remote <Resource> resource |
| 109 | Discovery | TD_M2M_SH_09 | AE discovers accessible resources residing in the remote Hosting CSE using multiple Filter Criteria |
| 110 | Unauthorized operation | TD_M2M_SH_10 | AE delete request is rejected after access rights verification using retargeting. |
| 111 | Notification | TD_M2M_SH_11 | AE receives a notification request from the remote hosting CSE |
| 112 | mgmtObj | TD_M2M_SH_05 | AE creates a <mgmtObj> resource |
| 113 |  | TD_M2M_SH_06 | AE updates a <mgmtObj> resource |
| 114 |  | TD_M2M_SH_07 | AE retrieves a <mgmtObj> resource |
| 115 |  | TD_M2M_SH_08 | AE deletes a <mgmtObj> resource |
| 116 | Announcement | TD_M2M_SH_12 | AE1 announces itself to CSE2 |
| 117 |  | TD_M2M_SH_13 | AE1 announces a child container to CSE2 |
| 118 |  | TD_M2M_SH_14 | AE1 announces an Optional Announce attribute to CSE2 |
| 119 |  | TD_M2M_SH_15 | AE2 retrieves an Announced Resource |
| 120 |  | TD_M2M_SH_16 | AE2 retrieves the original resource representation of an announced resource |
| 121 | fanOut | TD_M2M_SH_17 | AE creates a <contentInstance> resource in each group member, where some memberIDs are on a remoteCSE |
| 122 |  | TD_M2M_SH_18 | AE retrieves a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| 123 |  | TD_M2M_SH_19 | AE updates a <container> resource in each group member, where some memberIDs are on a remoteCSE |
| 124 |  | TD_M2M_SH_20 | AE deletes a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| 125 | Secure AE Registration | TD_M2M_SE_01 | AE uses Provisioned Symmetric Key Security Association Establishment Framework to enable mutual authentication with the Registrar CSE. Registrar CSE performs AE authorization check on incoming AE registration request. |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of new text 1-------------------------------------------

### 8.1.19	timeSeries Management

#### 8.1.19.1	timeSeries Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_90 |
| Objective: |  |  | AE creates a <timeSeries> resource in registrar CSE via a Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.30.1 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.38.2.1 |
| Pre-test conditions: |  |  | - AE has created a <AE> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | - op = 1 (Create) - to = {CSEBaseName}/URI of <AE> resource - fr = AE-ID - rqi = (token-string) - ty = 29 (timeSeries) - pc = Serialized representation of <timeSeries> resource |
| 3 |  | IOP Check | Registrar CSE successfully created the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | - rsc = 2001 (CREATED) - rqi = (token-string) same as received in request message - pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.2	timeSeries Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_91 |
| Objective: |  |  | AE retrieves information of a <timeSeries> resource via a Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.30.2 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.38.2.2 |
| Pre-test conditions: |  |  | - AE has created a <AE> resource on Registrar CSE - AE has created a <timeSereis> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | - op = 2 (Retrieve) - to = {CSEBaseName}/URI of <timeSeries> resource - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully received Retrieve Request of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | - rsc =2000 (OK) - rqi = (token-string) same as received in request message - pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Retrieve Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.3	timeSeries Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_92 |
| Objective: |  |  | AE updates attribute in <timeSeries> resource via a Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.30.3 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.38.2.3 |
| Pre-test conditions: |  |  | - AE has created a <AE> resource on Registrar CSE - AE has created a <timeSereis> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Update Request to the <timeSeries> resource to update the maxNrOfInstances |
| 2 | Mca | PRO Check Primitive | - op = 3 (Update) - to = {CSEBaseName}/URI of <timeSeries> resource - fr = AE-ID - rqi = (token-string) - pc = Serialized representation of maxNrOfInstances update(can be any other attribute) |
| 3 |  | IOP Check | Registrar CSE successfully updated the maxNrOfInstances of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | - rsc = 2004 (Updated) - rqi = (token-string) same as received in request message - pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Update Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.4	timeSeries Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_93 |
| Objective: |  |  | AE deletes a <timeSeries> resource via a Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.30.4 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.38.2.4 |
| Pre-test conditions: |  |  | - AE has created a <AE> resource on Registrar CSE - AE has created a <timeSeries> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Delete Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | - op = 4 (Delete) - to = {CSEBaseName}/URI of <timeSeries> resource - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully deleted the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | - rsc = 2002 (DELETED) - rqi = (token-string) same as received in request message - pc = empty |
| 5 |  | IOP Check | AE successfully received response of Delete Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.20	timeSeriesInstance Management

#### 8.1.20.1	timeSeriesInstance Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_94 |
| Objective: |  |  | AE sends Create Request of a <timeSeriesInstance> resource to a <timeSeries> resource in Registrar CSE. Registrar CSE creates the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.31.1 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.39.2.1 |
| Pre-test conditions: |  |  | - AE has created a <AE> resource on Registrar CSE - AE has created a <timeSeries> resource on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | - op = 1 (Create) - to = {CSEBaseName}/URI of <timeSeries> resource - fr = AE-ID - rqi = (token-string) - ty = 30 (timeSeriesInstance) - pc = Serialized representation of <timeSeriesInstance> resource |
| 3 |  | IOP Check | Registrar CSE successfully created <timeSeriesInstance> resource and updated currentNrOfInstances and currentByteSize of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | - rsc = 2001 (CREATED) - rqi = (token-string) same as received in request message - pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.20.2	timeSeriesInstance Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_95 |
| Objective: |  |  | AE retrieves information of a <timeSeriesInstance> resource via a Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.31.2 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.39.2.2 |
| Pre-test conditions: |  |  | - AE has created a <AE> resource on Registrar CSE - AE has created a <timeSeries> resource on registrar CSE - AE has created a <timeSeriesInstance> resource as child resource of the <timeSeries> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | - op = 2 (Retrieve) - to = {CSEBaseName}/URI of <timeSeriesInstance> resource - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully received Retrieve Request of the <timeSeriesInstance> resource |
| 4 | Mca | PRO Check Primitive | - rsc =2000 (OK) - rqi = (token-string) same as received in request message - pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Retrieve Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.20.3	timeSeriesInstance Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_96 |
| Objective: |  |  | AE sends Delete Request of a <timeSeriesInstance> resource in Registrar CSE. Registrar CSE delete the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.31.4 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.39.2.4 |
| Pre-test conditions: |  |  | - AE has created a <AE> resource on Registrar CSE - AE has created a <timeSeries> resource on registrar CSE - AE has created a <timeSeriesInstance> resource as child resource of the <timeSeries> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Delete Requeest a <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | - op = 4 (Delete) - to = {CSEBaseName}/URI of <timeSeriesInstance> resource - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully deleted <timeSeriesInstance> resource and updated currentNrOfInstances and currentByteSize of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | - rsc = 2002 (DELETED) - rqi = (token-string) same as received in request message - pc = empty |
| 5 |  | IOP Check | AE successfully received response of Delete Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.20.4	timeSeriesInstance Create when currentNrOfInstance equals to maxNrOfInstances in parent <timeSeries> resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_97 |
| Objective: |  |  | AE sends a <timeSeriesInstance> resource Create Request to a <timeSeries> resource which contains currentNrOfInstances value equals to that of maxNrOfInstances and Registrar CSE delets the oldest <timeSeriesInstance> resource from the <timeSeries> resource and then creates the requested <timeSeriesInstance> resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.31.1 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.38.2.1 |
| Pre-test conditions: |  |  | - AE has created a <AE> resource on Registrar CSE - AE has created a <timeSeries> resource on registrar CSE - AE has created several <timeSeriesInstance> resources and currentNrOfInstances of the <timeSeries> resource reach the maxNrOfInstances |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | - op = 1 (Create) - to = {CSEBaseName}/URI of <timeSeries> resource - fr = AE-ID - rqi = (token-string) - ty = 30 (timeSeriesInstance) - pc = Serialized representation of <timeSeriesInstance> resource |
| 3 |  | IOP Check | Registrar CSE successfully deleted <timeSeriesInstance> resource with the oldest dataGenerationTime and created <timeSeriesInstance> resource |
| 4 | Mca | PRO Check Primitive | - rsc = 2001 (CREATED) - rqi = (token-string) same as received in request message - pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### -----------------------End of new text 1---------------------------------------------