| Input Contribution |  |
| --- | --- |
| Meeting ID* | TST 35.1a |
| Title:* | TS-0013_Interop Test Cases for locationPolicy |
| Source:* | Preeti Rani (preetir@cdot.in), Suman Sheoran (ssheoran@cdot.in) |
| Date:* | 2018-06-18 |
| Input related to* | TS-0013 v.2.4.0 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Decision requested or recommendation:* | <A concise statement of the decision required or the recommended action to be taken> |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution proposes interop test cases for the <locationPolicy> resource management as a preparation of Interop#6 Event. Test cases are made based on the CRUD Operations.

Below test scenarios are newly proposed in this contribution.

- 8.1.19.1 LocationPolicy Create

- 8.1.19.2 LocationPolicy Retrieve

- 8.1.19.3 LocationPolicy Update

- 8.1.19.4 LocationPolicy Delete

- Section 8.1.5.7 Retrieve <latest> test case is further categorised based on the attribute locationID of the parent <container> resource. A new test case 8.1.5.7.1 is added for the case when locationID is configured and test case 8.1.5.7 is renumbered to 8.1.5.7.2.

As per TS-0001,

## 10.2.10	Location Management Procedures

#### 10.2.10.1	Procedure related to <locationPolicy> resource

##### 10.2.10.1.0	Overview

This clause introduces the procedures for obtaining and managing a target M2M Node's location information, which are associated with the <locationPolicy> resource that contains the method for obtaining and managing location information.

##### 10.2.10.1.1	Create <locationPolicy>

This procedure shall be used for creating a <locationPolicy> resource.

Table 10.2.10.1.1-1: <locationPolicy> CREATE

| <locationPolicy> CREATE |  |  |
| --- | --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |  |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: the address of the <CSEBase> resource Content: The representation of the <locationPolicy> resource described in clause 9.6.10 |  |
| Processing at Originator before sending Request | According to clause 10.1.1.1 |  |
| Processing at Receiver | - Check whether the Originator is authorized to request the procedure - Check whether the provided attributes of the <locationPolicy> resource represent a valid Request - Upon successful validation of the above procedures, the Hosting CSE creates <container> resource and a <locationPolicy> resource. Both of these resources shall be hosted locally on the Hosting CSE. The Hosting CSE shall maintain cross-reference between both resources: locationContainerID attribute for <locationPolicy> resource and locationID attribute for <container> resource - Check the defined locationSource attribute to determine which method is used. The locationSource attribute shall be set based on the capabilities of a target M2M Node, the required location accuracy of the Originator and the Underlying Network in which a target M2M Node resides: - For the Network-based case, if the locationServer is absent in the Originator's request the Hosting CSE shall either derive the locationServer value from the locationTargetID or be pre-provisioned with the identity of a locationServer. The Hosting CSE shall transform the request from the Originator into a Location Server request that includes the following attributes locationTargetID, locationServer that are defined in the <locationPolicy> resource. Additionally, the Hosting CSE shall also provide default values for other parameters (e.g. required quality of position) in the Location Server request [Error: Reference source not found] according to local policies. If the request which requests the location information of the target device towards the Location Server crosses over the Mcn reference point, then the Location Server in the Underlying Network performs positioning procedures, and returns the results over the Mcn reference point - The specific mechanism used to communicate with the network Location Server depends on the capabilities of the Underlying Network and other factors. For example, it could be either the OMA Mobile Location Protocol [Error: Reference source not found] or OMA RESTful NetAPI for Terminal Location [Error: Reference source not found] Check the assigned locationInformationType attribute and if the value of this attribute is Geo-fence event, following the steps below: The Hosting CSE shall check the target Node's capability (Positionable, Non-Positionable or both) by retrieving the stored <node> resource or <mgmtCmd> procedure(e.g. checking the Node's capability through RPC-based procedure) and the Hosting CSE shall create <mgmtCmd> resource type with appropriate configuration based on the node capability and attributes stored in the created <locationPolicy> resource (e.g. locationUpdatePeriod attribute of <locationPoicy> to execFrequency attribute of <mgmtCmd>) to obtain the Geo-fence relevant information (e.g. measurement or position fix) from the target Node. The node shall respond the information and the Hosting CSE shall create <execInstance> resource type as a placeholder for the information. The Hosting CSE shall forward this information to Geo-Fence Server (refer to locationServer attribute) and returns the results (e.g. event type) over the Mcn reference point. The result shall be stored in the created <container> resource as explained in clause 10.2.10.2.1. (See note) - For the Device-based case, this case is applicable if the Originator is ASN-AE and the ASN has location determination capabilities (e.g. GPS). The Hosting CSE is capable of performing positioning procedure using the module or technologies. For example, if the ASN has a GPS module itself, the ASN-CSE obtains the location information of Node from the GPS module through internal interfaces (e.g. System call or JNI [Error: Reference source not found]). The detail procedure is out-of-scope - For the Sharing-based case, this case shall be applicable if the Originator is an ADN-AE and the Hosting CSE is MN CSE and the ADN is a resource constrained node, no location determination capabilities (e.g. GPS) and Network-based positioning capabilities. Also according to the required location accuracy of the AE, the Originator may choose this case When the Hosting CSE receives the CREATE request and if the Hosting CSE can find the closest Node that is registered with the Hosting CSE and has location information from the Originator in the M2M Area Network, the location information of the closest Node shall be stored as the location information of the Originator, or if the Hosting CSE cannot find any closest Node or has no topology information, the location information of the Node of the Hosting CSE (MN) shall be stored as the location information of the Originator. The closest Node can be determined by the minimum hop based on the topology information stored in the <node> resource. |  |
| Information in Response message | The representation of the created <locationPolicy> resource |  |
| Processing at Originator after receiving Response | According to clause 10.1.1.1 |  |
| Exceptions | No change from the generic procedure |  |
| NOTE: The details of the mechanisms are addressed in the oneM2M TS-0004 [Error: Reference source not found]. |  |  |

##### 10.2.10.1.2	Retrieve <locationPolicy>

This procedure shall be used for retrieving an existing <locationPolicy> resource.

Originator: The Originator shall request to obtain <locationPolicy> resource information by using RETRIEVE operation. The Originator is either an AE or a CSE.

Receiver: The Receiver shall check if the Originator has RETRIEVE permission on the <locationPolicy> resource. Upon successful validation, the Hosting CSE shall respond to the Originator with the appropriate responses.

Table 10.2.10.1.2-1: <locationPolicy> RETRIEVE

| <locationPolicy> RETRIEVE |  |  |
| --- | --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |  |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the target <locationPolicy> resource |  |
| Processing at Originator before sending Request | None |  |
| Processing at Receiver | According to clause 10.1.2 |  |
| Information in Response message | According to clause 10.1.2 |  |
| Processing at Originator after receiving Response | None |  |
| Exceptions | According to clause 10.1.2 |  |

##### 10.2.10.1.3	Update <locationPolicy>

This procedure shall be used for updating an existing <locationPolicy> resource.

Originator: The Originator shall request to update attributes of an existing <locationPolicy> resource by using an UPDATE operation. The request shall address the specific <locationPolicy> resource of a CSE. The Originator may be either an AE or a CSE.

Receiver: The Receiver of an UPDATE request shall check whether the Originator is authorized to request the operation. The receiver shall further check whether the provided attributes of the <locationPolicy> resource represent a valid request for updating <locationPolicy> resource.

Table 10.2.10.1.3-1: <locationPolicy> UPDATE

| <locationPolicy> UPDATE |  |  |
| --- | --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |  |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the target <locationPolicy> resource Content: The attributes which are to be updated |  |
| Processing at Originator before sending Request | None |  |
| Processing at Receiver | According to clause 10.1.3 with the following: - If the value of locationUpdatePeriod attribute is updated to 0 or NULL, the Hosting CSE shall stop periodical positioning procedure and perform the procedure when Originator retrieves the <latest> resource of the linked <container> resource. See clause 10.2.10.2 for more detail - If the value of locationUpdatePeriod attribute is updated to bigger than 0 (e.g. 1 hour) from 0 or NULL, the Hosting CSE shall start periodical positioning procedure |  |
| Information in Response message | According to clause 10.1.3 |  |
| Processing at Originator after receiving Response | None |  |
| Exceptions | According to clause 10.1.3 |  |

##### 10.2.10.1.4	Delete <locationPolicy>

This procedure shall be used for deleting an existing <locationPolicy> resource.

Originator: The Originator shall request to delete an existing <locationPolicy> resource by using the DELETE operation. The Originator may be either an AE or a CSE. This request can be occurred when the locationSource attribute of the created <locationPolicy> resource is "sharing-based" and the Originator is an AE that disconnects from the registered MN-CSE.

Receiver: The Receiver shall check if the Originator has DELETE permission on the <locationPolicy> resource. Upon successful validation, the CSE shall remove the resource from its repository and shall respond to the Originator with appropriate responses.

Table 10.2.10.1.4-1: <locationPolicy> DELETE

| <locationPolicy> DELETE |  |  |
| --- | --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |  |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: the address of the target <locationPolicy> resource |  |
| Processing at Originator before Sending Request | None |  |
| Processing at Receiver | According to clause 10.1.4 |  |
| Information in Response message | According to clause 10.1.4 |  |
| Processing at Originator after receiving Response | Once the <locationPolicy> resource is deleted, the Receiver shall delete the associated resources (i.e. <container>, <contentInstance> resources). If the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource has been set with appropriate value, the Receiver shall tear down the session. The specific mechanism used to tear down the session depends on the support of the Underlying Network and other factors |  |
| Exceptions | According to clause 10.1.4 |  |

#### 10.2.10.2	Procedure when the <container> and <contentInstance> resource contain location information

##### 10.2.10.2.0	Overview

Since the actual location information of a target M2M Node shall be stored in the <contentInstance> resource as per the configuration described in the associated <locationPolicy> resource, this clause introduces the procedures related to the <contentInstance> and <container> resource.

##### 10.2.10.2.1	Procedure for <container> resource that stores the location information

This procedure is mainly triggered by the creation of <locationPolicy> resource. Based on the defined attributes related to the <container> resource such as 'locationContainerID' and 'locationContainerName', the Hosting CSE shall create <container> resource to store the location information in its child resource, <contentInstance> resource after the CSE obtains the actual location information of a target M2M Node. If the Originator provides the 'locationContainerName' and the given 'locationContainerName' does not exist in the Hosting CSE, the Hosting CSE shall set the 'resourceName' of the created <container> resource to the 'locationContainerName' provided by the Originator. If the given 'locationContainerName' already exists in the Hosting CSE, the Hosting CSE shall respond with an error following the general exceptions written in clause 10.1.1.1. If the Originator does not provide the 'locationContainerName' the Hosting CSE shall provide 'resourceName' for the created <container> resource. After the creation of the <container> resource, theresourceID attribute of the resource shall be stored in the 'locationContainerID'.

##### 10.2.10.2.2	Procedure for <contentInstance> resource that stores location information

After the <container> resource that stores the location information is created, each instance of location information shall be stored in the different <contentInstance> resources. In order to store the location information in the <contentInstance> resource, the Hosting CSE firstly checks the defined locationUpdatePeriod attribute. If a valid period value is set for this attribute, the Hosting CSE shall perform the positioning procedures as defined period value, locationUpdatePeriod, in the associated <locationPolicy> resource and stores the results (e.g. position fix and uncertainty) in the <contentInstanace> resource under the created <container> resource. However, if no value (e.g. null or zero) is set, the positioning procedure shall be performed when an Originator requests to retrieve the <latest> resource of the <container> resource and the result shall be stored as a <contentInstance> resource under the <container> resource.

As per TS-0004,

#### 7.4.10.2 <locationPolicy> resource specific procedure on CRUD Operations

##### 7.4.10.2.0	Introduction

This clause describes <locationPolicy> resource specific primitive behaviour for CRUD operations.

##### 7.4.10.2.1 Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The following <locationPolicy> resource type specific procedures shall be performed after Recv-6.5 and before Recv-6.6 generic procedures.

- The Hosting CSE shall create a <container> resource in which the actual location information will be stored. Then the Hosting CSE shall create a <locationPolicy> resource and shall fill in cross-references for both resources. Both of these resources shall be hosted locally on the Hosting CSE. The locationContainerID attribute of the <locationPolicy> resource shall contain the resourceID of the created <container> resource and the locationID attribute of the <container> resource shall contain the resourceID of the <locationPolicy> resource. The name of the created <container> resource shall be determined by the locationContainerName attribute if it is applicable.

- Check the locationSource and locationUpdatePeriod attributes:

a)	If the locationSource attribute is set by 'Network Based' and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the step 3.

b)	If the locationSource attribute is set by 'Device Based' and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the step 4.

c)	If the locationSource attribute is set by 'Sharing Based' and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the step 5.

d)	If the locationUpdatePeriod has more than one values, the first value in the list shall be used as the current location update period in step 3,4 and 5. In this case, based on the local context information of the Hosting CSE such as velocity, available memory, the Hosting CSE may choose one of the value out of the list to be the active location update period. If the device is moving at the high speed, it is expected that the location of the device would be update more frequently. The Hosting CSE would acquire the current velocity of the device and compare the value with some predefined value, depend on the result of the comparison, the Hosting CSE would choose a smaller value from the list if the current velocity is higher than the predefined value. Otherwise, the Hosting CSE would choose a larger value.

- The Hosting CSE shall retrieve the locationTargetID and locationServer attributes from the stored <location Policy> resource:

- If the locationServer is absent in the Originator's request, the Hosting CSE shall either derive the locationServer value from the locationTargetID or be pre-provisioned with the identity of a Location Server.

- In case either the locationTargetID or locationServer attribute cannot be obtained, the Hosting CSE shall reject the request with the Response Status Code indicating "BAD_REQUEST" error. Then, the Hosting CSE shall transform the location-acquisition request into Location Server request [Error: Reference source not found], using the attributes stored in <locationPolicy> resource. The Hosting CSE shall also provide default values for other required parameters (e.g. quality of position) in the Location Server request according to local policies.

- The Hosting CSE shall send this Location Server request to the location server using, for example, OMA Mobile Location Protocol [Error: Reference source not found] and OMA RESTful NetAPI for Terminal Location [Error: Reference source not found]. The location server performs positioning procedure based upon the Location Server request. Then continue with step 6.

- Based on the period information, locationUpdatePeriod attribute, this step can be periodically repeated or the location server can only notify the Hosting CSE of location information that performs periodically.

NOTE 1:	The location server performs the privacy control and only responds successfully if the positioning procedure is permitted.

NOTE 2:	The detailed information on how the Location Server request message is converted into OMA RESTful NetAPI for Terminal Location message is described in Error: Reference source not found.

- The Hosting CSE shall perform positioning procedure using location determination modules and technologies (e.g. GPS). Then continue with step 6:

- Based on the period information, locationUpdatePeriod attribute, this step can be periodically repeated.

NOTE 3:	The Hosting CSE can utilize the internal interface (e.g. System Call) to communicate with the modules and technologies. The detailed procedure is out of scope.

- The Hosting CSE shall collect information of topology of M2M Area Network using <node> resource and find the closest Node from the Originator that has registered with the Hosting CSE and has location information. The closest Node is determined by the minimum hop based on the collected topology information:

a)	If the Hosting CSE can find the closest Node from the Originator, the location information of the closest Node shall be stored as the location information of the Originator into a <contentInstance> resource under the created <container> resource.

b)	If the Hosting CSE cannot find the closest Node from the Originator, the location information of the Hosting CSE shall be stored as the location information of the Originator into a <contentInstance> resource under the created <container> resource.

- The Hosting CSE shall receive the corresponding response and transform it into a Response primitive:

a)	If the positioning procedure is failed, the Hosting CSE shall store a statusCode based on the error code in the locationStatus attribute in the created <locationPolicy> resource.

b)	If the positioning procedure is successfully complete which means that the Hosting CSE acquires the location information, The Hosting CSE shall store the acquired location information into a <contentInstance> resource under the created <container> resource.

##### 7.4.10.2.2 Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.10.2.3 Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.10.2.4 Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The procedure of the Receiver written in the clause Error: Reference source not found (from Rcv-D-1.0 to Rcv-D-10.0) shall be the same as initial steps. A following step is the <locationPolicy> resource type specific procedure for DELETE operation:

- Once the <locationPolicy> resource is deleted, the Receiver shall delete the associated resources (e.g. <container>, <contentInstance> resources). If the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource has been set with appropriate value, the Receiver shall tear down the session. The specific mechanism used to tear down the session depends on the support of the Underlying Network and other factors.

- --------------------------------------------------------------------------------------------------------------------------------------------

## Retrieve <latest>

As per TS-0001,

#### 10.2.22.1	Retrieve <latest>

If locationID of <container> is configured, the procedure specified in clause 10.2.10.2.2 shall apply.

If locationID of <container> is not configured, this procedure shall apply to the latest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If there is no <contentInstance> resource in the parent, then the Receiver shall responded with an error.

This procedure is the same as the procedures in clause 10.2.19.3 <contentInstance> RETRIEVE.

##### 10.2.10.2.2	Procedure for <contentInstance> resource that stores location information

After the <container> resource that stores the location information is created, each instance of location information shall be stored in the different <contentInstance> resources. In order to store the location information in the <contentInstance> resource, the Hosting CSE firstly checks the defined locationUpdatePeriod attribute. If a valid period value is set for this attribute, the Hosting CSE shall perform the positioning procedures as defined period value, locationUpdatePeriod, in the associated <locationPolicy> resource and stores the results (e.g. position fix and uncertainty) in the <contentInstanace> resource under the created <container> resource. However, if no value (e.g. null or zero) is set, the positioning procedure shall be performed when an Originator requests to retrieve the <latest> resource of the <container> resource and the result shall be stored as a <contentInstance> resource under the <container> resource.

As per TS-0004,

#### 7.4.27.2 Resource Specific Procedure on CRUD Operations

##### 7.4.27.2.1 Introduction

Th<latest>is clause describes <latest> resource specific behaviour for operations. Among operations, only Retrieve and Delete operations shall be allowed for the <latest> resource.

##### 7.4.27.2.2 Create

Originator:

The <latest> resource shall not be created via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

a)	"Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

b)	"Send the Response primitive".

##### 7.4.27.2.3 Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

If the locationID of <container> is configured and the value of locationUpdatePeriod is marked '0' or not defined and locationSource attribute is 'Network Based', the following <latest> resource type specific procedures shall be performed after Recv-6.3 and before Recv-6.6 generic procedures:

- The Hosting CSE shall transform the location-acquisition request into Location Server request [Error: Reference source not found], using the attributes stored in <locationPolicy> resource. The Hosting CSE shall also provide default values for other required parameters (e.g. quality of position) in the Location Server request according to local policies:

- The Hosting CSE shall send this Location Server request to the location server using, for example, OMA Mobile Location Protocol [Error: Reference source not found] and OMA RESTful NetAPI for Terminal Location [Error: Reference source not found]. The location server performs positioning procedure based upon the Location Server request. Then go to Step 3).

- The Hosting CSE shall receive the corresponding response and transform it into a Response primitive:

a)	If the positioning procedure is failed, the Hosting CSE returns the response with a Response Status Code based on the error code and shall store a statusCode based on the error code in the locationStatus attribute in the <locationPolicy> resource.

b)	If the positioning procedure is successfully completed which means that the Hosting CSE acquires the location information, the Hosting CSE shall perform the procedures of creating a <contentInstance> as described in clause Error: Reference source not found according to the acquired location information and perform the normal Recv-6.5 procedure.

If locationID of <container> is not configured, or the locationID of <container> is configured and the value of locationUpdatePeriod is greater than zero, or locationSource attribute is Device Based' or Sharing Based:

- No change from the generic procedures in clause Error: Reference source not found except the following modification:

- Recv-6.2:

- Check the existence of the latest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If the resource exists, the subsequent procedures of the Receiver (i.e. after Recv-6.2) shall be performed for the resource. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

- Recv-6.3:

- Addition to normal procedure of Recv-6.3 "Check authorization of the Originator": Check the value of disableRetrieval attribute, and if the value is True then the Hosting CSE shall reject the request with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

## -----------------------Start of change 1 -----------------------------------------

# 6	Test Description Summary

## 6.1	Tests list

| Nb | Procedure/Resource | TD ID | TD Description |
| --- | --- | --- | --- |
| 1 | CSEBase Management | TD_M2M_NH_01 | AE retrieves the CSEBase resource |
| 2 | RemoteCSE | TD_M2M_NH_02 | Registree CSE registers to Registrar CSE |
| 3 |  | TD_M2M_NH_03 | Registree CSE retrieves RemoteCSE from Registrar CSE |
| 4 |  | TD_M2M_NH_04 | Registree CSE updates RemoteCSE from Registrar CSE |
| 5 |  | TD_M2M_NH_05 | Registree CSE deletes RemoteCSE from Registrar CSE |
| 6 | Application Entity | TD_M2M_NH_06 | AE registers to its registrar CSE via an AE Create Request |
| 7 |  | TD_M2M_NH_07 | AE retrieves <AE> resource via an AE Retrieve Request |
| 8 |  | TD_M2M_NH_08 | AE updates attribute in <AE> resource via an AE Update Request |
| 9 |  | TD_M2M_NH_09 | AE de-registers by deleting <AE> resource via an AE Delete Request |
| 10 | Container | TD_M2M_NH_10 | AE creates a container resource in registrar CSE via a container Create Request |
| 11 |  | TD_M2M_NH_11 | AE retrieves information of a container resource via a container Retrieve Request |
| 12 |  | TD_M2M_NH_12 | AE updates attribute in application resource via a container Update Request |
| 13 |  | TD_M2M_NH_13 | AE deletes a specific container resource via a container Delete Request |
| 14 | ContentInstance | TD_M2M_NH_14 | AE adds a contentInstance resource <contentInstance> to a specific container in Registrar CSE via a contentInstance Create Request and the registrar CSE updates the parent <container> resource with stateTag, and currentNrOfInstances, CurrentByteSize attributes correspondingly |
| 15 |  | TD_M2M_NH_15 | AE retrieves information of a contentInstance resource via a contentInstance Retrieve Request |
| 16 |  | TD_M2M_NH_17 | AE deletes contentInstance resource via a Delete Request and the registrar CSE updates the parent <container> resource with currentNrOfInstances, and CurrentByteSize attribute correspondingly |
| 17 |  | TD_M2M_NH_49 | AE deletes a <latest> resource in a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| 18 |  | TD_M2M_NH_50 | AE deletes a <oldest> resource in a <container> resource and the Registrar CSE points an oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| 19 |  | TD_M2M_NH_51 | AE sends a <contentInstance> CREATE request to a <container> which contains attribute currentNrOfInstances whose value equals to that of maxNrOfInstances and Registrar CSE deletes the oldest <contentInstance> from the parent <container> and then creates the requested <contentInstance> resource |
|  |  | TD_M2M_NH_93 | AE retrieves a <latest> resource of a <container> for which attribute locationID is configured, value of locationUpdatePeriod is marked '0' or not defined and locationSource attribute is 'Network Based'. |
| 20 |  | TD_M2M_NH_71 | AE retrieves a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| 21 |  | TD_M2M_NH_72 | AE retrieves a <oldest> resource of a <container> and the Registrar CSE points a oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| 22 | Discovery | TD_M2M_NH_18 | AE discovers resources residing in Registrar CSE |
| 23 |  | TD_M2M_NH_19 | AE discovers accessible resources residing in Registrar CSE using the label filter criteria |
| 24 |  | TD_M2M_NH_20 | AE discovers accessible resources residing in Registrar CSE limiting the number of matching resources to the specified value. |
| 25 |  | TD_M2M_NH_21 | AE discovers accessible resources residing in Registrar CSE using multiple Filter Criteria |
| 26 |  | TD_M2M_NH_58 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 1 |
| 27 |  | TD_M2M_NH_59 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 2 |
| 28 |  | TD_M2M_NH_60 | AE1 discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 3 |
| 29 |  | TD_M2M_NH_61 | AE discovers accessible resources residing in Registrar CSE using the offset filter criteria value set to 3 |
| 30 |  | TD_M2M_NH_62 | AE discovers all the accessible resources residing in Registrar CSE using the offset filter criteria |
| 31 | Subscription | TD_M2M_NH_22 | AE creates a subscription to Application Entity resource via subscription Create Request |
| 32 |  | TD_M2M_NH_23 | AE retrieves information about a subscription via subscription Retrieve Request such as expirationTime, labels, etc. |
| 33 |  | TD_M2M_NH_24 | AE updates information about a subscription via subscription Retrieve Request |
| 34 |  | TD_M2M_NH_25 | AE cancels subscription via an subscription Delete Request |
| 35 | AccessControlPolicy | TD_M2M_NH_26 | AE creates an accessControlPolicy resource |
| 36 |  | TD_M2M_NH_27 | AE retrieves accessControlPolicy resource |
| 37 |  | TD_M2M_NH_28 | AE updates attribute in accessControlPolicy resource |
| 38 |  | TD_M2M_NH_29 | AE deletes accessControlPolicy resource |
| 39 |  | TD_M2M_NH_30 | AE delete request is rejected due to accessControlPolicy |
| 40 |  | TD_M2M_NH_73 | AE delete request is rejected due to accessControlPolicy (accessControlOriginators) |
| 41 |  | TD_M2M_NH_74 | AE delete request is allowed due to accessControlPolicy |
| 42 | Group | TD_M2M_NH_31 | AE creates a group resource |
| 43 |  | TD_M2M_NH_32 | AE retrieves group resource |
| 44 |  | TD_M2M_NH_33 | AE updates attribute in group resource |
| 45 |  | TD_M2M_NH_34 | AE deletes group resource |
| 46 | Node | TD_M2M_NH_35 | AE creates a node resource |
| 47 |  | TD_M2M_NH_36 | AE retrieves node resource |
| 48 |  | TD_M2M_NH_37 | AE updates attribute in node resource |
| 49 |  | TD_M2M_NH_38 | AE deletes node resource |
| 50 | PollingChannel | TD_M2M_NH_39 | AE creates a <pollingChannel> resource in registrar CSE via a Create Request |
| 51 |  | TD_M2M_NH_40 | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| 52 |  | TD_M2M_NH_41 | AE updates attribute in pollingChannel resource via a Update Request |
| 53 |  | TD_M2M_NH_42 | AE deletes a pollingChannel resource via a Delete Request |
| 54 |  | TD_M2M_NH_43 | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| 55 | FanoutPoint | TD_M2M_NH_44 | AE creates a <contentInstance> resource in each group member |
| 56 |  | TD_M2M_NH_45 | AE retrieves the <container> resource from in each group member |
| 57 |  | TD_M2M_NH_46 | AE updates an <container> resource of each member resource |
| 58 |  | TD_M2M_NH_47 | AE deletes a <container> of each member |
| 59 | Notification | TD_M2M_NH_48 | AE receives a notification request from the HOST CSE |
| 60 |  | TD_M2M_NH_80 | AE2 sends maxNrOfInstances UPDATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has specific setting in eventNotificationCriteria, Hosting CSE send notification to AE1 |
| 61 |  | TD_M2M_NH_81 | AE2 sends DELETE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Resource’, Hosting CSE send notification to AE1 |
| 62 |  | TD_M2M_NH_82 | AE2 sends <contentInstance> CREATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Create of Direct Child Resource’, Hosting CSE send notification to AE1 |
| 63 |  | TD_M2M_NH_83 | AE2 sends DELETE request to the <contentInstance> which located under the subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Direct Child Resource’, Hosting CSE send notification to AE1 |
| 64 | FlexContainer | TD_M2M_NH_52 | AE creates a flexcontainer resource in Registrar CSE via a flexcontainer Create Request |
| 65 |  | TD_M2M_NH_53 | AE retrieves information of a flexContainer resource via a flexContainer Retrieve Request |
| 66 |  | TD_M2M_NH_54 | AE updates attribute in application resource via a flexContainer Update Request |
| 67 |  | TD_M2M_NH_55 | AE deletes a specific container resource via a container Delete Request |
| 68 |  | TD_M2M_NH_56 | AE receives a notification request on flexContainer update from the HOST CSE |
| 69 |  | TD_M2M_NH_57 | AE discovers accessible resources residing in Registrar CSE using attribute filter criteria which has a customAttribute name and value assigned to it. |
| 70 | External Management Operations | TD_M2M_NH_63 | AE creates a mgmtCmd resource |
| 71 |  | TD_M2M_NH_64 | AE retrieves mgmtCmd resource |
| 72 |  | TD_M2M_NH_65 | AE updates attribute (not with 'true' in execEnable attribute) in mgmtCmd resource |
| 73 |  | TD_M2M_NH_66 | AE updates attribute (with 'true' in execEnable attribute) in mgmtCmd resource |
| 74 |  | TD_M2M_NH_67 | AE deletes mgmtCmd resource |
| 75 |  | TD_M2M_NH_68 | AE retrieves execInstance resource |
| 76 |  | TD_M2M_NH_69 | AE updates attribute 'execDisable' to true in execInstance resource to cancel pending management command. |
| 77 |  | TD_M2M_NH_70 | AE deletes execInstance resource |
| 78 | SemanticDescriptor Management | TD_M2M_NH_75 | AE creates a SemanticDescriptor resource in Registrar CSE via a SemanticDescriptor Create Request |
| 79 |  | TD_M2M_NH_76 | AE retrieves information of a semanticDescriptor resource via a semanticDescriptor Retrieve Request |
| 80 |  | TD_M2M_NH_77 | AE updates attribute in <semanticDescriptor> resource via a semanticDescriptor Update Request |
| 81 |  | TD_M2M_NH_78 | AE deletes SemanticDescriptor resource via a SemanticDescriptor Delete Request |
| 82 | Semantic Resource Discovery | TD_M2M_NH_79 | AE discovers accessible resources residing in Registrar CSE using the semanticFilter filter criteria |
| 83 | ResultContent | TD_M2M_NH_84 | Check creation of <container> resource with result content set to 0(nothing) |
| 84 |  | TD_M2M_NH_85 | Check creation of <container> resource with result content set to 1(attributes) |
| 85 |  | TD_M2M_NH_86 | Check creation of <container> resource with result content set to 2(hierarchical address) |
| 86 |  | TD_M2M_NH_87 | Check creation of <container> resource with result content set to 3(hierarchical address and attributes) |
| 87 |  | TD_M2M_NH_88 | Check retrievability of <container> resource with result content set to 4(attributes and child resources) |
| 88 | LocationPolicy | TD_M2M_NH_89 | AE creates a <locationPolicy> resource in registrar CSE via a locationPolicy Create Request |
| 89 |  | TD_M2M_NH_90 | AE retrieves information of a <locationPolicy> resource via a locationPolicy Retrieve Request |
| 90 |  | TD_M2M_NH_91 | AE updates attribute in <locationPolicy> resource via a locationPolicy Update Request |
| 91 |  | TD_M2M_NH_92 | AE deletes a specific <locationPolicy> resource via a locationPolicy Delete Request |
| 92 | Synchronous request | TD_M2M_NB_01 | AE creates a container resource using non-blocking synchronous request in registrar CSE |
| 93 |  | TD_M2M_NB_02 | AE retrieves a Container resource using non-blocking synchronous request in registrar CSE |
| 94 |  | TD_M2M_NB_03 | AE updates a Container resource using non-blocking synchronous request in registrar CSE |
| 95 |  | TD_M2M_NB_04 | AE deletes a Container resource using non-blocking synchronous request |
| 96 | Asynchronous request | TD_M2M_NB_05 | AE creates a container resource using non-blocking asynchronous request |
| 97 |  | TD_M2M_NB_06 | AE retrieves a Container resource using non-blocking asynchronous request |
| 98 |  | TD_M2M_NB_07 | AE updates a Container resource using non-blocking asynchronous request |
| 99 |  | TD_M2M_NB_08 | AE deletes a Container resource using non-blocking asynchronous request |
| 100 | Retargeting | TD_M2M_SH_01 | AE creates a remote <Resource> resource |
| 101 |  | TD_M2M_SH_02 | AE retrieves a remote <Resource> resource |
| 102 |  | TD_M2M_SH_03 | AE updates a remote <Resource> resource |
| 103 |  | TD_M2M_SH_04 | AE delete a remote <Resource> resource |
| 104 | Discovery | TD_M2M_SH_09 | AE discovers accessible resources residing in the remote Hosting CSE using multiple Filter Criteria |
| 105 | Unauthorized operation | TD_M2M_SH_10 | AE delete request is rejected after access rights verification using retargeting. |
| 106 | Notification | TD_M2M_SH_11 | AE receives a notification request from the remote hosting CSE |
| 107 | mgmtObj | TD_M2M_SH_05 | AE creates a <mgmtObj> resource |
| 108 |  | TD_M2M_SH_06 | AE updates a <mgmtObj> resource |
| 109 |  | TD_M2M_SH_07 | AE retrieves a <mgmtObj> resource |
| 110 |  | TD_M2M_SH_08 | AE deletes a <mgmtObj> resource |
| 111 | Announcement | TD_M2M_SH_12 | AE1 announces itself to CSE2 |
| 112 |  | TD_M2M_SH_13 | AE1 announces a child container to CSE2 |
| 113 |  | TD_M2M_SH_14 | AE1 announces an Optional Announce attribute to CSE2 |
| 114 |  | TD_M2M_SH_15 | AE2 retrieves an Announced Resource |
| 115 |  | TD_M2M_SH_16 | AE2 retrieves the original resource representation of an announced resource |
| 116 | fanOut | TD_M2M_SH_17 | AE creates a <contentInstance> resource in each group member, where some memberIDs are on a remoteCSE |
| 117 |  | TD_M2M_SH_18 | AE retrieves a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| 118 |  | TD_M2M_SH_19 | AE updates a <container> resource in each group member, where some memberIDs are on a remoteCSE |
| 119 |  | TD_M2M_SH_20 | AE deletes a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| 120 | Secure AE Registration | TD_M2M_SE_01 | AE uses Provisioned Symmetric Key Security Association Establishment Framework to enable mutual authentication with the Registrar CSE. Registrar CSE performs AE authorization check on incoming AE registration request. |

### -----------------------End of Change 1---------------------------------------------

## -----------------------Start of new text 1-------------------------------------------

### 8.1.19	Location Management

#### 8.1.19.1	LocationPolicy Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_89 |
| Objective: |  |  | AE creates a <locationPolicy> resource in registrar CSE via a locationPolicy Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.10.1.1 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.10.2.1 |
| Pre-test conditions: |  |  | - AE has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <locationPolicy> |
| 2 | Mca | PRO Check Primitive | - op = 1 (Create) - to = {CSEBaseName} - fr = AE-ID - rqi = (token-string) - ty = 10 (LocationPolicy) - pc = Serialized representation of <locationPolicy> resource |
| 3 |  | IOP Check | Check if possible that the <locationPolicy> resource is created in registrar CSE. - Check if possible that the <container> resource is created on registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively. |
| 4 | Mca | PRO Check Primitive | - rsc = 2001 (CREATED) - rqi = (token-string) same as received in request message - pc = Serialized representation of <locationPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.2	LocationPolicy Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_90 |
| Objective: |  |  | AE retrieves information of a <locationPolicy> resource via a locationPolicy Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.10.1.2 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.10.2.2 |
| Pre-test conditions: |  |  | - AE has created an Application Entity resource <AE> on Registrar CSE - AE has created a locationPolicy resource <locationPolicy> on Registrar CSE - AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <locationPolicy> |
| 2 | Mca | PRO Check Primitive | - op = 2 (Retrieve) - to = {CSEBaseName}/URI of <locationPolicy> resource - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 | Mca | PRO Check Primitive | - rsc =2000 (OK) - rqi = (token-string) same as received in request message - pc = Serialized representation of <locationPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.3	LocationPolicy Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_91 |
| Objective: |  |  | AE updates attribute in <locationPolicy> resource via a locationPolicy Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.10.1.3 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.10.2.3 |
| Pre-test conditions: |  |  | - AE has created an Application Entity resource <AE> on Registrar CSE - AE has created a locationPolicy resource <locationPolicy> on Registrar CSE - AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a locationPolicy Update Request to update the lifetime of the resource. |
| 2 | Mca | PRO Check Primitive | - op = 3 (Update) - to = {CSEBaseName}/URI of <locationPolicy> resource - fr = AE-ID - rqi = (token-string) - pc = Serialized representation of updated <locationPolicy> resource |
| 3 |  | IOP Check | Check if possible that the < locationPolicy > resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | - rsc = 2004 (Updated) - rqi = (token-string) same as received in request message - pc = Serialized representation of <locationPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.4	LocationPolicy Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_92 |
| Objective: |  |  | AE deletes a specific <locationPolicy> resource via a locationPolicy Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.10.1.4 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.10.2.4 |
| Pre-test conditions: |  |  | - AE has created an Application Entity resource <AE> on Registrar CSE - AE has created a locationPolicy resource <locationPolicy> on Registrar CSE - AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a locationPolicy Delete Request |
| 2 | Mca | PRO Check Primitive | - op = 4 (Delete) - to = {CSEBaseName}/URI of <locationPolicy> resource - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 |  | IOP Check | Check if possible that the <locationPolicy> resource is deleted in registrar CSE. |
| 4 | Mca | PRO Check Primitive | - rsc = 2002 (DELETED) - rqi = (token-string) same as received in request message - pc = empty |
| 5 |  | IOP Check | Check if possible that the <locationPolicy> resource has been removed in registrar CSE. |
| 6 |  | IOP Check | Check if possible that the associated resources (e.g. <container>, <contentInstance> resources) are removed from the registrar CSE. |
| 7 |  | IOP Check | Check if possible that if the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource were set with appropriate value, the session with underlying network are torn down. |
| 8 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### -----------------------End of new text 1---------------------------------------------

## -----------------------Start of change 2--------------------------------------------

#### 8.1.5.7	<latest> ContentInstance Retrieve

##### 8.1.5.7.1 Attribute locationID of the <container> resource configured

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_93 |
| Objective: |  |  | AE retrieves a <latest> resource of a <container> for which attribute locationID is configured, value of locationUpdatePeriod is marked '0' or not defined and locationSource attribute is 'Network Based'. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.22.1 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.27.2.2 |
| Pre-test conditions: |  |  | - AE has created an Application Entity resource <AE> on Registrar CSE - AE has created a container resource <container> on Registrar CSE - AE has created a <locationPolicy> resource on Registrar CSE having its resourceID and locationContainerID attribute set to locationID and resourceID attribute of the <container> resource respectively. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <latest> |
| 2 | Mca | PRO Check Primitive | - op = 2 (Retrieve) - to = {CSEBaseName}/URI of <container> resource/la - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 | Mca | PRO Check Primitive | - rsc =2000 (OK) - rqi = (token-string) same as received in request message - pc = Serialized representation of latest <contentInstance> resource, created after acquiring location info from Location Server, in response to Location Request made by the CSE, using the attributes stored in the <locationPolicy> resource. |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

##### 8.1.5.7.2 Attribute locationID of the <container> resource not configured

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_71 |
| Objective: |  |  | AE retrieves a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [Error: Reference source not found], clause 10.2.22.1 oneM2M TS-0004 [Error: Reference source not found], clause 7.4.27.2.2 |
| Pre-test conditions: |  |  | - AE has created an Application Entity resource <AE> on Registrar CSE - AE has created a container resource <container> on Registrar CSE - AE has created multiple contentInstance resources <contentInstance> as child resource of <container> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <latest> |
| 2 | Mca | PRO Check Primitive | - op = 2 (Retrieve) - to = {CSEBaseName}/URI of <container> resource/la - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 | Mca | PRO Check Primitive | - rsc =2000 (OK) - rqi = (token-string) same as received in request message - pc = Serialized representation of latest <contentInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### -----------------------End of change 2---------------------------------------------