| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TST #36 |
| Source:* | In Song Lee, KETI, insong@keti.re.kr Il Yeup Ahn, KETI, iyahn@keti.re.kr |
| Date:* | 2018-07-16 |
| Reason for Change/s:* | Adding explanation part of resource discovery |
| CR against: Release* | Rel-1 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0051-v0.1.0 |
| Clauses * | 6.2.8.0 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution is adding discovery resource explanation with filterUsage and filter criteria.

This will be helpful for reader to understand discovery service clearly. Other resourcetypes in the TR-0051 which do not have enough explanation will be handled in the next contribution.

### -----------------------Start of change 1-------------------------------------------

### 6.2.8	Resource discovery

#### 6.2.8.0	Introduction

The discovery is one of the common service functions which searches information about application and services. An originator can receive the matching information according to the filter criteria, by sending the discovery request. The format of a query string has both target resource address and filter criteria information; e.g. /mn-name?fu=2&ty=2.

The filterUsage can be set to retrieve any preferred format of the discovery response. The filterUsage value is specified in table 6.2.8.0-1. When filterUsage sets to 1, the response of the discovery is represented with a format of the URI list and all URIs of discovered resources is listed in the response. And when filterUsage sets to 2, the response contains attributes of the resources that matched with presented filter criteria conditions.

Table 6.2.8.0-1: Interpretation of filterUsage

| Interpretation | Value | Note |
| --- | --- | --- |
| Discovery Criteria | 1 |  |
| Conditional Retrieval | 2 | Default |

Filter criteria are set to search the resources with specific conditions. For example, AE resources can be found by setting the resourceType to 2. Filter criteria conditions are listed in table 6.2.8.0-2. More filter criteria parameters are specified in clause 8.1.2-2 in oneM2M TS-0001 [1].

Table 6.2.8.0-2: Filter criteria conditions

| Condition tag | Short Name | Multiplicity | Description |
| --- | --- | --- | --- |
| Matching Conditions |  |  |  |
| createdBefore | crb | 0..1 | The creationTime attribute of the matched resource is chronologically before the specified value. |
| createdAfter | cra | 0..1 | The creationTime attribute of the matched resource is chronologically after the specified value. |
| modifiedSince | ms | 0..1 | The lastModifiedTime attribute of the matched resource is chronologically after the specified value. |
| unmodifiedSince | us | 0..1 | The lastModifiedTime attribute of the matched resource is chronologically before the specified value. |
| stateTagSmaller | sts | 0..1 | The stateTag attribute of the matched resource is smaller than the specified value. |
| stateTagBigger | stb | 0..1 | The stateTag attribute of the matched resource is bigger than the specified value. |
| expireBefore | exb | 0..1 | The expirationTime attribute of the matched resource is chronologically before the specified value. |
| expireAfter | exa | 0..1 | The expirationTime attribute of the matched resource is chronologically after the specified value. |
| labels | lbl | 0..1 | The labels attribute of the matched resource matches the specified value. |
| resourceType | ty | 0..n | The resourceType attribute of the matched resource is the same as the specified value. It also allows differentiating between normal and announced resources. |
| sizeAbove | sza | 0..1 | The contentSize attribute of the <contentInstance> matched resource is equal to or greater than the specified value. |
| sizeBelow | szb | 0..1 | The contentSize attribute of the <contentInstance> matched resource is smaller than the specified value. |
| Filter Handling Conditions |  |  |  |
| limit | lim | 0..1 | The maximum number of resources to be included in the filtering result. This may be modified by the Hosting CSE. When it is modified, then the new value shall be smaller than the suggested value by the Originator. |
| level | lvl | 0..1 | The maximum level of resource tree that the Hosting CSE shall perform the operation starting from the target resource (i.e. To parameter). This shall only be applied for Retrieve operation. The level of the target resource itself is zero and the level of the direct children of the target is one. |

As a initial condition to use discovery function, CSEBase need to have resources. Table 6.2.8.0-3 has detailed information of resources which will be used in this clause.

Table 6.2.8.0-3: Resource Specifications

| Resource Name | Resource attributes in JSON format |
| --- | --- |
| CSEBase | { "m2m:cb": { "pi": null, "ty": 5, "ct": "20180302T070445", "ri": "CSE9486743758493047362", "rn": "mn-name", "lt": "20180302T070445", "lbl": [ "mn-name" ], "cst": 1, "csi": "/mn-name", "srt": [ 1, 2, 3, 4, 5, 9, 10, 13, 14, 16, 17, 23 ], "poa": [ "http://192.168.0.10:8282" ] } } |
| ae_actuator | { "m2m:ae": { "pi": "mnID", "ty": 2, "ct": "20180404T083025", "ri": "CAE0120180404T0830251405122594272800_cse01", "rn": "ae_actuator", "lbl": [ "actuator", "light" ], "lt": "20180406T083320", "et": "20221231T235959", "api": "A01.com.company.Light", "aei": "CAE0120180404T0830251405122594272800_cse01", "rr": false } } |
| cnt_light1 | { "m2m:cnt": { "pi": "CAE0120180404T0830251405122594272800_cse01", "ty": 3, "ct": "20180406T085318", "ri": "cnt20180406T0853181405855183193600_cse01", "rn": "cont_light1", "lt": "20180406T085318", "et": "20201231T235959", "lbl": [ "indoor_light" "actuator" "room1" ], "st": 5, "cr": "S20170717074825768bp2l", "mni": 10000, "mbs": 60000000, "mia": 1600, "cni": 5, "cbs": 10 } } |
| cnt_light2 | { "m2m:cnt": { "pi": "CAE0120180404T0830251405122594272800_cse01", "ty": 3, "ct": "20180405T085318", "ri": "cnt20180406T0853181405855183193600_cse01", "rn": "cont_light2", "lt": "20180406T085318", "et": "20201231T235959", "lbl": [ "outdoor_light" "actuator" ], "st": 4, "cr": "S20170717074825768bp2l", "mni": 10000, "mbs": 60000000, "mia": 1600, "cni": 10, "cbs": 20 } } |
| ae_sensor | { "m2m:ae": { "pi": "mnID", "ty": 2, "ct": "20180404T083320", "ri": "CAE0120180404T0833201405122522252800_cse01", "rn": "ae_sensor", "lbl": [ "sensor", "temperature" ], "lt": "20180404T083320", "et": "20221231T235959", "api": "A01.com.company.Temperature", "aei": "CAE0120180404T0833201405122522252800_cse01", "rr": false } } |
| cnt_temp1 | { "m2m:cnt": { "pi": "CAE0120180404T0833201405122522252800_cse01", "ty": 3, "ct": "20180406T085712", "ri": "cnt20180406T0857121405855183193600_cse01", "rn": "cont_temp1", "lt": "20180406T085712", "et": "20201231T235959", "lbl": [ "indoor_temperature" "sensor" "room2" ], "st": 8, "cr": "S20170717074825768bp2l", "mni": 10000, "mbs": 60000000, "mia": 1600, "cni": 10, "cbs": 20 } } |
| cnt_temp2 | { "m2m:cnt": { "pi": "CAE0120180404T0833201405122522252800_cse01", "ty": 3, "ct": "20180406T085820", "ri": "cnt20180406T0858201405855563993600_cse01", "rn": "cont_temp2", "lt": "20180406T085820", "et": "20211231T235959", "lbl": [ "outdoor_temperature" "sensor" ], "st": 9, "cr": "S20170717074825768bp2l", "mni": 10000, "mbs": 60000000, "mia": 1600, "cni": 15, "cbs": 30 } } |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0001: "Functional Architecture".

[2]				 	oneM2M TS-0004: "Service Layer Core Protocol Specification".

### -----------------------End of change 2---------------------------------------------