| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#38.2 |
| Source:* | NTT |
| Date:* | 2019-01-24 |
| Contact:* | Kei Harada(kei.harada.by@hco.ntt.co.jp), NTT, Hiroyuki Maeomichi(maeomichi.hiroyuki@lab.ntt.co.jp), NTT, |
| Reason for Change/s:* | To modify the condition tags of filter criteria. |
| CR against: Release* | R4 |
| CR against: WI* | Active < Work Item number(optional)> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No STE Small Technical Enhancements / < Work Item number(optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V3.9.0 |
| Clauses * | 7.3.3.17.0 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0001 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

There are some limitations in Filter Criteria parameter.

At first, attribute operator exists but it is used for only exact matching (=). For attribute of number type, it needs comparison operators. We already have tags for comparison matching (e.g. xxBefore/After/Smaller/Bigger condition tags) for some specific attributes (creation Time, state Tag and so on). This CR aimes exapnding comparision matching to general attributes.

Moreover, the current specification doesn’t support complex queries including parentheses or more than one logical operators  (e.g., (A=n1 OR B=n2) AND C<n3).

This CR introduces new filter syntax (LDAP) to enable these queries.

There are some filelds specifiyng various query syntax, contentFilterSyntax and contentFilterQurery and they are supposed to be used for only for content query. It is possible to make them generic by renaming customFilterSyntax and customeFilterQuery. Short names for the parameters are kept same.

This proposal is based on ARC-2018-0098R01-Modifying_condition_tags_of_Filter_Criteria.

R01: Modify the Targeted Resource Attribute.

R02: Add the Change4 and Modify typos of comparisons.

## ------------------------------Start of change 1-------------------------------------

### 6.3.4	oneM2M enumerated data types

##### 6.3.4.2.37	m2:contentFilterSyntax

Used for contentFilterSyntax element in Filter Criteria primitive parameter.

Table 6.3.4.2.37-1: Interpretation of contentFilterSyntax

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | JSON_PATH_SYNTAX | 'jsonpath' query syntax. See Annex K.2 of oneM2M TS-0001 [6] for information about the jsonpath syntax. |
| 2 | LDAP_FILTER_SYNTAX | ‘ldapfilter’ query syntax. See Annex K.3 of oneM2M TS-0001 [6] for information about the ldapfilter syntax. |

## ------------------------------End of change 1-------------------------------------

## -----------------------------Start of change 2--------------------------------------

### 7.3.3.17	Using Filter Criteria for identification of target resources

#### Introduction

When the Filter Criteria primitive parameter is present in a request primitive, it shall be applied for identification of the applicable target resources of the respective operation. This may apply to Retrieve, Delete, Discovery and Semantic Resource Discovery operations as specified in clauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found respectively.

The Filter Criteria primitive parameter defines matching conditions on resource attributes and filter handling conditions. Matching conditions are evaluated against resources and, when true, determine the matched resources. The filter handling conditions provide additional input applied to the matched resource set to determine the filtering result (e.g. maximum number of resources to be included in the filtering result). The filtering result may be composed of one or more resources and shall be used as the target of the operation. Table 7.3.3.17.0-2 summarizes the various filter criteria and conditions. Each row in the table represents a different filter condition type.

If multiple matching conditions of the same type (i.e. same condition tag) are present in the Filter Criteria parameter, these shall be combined by applying logical OR operation. This applies to condition tags labels, resourceType, contentType or attribute for multiplicity n > 1.

If multiple matching conditions of different type (i.e. different condition tags) are present in the Filter Criteria parameter, then the combined condition shall be derived by applying the logical operation specified by the filterOperation condition. By default logical AND operation shall be used if the filterOperation condition is not present.

EXAMPLE:

- labels=floor1, stateTagSmaller=3 will match if both conditions are true [default AND when filterOperation is not specified]

- labels=floor1, stateTagSmaller=3, filterOperation=1 will match if both conditions are true

- labels=floor1, stateTagSmaller=3, filterOperation=2 will match if either condition is true

- labels=floor1, labels=floor2, filterOperation=1 will match if either condition is true [ filterOperation has no effect when all condition tags are the same]

- labels=floor1, stateTagSmaller=3, labels=floor2, filterOperation=2 will match if any of these conditions are true resource has [labels with value "floor1" OR "floor2"] OR stateTagSmaller than 3.

Table 7.3.3.17.0-2: Summary on Filter conditions

| Condition Tag | Multiplicity | Targeted Resource Attribute | Matching Condition |
| --- | --- | --- | --- |
| createdBefore, | 0..1 | creationTime | creationTime < createdBefore, see clause Error: Reference source not found |
| createdAfter | 0..1 |  | createdAfter ≤ creationTime , see clause Error: Reference source not found |
| unmodifiedSince | 0..1 | lastModifiedTime | lastModifiedTime < unmodifiedSince, see clause Error: Reference source not found |
| modifiedSince | 0..1 |  | unmodifiedSince ≤ lastModifiedTime, see clause Error: Reference source not found |
| stateTagSmaller | 0..1 | stateTag | stateTagSmaller < stateTag, see clause Error: Reference source not found |
| stateTagBigger | 0..1 |  | stateTag ≤ stateTagBigger, see clause Error: Reference source not found |
| expireBefore | 0..1 | expirationTime | expirationTime < expireBefore, see clause Error: Reference source not found |
| expireAfter | 0..1 |  | expireAfter ≤ expirationTime , see clause Error: Reference source not found |
| labels | 0..1 | labels | see clause Error: Reference source not found |
| childLabels | 0..1 |  | see clause 7.3.3.17.5 |
| parentLabels | 0..1 |  | see clause 7.3.3.17.5 |
| resourceType | 0..1 | resourceType | see clause Error: Reference source not found |
| childResourceType | 0..1 |  | see clause 7.3.3.17.6 |
| parentResourceType | 0..1 |  | see clause 7.3.3.17.6 |
| sizeBelow | 0..1 | contentSize | contentSize < sizeBelow, see clause Error: Reference source not found |
| sizeAbove | 0..1 |  | sizeAbove ≤ contentSize, see clause Error: Reference source not found |
| typeOfContent | 0..n | contentInfo | matched with typeOfContent component in contentInfo, see clause Error: Reference source not found |
| attribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause Error: Reference source not found |
| attributeBelow | 0..n | numerical attributes or attributes which data type is m2m: timestamp or m2m:absTimestamp (see Matching Condition) | resource attribute < name and value of Filter Criteria attribute. When the target attribute is numerical, the comparison is based on the numeric order (e.g. 1<02=2<12) not lexicographic order (e.g. 02<1<12<2). When the target attribute’s type is m2m:timestamp or m2m:absTimestamp, the comparision is based on the time order. |
| attributeAbove | 0..n | numerical attributes or attributes which data type is m2m: timestamp or m2m:absTimestamp (see Matching Condition) | name and value of Filter Criteria attribute ≤ resource attribute. When the target attribute is numerical, the comparison is based on the numeric order (e.g. 1<02=2<12) not lexicographic order (e.g. 02<1<12<2). When the target attribute’s type is m2m:timestamp or m2m:absTimestamp, the comparision is based on the time order. |
| childAttribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause 7.3.3.17.9 |
| parentAttribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause 7.3.3.17.9 |
| limit | 0..1 | (not applicable) | Constraint on maximum number of targeted resources, see clause Error: Reference source not found |
| filterUsage | 0..1 | (not applicable) | Indicator specifying the use case of Filter Criteria parameters |
| semanticsFilter | 0..n | (not applicable) | Filtering conditions expressed in SPARQL Error: Reference source not found. These are applicable to the descriptor attribute of <semanticDesciptor> children associated with discoverable resources. When multiple semanticsFilter elements are provided, the matching condition is fulfilled if any of the individual conditions is matched. |
| filterOperation | 0..1 | (not applicable) | Indicates the logical operation (AND/OR) to be used for different conditions. The default value is logical AND. |
| customFilterSyntax | 0..1 | (not applicable) | Indicates the Identifier for syntax to be applied for discovery based operations. |
| customFilterQuery | 0..1 | content or (variable) (see Matching Condition) | The query string shall be specified when customFilterSyntax parameter is present. See clause for applicable syntax for discovery based operations. If customFilterSyntax value is ‘JSON_PATH_SYNTAX’, targeted attribute is only content. If customFilterSyntaxvalue is ‘LDAP_FILTER_SYNTAX’, targeted attribute is (variable). |
| level | 0..1 | (not applicable) | Constraint on maximum number of levels in the resource tree that retrieve operation shall span, see clause Error: Reference source not found. |
| offset | 0..1 | (not applicable) | The number of direct child and descendant resources the Hosting CSE shall skip over and not include within a retrieve response, see clause Error: Reference source not found. |
| applyRelativePath | 0..1 | (not applicable) | A resource tree relative path (e.g. ../tempContainer/LATEST) which applies after all the matching conditions have been used (i.e. a matching result has been obtained). See clause 7.2.2.17.17. |
| NOTE: (variable) is including [customAttribute] of <flexContainer> and <mgmtObject>. |  |  |  |

## ------------------------------End of change 2--------------------------------------

## ------------------------------Start of change 3-------------------------------------

#### 7.3.3.17	Using Filter Criteria for identification of target resources

#### 7.3.3.17.13	Conditions on discovery based operations

##### 7.3.3.17.13.1	Introduction

The Filter Criteria elements customFilterSyntax and customFilterQuery are used for  discovery based operations .

The customFilterSyntax element specifies the syntax to be used by the customFilterQuery element to express thediscovery conditions. A customFilterQuery shall be included if the customFilterSyntax parameter is present.

The customFilterQuery element contains the query expression for the discovery.

##### 7.3.3.17.13.2	JSON path syntax

When the customFilterySytax element has the value 'JSON_PATH_SYNTAX', the customFilterQuery element shall be expressed using 'jsonpath' query syntax (see Annex K.2 oneM2M 0001[6]). If the Hosting CSE does not support this syntax, NOT_IMPLEMENTED error shall be returned.

7.3.3.17.13.3	LDAP filter syntax

When the customFilterySytax element has the value 'LDAP_FILTER_SYNTAX', the customFilterQuery element shall be expressed using 'ldapfilter' query syntax (see Annex K.3 oneM2M 0001[6]). If the Hosting CSE does not support this syntax, NOT_IMPLEMENTED error shall be returned.

## ------------------------------End of change 3--------------------------------------

## ------------------------------Start of change 4-------------------------------------

#### 7.3.3.17.18	Conditions on numerical attributes or attributes which data type is m2m: timestamp or m2m:absTimestamp

Each of the Filter Criteria elements attributeAbove and attributeBelow define a number range which is tested against the numerical attributes or attributes which data type is m2m:timestamp or m2m:absTimestamp of the applicable resources in order to determine the matching resources.

The query should be following:

attributeAbove = (a certain attribute), (value)

attributeBelow = (a certain attribute), (value)

For example:

attributeAbove = eventStart, 20141003T112032

This filter criterion shall be satisfied if any of the following three conditions is fulfilled:

- only attributeBelow given in Filter Criteria: 
a certain attribute < attributeBelow

- only attributeAbove given in Filter Criteria: 
attributeAbove ≤ a certain attribute

- both, stateTagSmaller and stateTagBigger given in Filter Criteria:
(attributeAbove ≤ a certain attribute) AND (a certain attribute < attributeBelow)

NOTE:	In case 3) the Filter Criteria will only generate a match if attributeAbove < attributeBelow.

## ------------------------------End of change 4-------------------------------------