| Input Contribution |  |
| --- | --- |
| Meeting ID* | ARC 38 |
| Title:* | User Based Access Control |
| Source:* | Dale Seed, Convida, Seed.Dale@convidawireless.com |
| Date:* | 2018-12-01 |
| Input related to* | WI-0083 - oneM2M Service Subscribers and Users (TR-0054) |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) |  |
| Decision requested or recommendation:* | Approval for insertion into TR-0054 |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution proposes user-based access control policy functionality that enhances the <accessControlPolicy> resource with the capability to specify privileges based on the identity of M2M Service Users and their M2M-User-IDs.

### -----------------------Start of change 1-------------------------------------------

### User Based Access Control Policies

To enable a Hosting CSE with support for granting access to a resource based on the M2M-User-ID of the Originator, the following enhancements to the <accessControlPolicy> resource can be made.

The accessControlOriginators parameter in an access-control-tuple can be extended to support M2M-User-ID.  The accessControlOriginators is a mandatory parameter in an access-control-rule-tuple. It represents the set of Originators that shall be allowed to use this access control rule. The set of Originators is described as a list of parameters, where the types of the parameter can vary within the list. Table 7.2.2.4.1-1 describes the supported types of parameters in accessControlOriginators.  By extending the accessControlOriginators parameter to support M2M-User-ID, access control rules can be defined with a higher level of abstraction.   For example, grant access to requests originating from a particular M2M Service User regardless of the application the M2M Service User is using.

Table 7.2.2.4.1-1: Types of Parameters in accessControlOriginators

| Name | Description |
| --- | --- |
| UserID | M2M-User-ID of the Originator |

The accessControlContexts parameter in an access-control-tuple can be extended to support M2M-User-ID. The accessControlContexts is an optional parameter in an access-control-rule-tuple that contains a list, where each element of the list, when present, represents a context that is permitted to use this access control rule.  This request context is used to further qualify the accessControlOriginators parameter.   The accessControlContexts is described by a set of parameters as shown in Table 7.2.2.4.1-2.  By extending the accessControlContexts parameter to support M2M-User-ID, finer grain access control rules can be defined.  For example, grant access to requests originating from a particular AE that is currently being used by a particular user.

Table 7.2.2.4.1-2: Types of Parameters in accessControlContexts

| Name | Description |
| --- | --- |
| accessControlUserIDs | Represents a M2M-User-ID constraint which is compared against the M2M-User-ID of the Originator of the request. |

-----------------------End of change 1-------------------------------------------