| Input Contribution |  |
| --- | --- |
| Meeting ID* | SDS#39 |
| Title:* | Alternative solution for resource offloading |
| Source:* | JaeSeung Song, KETI, jssong@sejong.ac.kr Lewis Nkenyereye, KETI, nkenyele@sejong.ac.kr Youngjin Na, Hyundai Motors, yjra@hyundai.com Minbyeong Lee, Hyundai Motors, minbyeong.lee@hyundai.com |
| Date:* | 2019-02-17 |
| Input related to* | WI-0080 Edge and Fog Computing TR-0052 V 0.5.0 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) |  |
| Decision requested or recommendation:* | Agree for inclusion in TR-0052 |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution provides an alternative solution for resource and service offoading to Edge/Fog through introducing a new resource, the <resourceOffloading> resource.

### -----------------------Start of change 1-------------------------------------------

# 9	Proposed Solutions

Editor’s Note: The section provides solutions to the Key Issues identified for employing Edge and Fog technologies in oneM2M.

## 9.X	Solution X: Edge/Fog Offloading using a New Resource Type

Editor’s Note: Each Solution section references one or more Key Issues that it addresses and provides a brief solution description.

This solution addresses Key Issue 1. This solution proposes an Edge/Fog offloading mechanism allowing a resource can be offloaded to a remote CSE to support Edge/Fog computing. In this case all the selected resources are subjected to be transferred to a given remote CSE. A relationship between the original resource and offloaded resource is specified together with a synchronization mechanism.

A new resource type is introduced to support resource offloading. In TS-0001, oneM2M resources are categorised into three resources:

- Normal resources

- Virtual resources

- Announced resources

This solution introduces the fourth type resources called “Offloaded resources”.

An offloaded resource contains a set of attributes of the original resource. An offloaded resource is updated directly by oneM2M applications and original resource is updated by the CSE hosting offloaded resource. The offloaded resource contains a link to the original resource.

Editor’s Note: Further mechanisms (e.g. using a dedicated resource for offloading) to support offloading need to be investigated.

### Solution Applicability

Editor’s Note: The Solution Applicability states which Key Issues are addressed by the solution.

This solution addresses the key issue 1 to support.

### Solution Description

Editor’s Note: This section provides a concise description of the solution which provides enough detail for further stage 2 development.

This solution requires a set of extension on different oneM2M entities.

AE: AE can trigger resource offloading via sending a request message to its hosting CSE with the following information.

- Target CSE: is a CSE serviing offloaded resources.

- Target resources: contain a set of resources to be offloaded at the target CSE

- Synchronization mode: specifies how to synchronize between offloaded and original resources

The original CSE manages its resources indicated as target offload resources as follows:

- Indicate offloaded resources to show the resources are currently offloaded or not

- Synchronization time shows when the original resources have synchronized with the offloaded resources

- Status of resource offloading to show how to handle requests to the original resource while offloaded

- Link to the target CSE

The target CSE manages its offloaded resources as follows:

- Link to the original CSE

- For each offloaded <resourceType>, the addition of suffix “Offload” to the original <resourceType> shall be used to indicate its associated offloaded resource type. For example, resource <containerOffload> shall indicate the offloaded resource type for <container> resource; <groupOffload> shall indicate offloaded resource type for <group> resource, etc.

- Synchronization with its original resources

  - Periodic: updates to the announced resources are periodically applied to the original resources

  - Update: when there is any update to the announced resources, the changes are applied to the original resources immediately

  - Finish: the original resources are updated only at the time the announced resources are deleted from the remote CSE and the Edge/Fog service is terminated. If an Application receiving the Edge/Fog service via the announced resources is moved to another Edge/Fog node, all the announced resources to the remote CSE are transferred to a new remote CSE. Then the link at the original resource is also updated to point the new remote CSE.