| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#39 |
| Source:* | Feng Zhang (zhangfeng49@huawei.com) Yongjing Zhang(zhangyongjing@huawei.com) |
| Date:* | 2019-02-11 |
| Reason for Change/s:* | To introduce the solution of ontology mapping to meet the requirements ONT-018 & ONT-019. This CR provides new resource types for ontology mapping. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0053> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0033 V4.2.1 |
| Clauses * | 7.3.x |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

To enable the semantic interoperability between different ontologies, ontology mapping is a prerequisite. It’s an important method to identify the commonality, similarity as well as inclusion relationships between ontologies, so that the data described in one ontology can be consumed meaningfully by the application who understand only another ontology.

Corresponding requirements has been accepted as ONT-018 and ONT-19 in TS-0002. The supporting use cases can be found in TR-0001 clause 12.23 – 12.26.

| Requirement ID | Description | Release |
| --- | --- | --- |
| ONT-018 See REQ-2018-0057R01 | The oneM2M system shall support semantic query and discovery across heterogeneous ontologies including support of automatic ontology mapping and semantic reasoning. | 4 |
| ONT-019 REQ-2018-0058R01 | The oneM2M system shall be able to support semantic control of devices with support of addtional capabilities e.g. automatic ontology mapping and semantic reasoning. | 4 |

This CR provides new resource types for ontology mapping in accordance with the procedures defined in CR SDS-2019-0032.

### -----------------------Start of change 1-------------------------------------------

- 7.3	Resources related to semantics

- 7.3.x	Resource <ontologyMapping>  and <ontologyMappingAlgorithmRepository>

The <ontologyMapping> resource is used for the mapping between two different ontologies. It contains necessary input parameters, such as the Resource-IDs of the source and target ontologies and the mapping method description (which includes the parameters of mapping policy and mapping algorithm), to perform ontology mapping. It also contains the output ontology mapping results.

The <ontologyMappingAlgorithmRepository> resource is used for storing a collection of ontology mapping algorithms which are represented as <ontologyMappingAlgorithm> resources, which are referenced from the <ontologyMapping> resource.

Figure 7.3.x-1: Resource structure of <ontologyMapping> and <ontologyMappingAlgorithmRepository>

The <ontologyMapping> resource contains the child resources specified in table 7.3.x-1.

Table 7.3.x-1: Child resources of <ontologyMapping> resource

| Child Resources of <ontologyMapping> | Child Resource Type | Multiplicity | Description | <ontologyMappingAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See [Error: Reference source not found], clause 9.6.8 where the type of this resource is described. | <subscription> |

The <ontologyMapping> resource contains the attributes specified in table 7.3.x-2.

Table 7.3.x-2: Attributes of <ontologyMapping> resource

| Attribute Name | Multiplicity | RW/RO/WO | Description | <ontologyMappingAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| parentID | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| accessControlPolicyIDs | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| lastModifiedTime | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| labels | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| sourceOntology | 1 | RW | Contains the resourceID of the source ontology to be mapped. | OA |
| targetOntology | 1 | RW | Contains the resourceID of the target ontology to be mapped. | OA |
| mappingPolicy | 1 | RW | Determines the policy of applying the ontology mapping algorithms between the sourceOntology and targetOntology. It’s an enumeration type and the value can be: - single: only one mapping algorithm is used. - multiple: two or more mapping algorithms of different types are used. - traversal: all provided mapping algorithms are used in a traversal way. | OA |
| mappingAlgorithmLinks | 1 | RW | Links to a <ontologyMappingAlgorithmRepository> resource, or one or multiple <ontologyMappingAlgorithm> resources; | OA |
| mappingResultFormat | 1 | RW | Indicates the required format of the generated content in the mappingResult attribute. | OA |
| mappingResult | 1 | RO | Stores the mapping results (e.g. RDF/XML) generated by the hosting CSE. | OA |

The  <ontologyMappingAlgorithmRepository> resource contains the child resources specified in table 7.3.x-3.

Table 7.3.x-3: Child resources of <ontologyMappingAlgorithmRepository> resource

| Child Resources of <ontologyMapping> | Child Resource Type | Multiplicity | Description | <ontologyMappingAlgorithmRepositoryAnnc>Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <ontologyMappingAlgorithm> | 0..n | Each <ontologyMappingAlgorithm> represents an ontology mapping algorithm. | <ontologyMappingAlgorithmAnnc> |
| [variable] | <subscription> | 0..n | See [Error: Reference source not found], clause 9.6.8 where the type of this resource is described. | <subscription> |

The  <ontologyMappingAlgorithmRepository> resource contains the attributes specified in table 7.3.x-5.

Table 7.3.x-4: Attributes of <ontologyMappingAlgorithmRepository> resource

| Attribute Name | Multiplicity | RW/RO/WO | Description | <ontologyMappingAlgorithmAnnc>Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| parentID | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| accessControlPolicyIDs | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| lastModifiedTime | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| labels | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |

The <ontologyMappingAlgorithm> resource contains no child resources. It contains the attributes specified in table 7.3.x-5.

Table 7.3.x-5: Attributes of <ontologyMappingAlgorithm> resource

| Attribute Name | Multiplicity | RW/RO/WO | Description | <ontologyMappingAlgorithmAnnc>Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| parentID | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| accessControlPolicyIDs | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| lastModifiedTime | 1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| labels | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See [Error: Reference source not found], clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See [Error: Reference source not found], clause 9.6.1.3. | NA |
| executable | 0..1 | RW | Contains the binary executable of the ontology mapping algorithm. For a pre-configured algorithm already stored in the system, this attribute can be omitted. The system can invoke the algorithm according to the resourceName or resourceID attribute based on its local policy configuration. | OA |
| algorithmType | 0..1 | RW | Indicates the type of the ontology mapping algorithm. It’s an enumeration type with possible values as follows: - linguistic-feature extraction algorithm: compute the similarity based on linguistic string distance between the concepts of the source and target ontologies. - structural-feature extraction algorithm: compute the similarity based on the graph structure between the source and target ontologies. - external resource acquisition algorithm: compute the similarity based on external resources (e.g. well-known dictionary, knowledge base, expert system) - logical reasoning algorithm: compute the similarity based on the reasoning of description logic. - others: any other types that are not specified here. This attribute can be used by an originator for discovering and selecting the candidate algorithms as the input to the ontology mapping task. It can also be used by the hosting CSE that performs the ontology mapping task to determine the proper ontology mapping algorithms according to the specified mapping policy. | OA |
| description | 0..1 | RW | Human readable description of the ontology mapping algorithm. | OA |

### -----------------------End of change 1-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?