| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TP-39 |
| Source:* | Poornima Shandilya, C-DOT, poornima@cdot.in Chaitan Kumar Yadav , C-DOT, chaitan.yadav@cdot.in Siddharth Trikha, C-DOT, strikha@cdot.in Anupama Chopra, CDOT, anupama@cdot.in |
| Date:* | 2019-02-04 |
| Reason for Change/s:* | Refer to Introduction section. |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: SDS-2019-0034 STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0001 Version 2.21.0 |
| Clauses * | 9.6.8 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N.A. |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Notification content type is governed by subscription’ notificationContentType attribute, for the scenario where subscription’s notificationEventType  is set to ‘Update to attributes of the subscribed-to resource’ , ‘Creation of a direct child of the subscribed-to resource’ and ‘Deletion of a direct child of the subscribed-to resource’and notificationContentType set to ‘modified attributes’ , if event type ‘Creation of a direct child of the subscribed-to resource’ or ‘Deletion of a direct child of the subscribed-to resource’ occurs as per specs, notification payload would contain empty payload because there is no updated attributes. This CR attempts to resolve this scenario.

Revision post discussion:

It was discussed to add a table for mapping of notification content type and notification event type like we have a table for result content mapping for various operations.

So revision adds corresponding table for mapping of notification content type and event type.

### -----------------------Start of change 1-------------------------------------------

### 9.6.8	Resource Type subscription

The <subscription> resource contains subscription information for its subscribed-to resource.

The <subscription> resource shall be represented as child resource of the subscribed-to resource. For example, <container> resource has <subscription> resource as a child resource (see clause 9.6.6). A <subscription> resource shall be deleted when the parent subscribed-to resource is deleted.

The <subscription> resource shall represent a subscription to a subscribed-to resource. An Originator shall be able to create a resource of <subscription> resource type when the Originator has RETRIEVE privilege to the subscribe-to resource. The Originator which creates a <subscription> resource becomes the resource subscriber.

Each <subscription> may include notification policies that specify which, when, and how notifications are sent. These notification policies may work in conjunction with CMDH policies.

When a <subscription> resource is deleted, a Notify request shall be sent to the subscriberURI if it is provided by the Originator.

Figure 9.6.8-1: Structure of <subscription> resource

The <subscription> resource shall contain the child resources specified in table 9.6.8-1.

Table 9.6.8-1: Child resources of <subscription> resource

| Child Resources of <subscription> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| notificationSchedule | <schedule> | 0..1 | In the context of the <subscription> resource, the notificationSchedule specifies when notifications may be sent by the Hosting CSE to the notificationURI(s). See clause 9.6.9. |
| [variable] | <notificationTargetMgmtPolicyRef> | 0..n | See clause 9.6.31 for this type of resource. |
| ntsr | <notificationTargetSelfReference> | 1 | See clause 9.6.34 for this type of resource. |

The <subscription> resource shall contain the attributes specified in table 9.6.8-2.

Table 9.6.8-2: Attributes of <subscription> resource

| Attributes of <subscription> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. If no accessControlPolicyIDs value is configured, the accesControlPolicyIDs of the parent resource shall be applied for privilege checking. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creator | 0..1 | WO | See clause 9.6.1.3. |
| eventNotificationCriteria | 0..1 | RW | This attribute (notification policy) indicates the event criteria for which a notification is to be generated. |
| expirationCounter | 0..1 | RW | This attribute (notification policy) indicates that the subscriber wants to set the life of this subscription to a limit of a maximum number of notifications. When the number of notifications sent reaches the count of this counter, the <subscription> resource shall be deleted, regardless of any other policy. |
| notificationURI | 1 (L) | RW | This attribute shall be configured as a list consisting of one or more targets that the Hosting CSE shall send notifications to. A target shall be formatted as a oneM2M compliant Resource-ID as defined in clause 7.2 or as an identifier compliant with a oneM2M supported protocol binding (e.g. http, coap, mqtt). If a target is formatted as a oneM2M compliant Resource-ID, then the target shall be formatted as a structured or unstructured CSE-Relative-Resource-ID, SP-Relative-Resource-ID, and/or Absolute-Resource-ID. A Hosting CSE shall use this information to determine proper pointOfAccess, requestReqchability and/or pollingChannel information needed to send a notification to the target. The following is an example: - /CSE0001/AE0001 For a target that is formatted as an identifier compliant with a oneM2M supported protocol binding, the details of this format are defined by the respective oneM2M protocol specification. The following is an example of an HTTP URI compliant with oneM2M HTTP protocol binding: - https://172.25.30.25:7000/notification/handler For a subscription to a <fanOutPoint> Resource, if subscription resource in request contains a notificationForwardingURI, then the Group Hosting CSE shall configure the notificationURI of the fanout subscription request with an address specified by the Group Hosting CSE that can be used by the Group Hosting CSE to receive aggregated notifications. |
| groupID | 0..1 | RW | The ID of a <group> resource in case the subscription is made through a group. This attribute may be used in the Filter Criteria to discover all subscription resources created via a <fanOutPoint> resource to a specific groupID. |
| notificationForwardingURI | 0..1 | RW | The attribute is a forwarding attribute that shall be present only for group related subscriptions. It represents the resource subscriber notification target. It shall be used by group Hosting CSE for forwarding aggregated notifications. See clauses 10.2.7.11 and 10.2.7.12. This attribute shall be configured with the target of the subscriber if the subscriber intends for the Group Hosting CSE to aggregate the notifications. The target is used by the Group Hosting CSE to determine where to send aggregated notifications. A target shall be formatted as a oneM2M compliant Resource-ID as defined in clause 7.2 or as an identifier compliant with one of the oneM2M supported protocol bindings (the detailed format of which are defined by each respective oneM2M protocol binding specification). |
| batchNotify | 0..1 | RW | This attribute (notification policy) indicates that the subscription originator wants to receive batches of notifications rather than receiving them one at a time. This attribute includes : the number of notifications to be batched for delivery and the duration. When only the number is specified by the subscription originator, the Hosting CSE shall set the default duration given by M2M Service Provider. If batchNotify is used simultaneously with latestNotify, only the latest notification shall be sent and have the Event Category set to "latest". |
| rateLimit | 0..1 | RW | This attribute (notification policy) indicates that the subscriber wants to limit the rate at which it receives notifications. This attribute expresses the subscriber's notification policy and includes two values: a maximum number of events that may be sent within some duration, and the rateLimit window duration. When the number of generated notifications within the rateLimit window duration exceeds the maximum number, notification events are temporarily stored, until the end of the window duration, when the sending of notification events restarts in the next window duration. The sending of notification events continues as long as the maximum number of notification events is not exceeded during the window duration. The rateLimit policy may be used simultaneously with other notification policies. |
| preSubscriptionNotify | 0..1 | WO | This attribute (notification policy) indicates that the subscriber wants to be sent notifications for events that were generated prior to the creation of this subscription. This attribute has a value of the number of prior notification events requested. If up-to-date caching of retained events is supported on the Hosting CSE and contains the subscribed events then prior notification events will be sent up to the number requested. The preSubscriptionNotify policy may be used simultaneously with any other notification policy. |
| pendingNotification | 0..1 | RW | This attribute (notification policy), if set, indicates how missed notifications due to a period of no connectivity are handled (according to the reachability and notification schedules). The possible values for pendingNotification are: - "sendLatest"; - "sendAllPending". This policy depends upon caching of retained notifications on the hosted CSE. When this attribute is set to "sendLatest", only the last notification shall be sent and it shall have the Event Category set to "latest". If this attribute is not present, the Hosting CSE sends no missed notifications. This policy applies to all notifications regardless of the selected delivery policy (batchNotify, latestNotify, etc.) Note that unreachability due to reasons other than scheduling is not covered by this policy. |
| notificationStoragePriority | 0..1 | RW | Indicates that the subscriber wants to set a priority for this subscription relative to other subscriptions belonging to this same subscriber. This attribute sets a number within the priority range. When storage of notifications exceeds the allocated size, this policy is used as an input with the storage congestion policy (notificationCongestionPolicy) specified in clause 9.6.3 to determine which stored and generated notifications to drop and which ones to retain. |
| latestNotify | 0..1 | RW | This attribute (notification policy) indicates if the subscriber wants only the latest notification. If multiple notifications of this subscription are buffered, and if the value of this attribute is set to true, then only the last notification shall be sent and it shall have the Event Category value set to "latest". |
| notificationContentType | 1 | RW | Indicates a notification content type that shall be contained in notifications. The allowed values are: - "modified attributes"; - "all attributes"; - "ID" of the resource indicated in the notificationEventType condition. For a list of the default and allowed values of notificationContentType for each of the supported values of notificationEventType refer to Table 9.6.8-4. |
| notificationEventCat | 0..1 | RW | This attribute (notification policy) indicates the subscriber's requested Event Category to be used for notification messages generated by this subscription. |
| subscriberURI | 0..1 | WO | This attribute shall be configured with the target of the subscriber. The target is used by the Hosting CSE to determine where to send a notification when the subscription is deleted. A target shall be formatted as a oneM2M compliant Resource-ID as defined in clause 7.2 or as an identifier compliant with one of the oneM2M supported protocol bindings (the detailed format of which are defined by each respective oneM2M protocol binding specification). |

Table 9.6.8-3 describes the eventNotificationCriteria conditions.

Table 9.6.8-3: eventNotificationCriteria conditions

| Condition tag | Multiplicity | Matching condition |
| --- | --- | --- |
| createdBefore | 0..1 | The creationTime attribute of the resource is chronologically before the specified value. |
| createdAfter | 0..1 | The creationTime attribute of the resource is chronologically after the specified value. |
| modifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically after the specified value. |
| unmodifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically before the specified value. |
| stateTagSmaller | 0..1 | The stateTag attribute of the resource is smaller than the specified value. |
| stateTagBigger | 0..1 | The stateTag attribute of the resource is bigger than the specified value. |
| expireBefore | 0..1 | The expirationTime attribute of the resource is chronologically before the specified value. |
| expireAfter | 0..1 | The expirationTime attribute of the resource is chronologically after the specified value. |
| sizeAbove | 0..1 | The contentSize attribute of the <contentInstance> resource is equal to or greater than the specified value. |
| sizeBelow | 0..1 | The contentSize attribute of the <contentInstance> resource is smaller than the specified value. |
| notificationEventType | 0..5 | The type of event. Possible notification event type values are: a) Update to attributes of the subscribed-to resource. b) Deletion of the subscribed-to resource. c) Creation of a direct child of the subscribed-to resource. d) Deletion of a direct child of the subscribed-to resource. e) An attempt to retrieve a <contentInstance> direct-child-resource of a subscribed-to <container> resource is performed while this <contentInstance> child resource is an obsolete resource or the reference used for retrieving this resource is not assigned. This retrieval is performed by a RETRIEVE request targeting the subscribed-to resource with the Result Content parameter set to either "child-resources" or "attributes+child-resources". The other conditions in eventNotificationCriteria conditions apply to the selected notificationEventType. For example, if notificationEventType is "Creation of a direct child of the subscribed-to resource" then other eventNotificationCriteria conditions is applied to the direct child resources of the subscribed-to resource. If this condition is not specified, the default value is "Update to attributes of the subscribed-to resource". The notion of "obsolete resource" is defined in clause 9.6.1.3.2 (Common attributes). |
| operationMonitor | 0..5 | The operations accessing the subscribed-to resource matches with the specified value. It allows monitoring which operation is attempted to the subscribed-to resource regardless of whether the operation is performed. This feature is useful when to find malicious AEs. Possible string arguments are: create, retrieve, update, delete. |
| attribute | 0..n | A list of attribute names of a subscribed-to-resource. This list is only applicable when notificationEventType has a value of "Update to attributes of the subscribed-to resource". If this list is present, then it is used to specify a subset of a subscribed-to-resource's attributes for which updates shall result in a notification. If ANY attribute specified on this list is updated, then a notification shall be generated. If an attribute that is not specified in this list is updated, then a notification shall not be generated. If this list is not presented, then the default attribute list is the full set of a subscribed-to-resource's attributes. If ANY attribute of a subscribed-to-resource is updated, then a notification shall be generated. |
| missingData | 0..1 | The missingData includes two values: a minimum specified missing number of the Time Series Data within the specified window duration, and the window duration. The condition only applies to subscribed-to resources of type <timeSeries>. The first detected missing data point starts the timer associated with the window duration. The window duration is restarted upon its expiry until such time as the entire subscription is terminated or not refreshed. More details about NOTIFICATIONS related to data reporting is found in clause 10.2.39. |
| filterOperation | 0..1 | Indicates the logical operation (AND/OR) to be used for different condition tags. The default value is logical AND. |

The rules when multiple conditions are used together shall be as follows:

- Different condition tags shall use the "AND/OR" logical operation based on the filterOperation specified.

- Same condition tags shall use the "OR" logical operation.

No mixed AND/OR filter operation will be supported.

Table 9.6.8-4 defines the default and allowed values of notificationContentType for each of the supported values of notificationEventType.

Table 9.6.8-4: Default and allowed values of notificationContentType

| notificationEventType notificationContentType | a | b | c | d | e |
| --- | --- | --- | --- | --- | --- |
| "modified attributes"; | valid | n/a | n/a | n/a | n/a |
| "all attributes"; | valid (default) | valid (default) | valid (default) | valid (default) | valid (default) |
| "ID" of the resource indicated in the notificationEventType condition. | valid | valid | valid | valid | valid |

### -----------------------End of change 1---------------------------------------------

No mixed AND/OR filter operation will be supported.CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?