| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TP-39 |
| Source:* | Poornima Shandilya, C-DOT, poornima@cdot.in Chaitan Kumar Yadav , C-DOT, chaitan.yadav@cdot.in Siddharth Trikha, C-DOT, strikha@cdot.in Prateek Varshney, C-DOT, prateekv@cdot.in Anupama Chopra,C-DOT, anupama@cdot.in |
| Date:* | 2019-01-29 |
| Reason for Change/s:* | Refer to Introduction section. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 Version 3.10.0 |
| Clauses * | 7.4.8.2.1, 7.4.8.2.3, 7.5.1.2.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N.A. |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The CR proposes to add validations/clarifications in TS-0004 regarding CSE actions if incorrect values are set in eventNotificationCriteria.

Below is the <subscription> resource from TS-0001:

Table 9.6.8-3: eventNotificationCriteria conditions

| Condition tag | Multiplicity | Matching condition |
| --- | --- | --- |
| createdBefore | 0..1 | The creationTime attribute of the resource is chronologically before the specified value. |
| createdAfter | 0..1 | The creationTime attribute of the resource is chronologically after the specified value. |
| modifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically after the specified value. |
| unmodifiedSince | 0..1 | The lastModifiedTime attribute of the resource is chronologically before the specified value. |
| stateTagSmaller | 0..1 | The stateTag attribute of the resource is smaller than the specified value. |
| stateTagBigger | 0..1 | The stateTag attribute of the resource is bigger than the specified value. |
| expireBefore | 0..1 | The expirationTime attribute of the resource is chronologically before the specified value. |
| expireAfter | 0..1 | The expirationTime attribute of the resource is chronologically after the specified value. |
| sizeAbove | 0..1 | The contentSize attribute of the <contentInstance> resource is equal to or greater than the specified value. |
| sizeBelow | 0..1 | The contentSize attribute of the <contentInstance> resource is smaller than the specified value. |
| notificationEventType | 0..6 | The type of event that shall trigger a notification. If multiple notificationEventType tags are present, a notification shall be triggered if any of the configured events occur. Note that not all permutations of event type are meaningful. Possible notification event type values are: - Update to attributes of the subscribed-to resource - Deletion of the subscribed-to resource, - Creation of a direct child of the subscribed-to resource, - Deletion of a direct child of the subscribed-to resource - An attempt to retrieve a <contentInstance> direct-child-resource of a subscribed-to <container> resource is performed while this <contentInstance> child resource is an obsolete resource or the reference used for retrieving this resource is not assigned. This retrieval is performed by a RETRIEVE request targeting the subscribed-to resource with the Result Content parameter set to either "child-resources" or "attributes+child-resources". This value for the eventNotificationType tag implies that the subscribed-to resource shall be an <container> resource. Otherwise this setting is not valid. - Trigger Received targeting the MN/ASN-AE associated with the <AE> parent resource. This implies that the subscribed-to resource shall be an <AE> resource instance. Otherwise this setting is not valid. - Update to attributes of the subscribed-to resource with blocking of the triggering UPDATE operation. For this eventNotificationType value setting, only one single Notification Target shall be present in the notificationURI attribute – see notificationURI attribute definition. This value for the eventNotificationType tag shall not be combined with any other eventNotificationType tag value. This value for notificationEventType establishes a subscription that is triggered for the same events as for the value “Update to attributes of the subscribed-to resource”. However, upon occurrence of a triggering UPDATE operation that has been validated and results in an authorized UPDATE operation, the triggering UPDATE operation shall be blocked by the Hosting CSE until a notification request was sent out and a corresponding response message was received or a timeout happens. When the response status code of the notification response message indicates a successful notification reception in combination with a successful notification action taken by the Notification Target entity, the triggering UPDATE operation shall be completed with a successful update of the targeted attribute(s). If the notification response message indicates an unsuccessful notification reception or a successful notification reception with unsuccessful notification action by the targeted entity or times out, the blocked UPDATE operation shall be completed with no success and no change of the targeted attribute(s). For any subscribed-to resource there shall exist a maximum of one subscription with this setting of notificationEventType. All other notification policies shall not be allowed when this setting of notificationEventType is used. The notificationContentType shall be “modified attibutes”. When an UPDATE operation has been blocked due to triggering this type of notification, any other occurring UPDATE or DELETE requests to the same resource shall be handled only after the blocked UPDATE operation has been completed. The other conditions in eventNotificationCriteria conditions apply within the scope of the selected notificationEventType. For example, if notificationEventType is "Creation of a direct child of the subscribed-to resource" then other eventNotificationCriteria conditions is applied to the direct child resources of the subscribed-to resource. If this condition is not specified, the default value is "Update to attributes of the subscribed-to resource". The notion of "obsolete resource" is defined in clause 9.6.1.3.2 (Common attributes). |
| operationMonitor | 0..n | The operations and/or the Originators accessing the subscribed-to resource matches with the specified value. It allows monitoring which operation and/or which Originator is attempting to the access subscribed-to resource regardless of whether the operation is performed. This feature is useful to detect AEs that send requests to a subscribed-to resource and that result in a successful or failure response. Possible arguments are operation(s) (e.g.: CREATE, RETRIEVE, UPDATE, DELETE, NOTIFY) and/or Originator identifier(s). If a set of Originator identifier(s) is included in this tag and no operations are listed, any operations initiated from any of the indicated Originator(s) shall trigger a notification. If a set of operation(s) is included in this tag and no Originator identifier, any of the listed operations shall trigger a notification. If both, a set of Originator identifiers and a set of operations are listed, then any of the listed operations initiated from any of the listed Originators shall trigger the notification. When the notificationEventType tag is present in the eventNotificationCriteria, the value of the operationsMonitor tag is ignored if present in the eventNotificationCriteria attribute. |
| Attribute | 0..n | A list of attribute names of a subscribed-to-resource. This list is only applicable when notificationEventType has a value of "Update to attributes of the subscribed-to resource". or “Update to attributes of the subscribed-to resource with blocking of the triggering UPDATE operation”. If this list is present, then it is used to specify a subset of a subscribed-to resource's attributes for which updates shall result in a notification. If ANY attribute specified on this list is updated, then a notification shall be generated. If an attribute that is not specified in this list is updated, then a notification shall not be generated. If this list is not presented, then the default attribute list is the full set of a subscribed-to resource's attributes. If ANY attribute of a subscribed-to resource is updated, then a notification shall be generated. |
| childResourceType | 0.. 1 (L) | A list of resource types. This list is only applicable when notificationEventType has a value of "Creation of a direct child of the subscribed-to resource ". If this list is present, then it is used to specify a subset of resource type for direct child resource of which creation shall result in a notification. If ANY resource type specified on this list is created, then a notification shall be generated. If a resource type that is not specified in this list is created, then a notification shall not be generated. If this list is not present, then the default resource type list is the full set of a direct child resource. |
| missingData | 0..1 | The missingData includes two values: a minimum specified missing number of the Time Series Data within the specified window duration, and the window duration. The condition only applies to subscribed-to resources of type <timeSeries>. The first detected missing data point starts the timer associated with the window duration. The window duration is restarted upon its expiry until such time as the entire subscription is terminated or not refreshed. More details about NOTIFICATIONS related to data reporting is found in section 10.2.4.29 |
| filterOperation | 0..1 | Indicates the logical operation (AND/OR) to be used for the condition tags createdBefore, createdAfter, modifiedSince, unmodifiedSince, stateTagSmaller, stateTagBigger, expireBefore, expireAfter, sizeAbove, sizeBelow. The default value is logical AND. |

### The approach taken in this CR is to ignore such values in line with approach given for handling of operationMonitor in TS-0001 or other approach could be to reject such request.

### -----------------------Start of change 1-------------------------------------------

#### 7.4.8.2 <subscription> resource specific procedures for CRUD operations

##### 7.4.8.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The following are additional Hosting CSE procedures to the generic resource handling procedures (Error: Reference source not found in clause Error: Reference source not found). The additional procedures shall be inserted from Recv-6.2 to Recv-6.8 as below.

The resource handling procedure for the Hosting CSE which receives <subscription> CREATE request shall perform the following procedures in order:

- Recv-6.2

- Recv-6.3

- Check if the subscribed-to resource, addressed in the To parameter in the Request, is subscribable. Subscribable resource types are defined in oneM2M TS-0001 [Error: Reference source not found], they have <subscription> resource types as their child resources.

If it is not subscribable, the Hosting CSE shall return the Notify response primitive with a Response Status Code indicating "TARGET_NOT_SUBSCRIBABLE" error.

- Check if the Originator has privileges for retrieving the subscribed-to resource.

If the Originator does not have the privilege, the Hosting CSE shall return the response primitive with Response Status Code indicating "ORIGINATOR_HAS_NO_PRIVILEGE" error.

- If any of the notificationURI entries are not the Originator, the Hosting CSE may send a Notify request primitive to each of them with the verificationRequest parameter set to true (see clause 7.5.1.2.3).

a)	If the Hosting CSE cannot send one or more Notify request primitives, the Hosting CSE shall return the response primitive with a Response Status Code indicating "SUBSCRIPTION_VERIFICATION_INITIATION_FAILED" error.

b)	If the Hosting CSE sent all the Notify request primitives, the Hosting CSE shall check if each Notify response primitive contains a Response Status Code indicating "OK". If not, the Hosting CSE shall return the Create response primitive containing the Response Status Code from the first unsuccessful Notify response primitive.

- Recv-6.4

- Recv-6.5

If the Originator provides  a value of childResourceType which is not a valid child of the subscribed-to resource, the request shall be rejected with a “BAD_REQUEST” Response Status Code.

If both the notificationEventType and operationMonitor are present in the Request, the request shall be rejected with a “BAD_REQUEST” Response Status Code.

If the Originator does not provide notificationContentType, the Hosting CSE shall set it as 'all attributes'.

If the notificationURI is not the Originator, the Hosting CSE shall store the Originator ID as the <subscription> resource's creator attribute.

If the batchNotify attribute is present in the Request but batchNotify/duration is not provided by the Originator, the Hosting CSE shall set the value of batchNotify/duration to the default duration as given by the M2M Service Provider.

If the associatedCrossResourceSub is provided, check that the Hosting CSE ID value in the associatedCrossResourceSub is the same as the From parameter of the request.

- Recv-6.6

- Recv-6.7

- Recv-6.8

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

##### 7.4.8.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The following are additional Hosting CSE procedures to the generic resource handling procedures in clause Error: Reference source not found.

- Recv-6.5.

If the Originator provides a  value of childResourceType which is not a valid child of the subscribed-to resource,the request shall be rejected with a “BAD_REQUEST” Response Status Code.

If the UPDATE operation would result in both operationMonitor and notificationEventType being present in the resource, the request shall be rejected with a “BAD_REQUEST” Response Status Code.

- If a new associatedCrossResourceSub is provided, check that the Hosting CSE ID value in the associatedCrossResourceSub is the same as the From parameter of the request. If a <crossResourceSubscription> Hosting CSE ID is removed from associatedCrossResourceSub, the Hosting CSE shall send a Notify request for Subscription Deletion using the procedures in clause 7.5.1.2.4 to the <crossResourceSubscription> Hosting CSE.

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3---------------------------------------------

##### 7.5.1.2.2 Notification for <subscription> resources

When the notification message is forwarded or aggregated by transit CSEs, the Originator or a transit CSE shall check whether there are notification policies to enforce between subscription resource Hosting CSE and the notification target. In that case, the transit CSE as well as the Originator shall process Notify request primitive(s) by using the corresponding policy and send processed Notify request primitive(s) to the next CSE with notification policies related to the enforcement so that the transit CSE is able to enforce the policy defined by the subscriber. The notification policies related to the enforcement at this time is verified by using the subscription reference in the Notify request primitive. In the notification policies, the latestNotify attribute is only enforced in the transit CSE as well as the Originator.

If Event Category parameter is set to 'latest' in the notification request primitive, the transit CSE as well as Originator shall cache the most recent Notify request. That is, if a new Notify request is received by the CSE with a subscription reference that has already been buffered for a pending Notify request, the newer Notify request will replace the buffered older Notify request.

Originator:

When an event is generated, the Originator shall execute the following steps in order:

Step 1.0	Check the eventNotificationCriteria attribute of the <subscription> resource associated with the modified resource:

- If the eventNotificationCriteria attribute is set, then the Originator shall check whether the corresponding event matches with the event criteria.

- If notificationEventType is not set within the eventNotificationCriteria attribute, the Originator shall use the default setting of 'Update_of_Resource' to compare against the event.

- If the notificationEventType has either an explicit or default value of 'Update_of_Resource' and the attribute condition is also present then the matching event shall only be detected if one of the attributes in the list has been updated. If the attribute condition is not present then a matching event is generated whenever any attribute has been updated.

- If the notificationEventType has value of ‘Create_of_Direct_Child_Resource' and the childResourceType condition is also present, then the matching event shall only be detected if one of the child resource types present in the list has been created. If the childResourceType condition is not present then a matching event is generated whenever any child resource is created.

- If the event matches, go to the step 2.0. Otherwise, the Originator shall discard the corresponding event.

- If the eventNotificationCriteria attribute is not configured, the Originator shall use the default setting of 'Update_of_Resource' for the notificationEventType and then continue with the step 2.0.

Step 2.0	The Originator shall check the notification policy as described in the below steps, but the notification policy may be checked in different order. After checking the notification policy in step 2.0 (i.e. from step 2.1 to step 2.6), then continue with step 3.0.

Step 2.1	The Originator shall determine the type of the notification per the notificationContentType attribute. The possible values of for notificationContentType attribute are 'Modified Attributes', 'All Attributes', 'ResourceID' or 'Trigger Payload'. This attribute may be used jointly with eventType attribute in the eventNotificationCriteria to determine if it is the attributes/resourceID of the subscribed-to resource or the attributes/resourceID of the child resource of the subscribed-to resource that shall be returned in the notification:

- If the value of notificationContentType is set to 'Modified Attributes', the Notify request primitive shall include the partial resource containing modified attribute(s) only (Refer to clause Error: Reference source not found for response content description).

- If the value of notificationContentType is set to 'All Attributes', the Notify request primitive shall include the complete resource with all attributes (Refer to clause Error: Reference source not found for response content description).

- If the value of notificationContentType is set to 'ResourceID', the Notify request primitive shall include the URI of the resource (Refer to clause Error: Reference source not found for response content description).

- If the value of notificationContentType is set to 'Trigger Payload', the Notify request primitive shall include the trigger payload (Refer to clause 9.2.1 for trigger payload description).

Step 2.2	Check the notificationEventCat attribute:

- If the notificationEventCat attribute is set, the Notify request primitive shall employ the Event Category parameter as given in the notificationEventCat attribute. Then continue with the step 2.3.

- If the notificationEventCat attribute is not configured, then continue with step 2.3.

Step 2.3	Check the latestNotify attribute:

- If the latestNotify attribute is set, the Originator shall assign Event Category parameter of value 'latest' of the notifications generated pertaining to the subscription created.

Step 2.4	Check the batching notifications policy:

- See details in oneM2M TS-0001 [Error: Reference source not found], clause 10.2.10.7.

NOTE:	The use of some attributes such as rateLimit and preSubscriptionNotify is not supported in the present document.

Step 2.5	Check the notificationURI attribute:

- The Originator shall fetch the notificationURI attribute and set the value to the To parameter of the Notify request. When the notificationURI attribute contains more than one target, the Originator shall generate each Notify request per target.

- If the notificationURI attribute includes the notification serialization indication, in form of key-value pair, e.g. "ct=json", after the delimiter "?", the Originator shall serialize the notification for the notification target in that serialization type. The delimiter with the serialization indication shall be removed when the target is set to the To parameter of the Notify request. Then continue with step 3.0.

Step 3.0	The Originator shall check the notification and reachability schedules, but the notification schedules may be checked in different order:

- If the <subscription> resource associated with the modified resource includes a <notificationSchedule> child resource, the Originator shall check the time periods given in the scheduleElement attribute of the <notificationSchedule> child resource.

- Also, the Originator shall check the reachability schedule associated with the Receiver by exploring its <schedule> resource. If reachability schedules are not present in a Node then that Entity is considered to be always reachable.

- If notificationSchedule and reachability schedule indicate that message transmission is allowed, then proceed with step 5.0. Otherwise, proceed with step 4.0.

- In particular, if the notificationEventCat attribute is set to 'immediate' and the <notificationSchedule> resource does not allow transmission, then go to step 5.0 and send the corresponding Notify request primitive by temporarily ignoring the Originator's notification schedule.

Step 4.0	Check the pendingNotification attribute:

- If the pendingNotification attribute is set, then the Originator shall cache pending Notify request primitives according to the pendingNotification attribute. The possible values are 'sendLatest' and 'sendAllPending'. If the value of pendingNotification is set to 'sendLatest', the most recent Notify request primitive shall be cached by the Originator and it shall set the Event Category parameter to 'latest'. If pendingNotification is set to 'sendAllPending', all Notify request primitives shall be cached by the Originator. If the pendingNotification attribute is not configured, the Originator shall discard the corresponding Notify request primitive. The processed Notify request primitive by the pendingNotification attribute is sent to the Receiver once message transmission becomes possible (see the step 6.0).

Step 5.0	Check the expirationCounter attribute:

- If the expirationCounter attribute is set, then it shall be decreased by one when the Originator successfully sends the Notify request primitive. If the counter equals to zero('0'), the corresponding <subscription> resource shall be deleted. Then end the 'Compose Notify Request Primitive' procedure.

- If the expirationCounter attribute is not configured, then end the 'Compose Notify Request Primitive' procedure.

Originator:

When message transmission becomes possible, the Originator shall execute the following steps in order:

Step 6.0	If the pendingNotification attribute is set, the Originator shall send the processed Notify request primitive by the pendingNotification attribute and then continue with the step 7.0

Step 7.0	Check the expirationCounter attribute:

- If the expirationCounter attribute is set, then its value shall be decreased by one when the Originator successfully sends the Notify request primitive. If the counter meets zero, the corresponding <subscription> resource shall be deleted. Then end the 'Compose Notify Request Primitive' procedure.

- If the expirationCounter attribute is not configured, then end the 'Compose Notify Request Primitive' procedure.

Receiver:

When the Hosting CSE receives a Notify request primitive, the Hosting CSE shall check validity of the primitive parameters. In case the Receiver is a transit CSE which forwards or aggregates Notify request primitives before sending to the subscriber or other transit CSEs, upon receiving the Notify request primitive with the Event Category parameter set to 'latest', the Receiver shall identify the latest Notify request primitive with the same subscription reference while storing Notify request primitives locally. When the Receiver as a transit CSE needs to send pending Notify request primitives, it shall send the latest Notify request primitive. When the Receiver as a transit CSE needs to send Notify request primitives, it shall use one of the serializations specified in the subscriber or other transit CSE contentSerialization attribute. If there is no contentSerialization value specified the transit CSE may use any serialization format.

### -----------------------End of change 3---------------------------------------------

No mixed AND/OR filter operation will be supported.CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?