| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS-39 |
| Source:* | Poornima Shandilya, C-DOT, poornima@cdot.in Kapil Badwal, C-DOT, kapil@cdot.in Suman Sheoran, C-DOT, ssheoran@cdot.in Anupam Chopra, C-DOT, anupama@cdot.in |
| Date:* | 2019-01-29 |
| Reason for Change/s:* | Refer to Introduction section. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 Version 3.10.0 |
| Clauses * | 7.3.2.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N.A. |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The CR proposes to add some validations regarding From parameter as mentioned in TS-0001.

The format (i.e. CSE-relative, SP-relative or absolute) of resource identifier (e.g. the To parameter, accessControlPolicyIDs attribute) shall be correctly set by the Originator in an initial request, while the format of AE-ID or CSE-ID in the From parameter shall be set in a shortest format by the Originator in the initial request and it shall be converted in another format by the Registrar CSE or IN-CSE as the following. When an AE is the Originator, the From parameter shall be in AE-ID-Stem. When the Registrar CSE receives the request, it shall convert the format into SP-relative AE-ID in case the stem is CSE-relative and the To parameter refers to a resource hosted by a different CSE. When an CSE is the Originator, the From parameter shall be in SP-relative CSE-ID. The IN-CSE shall convert the format of the From parameter in a request that is received from SP-relative to absolute if the To parameter refers to a resource is hosted by a CSE in a different M2M Service Provider Domain.

### -----------------------Start of change 1-------------------------------------------

#### 7.3.2.1	Check the validity of received request primitive

The validity checking of the message carrying the received request primitive is specified by the protocol mapping Technical Specifications (CoAP binding [Error: Reference source not found], HTTP binding [Error: Reference source not found], and MQTT binding [Error: Reference source not found]).

If the Request Expiration Timestamp is given in the request and expired, the Receiver CSE shall reject the request with an "REQUEST_TIMEOUT" Response Status Code parameter value.

If the From parameter is not present in the request except AE Create request, the Receiver CSE shall reject the request with "BAD_REQUEST" Response Status Code parameter value.

If the received request is communicated within an established Security Association (oneM2M TS-0003 [Error: Reference source not found]), and:

- the Receiver knows that the Registree using the established Security Association is an AE; and

- the From parameter is not in CSE-relative AE-ID Stem format

then the request shall be rejected with an “BAD_REQUEST” Response Status Code parameter value.

If the received request is communicated within an established Security Association, and:

- the Receiver knows that the Registree using the established Security Association is a CSE; and

- the Receiver knows the CSE -ID of the Registree using the established Security Association; and

- the Receiver knows that the Originator of the request is its Registree; and

- the From parameter is in Absolute Form

then the request shall be rejected with an “BAD_REQUEST” Response Status Code parameter value.

If the received request is communicated within an established Security Association (oneM2M TS-0003 [Error: Reference source not found]), and:

- the Receiver knows that the Registree using the established Security Association is an AE; and

- the Receiver knows the AE-ID(s) of the Registree using the established Security Association; and

- the From parameter does not match the allowed AE-ID(s) of the Registree using the established Security Association;

then the request shall be rejected with an "ORIGINATOR_HAS_NOT_REGISTERED " Response Status Code parameter value.

If the received request is communicated within an established Security Association, and:

- the Receiver knows that the Registree using the established Security Association is a CSE; and

- the Receiver knows the CSE -ID of the Registree using the established Security Association; and

- if one of the following applies:

- the From parameter is an CSE-ID that matches one of the Receiver's Registree CSE's CSE-ID other than the CSE-ID of the Registree using the established Security Association, or

- the From parameter is an CSE-Relative C-Type AE-ID-Stem, or

- the From parameter is an SP-Relative AE-ID or Absolute AE-ID with a C-Type AE-ID-Stem, and the CSE-ID portion of the From parameter matches one of the Receiver's Registree CSE's CSE-ID other than the CSE-ID of the Registree for the established Security Association;

then the request shall be rejected with an "ORIGINATOR_HAS_NOT_REGISTERED" Response Status Code parameter value.

NOTE:	An SP-Relative-AE-ID or Absolute AE-ID with a C-Type AE-ID-Stem always includes a CSE-ID portion (see oneM2M TS-0001 [Error: Reference source not found]).

If the received request is communicated outside of an established Security Association, and:

- If the From parameter includes an AE-ID; and

- The request is not a CREATE <AE> Request; and

- the From parameter is not in CSE-relative AE-ID Stem format;

then the request shall be rejected with a "BAD_REQUEST" Response Status Code parameter value.

If the received request is communicated outside of an established Security Association, and:

- If the From parameter includes an AE-ID; and

- The request is not a CREATE <AE> Request; and

- The From parameter does not match the AE-ID of an AE currently registered to the Receiver;

then the request shall be rejected with a "ORIGINATOR_HAS_NOT_REGISTERED" Response Status Code parameter value.

If the received request is communicated outside of an established Security Association, and the From parameter includes a CSE-ID, then the request shall be rejected with an "SECURITY_ASSOCIATION_REQUIRED" Response Status Code parameter value.

If a received request needs to be forwarded to another CSE and if CMDH processing is supported, then in addition, the "CMDH message validation procedure" defined in clause Error: Reference source not found shall be carried out.

If the message is not valid, the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

If Resource Type is not present or is invalid in a CREATE request, the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

If the Filter Criteria parameter is included in a CREATE request, the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

If the Result Content is invalid for a given operation (refer oneM2M TS-0001 [Error: Reference source not found], Table 8.1.2-1: Summary of Result Content Values) then the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

If the receiver does not support the content format (i.e. type of serialization) requested by the originator, the request may be rejected with a Response Status Code indicating "NOT_ACCEPTABLE" error.

If the receiver does not support the content format sent by the originator, the request may be rejected with a Response Status Code indicating “UNSUPPORTED_MEDIA_TYPE” error.

If the Release Version Indicator is not present in the request the Receiver CSE shall add the Release Version Indicator with value set to 1.

If the Receiver CSE is the Hosting CSE and it does not support the release value present in the Release Version Indicator then the request shall be rejected with a Response Status Code indicating "RELEASE_VERSION_NOT_SUPPORTED" error.

### -----------------------End of change 1---------------------------------------------

No mixed AND/OR filter operation will be supported.CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?