| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #39 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com |
| Date:* | 2019-02-11 |
| Reason for Change/s:* | The CR is to add new feature of 3GPP QoS interworking. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0058 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0024 |
| Clauses * | 8.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TR-0024 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

When creating QoS, the creator focus on the QoS between the Originator and the targeted node for the specified resource type. This solution supports the E2E QoS creation, but only supports the QoS flow between IN-CSE and ADN-AE or ASN/MN-CSE which is 3GPP UE.

QoS Requirements in TS-0002:

| OSR-038 | The oneM2M System shall support the inclusion of M2M Application's QoS preference in service requests to Underlying Networks. | Not implemented |
| --- | --- | --- |
| OSR-039 | The oneM2M System shall be able to authorize service requests with QoS preference at service level, but shall pass M2M Application's QoS preference in service requests to Underlying Network for authorization and granting or negotiation of the service QoS requests. | Not implemented |
| OSR-042 | The oneM2M System shall be able to support different QoS-levels specifying parameters, such as guaranteed bitrate, delay, delay variation, loss ratio and error rate, etc. | Not implemented |

### -----------------------Start of change 1-------------------------------------------

## 8.x	Setting up an AS session with required QoS procedure

### 8.x.4 oneM2M solution

#### 8.x.4.1	Resource Structure

##### 8.x.4.1.1 Resource Type E2EQoS

The <E2EQoS> resource is used to request that the IN-CSE negotiates a QoS for a resource when is operated between the creator and the targeted entity, with the Underlying Network. The resource attributes provide the End to End information before the resource is actually operated.

The <E2EQoS> resource contains the child resources specified in table 8.x.4.1.1-1.

Table 8.x.4.1.1-1: Child resources of <E2EQoS> resource

| Child Resources of < E2EQoS > | Child Resource Type | Multiplicity | Description | <E2EQoS> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8. | <subscription> |

The <E2EQoS> resource contains the attributes specified in table 8.x.4.1.1-2.

Table 8.x.4.1.1-2: Attributes of <E2EQoS> resource

| Attributes of <E2EQoS> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 |
| resourceID | 1 | RO | See clause 9.6.1.3 |
| resourceName | 1 | WO | See clause 9.6.1.3 |
| parentID | 1 | RO | See clause 9.6.1.3 |
| creationTime | 1 | RO | See clause 9.6.1.3 |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 |
| expirationTime | 1 | RW | See clause 9.6.1.3 |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| announceTo | 0..1(L) | RW | See clause 9.6.1.3 |
| supportedResourceType | 1(L) | RW | See clause 9.6.1.3 |
| qosLevel | 1 | RW | Indicates the QoS level.It’s linked to the QoS information which has been configured in oneM2M system: 0~200: indicates 3GPP QoS level mapped with 3GPP QoS Class Identifier which is specified in 3GPP TS23.203 [i.11]. >200: reserved for future use. |
| usageThreshold | 0..1 | RW | Time period and/or traffic volume in which the QoS is to be applied. It’s specified in 3GPP TS 29.122[i.aa] |
| qosStatus | 0..1(L) | RW | List of arrays which consist of two elements: QoS status and actions according to the status. For example, when the usage of the QoS is exhausted , the IN-CSE shall delete the QoS with the underlying network. |
| E2EInfo | 1(L) | RW | Indicates the targeted entity and flow direction which QoS is applied for. List of arrays which consist of two elements: direction and targetEntityID. direction: Indicates the direction of resource operation. - In：from Originator entity to Target entity - Out：from target entity to Originator entity - Both: from Originator entity to Target entity and from target entity to Originator entity targetEntityID: Indicates the targeted entity ID of this E2E information. |

8.x.4.1.2 Resource Type CSEBase

Table 8.x.4.1.2-1 Child resources of <CSEBase> resource

| Child Resources of <CSEBase> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <E2EQoS> | 0..1 | See clause 9.6.aa |
| [variable] | <3GPPQoS> | 0..n | See clause 8.x.4.1.3 |

##### 8.x.4.1.3 Resource Type remoteCSE

Table 8.x.4.1.3-1: Child resources of <remoteCSE> resource

| Child Resources of <remoteCSE> | Child Resource Type | Multiplicity | Description | <remoteCSEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <E2EQoS> | 0..1 | See clause 8.x.4.1.1 | <E2EQoSAnnc> |

##### 8.x.4.1.4 Resource Type AE

Table 8.x.4.1.4-1: Child resources of <AE> resource

| Child Resources of <AE> | Child Resource Type | Multiplicity | Description | <AEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <E2EQoS> | 0..1 | See clause 8.x.4.1.1 | <E2EQoSAnnc> |

#### 8.x.4.2 	Create E2E QoS procedure

Figure 8.x.4.2-1: Create E2E QoS procedure

Step 0.a: The IN-AE performs the oneM2M registration procedure and the IN-CSE creates <3GPPQoS> resource.

The IN-AE performs the oneM2M registration procedure. The IN-CSE hosts the corresponding <AE> resource for the registree. The IN-CSE pre-defines the 3GPP QoS Information according to the local policy.

Table 8.x.4.2-1: 3GPP QoS Information

| Information | Multiplicity | Description |
| --- | --- | --- |
| qosLevel | 1 | Indicates the QoS level according to the 3GPP QCI. |
| QCI | 1 | QoS Class Identifier which is specified in 3GPP TS23.203 [i.11].e.g. QCI 1 is used for Conversational Voice. |
| Max-Requested-Bandwidth-UL | 0..1 | Indicates the maximum requested bandwidth in bits per second for an uplink IP flow. Refer to 3GPP TS29.214 [i.xx]. |
| Max-Requested-Bandwidth-DL | 0..1 | Indicates the maximum bandwidth in bits per second for a downlink IP flow. Refer to 3GPP TS29.214 [i.xx]. |
| Priority-Sharing-Indicator | 0..1 | It is used to indicate that the related media component can use the same Allocation and Retention Priority as media component(s). Refer to 3GPP TS29.214 [i.xx]. |
| Pre-emption-Capability | 0..1 | Indicates whether a service data flow can get resources that were already assigned to another service data flow with a lower priority level. Refer to 3GPP TS 29.212[i.zz]. |
| Pre-emption-Vulnerability | 0..1 | Indicates whether a service data flow can lose the resources assigned to it in order to admit a service data flow with higher priority level. Refer to 3GPP TS 29.212[i.zz]. |

Step1-2: The IN-AE sends the <E2EQoS> resource CREATE request, the < E2EQoS > includes the following setting:

- supportedResourceType shall be set to the resource type in which the QoS is to applied, e.g. mgmtCmd and locationPolicy

- qosLevel shall be set to the value which is mapped with 3GPP QCI according to the resource type.

- usageThreshold shall be set to the Time period and/or traffic volume in which the QoS is to be applied.

- status of qosStatus shall be set to USAGE_EXHUSTED, and action of qosStatus shall be set to DELETE.

- direction of the E2EInfo shall be set to the value according to the resource type, e.g. In means from IN-AE to target ASN/MN-CSE or ADN-AE. targetEntityID of the E2EInfo shall be set to the entity ID of target ASN/MN-CSE or ADN-AE in which the QoS is to be applied.

The IN-CSE creates the child resource <E2EQoS> of the resource <AE>, and QoS information associated with the resource type according to the request.

8.x.4.3 	Setting up 3GPP session with required QoS procedure

Figure 7.9.3.2-1: Setting up 3GPP session with required QoS

Step 1: The IN-AE sends the resource operation request to the target ASN/MN-CSE or ADN-AE.

The IN-AE sends the resource (e.g. mgmtCmd) operation request with the resource ID to the target ASN/MN-CSE or ADN-AE.

Step 2: The IN-CSE checks whether there is the QoS information associated with the resource in the request; if there is, the IN-CSE gets the registration information of target ASN/MN-CSE or ADN-AE.

The IN-CSE gets the <E2EQoS> of the IN-AE and the and check if the resource type in the operation request is included in the supportedResourceType. If it is included, the IN-CSE check the transport link between the target ASN/MN-CSE or ADN-AE and IN-CSE:

CaseA: If it is TCP:

- Step 3.1a (Optional): IN-CSE initials TCP connectivity establishment with target ASN/MN-CSE or ADN-AE if there is no transport link;

- Step 3.2a: IN-CSE gets the IP 5-tuples of TCP and maps it to 3GPP flow information.

CaseA: If it is UDP:

- Step 3b: IN-CSE gets the IP 5-tuples of UDP and maps it to 3GPP flow information.

Step4: The IN-CSE sends the On-demand QoS request to SCEF to negotiate the QoS.

The IN-CSE sends an AsSessionWithQoSSubscription request to the SCEF according to the registration network information of the target ASN/MN-CSE or ADN-AE to set up an session request with specific QoS for the service provided by the IN-CSE.

The AsSessionWithQoSSubscription request from the IN-CSE to the SCEF shall comply with i.53GPP TS 29.122 [i.aa]. The AsSessionWithQoSSubscription request shall include:

- notificationDestination shall be set to a URI that the SCEF can target QoS status notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

- flowInfo shall be set to the value of IP 5-tuples between IN-CSE and target ASN/MN-CSE or ADN-AE. For example: The direction of E2EInfo is In which means from the IN-AE to the target ASN/MN-CSE or ADN-AE and parameter mappings are as below:

| - 3GPP API parameter | - Value |
| --- | --- |
| - flowId | - Assigned by the IN-CSE according local policy. |
| - Direction | - Out (downlink IP flows) |
| - Source IP address | - IP address of IN-CSE |
| - destination IP address | - IP address of target ASN/MN-CSE or ADN-AE |
| - Protocol | - TCP/UDP |
| - Source port | - Port number of IN-CSE |
| - destination port | - Port number of target ASN/MN-CSE or ADN-AE |

- qosReference shall be set to the value of qosLevel of <E2EQoS>.

- ueIpv4Addr or ueIpv6Addr shall be set to the value of  the IP address of target ASN/MN-CSE or ADN-AE

- usageThreshold shall be set to the value of usageThreshold of <E2EQoS>

- sponsorInfo shall be set to the value of sponsor information.

Step 5: 3GPP initiated IP-CAN Session Modification according to the QoS request.

The SCEF interacts with the 3GPP entity triggers 3GPP initiated IP-CAN Session Modification according to the QoS request defined in 3GPP TS 23.203[i.11].

Step 6: SCEF sends the On-demand QoS response to IN-CSE.

The SCEF sends an AsSessionWithQoSSubscription response that contains information as specified in 3GPP TS 29.122 [i.aa] to the IN-CSE.

Step7: Message delivery via 3GPP network with required QoS.

IN-CSE sends the resource operation request to the target ASN/MN-CSE or ADN-AE via 3GPP network with required QoS and the target ASN/MN-CSE or ADN-AE sends response to IN-CSE.

Step8: IN-CSE sends response to the IN-AE.

#### 8.x.4.4 	3GPP QoS status monitoring and report procedure

Figure 7.9.3.3-1: 3GPP QoS status report procedure

Step 1: The 3GPP network entities detect the IP-CAN session modification.

The 3GPP entities may notify the SCEF about bearer level events for the Rx session (e.g., transmission resources are released/lost) with an IP-CAN Session Modification as described in TS 23.203[i.11].

Step 2: SCEF sends Status notification message to IN-CSE.

When the SCEF receives information of status change in step 1, the SCEF creates and sends an Event Notification message to the IN-CSE as specified in 3GPP TS 29.122 [i.aa].

The Event Notification request includes the following:

- event is configured with the value as specified in 3GPP TS 29.122 [i.aa].

- accumulatedUsage is configured the usage of usageThreshold  when event is USAGE_REPORT.

- flowIds is configured with the same value of flowId in the AsSessionWithQoSSubscription request.

Step 3: IN-CSE checks the qosStatus and performs the action according to the configuration.

When the IN-CSE receives the Event Notification, the IN-CSE will map the event to the qosStatus of <E2EQoS> and performs the action according to the status.

Step 4 (Optional): IN-CSE requests to delete On-demand QoS.

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this AsSessionWithQoSSubscription.  This step may be triggered by the IN-CSE when message delivery completes or the action configured in qosStatus.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

# 2	References

## 2.1	Normative references

As informative publications shall not contain normative references this clause shall remain empty.

Not applicable.

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	oneM2M TS-0002: "Requirements".

[i.3]	3GPP TS 22.101: "Service aspects; Service principles (Release 13)".

[i.4]	3GPP TS 22.115: "Service aspects; Charging and billing (Release 13)".

[i.5]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications (Release 14)".

[i.6]	OMA API Inventory.

NOTE:	Available at http://technical.openmobilealliance.org/Technical/technical-information/oma-api-program.

[i.7]	OMA Service Exposure Framework.

NOTE:	Available at http://member.openmobilealliance.org/ftp/Public_documents/ARCH/ServiceExposure.

[i.8]	OMA Exposing Network Capabilities to M2M.

NOTE:	Available at http://member.openmobilealliance.org/ftp/Public_documents/ARCH/ENCap-M2M.

[i.9]	oneM2M TS-0001: "Functional Architecture ".

[i.10]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications (Release 13)".

[i.11]	3GPP TS 23.203: "Policy and charging control architecture (Release 15)".

[i.12]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC); Stage 1".

[i.13]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[i.14]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[i.15]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[i.xx]                    3GPP TS 29.214: Technical Specification Group Core Network and Terminals; Policy and Charging Control over Rx reference point (Release 15)

[i.zz]                    3GPP TS 29.212: Technical Specification Group Core Network and Terminals; Policy and Charging Control (PCC); Reference points (Release 15)

[i.aa]                    3GPP TS 29.122: T8 reference point for Northbound APIs (Release 15)

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?