| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #39 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com |
| Date:* | 2019-02-11 |
| Reason for Change/s:* | The CR is to add new feature of 3GPP QoS interworking. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0058 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0024 |
| Clauses * | 8.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TR-0024 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

When creating QoS, the creator focus on the QoS between the Originator and the targeted node for the specified resource type. This solution supports the E2E QoS creation, but only supports the QoS flow between IN-CSE and ADN-AE or ASN/MN-CSE which is 3GPP UE.

QoS Requirements in TS-0002:

| OSR-038 | The oneM2M System shall support the inclusion of M2M Application's QoS preference in service requests to Underlying Networks. | Not implemented |
| --- | --- | --- |
| OSR-039 | The oneM2M System shall be able to authorize service requests with QoS preference at service level, but shall pass M2M Application's QoS preference in service requests to Underlying Network for authorization and granting or negotiation of the service QoS requests. | Not implemented |
| OSR-042 | The oneM2M System shall be able to support different QoS-levels specifying parameters, such as guaranteed bitrate, delay, delay variation, loss ratio and error rate, etc. | Not implemented |

R01:

Delete the service plow in this contribution.

### -----------------------Start of change 1-------------------------------------------

## 8.x	Setting up an AS session with required QoS procedure

### 8.x.4 oneM2M solution

#### 8.x.4.1	Resource Structure

##### 8.x.4.1.1 Resource Type e2eQosSession

The <e2eQosSession> resource defines end-to-end (E2E) QoS session requirements for the exchange of oneM2M request and response primitives between oneM2M entities. This resource consists of a set of QoS parameters and an applicable set of oneM2M entities that exchange oneM2M requests and responses with one another and function as the session endpoints.  A Hosting CSE uses this resource to configure and manage E2E QoS between these session endpoints.  For example, the Hosting CSE can manage the establishment and tear-down of QoS session(s) in underlying network(s) that interconnect oneM2M entities with one another. When establishing a QoS session in an underlying network, a Hosting CSE can configure the QoS parameters based on the requirements defined by the <e2eQosSession> resource.  For cases where the oneM2M entities are separated by multiple hops, each hop may require the establishment and configuration of a separate underlying network QoS session.   For this case, a Hosting CSE can coordinate with its registrar and registree CSEs to assist it with the configuration of a QoS session in each respective underlying network involved in a multi-hop E2E QoS session such that the E2E QoS requirements defined within the <e2eQosSession> can be satisfied across the multiple hops.

Note - The details for how an E2E QoS session is established across multiple hops is for FFS.

The <e2eQosSession> resource contains the child resources specified in table 8.x.4.1.1-1.

Table 8.x.4.1.1-1: Child resources of <e2eQosSession> resource

| Child Resources of <e2eQosSession> | Child Resource Type | Multiplicity | Description | <e2eQosSession> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8. | <subscription> |

The <e2eQosSession> resource contains the attributes specified in table 8.x.4.1.1-2.

Table 8.x.4.1.1-2: Attributes of <e2eQosSession> resource

| Attributes of <e2eQosSession> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 |
| resourceID | 1 | RO | See clause 9.6.1.3 |
| resourceName | 1 | WO | See clause 9.6.1.3 |
| parentID | 1 | RO | See clause 9.6.1.3 |
| creationTime | 1 | RO | See clause 9.6.1.3 |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 |
| expirationTime | 1 | RW | See clause 9.6.1.3 |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| announceTo | 0..1(L) | RW | See clause 9.6.1.3 |
| supportedResourceID | 1(L) | RW | See clause 9.6.1.3. Indicates the resource ID list which the E2E QoS applies for. |
| e2eQosRequirements | 1 | RW | Indicates the E2E QoS requirements expressed as a list. Each element in the list consists of the following information. - Source Endpoint – the identifier (AE-ID or CSE-ID) of the entity initiating requests associated with this E2E QoS session. It could be the same with the Creator of the requrest. - Destination Endpoints – the identifiers (AE-IDs or CSE-IDs) of the target entities associated with this E2E QoS Session - QoS Level (mandatory): Represents required QoS level. Expressed as a range from 0 (lowest) -100 (highest) - Time Period (optional): Represents required time windows of when the QoS level needs to be active. - Traffic Volume (optional): Represents required traffic volumes in terms of number of requests or total bytes allowed of when the QoS level needs to be active. |
| qosPolicy | 0..1(L) | RW | List of arrays which consist of two elements: - QoS session status: Indicates the status of the QoS session of which the IN-CSE negotiates with the underlying network. The predefined values are: - EXHAUSTED: the time window is time over or the traffic volume is exhauseted. - FAILED: the QoS session negotiation is failed between the IN-CSE and the underlying network. - Action: Indicates the operation the IN-CSE shall take according to the status. The pre-defined values are: - Notify: Notify the QoS status to the Source Endpoint. - Delete: Delete the QoS session with the underlying network. For example, when QoS session status is EXHAUSTED, the Action is Delete which means the IN-CSE shall delete the QoS session with the underlying network when the usage of the QoS is exhausted. |

8.x.4.1.2 Resource Type CSEBase

Table 8.x.4.1.2-1 Child resources of <CSEBase> resource

| Child Resources of <CSEBase> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <e2eQosSession> | 0..1 | See clause 9.6.aa |

##### 8.x.4.1.3 Resource Type remoteCSE

Table 8.x.4.1.3-1: Child resources of <remoteCSE> resource

| Child Resources of <remoteCSE> | Child Resource Type | Multiplicity | Description | <remoteCSEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <e2eQosSession> | 0..1 | See clause 8.x.4.1.1 | < e2eQosSession Annc> |

##### 8.x.4.1.4 Resource Type AE

Table 8.x.4.1.4-1: Child resources of <AE> resource

| Child Resources of <AE> | Child Resource Type | Multiplicity | Description | <AEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <e2eQosSession> | 0..1 | See clause 8.x.4.1.1 | < e2eQosSession Annc> |

### -----------------------End of change 1---------------------------------------------

-

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?