| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #39 |
| Source:* | Youngjin Na, Hyundai Motor Company, yjra@hyundai.com Minbyeong Lee, Hyundai Motor, minbyeong.lee@hyundai.com JaeSeung Song, KETI, jssong@sejong.ac.kr |
| Date:* | 2019-02-11 |
| Reason for Change/s:* | Need improvements on Action triggering |
| CR against: Release* | Release 4 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| _CR against: TS/TR* | TS 0001 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0001 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution contains several proposals to enhance action triggering mechanisms specified in TS-0001.

1. Action triggering for multiple, ordered, and time-based operations

In addition, there exist a case where multiple operations should be performed based on a specific order or specific time. For example, when a vehicle is near to the home (satisfying evalCriteria), multiple operations can be executed in a specific order such as 1) turning on air conditioning, and 2) turninging on the light. In this case, the operation 1) should be done before operation 2) because operation 1) needs more time to perform. Additioanlly the operation 2) can be executed with a specific operation time. Tp support this, operationResourceID and operation should have 0..1(L) type multiplicity and  objectResouceIDs should have ordered list and triggering time information.

2. Introducing time window mechanism between dependent events into <dependency> resourceOne or more <dependency> may exist to provide complex events for triggering. Multiple events may occur at difference time intervals. Thus a mechanism supporting time widnow mechanism can be used in determining whether to send action triggering or not within a given timeframe. The hosting CSE issues action triggering when evalCriteria is met within the given time window. The timeWindowSize can be set to zero. To implement such a time window with zero value, when one of the designated event conditions is triggered, the resource host has to check if other event criteria are also met. The action triggering is sent if the conditions are substantially simultaneously satisfied.

### -----------------------Start of change 1-------------------------------------------

### 9.6.X	Resource Type action

The <action> resource enables the system to specify which actions or operations to be performed on system resources (e.g. devices or services) based on monitored events. The events may include state changes of subject resources, requests from applications, etc. Once the event occurs, and if specified <dependency> conditions are met, the Hosting CSE sends a primitive defined by <action> resource attributes.

These child resources and attributes provide information about:

- The subject resource, which is the resource monitored to determine if a primary event occurs. Action is to be triggered if the primary event is the change of the state of the subject resource .

- The object resource, which is the resource which is the target of the triggered action

- The input resource, which is a resource which may provide input parameters for the action. Alternatively, an input value may be provided.

- Conditions to be monitored in order to determine if the primary event occurs

- Dependencies to be evaluated in order to determine if the action is to be conditionally performed.

- Parameters providing priorities of the action and priorities of the dependencies.

The <action> resource shall contain the child resources specified in table 9.6.X-1.

Table 9.6.X-1: Child resources of <action> resource

| Child Resources of <action> | Child Resource Type | Multiplicity | Description | <actionAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <dependency> | 0..n | See clause 9.6.Y | <dependencyAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |

The <action> resource shall contain the attributes specified in table 9.6.X-2.

Table 9.6.X-2: Attributes of <action> resource

| Attributes of <action> | Multiplicity | RW/ RO/ WO | Description | <actionAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| Labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| stateTag | 1 | RO | See clause 9.6.1.3. | OA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| Creator | 1 | RO | See clause 9.6.1.3. | NA |
| actionSubjectResource | 0..1 | RW | The resource that is the subject to be monitored for triggering the evaluation of the eventCriteriaevalCriteria attribute. The action indicated by the operation attribute Action is to be triggered based on the change of the state of the resource indicated the actionSubject Resource attribute.The subject can be any resource in the system. If the attribute is missing the subject is the parent resource. | OA |
| actionPriority | 0..1 | RW | Attribute indicating the priority of the action when compared with the other actions with the same event expression (subjectResourceID and evalCriteria), with highest priority indicated by the lowest value. Local policies are applied for equal or unspecified priorities. | OA |
| subjectResourceID | 0..1 | RW | The resourceID of the resource that is the subject of monitoring for evaluation of the evalCriteria attribute. The subject can be any resource in the system. If the attribute is missing the subject is the parent resource of this <action> resource. | OA |
| evalCriteria | 0..1 | RW | This attribute provides the conditions determining if the action is to be conditionally triggered to the object resource. The evalCriteria attribute, in conjunction with the subjectResourceID, forms the event expression that is being monitored. See further description below and in table 9.6.X-3 | OA |
| evalMode | 1 | RW | Attribute provides the macro control mode of the evaluation of the <action> resource. Some values are: 0 = off 1 = once 2 = periodic 3 = continuous | OA |
| evalControlParam | 0..1 | RW | When evalMode= periodic, evalControlParam represents the periodicity. When evalMode= continuous the evaluation is restarted as soon as an action has been triggered. In this case, if evalControlParam is specified, it determines the number of times the Hosting CSE shall trigger the event , otherwise it is repeated indefinitely. When evalMode is off or once this attribute can be ignored. | OA |
| evalPriority | 0..1 (L) | RW | List of priorities to be applied for the evaluation of the dependencies . This is an ordered list of the resourceIDs of the child <dependency> resources. | OA |
| objectResourceID | 0..1(L) | RW | The resourceID(s) of the target of the primitive to be sent when the event specified by evalCriteria occurs at the subject resource. The target may be specified also as the URI of a resource attribute. This attribute could include an ordered list. | MA |
| actionPrimitive | 1 | RW | This attribute stores the entire content of the primitive associated with this action with its parameters (e.g. the op parameter indicating the operation to be performed on the resource identified by objectResourceID), with some parameters being overwritten by the Hosting CSE when the action is triggered as follows: - the to and resource type parameters will be derived from objectResourceID attribute. - the content parameter is derived from the input attribute. When objectResourceID or content the input attributes are empty, the respective parameters are maintained as indicated by actionPrimitive. | MA |
| actionExecTime | 0..1(L) | RW | This attribute indicates the time and duration of an action when the actionPrimitive for object resource is triggered. | OA |
| input | 0..1 (L) | RW | Input parameters for the operation, used by the Hosting CSE to overwrite the content parameter of the actionPrimitive before sending the primitive, at the time that the action is triggered. The attribute could be one of the following: - a string - a resourceID - the URI of a resource attribute. The Hosting CSE overwrites the content parameter of actionPrimitive with the following (respectively): - the given string - the resource representation of the resource with the given resourceID. - the value of the given resource attribute | OA |
| outputResourceID | 0..1 | RW | The resourceID of a resource where the results of the action are stored. | OA |
| actionResult | 1 | RW | The result of the action. If outputResourceID is set, the Hosting CSE performs a RETRIEVE based on it when the response primitive of the action primitive is received and stores its content. Otherwise, the response primitive of the action primitive is stored in this attribute. | OA |

The conditions represented in the evalCriteria attribute determine if the action is to be conditionally triggered.

The evalCriteria attribute is comprised of triples (subject, operator, threshold) with the parameters shown and described in table 9.6.X-3.

Table 9.6.4-3: Parameters in evalCriteria triple

| Name | Description |
| --- | --- |
| subject | This parameter shall indicate an attribute of the subjectResourceID resource e.g. “tempContainer.content” |
| operator | This parameter is a keyword used to construct the evaluation logic, e.g. ‘equals’, ‘not equals’, ‘greater than’, ‘less than’, ‘greater or equal’, ‘less or equal’, ‘string match’, etc. |
| threshold | This parameter provides a value used to evaluate the criteria. The value type shall match the type of the attribute used as subject. |

### 9.6.Y	Resource Type dependency

The <dependency> resource represents  conditions that determines if the action or operation specified in the parent <action> resource is triggered. One or more <dependency> resources may exist to provide complex events for  triggering. The <dependency> resources provide conditions that always need to be checked before the action or operation is triggered. The difference between the evalCriteria  specified by an <action> resource and the one specified by a  <dependency> resource is that the former applies to the subject resource and triggers an evaluation, while  the latter applies to other resources which provide conditions used in evaluation.

The <dependency> resource shall contain the attributes specified in table 9.6.Y-2.

Table 9.6.Y-2: Attributes of <dependency> resource

| Attributes of <action> | Multiplicity | RW/ RO/ WO | Description | <actionAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. If no accessControlPolicyIDs value is configured, the accessControlPolicyIDs of the parent resource shall be applied for privilege checking. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| stateTag | 1 | RO | See clause 9.6.1.3. | OA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3. | NA |
| logic | 0..1(L) | RW | Indicates if the dependency is SUFFICIENT or NECESSARY. The operand of the related parent <action> resource needs to match: At least one of the SUFFICIENT dependencies OR All the NECESSARY dependencies | OA |
| evalCriteria | 0..1(L) | RW | Attribute providing conditions determining if the dependency matches the criteria. The evalCriteria attribute in conjunction with the dependentResource attribute forms the dependency to be evaluated. See further description in table 9.6.X-3. | OA |
| dependentResourceID | 1 | RW | The resourceID of the resource for which the evalCriteria is evaluated for the dependency. | MA |
| timeWindowSize | 0..1(L) | RW | This attribute indicates the time duration (e.g. in seconds) defining the time window size of sending the correspond action triggering shall be generated. | OA |

### -----------------------End of change 1-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?