| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | SDS #39 |
| Source:* | Convida Wireless |
| Date:* | 2019-02-10 |
| Contact:* | Chonggang Wang <wang.chongggang@convidawireless.com> Xu Li <li.xu@convidawireless.com> |
| Reason for Change/s:* | This contribution brings some clean ups on semantic mashup related resources. |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <WI-0053> MNT Maintenance / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V3.10.0 |
| Clauses/Sub Clauses* | 7.4.49.2.4, 6.3.4, 7.4.50.1, 7.4.50.2, 7.4.51.2, 6.3.5, 8.2.5 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution does a number of clean ups in order to address some recently received comments. In particular, the details of this contribution are as follows:

- Change #1 is to address the following comment:

- For the sentence below in 7.4.49, the comment we received was “I think the use of “notify” could be confusing here. I suggest this paragraph be reworded”

If the <semanticMashupJobProfile> resource to be deleted has a smiID attribute, the Hosting CSE shall notify each <semanticMashupInstance> resource that is referenced by the smiID attribute of the removal of the <semanticMashupJobProfile> by updating the value of smjpID attribute of those <semanticMashupInstance> resources to "NULL".

- Changes #2 and #3 are to address the following comment:

  - For the mashupMember description, the comment we received was “Need more details on how this serialization is supposed to work.  This reads as if it just serializes a single member, but it’s supposed to contain a list of URIs and member content – are they all contained in one RDF graph somehow – if so where is that defined?”

- Change #4 is to address the following comment:

  - For the periodForResultGen, the comment we received was “This conflicts with the datatype of xs:duration, since xs:duration format includes the time units (e.g. you can have a duration of P1D meaning 1 day.  I think that “Time unit is in second” should be deleted”

- Change #5 is to address the following comment:

  - For the sentence below in the CREATE procedure of 7.4.50, the comment we received was “Does this mean they get added to any values that were provided in the Create request primitive?”

- Change #6 is to address the following comment:

- For this text below in the UPDATE procedure of 7.4.50, the comment we received was “What does this mean? Does it replace an existing semanticMashupResult child (if so, which one) or add a new one?”

- For this text below in the UPDATE procedure of 7.4.50, the comment we received was “I assume that this change is applied to the existing mashupMember values. What happens if the update supplies new values for both mashupMember and memberStoreType? Do they have to be compatible?” – Answer: We do not have changes for this question, see the in-line comments.

For this text below in the UPDATE procedure of 7.4.50, the comment we received was “Does this affect the semanticMashupResult children that have already been created?” – Answer: No, since as described in the Table 9.6.54-1 of TS-0001, a <semanticMashupInstance> resource may have multiple <semanticMashupResult> child resources, which means that every time the Hosting CSE re-calculates the semantic mashup result, a new <semanticMashupInstance> child resource will be created to store the latest result.

No change from the generic procedures in clause 7.2.2.2 with the following exceptions:

- Primitive specific operation on Recv-6.4 "Check validity of resource representation for the given resource type":

- If the updated attribute in the Request message is smjpInputParameter, the Hosting CSE shall re-calculate the semantic mashup result.

- If the updated attribute in the Request message is mashupMember and resultGenType is WHEN_A_MASHUP_MEMBER_IS_UPDATED, the Hosting CSE shall re-calculate the semantic mashup result. [Explnation: if mashupMeber is updatd, it will be handed by this step, i.e., to caculate the new mashup result.]

- If the updated attribute in the Request message is memberStoreType, the Receiver shall change the way mashup member resources are represented by the mashupMember attribute. [Explanation: if memberStoreType is updated, it will be applied to all the current/latest mashupMembers. In other words, if mashupMember is updated, it was already handled by the previous step and then this step will be applied to all the lastest mashup members.]

- If the updated attribute in the Request message is resultGenType, the Receiver shall change the way that the semantic mashup result is generated accordingly.

- Change #7 is to address the following comment:

- For this text below in the RETRIEVE procedure of 7.4.51, the comment we received was “why is this a “May”? Is that just referring to the fact that the request might not being asking for child resources? If so then we don’t really need to include these words, since that’s just the standard procedure?”

- For this text below in the RETRIEVE procedure of 7.4.51, the comment we received was “The words “generate/refresh” need clarification. If you follow the steps literally each time they execute they add a new <semanticMashupResult>. Is that the intention, or is the intention that only the most recent result is kept?”

- Check the mashupMember attribute of the parent <semanticMashupInstance> resource.

- Collect latest data from each of the mashup members. If data cannot be collected from all of the mashup members, the Hosting CSE shall generate a Response Status Code indicating a "MASHUP_MEMBER_NOT_FOUND" error.

- Retrieve the mashup function from the functionDescriptor attribute of the <semanticMashupJobProfile> resource that is specified by the smjpID attribute of the parent <semanticMashupInstance> resource.

- Apply the mashup function with the data collected from the mashup members and produce a mashup result.

- Create a <semanticMashupResult> child resource under the parent <semanticMashupInstance> resource, and use it to store the newly-generated mashup result.

- Perform Recv-6.7 "Create a success response" where the Response may include the newly-generated mashup result.

- Perform Recv-6.8 and the procedure is terminated.

- Note that, in case of the resultGenType attribute of the parent <semanticMashupInstance> resource is set to PERIODICALLY, steps 1) through 5) will be executed periodically by the Hosting CSE in order to generate/refresh the mashup result.

- -----------------------Start of change 1-------------------------------------------

##### 7.4.49.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause 7.2.2.2 with the following exceptions:

- Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

a)	If the <semanticMashupJobProfile> resource to be deleted has a smiID attribute, the Hosting CSE shall update each <semanticMashupInstance> resource that is referenced by the smiID attribute of the removal of the <semanticMashupJobProfile> by updating the value of smjpID attribute of those <semanticMashupInstance> resources to "NULL".

- -----------------------End of change 1-------------------------------------------

- -----------------------Start of change 2-------------------------------------------

The oneM2M-defined enumeration data types are defined in clause Error: Reference source not found.

Table 6.3.3-1: oneM2M Simple Data Types

| XSD type name | Type Name |  |  | Examples |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| m2m:resourceName | Resource name |  |  | myLightBulb 123Sensor |  | Used for resource name attribute. This shall be formed by (ALPHA / DIGIT) *(ALPHA / DIGIT / "-" / "." / "_") as described in clause Error: Reference source not found |
| m2m:ID |  | Generic ID | //globalm2m.org |  | Used to represent generic IDs generated and used within oneM2M (M2M-SP-ID) |  |
|  |  |  | //globalm2m.org/C190XX7T |  | (CSE-ID) |  |
|  |  |  | //globalm2m.org/CSE1/123A38ZZY |  | (AE-ID) |  |
| m2m:nodeID | Node ID |  |  | urn:gsma:imei:90420156-025763-0;svn=42 |  | Used for Node IDs. The constraints on this type are different from those on Generic IDs (IMEI as node ID) |
| m2m:deviceID | Device ID |  |  | urn:dev:ops:012345-Set%2DTop%2DBox-0123456789 |  | A Device ID uniquely identifies a device using a URN. The format of the URN is one of IETF RFC 4122 [Error: Reference source not found] UUID, OPS URN, OS URN, IMEI URN, ESN URN, or MEID URN.A |
| m2m:externalID |  | M2M-EXT-ID | 123456789@domain.com |  | The External Identifier allows the Underlying Network to identify the M2M Device (e.g. ASN, MN) associated with the CSE-ID or AE-ID. In the 3GPP case, the External Identifier is specified in 3GPP TS 23.003 [Error: Reference source not found] |  |
|  |  | 3GPP external Group ID | 123456789@domain.com |  | In the 3GPP multicast case, the External Group Identifier is used in the group message delivery procedure and specified in 3GPP TS 23.682 [Error: Reference source not found] |  |
| m2m:requestID | Request ID |  |  | ab3f124a, CSE1/98821 |  | Used for Request IDs. This type may include the ID of the target CSE as well as a part that varies for each ID |
| m2m:nhURI | Non Hierarchical Identifier |  |  | /CSE090112/ C190XX7T |  | Used where a resourceID is required to be non-hierarchical |
| m2m:acpType | List of ACP Resource IDs |  |  | //IN-CSEID.m2m.myoperator.org/93405 |  | Used to represent a list of AccessControlPolicy identifiers. The list shall contain at least one member |
| m2m:labels | list of xs:token |  |  | printers networkwifi1 home_energy (key-only) domain:home color:red (key-value pair) |  | A list of tokens used for describing and discovering resources (searching wifi connected printer from vendor 1) Each token can have two formats, key-only and key-value pair. In the case of key-value pair, key and value are separated by ":". The key portion does not contain ":". The list shall contain at least one member |
| m2m:triggerRecipientID | Trigger Recipient Identifier |  |  | 3010 |  | Used when device triggering services are requested from the Underlying Network, to identify an instance of an ASN/MN-CSE on an execution environment, to which the trigger is routed. Defined as port number in the range 0 to 65535 |
| m2m:listOfM2MID | List of M2M identifiers |  |  |  |  | xs:list of elements of data type m2m:ID. The list shall contain at least one member |
| m2m:listOfURIs | List of any URI |  |  |  |  | xs:list of elements of data type xs:anyURI. The list shall contain at least one member |
| m2m:listOfDuration | List of durations |  |  |  |  | xs:list of elements of data type xs:duration. The list shall contain at least one member |
| m2m:resourceTypeList | List of resource types |  |  |  |  | xs:list of elements of data type m2m:resourceType. The list shall contain at least one member |
| m2m:listOfMinMax | List of Time Limits |  |  | 10 2560 |  | xs:list of two xs:long values defining min and max limits of time intervals in units of milliseconds (value -1 representing infinite time) |
| m2m:ipv4 | IPv4 address string with optional CIDR suffix |  |  | 10.125.0.0/16,122.77.12.1 |  | Used in m2m:accessControlRules specified in clause Error: Reference source not found |
| m2m:ipv6 | IPv6 address string with optional CIDR suffix |  |  | ::/0, Fadf:ddd0::/32, abcd:ffff:abb0:aaaa::/64 |  | Used in m2m:accessControlRules specified in clause Error: Reference source not found |
| m2m:countryCode | Country Code |  |  | KR |  | 2-character country code as defined by ISO 3166-1 [Error: Reference source not found] |
| m2m:pointOfAccess | single point of access of an AE or CSE |  |  | http://172.25.0.10:8080/xyz or coap://m2m.sp.com:5683 or mqtt://172.25.0.10:1883 or ws://10.222.254.26:80 |  | A point of access is represented as a URI that shall contain the underlying transport protocol (in either lowercase or uppercase spelling), the IP address (or an FQDN in all lowercase) and optionally a port number and/or path. No whitespace characters are allowed. The protocol binding specifications may give additional instructions on how the URI is. interpreted |
| m2m:poaList | List of pointOfAccess strings |  |  | http://172.25.0.10:8080/xyz coap://m2m.sp.com:5683 mqtt://172.25.0.10:1883 |  | xs:list of elements of data type m2m:pointOfAccess. The list shall contain at least one member |
| m2m:timestamp | Time stamp string |  |  | 20141003T112032 |  | DateTime string using 'Basic Format' specified in ISO 8601 [Error: Reference source not found]. Time zone shall be interpreted as UTC timezone. See below for more details |
| m2m:absRelTimestamp | absolute or relative time stamp string |  |  | 20141003T112032 (absolute time),or 3600000 (relative time) |  | defined as xs:union of m2m:timestamp and xs:long data types |
| m2m:typeOfContent | Type of Content |  |  | application/xml |  | The media type shall be an IANA registered Media Types name, or an experimental Media Type (see [Error: Reference source not found]) ':' |
| m2m:serializations | Serialization types |  |  | application/xml application/json application/cbor |  | A list of IANA registered media types that can be used for serialization of primitives. The permitted values are: - application/xml - application/json - application/cbor The list shall contain at least one member |
| m2m:contentInfo | Content Information |  |  | application/xml:1 application/xml:1:0 application/xml:1:5 |  | A string consisting of a media type followed by a m2m:encodingType and optional m2m:contentSecurity, each separated by ':' character. If the m2m:contentSecurity value is not present, then the preceding ':' shall also be not present. If the m2m:contentSecurity value is not present then this has the same interpretation as a value of 0 for m2m:contentSecurity. See note |
| m2m:protocolList | List of protocols |  |  | application/x-alljoyn;version=1.0 application/x-echonet-lite;version=1.0 |  | A list of MIME types for all communication protocols supported by the device |
| m2m:eventCat | Event Category |  |  | 2 |  | Either: - One of the values from m2m:stdEventCats or - A user-defined category in the range 100-999 |
| m2m:eventCatWithDef | Event Category with default |  |  | 0 |  | Either: - A value from m2m:eventCat , or - The value 0 which has the special meaning "default" |
| m2m:listOfEventCat | List of (applicable) Event Categories |  |  | 1 101 |  | xs:list of elements of data type m2m:eventCat. The list shall contain at least one member |
| m2m:listOfEventCatWithDef | List of m2m:eventCatWithDef |  |  | 0 1 101 |  | xs:list of elements of data type m2m:eventCatWithDef. The list shall contain at least one member |
| m2m:scheduleEntry | Schedule Entry |  |  | * 0-5 2,6,10 * * * * |  | The string is used to describe a duration of enablement. The string format is described in clause 7.4.9.1 |
| m2m:attributeList | List of xs:NCName |  |  | poa rr |  | Used for the Content parameter of Retrieve request primitives and in m2m:eventNotificationCriteria. Attributes represented with their short names. The list shall contain at least one member |
| m2m:roleID | Role-ID |  |  | 1234abcd@role-issuer.com |  | A string pattern consisting of a name (the issuerRelativeID) and an FQDN in all lowercase (the issuerID) separated by the ‘@' character, not including any whitespace characters. The issuerRelativeID shall be comprised of any combination of the Roman alphabet, numerals, '.', '_' and '-' characters |
| m2m:descriptorRepresentation | Semantic content representation |  |  | application/rdf+xml:1 |  | A string consisting of a media type followed by a m2m:encoding separated by ':' character. The only permitted value is application/rdf+xml:1 |
| m2m:sparql | SPARQL content |  |  | SELECT ?functionality WHERE { ?functionality rdf:type base:Measuring. ?functionality base:refersTo ?aspect. ?aspect rdf:type instance:Temperature } |  | The string is used for SPARQL content, e.g. in semanticsFilter |
| m2m:missingDataList | List of absolute timestamp or list of relative timestamp |  |  | absolute time: 20141103T111832 20141103T112435 20141103T113633 or relative time: 10000 10005 10020 |  | Used for storing the time information of missing data points in Time Series defined as xs:union of list of m2m:timestamp and list of xs:duration data types. The list shall contain at least one member |
| m2m:tokenID | Token-ID |  |  | 1234abcd@token-issuer.com |  | A string pattern consisting of a name (the issuerRelativeID) and an FQDN in all lowercase (the issuerID) separated by the ‘@' character, not including any whitespace characters. See constraints above for the issuerRelativeID |
| m2m:dynAuthJWT | JSON Web Token (JWT), which uses either JSON Web Encryption (JWE) Compact Serialization JSON Web Signature (JWS) Compact Serialization |  |  | See m2m:e2eCompactJWE and m2m:e2eCompactJWS |  | Defined as xs:union of m2m:e2eCompactJWE and m2m:e2eCompactJWS |
| m2m:e2eCompactJWS | JSON Web Signature (JWS) Compact Serialization, used in End-to-End Security Features oneM2M TS-0003 [Error: Reference source not found] |  |  | eyJ0eXAiOiJK. eyJpc3MiOiJqb2UiLA0KIC. dBjftJeZ4CVP (line breaks for display purposes only) |  | Of the form [a].[b].[c], where components [a] and [c] are non-empty, while component [b] can be either empty or not empty. When not empty, each component is base64url encoded (IETF RFC 4648 [Error: Reference source not found]). See IETF RFC 7515 [Error: Reference source not found] |
| m2m:e2eCompactJWE | JSON Web Encryption (JWE) Compact Serialization, used in End-to-End Security Features oneM2M TS-0003 [Error: Reference source not found] |  |  | eyJ0eXAiOiJK. eyJpc3MiOiJqb2UiLA0KIC. dBjftJeZ4CVP. 5eym8TW_c8SuK. SdiwkIr3a. XFBoMYUZo (line breaks for display purposes only) |  | Of the form [a].[b].[c].[d].[e], where components [a] and [d] are non-empty, while components [b], [c] and [e] can be empty or not empty. When not empty, each component is base64url encoded (IETF RFC 4648 [Error: Reference source not found]). See IETF RFC 7516 [Error: Reference source not found] |
| m2m:signatureList | List of signatures generated using a certificate or MIC generated using a symmetric key. It is used in Authorization Relationship Mapping |  |  | i6watmQQQ1y3GB-VsWq5fJKzQcBB4jRfH1bfJFj0JtFVtLotttzYyA== IWijxQjUrcXBYoCei4QxjWo9Kg8D3p9tlWoT4t0_gyTE96639In0FZFY2_rvP-_bMJ01EArmKZsR5VW3rwoPxw== (line breaks for display purposes only) |  | Each signature or MIC in the list is represented as a string which is base64url encoded(IETF RFC 4648 [Error: Reference source not found]). The list shall contain at least one member |
| m2m:locationTargetID | The identifier to be used for retrieving the location information of a remote Node or device of underlying network |  |  | urn:gsma:imei:90420156-025763-0;vers=0 or 123456789@domain.com;svn=42 or 8617791450839 |  | defined as xs:union of m2m:nodeID and m2m:externalID and MSISDN |
| m2m:releaseVersion | Service Layer Release Version |  |  | 3 or 2a |  | This parameter is set to the release version that the primitive complies with |
| m2m:supportedReleaseVersions | List of supported Release Versions |  |  | applicable list elements: 1, 2, 2a, 3 |  | This list includes the release versions supported by AE or CSE. The list shall contain at least one member |
| m2m:TMGI | Temporary Mobile Group Identity allocated to the MBMS bearer. |  |  | F2003090156 |  | A string assigned by the 3GPP network used to identify the MBMS Bearer Service. The format is defined in 3GPP TS 23.003 [Error: Reference source not found] |
| m2m:sessionDescription | Session Description |  |  | o=user 2890844526 2890844526 IN IP4 10.1.1.1 s=stream c=IN IP4 10.1.1.1 t=0 0 m=video 5600 RTP/AVP 96 a=rtpmap:96 H264/90000 |  | The description format is a multi-lined text string as defined in Session Description Protocol (IETF RFC 4566 [Error: Reference source not found]) |
| m2m:sessionCapabilities | Session Capability |  |  | audio:AVP video:RTS/AVP |  | Pair(s) of media type and corresponding protocol as defined in Session Description Protocol (IETF RFC 4566 [Error: Reference source not found]. The delimiter between the media type and the protocol is colon |
| NOTE: The media type and m2m:encodingType in m2m:contentInfo describe the content data to which the End-to-End Security of Data (ESData) processing, if any, was applied as indicated by m2m:contentSecurity. The m2m:contentInfo indicates a sequence of processes to be applied to the content after being obtained from the CSE. First, the ESData processing (if any) as indicated by m2m:contentSecurity is applied. The result of this processing then has transfer decoding (if any) applied as indicated by m2m:encodingType. The result of this processing is then processed according to the media type. |  |  |  |  |  |  |

#### 6.3.5.XX	m2m:mashupMembers

Used for the mashupMember attribute of the <semanticMashupInstance> resource.

Table 6.3.5.XX-2: Type Definition of m2m:mashupMembers

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| mashupMember | (anonymous) | 1..n |  |
| mashupMember/memberURI | xs:anyURI | 1 |  |
| mashupMember/memberValue | xs:integer | 0..1 |  |

### 8.2.5	Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-3: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| createdBefore | filterCriteria, eventNotificationCriteria | crb |
| createdAfter | filterCriteria, eventNotificationCriteria | cra |
| modifiedSince | filterCriteria, eventNotificationCriteria | ms |
| unmodifiedSince | filterCriteria, eventNotificationCriteria | us |
| stateTagSmaller | filterCriteria, eventNotificationCriteria | sts |
| stateTagBigger | filterCriteria, eventNotificationCriteria | stb |
| expireBefore | filterCriteria, eventNotificationCriteria | exb |
| expireAfter | filterCriteria, eventNotificationCriteria | exa |
| labels | filterCriteria, eventNotificationCriteria | lbl* |
| labelsQuery | filterCriteria | lbq |
| resourceType | filterCriteria, accessControlObjectDetails | ty* |
| sizeAbove | filterCriteria, eventNotificationCriteria | sza |
| sizeBelow | filterCriteria, eventNotificationCriteria | szb |
| contentType | filterCriteria | cty |
| limit | filterCriteria | lim |
| attribute | filterCriteria, eventNotificationCriteria | atr |
| contentFilterSyntax | filterCriteria | cfs |
| contentFilterQuery | filterCriteria | cfq |
| level | filterCriteria | lvl |
| offset | filterCriteria | ofst |
| notificationEventType | eventNotificationCriteria, notificationEvent | net |
| operationMonitor | eventNotificationCriteria, notificationEvent | om |
| representation | notificationEvent | rep |
| filterUsage | filterCriteria | fu* |
| eventCatType | eventCat | ect |
| eventCatNo | eventCat | ecn |
| number | batchNotify | num |
| duration | batchNotify | dur |
| notification | aggregatedNotification, Request Primitive Content | sgn |
| notificationEvent | notification | nev |
| verificationRequest | notification | vrq |
| subscriptionDeletion | notification | sud |
| subscriptionReference | notification | sur |
| creator | notification | cr* |
| notificationForwardingURI | notification | nfu* |
| notificationTarget | notification | ntt |
| targetRemovalRequest | notification | trr |
| targetRemovalAllowance | notification | tra |
| IPEDiscoveryRequest | notification | idr |
| AERegistrationPointChange | notification | aerp |
| AEReferenceIDChange | notification | aerid |
| trackingID1 | notification | tid1 |
| trackingID2 | notification | tid2 |
| filterCriteria | IPEDiscoveryRequest | fc* |
| operation | operationMonitor, dynAuthDasRequest | op* |
| operations | operationMonitor | ops |
| originator | operationMonitor, IPEDiscoveryRequest, dynAuthDasRequest | or* |
| action | actionStatus | acn* |
| status | actionStatus | sus* |
| childResource | All except execInstance, announced resource, management resources from firmware | ch |
| accessControlRule | privileges, selfPrivileges | acr |
| accessControlOriginators | accessControlRule | acor |
| accessControlOperations | accessControlRule | acop |
| accessControlContexts | accessControlRule | acco |
| accessControWindow | accessControlContexts | actw |
| accessControlIpAddresses | accessControlContexts | acip |
| ipv4Addresses | accessControlIpAddress | ipv4 |
| ipv6Addresses | accessControlIpAddress | ipv6 |
| accessControlLocationRegion | accessControlContexts | aclr |
| countryCode | accessControlLocationRegion | accc |
| circRegion | accessControlLocationRegion | accr |
| name | attribute, anyArgType, mgmtLinkRef, childResourceRef, contentRef | nm* |
| specializationID | childResourceRef, accessControlObjectDetails | spid |
| value | attribute | val |
| type | anyArgType | typ |
| maxNrOfNotify | rateLimit | mnn |
| timeWindow | rateLimit | tww |
| scheduleEntry | scheduleElement | sce |
| aggregatedNotification | Request Primitive Content | agn |
| attributeList | Request Primitive Content | atrl |
| securityInfo | Request Primitive Content, Response Primitive Content | seci |
| aggregatedResponse | Response Primitive Content | agr |
| resource | Response Primitive Content | rce |
| URIList | Response Primitive Content | uril |
| debugInfo | Response Primitive Content | dbg |
| queryResult | Response Primitive Content | qres |
| anyArg | resetArgsType, rebootArgsType, uploadArgsType, downloadArgsType, softwareInstallArgsType softwareUpdateArgsType, softwareUninstallArgsType, execReqArgsListType | any |
| fileType | downloadArgsType | ftyp |
| URI | resourceWrapper, dynAuthTokenReqInfo | uri |
| URL | downloadArgsType | url* |
| username | uploadArgsType, downloadArgsType, softwareUpdateArgsType, softwareUninstallArgsType | unm |
| password | uploadArgsType, downloadArgsType, softwareUpdateArgsType, softwareUninstallArgsType | pwd |
| filesize | downloadArgsType | fsi |
| targetFile | downloadArgsType | tgf |
| delaySeconds | downloadArgsType | dss |
| successURL | downloadArgsType | surl |
| startTime | downloadArgsType | stt |
| completeTime | downloadArgsType | cpt |
| UUID | softwareInstallArgsType softwareUpdateArgsType, softwareUninstallArgsType | uuid |
| executionEnvRef | softwareInstallArgsType softwareUpdateArgsType, softwareUninstallArgsType | eer |
| version | softwareUninstallArgsType, tokenClaimSet | vr* |
| reset | execReqArgsListType | rst |
| reboot | execReqArgsListType | rbo* |
| upload | execReqArgsListType | uld |
| download | execReqArgsListType | dld |
| softwareInstall | execReqArgsListType | swin |
| softwareUpdate | execReqArgsListType | swup |
| softwareUninstall | execReqArgsListType | swun |
| tracingOption | deliveryMetaData | tcop |
| tracingInfo | deliveryMetaData | tcin |
| responseTypeValue | responseTypeInfo | rtv |
| notificationURI | responseTypeInfo | nu |
| timeOfDay | deletionContexts | tod |
| locationRegions | deletionContexts | lr |
| URIReference | contentRef | urir |
| semanticsFilter | filterCriteria | smf |
| missingDataList | timeSeries | mdl |
| missingData | eventNotificationCriteria | md |
| tokenID | tokenClaimSet, dynAuthLocalTokenIdAssignments | tkid |
| holder | tokenClaimSet | tkhd* |
| issuer | tokenClaimSet | tkis* |
| notBefore | tokenClaimSet | tknb* |
| notAfter | tokenClaimSet | tkna* |
| tokenName | tokenClaimSet | tknm* |
| audience | tokenClaimSet | tkau* |
| permissions | tokenClaimSet | tkps* |
| extension | tokenClaimSet | tkex* |
| permission | tokenPermissions | pm |
| resourceIDs | tokenPermission | ris |
| privileges | tokenPermission, setOfPermissions | pv* |
| roleIDs | tokenPermission | rids* |
| localTokenIdAssignment | dynAuthLocalTokenIdAssignments | ltia |
| localTokenID | dynAuthLocalTokenIdAssignment | lti |
| dasInfo | dynAuthTokenReqInfo | dasi |
| dasRequest | dynAuthTokenReqInfo | daq |
| securedDasRequest | dynAuthTokenReqInfo | sdr |
| filterOperation | filterCriteria | fo |
| targetedResourceType | dynAuthDasRequest | trt |
| originatorIP | dynAuthDasRequest | oip* |
| ipv4Address | dynAuthDasRequest, ipAddress | ip4 |
| ipv6Address | dynAuthDasRequest, ipAddress | ip6 |
| originatorLocation | dynAuthDasRequest | olo* |
| originatorRoleIDs | dynAuthDasRequest | orid |
| requestTimestamp | dynAuthDasRequest | rts |
| targetedResourceID | dynAuthDasRequest | trid |
| proposedPrivilegesLifetime | dynAuthDasRequest | ppl |
| roleIDsFromACPs | dynAuthDasRequest | rfa |
| tokenIDs | dynAuthDasRequest | tids |
| dynamicACPInfo | dynAuthDasResponse | dai |
| grantedPrivileges | dynAuthDasResponse | gp |
| privilegesLifetime | dynAuthDasResponse | pl |
| tokens | dynAuthDasResponse | tkns |
| securityInfoType | securityInfo | sit |
| dasRequest | securityInfo | dreq |
| dasResponse | securityInfo | dres |
| dynAuthRelMapRequest | securityInfo | darq |
| dynAuthRelMapResponse | securityInfo | dars |
| esprimRandObject | securityInfo | ero |
| esprimObject | securityInfo | epo |
| escertkeMessage | securityInfo | eckm |
| resourceRef | listOfChildResourceRef | rrf |
| resourceRefList | Response Primitive Content | rrl |
| esprimRandID | originatorESPrimRandObject, receiverESPrimRandObject | esri |
| esprimRandValue | originatorESPrimRandObject, receiverESPrimRandObject | esrv |
| esprimRandExpiry | originatorESPrimRandObject, receiverESPrimRandObject | esrx |
| esprimKeyGenAlgID | originatorESPrimRandObject | esk |
| esprimKeyGenAlgIDs | receiverESPrimRandObject | esks |
| esprimProtocolAndAlgIDs | originatorESPrimRandObject, receiverESPrimRandObject | espa |
| supportede2ESecFeatures | e2eSecInfo | esf |
| certificates | e2eSecInfo | escert |
| sharedReceiverESPrimRandObject | e2eSecInfo | esro |
| networkAction | backOffParameters | nwa |
| initialBackoffTime | backOffParameters | ibt |
| additionalBackoffTime | backOffParameters | abt |
| maximumBackoffTime | backOffParameters | mbt |
| optionalRandomBackoffTime | backOffParameters | rbt |
| backOffParametersSet | backOffParameters | bops |
| dataLink | listOfDataLinks | dali |
| attributeName | dataLink | atn |
| dataContainerID | dataLink | dcid |
| accessControlAuthenticationFlag | accessControlRule | acaf |
| accessControlObjectDetails | accessControlRule | acod |
| dataLinkEntry | listOfDataLinks | dle |
| childResourceType | accessControlObjectDetails, eventNotificationCriteria, filterCriteria | chty |
| parentResourceType | filterCriteria | pty |
| childLabels | filterCriteria | clbl |
| parentLabels | filterCriteria | palb |
| childAttribute | filterCriteria | catr |
| parentAttribute | filterCriteria | patr |
| applyRelativePath | filterCriteria | arp |
| sessionDescription | sessionDescriptions | sdc |
| activityPattern | activityPatternElements | apt |
| stationaryIndication | activityPattern | sti |
| dataSizeIndicator | activityPattern | dsi |
| eventNotificationCriteriaEntry | eventNotificationCriteriaSet | encn |
| mashupMember | mashupMembers | mspm |
| memberURI | mashupMembers | muri |
| memberValue | mashupMembers | mvl |
| NOTE: * marked short names have been already assigned in attribute Table 8.2.3-1 to Table 8.2.3-6. |  |  |

- -----------------------End of change 2-------------------------------------------

- -----------------------Start of change 3-------------------------------------------

Table 7.4.50.1-4: Resource Specific Attributes of <semanticMashupInstance> resource

| Attribute Name |  | Request Optionality |  |  |  | Data Type |  | Default Value and Constraints |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Create |  | Update |  |  |  |  |
| smjpID | M |  | O |  | xs:anyURI |  | No default. |  |
| smjpInputParameter | M |  | O |  | xs:base64Binary |  | The content shall be serialized using application/rdf+xml media type. |  |
| memberStoreType | M |  | O |  | m2m:mashupMemberStoreType |  | No default. |  |
| mashupMember | O |  | O |  | m2m:mashupMembers |  | In case memberStoreType is set as '1' (URI_ONLY), this attribute shall be a list of URIs, e.g. [URI1, URI2, …]; In case memberStoreType is set as '2' (URI_AND_VALUE), this attribute shall be a list of <URI, value> pairs e.g. [<URI1, 34>, <URI2, 58>, ..] |  |
| resultGenType | M |  | O |  | m2m:mashupResultGenType |  | No default. |  |
| periodForResultGen | O |  | O |  | xs:duration |  | No default. Time unit is in second. |  |

- -----------------------End of change 3-------------------------------------------

- -----------------------Start of change 4-------------------------------------------

Table 7.4.50.1-5: Resource Specific Attributes of <semanticMashupInstance> resource

| Attribute Name |  | Request Optionality |  |  |  | Data Type |  | Default Value and Constraints |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Create |  | Update |  |  |  |  |
| smjpID | M |  | O |  | xs:anyURI |  | No default. |  |
| smjpInputParameter | M |  | O |  | xs:base64Binary |  | The content shall be serialized using application/rdf+xml media type. |  |
| memberStoreType | M |  | O |  | m2m:mashupMemberStoreType |  | No default. |  |
| mashupMember | O |  | O |  | m2m:mashupMember |  | In case memberStoreType is set as '1' (URI_ONLY), this attribute shall be of type m2m:listOfURIs; otherwise, it shall be of type xs:base64Binary and the content shall be serialized using application/rdf+xml media type. |  |
| resultGenType | M |  | O |  | m2m:mashupResultGenType |  | No default. |  |
| periodForResultGen | O |  | O |  | xs:duration |  | No default. |  |

- -----------------------End of change 4-------------------------------------------

- -----------------------Start of change 5-------------------------------------------

#### 7.4.50.2	<semanticMashupInstance> resource specific procedures for CRUD operations

##### 7.4.50.2.0	Introduction

This clause describes <semanticMashupInstance> resource specific primitive behaviour for CRUD operations.

##### 7.4.50.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause 7.2.2.2 with the following exception:

- Primitive specific operation on Recv-6.4 "Check validity of resource representation for the given resource type":

a)	The Hosting CSE shall check that the value of the smjpID attribute is not NULL and a valid SMJP can be retrieved by using the SMJP ID indicated in the smjpID attribute.

b)	The Hosting CSE shall check that the value of smjpInputParameter attribute conforms to the RDF/XML syntax as defined in RDF 1.1 XML Syntax [Error: Reference source not found] and that it meets the requirement described in the "inputDescriptor" attribute of the retrieved <semanticMashupJobProfile>.

c)	If any of the above checks fail, the Hosting CSE shall generate a Response Status Code indicating a "NOT_ACCEPTABLE" error.

d)	The Hosting CSE shall use the SPARQL query specified in the memberFilter attribute of the retrieved <semanticMashupJobProfile> to discover mashup member resources for the <semanticMashupInstance> to be created. The hosting CSE shall conduct semantic resource discovery on one or more CSEs by using the SPARQL query statement as the semantics filter to identify qualified mashup members.

e)	If not all of the required mashup members can be identified, the Hosting CSE shall generate a Response Status Code indicating a "MASHUP_MEMBER_NOT_FOUND" error.

f)	If all the mashup members are identified, the mashupMember attribute shall be updated. If the memberStoreType attribute has the value of “URI_ONLY”, then the URIs of those identified member resources (during step d)) will be stored in the mashupMember attribute. If the memberStoreType attribute has the value of “URI_AND_VALUE”, then the URIs as well as their values of those identified member resources (during step d)) will be stored in the mashupMember attribute.

- -----------------------End of change 5-------------------------------------------

- -----------------------Start of change 6-------------------------------------------

##### 7.4.50.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- Primitive specific operation on Recv-6.4 "Check validity of resource representation for the given resource type":

a)	If the updated attribute in the Request message is smjpInputParameter, the Hosting CSE shall re-calculate the semantic mashup result. A <semanticMashupInstance> resource may have multiple <semanticMashupResult> child resources, which means that every time the Hosting CSE re-calculate the semantic mashup result, a new <semanticMashupResult> child resource will be created to store the latest result.

b)	If the updated attribute in the Request message is mashupMember and resultGenType is WHEN_A_MASHUP_MEMBER_IS_UPDATED, the Hosting CSE shall re-calculate the semantic mashup result.

c)	If the updated attribute in the Request message is memberStoreType, the Receiver shall change the way mashup member resources are represented by the mashupMember attribute.

d)	If the updated attribute in the Request message is resultGenType, the Receiver shall change the way that the semantic mashup result is generated accordingly.

- -----------------------End of change 6-------------------------------------------

- -----------------------Start of change 7-------------------------------------------

##### 7.4.51.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The Receiver shall follow the steps from Recv-1.0 to Recv-6.3 specified in clause Error: Reference source not found Generic Resource Request Procedure for Receiver, with the following primitive specific operations:

After Recv-6.3 "Check authorization of the Originator":

- Check the mashupMember attribute of the parent <semanticMashupInstance> resource.

- Collect latest data from each of the mashup members. If data cannot be collected from all of the mashup members, the Hosting CSE shall generate a Response Status Code indicating a "MASHUP_MEMBER_NOT_FOUND" error.

- Retrieve the mashup function from the functionDescriptor attribute of the <semanticMashupJobProfile> resource that is specified by the smjpID attribute of the parent <semanticMashupInstance> resource.

- Apply the mashup function with the data collected from the mashup members and produce a mashup result.

- Create a <semanticMashupResult> child resource under the parent <semanticMashupInstance> resource, and use it to store the newly-generated mashup result.

- Perform Recv-6.7 "Create a success response".

- Perform Recv-6.8 and the procedure is terminated.

Note that, in case of the resultGenType attribute of the parent <semanticMashupInstance> resource is set to PERIODICALLY, steps 1) through 5) will be executed periodically by the Hosting CSEin order to generate the mashup result.

- -----------------------End of change 7-------------------------------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror crs been posted?

- Does this change request make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause?