| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #39 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com |
| Date:* | 2019-02-11 |
| Reason for Change/s:* | The CR is to add new feature of 3GPP QoS interworking. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0058 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0024 |
| Clauses * | 8.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TR-0024 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Splite the SDS-2019-0079 to two contributions. This contribution is the service flow part of QoS solution.

The this solution can support the two deployments as below:

- one hop bewteen the IN-CSE and ASN/MN-CSE

- two hops between the ASN/MN-CSE and the ASN/MN-CSE

The other deployments are FFS.

### -----------------------Start of change 1-------------------------------------------

## 8.x	Setting up an AS session with required QoS procedure

### 8.x.4 oneM2M solution

#### 8

#### 8.x.4.2 	Create E2E QoS procedure

Figure 8.x.4.2-1: Create E2E QoS procedure

Step 0.a: The IN-AE or ASN/MN-CSE performs the oneM2M registration procedure and the IN-CSE pre-defines the 3GPP QoS Information.

The IN-AE performs the oneM2M registration procedure. The IN-CSE hosts the corresponding <AE> resource for the registree. The IN-CSE pre-defines the 3GPP QoS Information according to the information from 3GPP operator.

Table 8.x.4.2-1: 3GPP QoS Information

| Information | Multiplicity | Description |
| --- | --- | --- |
| QosLevel | 1 | Indicates the QoS level of oneM2M according to the QCI of 3GPP. |
| QCI | 1(L) | QoS Class Identifier(s) which is specified in 3GPP TS23.203 [i.11].e.g. QCI 1 is used for Conversational Voice. |
| Max-Requested-Bandwidth-UL | 0..1 | Indicates the maximum requested bandwidth in bits per second for an uplink IP flow. Refer to 3GPP TS29.214 [i.xx]. |
| Max-Requested-Bandwidth-DL | 0..1 | Indicates the maximum bandwidth in bits per second for a downlink IP flow. Refer to 3GPP TS29.214 [i.xx]. |
| Priority-Sharing-Indicator | 0..1 | It is used to indicate that the related media component can use the same Allocation and Retention Priority as media component(s). Refer to 3GPP TS29.214 [i.xx]. |
| Pre-emption-Capability | 0..1 | Indicates whether a service data flow can get resources that were already assigned to another service data flow with a lower priority level. Refer to 3GPP TS 29.212[i.zz]. |
| Pre-emption-Vulnerability | 0..1 | Indicates whether a service data flow can lose the resources assigned to it in order to admit a service data flow with higher priority level. Refer to 3GPP TS 29.212[i.zz]. |

Step1: The IN-AE or ASN/MN-CSE sends the <e2eQosSession> resource CREATE request, the <e2eQosSession> includes the following setting:

- Supported ResourceID List of the e2eQosRequirements shall be set to the resource ID (s) to which the QoS is to apply

- Source Endpoint of the e2eQosRequirements shall be set to the entity ID of the IN-AE or ASN/MN-CSE.

- Destination Endpoint of the e2eQosRequirements shall be set to the entity ID of target ASN/MN-CSE to which the QoS is to be applied

- QoS Level of the e2eQosRequirements shall be set to the value which is mapped with 3GPP QCI according to the resource identifier.

- Time Period of the e2eQosRequirements shall be set to the time window when the QoS is to be applied.

- QoS session status of the qosPolicy shall be set to USAGE_EXHUSTED, and Action of qosPolicy shall be set to DELETE.

Step2：The IN-CSE validates the attributes of the <e2eQosSession> resource.

The IN-CSE checks the existence of the Supported ResourceID List, if any one resource ID in the list does not exist, the IN-CSE shall return the error to the IN-AE or ASN/MN-CSE. The IN-CSE check the entity type of the Source Endpoint and Destination Endpoint, if both entities are AE, the IN-CSE shall return the error to the the IN-AE or ASN/MN-CSE.

Step3 : The IN-CSE creates the child resource <e2eQosSession> of the resource <AE> or <remoteCSE>, and QoS information associated with the resource identifier in the request.

8.x.4.3 	Setting up 3GPP session with required QoS procedure

Figure 7.9.3.2-1: Setting up 3GPP session with required QoS

Step 1 (optional) : The IN-AE sends the resource operation request to the target ASN/MN-CSE.

The IN-AE sends the operation request with the resource ID to the target ASN/MN-CSE.

Step 2: The IN-CSE prepares the 3GPP session parameters according to the attributes of the <e2eQosSession>.

If the Time Period of the <e2eQosSession> is absent, the IN-CSE checks the parameters in the request with the attributes of the <e2eQosSession>: the IN-CSE gets the IN-AE ID from the From in the request and the targeted resource identifier and the targeted ASN/MN-CSE ID from the To in the request. Then the IN-CSE gets the targeted ASN/MN-CSE ID according to the targeted resource identifier. The IN-CSE compares the IN-AE ID, the targeted ASN/MN-CSE ID with each Source Endpoint and Destination Endpoint element of the <e2eQosSession>. If they matches with any one element, and the targeted resource identifier is included in the Supported ResourceID List, the IN-CSE gets the information of the network of which the targeted ASN/MN-CSE registrates and  checks the transport link between the target ASN/MN-CSE and IN-CSE.

If the Time Period of the <e2eQosSession> exists, the IN-CSE shall check the transport link between the targeted ASN/MN-CSE and IN-CSE.

CaseA: If it is TCP:

- Step 3.1a (Optional): IN-CSE initials TCP connectivity establishment with target ASN/MN-CSE or ADN-AE if there is no transport link;

- Step 3.2a: IN-CSE gets the IP 5-tuples of TCP and maps it to 3GPP flow information.

CaseA: If it is UDP:

- Step 3b: IN-CSE gets the IP 5-tuples of UDP and maps it to 3GPP flow information.

Step4: The IN-CSE sends the On-demand QoS request to SCEF to negotiate the QoS.

The IN-CSE sends an AsSessionWithQoSSubscription request to the SCEF according to the registration network information of the target ASN/MN-CSE or ADN-AE to set up an session request with specific QoS for the service provided by the IN-CSE.

The AsSessionWithQoSSubscription request from the IN-CSE to the SCEF shall comply with i.53GPP TS 29.122 [i.aa]. The AsSessionWithQoSSubscription request shall include:

- notificationDestination shall be set to a URI that the SCEF can target QoS status notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

- flowInfo shall be set to the value of IP 5-tuples between IN-CSE and target ASN/MN-CSE or ADN-AE. For example: The direction of E2EInfo is In which means from the IN-AE to the target ASN/MN-CSE or ADN-AE and parameter mappings are as below:

| - 3GPP API parameter | - Value |
| --- | --- |
| - flowId | - Assigned by the IN-CSE according local policy. |
| - Direction | - Out (downlink IP flows) |
| - Source IP address | - IP address of IN-CSE |
| - destination IP address | - IP address of target ASN/MN-CSE or ADN-AE |
| - Protocol | - TCP/UDP |
| - Source port | - Port number of the IN-CSE |
| - destination port | - Port number of the targeted ASN/MN-CSE or ADN-AE |

- qosReference shall be set to the value of QoS Level of the e2eQosRequirements which the IN-AE ID, the targeted ASN/MN-CSE ID or ADN-AE ID are the same with the Source Endpoint and Destination Endpoint.

- ueIpv4Addr or ueIpv6Addr shall be set to the value of  the IP address of target ASN/MN-CSE or ADN-AE

- usageThreshold shall be set to the value of usageThreshold of <e2eQosSession>

- sponsorInfo shall be set to the value of sponsor information.

Step 5: 3GPP initiated IP-CAN Session Modification according to the QoS request.

The SCEF interacts with the 3GPP entity triggers 3GPP initiated IP-CAN Session Modification according to the QoS request defined in 3GPP TS 23.203[i.11].

Step 6: SCEF sends the On-demand QoS response to IN-CSE.

The SCEF sends an AsSessionWithQoSSubscription response that contains information as specified in 3GPP TS 29.122 [i.aa] to the IN-CSE.

Step7: Message delivery via 3GPP network with required QoS.

IN-CSE sends the resource operation request to the target ASN/MN-CSE or ADN-AE via 3GPP network with required QoS session and the target ASN/MN-CSE or ADN-AE sends response to IN-CSE.

Step8: IN-CSE sends response to the IN-AE.

#### 8.x.4.4 	3GPP QoS status monitoring and report procedure

Figure 7.9.3.3-1: 3GPP QoS status report procedure

Step 1: The 3GPP network entities detect the IP-CAN session modification.

The 3GPP entities may notify the SCEF about bearer level events for the Rx session (e.g., transmission resources are released/lost) with an IP-CAN Session Modification as described in TS 23.203[i.11].

Step 2: SCEF sends Status notification message to IN-CSE.

When the SCEF receives information of status change in step 1, the SCEF creates and sends an Event Notification message to the IN-CSE as specified in 3GPP TS 29.122 [i.aa].

The Event Notification request includes the following:

- event is configured with the value as specified in 3GPP TS 29.122 [i.aa].

- accumulatedUsage is configured the usage of usageThreshold  when event is USAGE_REPORT.

- flowIds is configured with the same value of flowId in the AsSessionWithQoSSubscription request.

Step 3: IN-CSE checks the -	QoS session status and performs the action according to the configuration.

When the IN-CSE receives the Event Notification, the IN-CSE will map the event and accumulatedUsage to the QoS session status of <e2eQosSession> and performs the action according to the status.

| 3GPP event | oneM2M QoS session status |
| --- | --- |
| LOSS_OF_BEARER | FAILED |
| RELEASE_OF_BEARER | FAILED |
| USAGE_REPORT and duration or totalVolume of accumulatedUsage is 0 | EXHAUSTED |

Step 4 (Optional): IN-CSE requests to delete On-demand QoS.

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this AsSessionWithQoSSubscription.  This step may be triggered by the IN-CSE when message delivery completes or the action configured in <e2eQosSession>.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

# 2	References

## 2.1	Normative references

As informative publications shall not contain normative references this clause shall remain empty.

Not applicable.

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	oneM2M TS-0002: "Requirements".

[i.3]	3GPP TS 22.101: "Service aspects; Service principles (Release 13)".

[i.4]	3GPP TS 22.115: "Service aspects; Charging and billing (Release 13)".

[i.5]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications (Release 14)".

[i.6]	OMA API Inventory.

NOTE:	Available at http://technical.openmobilealliance.org/Technical/technical-information/oma-api-program.

[i.7]	OMA Service Exposure Framework.

NOTE:	Available at http://member.openmobilealliance.org/ftp/Public_documents/ARCH/ServiceExposure.

[i.8]	OMA Exposing Network Capabilities to M2M.

NOTE:	Available at http://member.openmobilealliance.org/ftp/Public_documents/ARCH/ENCap-M2M.

[i.9]	oneM2M TS-0001: "Functional Architecture ".

[i.10]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications (Release 13)".

[i.11]	3GPP TS 23.203: "Policy and charging control architecture (Release 15)".

[i.12]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC); Stage 1".

[i.13]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[i.14]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[i.15]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[i.aa]                    3GPP TS 29.122: T8 reference point for Northbound APIs (Release 15)

[i.xx]                    3GPP TS 29.214: Technical Specification Group Core Network and Terminals; Policy and Charging Control over Rx reference point (Release 15)

[i.zz]                    3GPP TS 29.212: Technical Specification Group Core Network and Terminals; Policy and Charging Control (PCC); Reference points (Release 15)

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?