| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#39 |
| Source:* | Moode Giribabu Naik (moode@cdot.in), Anupama Chopra(anupama@cdot.in), Poornima (poornima@cdot.in), Suman Singh Sheoran (ssheoran@cdot.in), Sachin(sachin@cdot.in) |
| Date:* | 2019-02-01 |
| Reason for Change/s:* | See Introduction |
| CR against: Release* | 3 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0006 v3_6_0 |
| Clauses * | 7.15,7.8 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | n/a |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Refer to the CR additional_parameters_required_for_CSE_AE_registration(R4).

### ********** Start of change 1 **********

## 7.15	Resource [registration]

The Resource [registration] represents the configuration information needed to register and AE or CSE with a Registrar CSE.

The Resource [registration] is a multi-instance object where the key of the object is the originatorID (i.e., AE-ID, CSE-ID). The following rules are used to determine the object instance based on the originatorID:

- When the originatorID resource is for a CSE-ID, the TR-069 object instance Device.X_oneM2M_org_CSE.{i} shall be used for the specified CSE-ID.

- When the originatorID resource is for an AE-ID, the TR-069 object instance Device.X_oneM2M_org_AE.{i} shall be used for the specified AE-ID.

The information shall be created using the Add Object RPC of TR-069 [Error: Reference source not found].

The information shall be deleted using the Delete Object RPC of TR-069 [Error: Reference source not found].

The information shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The capabilities shall be enabled and disabled using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.15-1: Resource [registration] for CSE

| Attribute Name of [registration] | BBF TR-181 [6] Parameter (X_oneM2M_org_CSE) |
| --- | --- |
| originatorID | ID – See description of the type of object to instantiate. |
| poA | PointOfAccess |
| CSEBase | CSEBase - Resource-ID of the CSEBase of Registrar CSE. |
| CSE-ID | CSEID - CSE-ID of the Registrar CSE. |
| externalID | ExternalID |
| triggerRecipientID | TriggerRecipientID |
| mgmtLink [authenticationProfile] | AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table) |

Table 7.15-2: Resource [registration] for AE

| Attribute Name of [registration] | BBF TR-181 [6] Parameter (X_oneM2M_org_AE) |  |
| --- | --- | --- |
| originatorID | ID – See description of the type of object to instantiate. |  |
| poA | PointOfAccess |  |
| CSEBase | CSEBase - Resource-ID of the CSEBase of Registrar CSE. |  |
| CSE-ID | CSEID - CSE-ID of the Registrar CSE. |  |
| appID | ApplicationID |  |
| mgmtLink [authenticationProfile] | AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table) |  |

### ********** End of change 1 **********

### ********** Start of change 2 **********

## 7.8	Resource [deviceCapability]

The Resource [deviceCapability] represents a capability of device that can be administratively enabled or disabled. The lists of capabilities that are managed are defined in the enumeration of the capabilityName attribute. The TR-181 [Error: Reference source not found] data model defines a subset of capabilities listed in the deviceCapability enumeration. The supported device capabilities within TR-181 [Error: Reference source not found] include:

- LAN Interfaces: USB, Wi-Fi, HomePlug, MoCA, UPA

- Hardware Capabilities: SmartCardReader

The information shall be retrieved using the GetParameterValues RPC of TR-069 [Error: Reference source not found].

The capabilities shall be enabled and disabled using the SetParameterValues RPC of TR-069 [Error: Reference source not found].

Table 7.8-1: Resource [deviceCapability]

| Attribute Name of [deviceCapability] | BBF TR-181 [6] Parameter |
| --- | --- |
| capabilityName | This attribute is fixed based on the value of the capabilityName attribute. |
| Attached | Returns “True” |
| capabilityActionStatus | - Status is defined as: - Success if the SetParameterValues RPC indicates that the operation was successful. - Failure if the response to the SetParameterValues RPCs indicates that the operation failed. - In process if the SetParameterValues RPC is initiated but the response to the SetParameterValues RPC has not been received. |
| currentState | USB: Device.USB.Interface.{i}.Enable Wi-Fi: Device.Wi-Fi.Radio.{i}.Enable HomePlug: Device.HomePlug.Interface.{i}.Enable MoCA: Device.MoCA.Interface.{i}.Enable UPA: Device.UPA.Interface.{i}.Enable SmartCardReader: Device.SmartCardReaders.SmartCardReader.{i}.Enable |
| enable | USB: Device.USB.Interface.{i}.Enable Wi-Fi: Device.Wi-Fi.Radio.{i}.Enable HomePlug: Device.HomePlug.Interface.{i}.Enable MoCA: Device.MoCA.Interface.{i}.Enable UPA: Device.UPA.Interface.{i}.Enable SmartCardReader: Device.SmartCardReaders.SmartCardReader.{i}.Enable |
| disable | Same parameter is used to disable a capability as the enable attribute. |

### ********** End of change 2**********

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?