| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 39 |
| Source:* | Bob Flynn, bob.flynn@convidawireless.com Kushal Shah, Radisys, Kushal.shah@radisys.com |
| Date:* | 2019-02-18 |
| Reason for Change/s:* | Addition of attribute “specializationID” to group resource so that if a group type is flexContainer or mgmtObj then it can be further identified with the specialization type of the resource. E.g .Group of Lights or group of deviceID |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: ARC-2019-0318 STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001V2_22_0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0004, addition of new attribute “specializationID” to group resource |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

A <group> resource is defined to support a resource type which is intended to ensure all group members are of the same type.

The current specifications do not specify the way to create a group resource with specialized type when the memberType is set to flexContainer or mgmtObj. This contribution aims to address this gap by adding an attribute “specializationID” to group resource. This attribute will hold the specialization type of flexContainer or mgmtObj resource. E.g Group of deviceLight or Group of deviceID

### -----------------------Start of change 1-------------------------------------------

Table 9.6.13-2: Attributes of <group> resource

| Attributes of <group> | Multiplicity | RW/ RO/ WO | Description | <groupAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3. | NA |
| memberType | 1 | WO | It is the resource type of the member resources of the group, if all member resources (including the member resources in any sub-groups) are of the same type. Otherwise, it is of type 'mixed'. | OA |
| specializationType | 0..1 | WO | When the memberType attribute value is set to “mgmtObj” this specializationTypeDmay be set to the specialization defined by the mgmtDefinition attribute. When the memberType attribute value is set to “flexContainer”, this specializationType may be set to the specialization defined by the containerDefinition attribute. | OA |
| currentNrOfMembers | 1 | RO | Current number of members in a group. It shall not be larger than maxNrOfMembers. | OA |
| maxNrOfMembers | 1 | RW | Maximum number of members in the <group>. | OA |
| memberIDs | 1 (L) | RW | List of member resource IDs referred to in the remaining of the present document as memberID. Each ID (memberID) should refer to a member resource or a (sub-) <group> resource of the <group> if memberID is suffixed with "/fopt". A <group> resource with an empty member list is allowed. | OA |
| membersAccessControlPolicyIDs | 0..1 (L) | RW | List of IDs of the <accessControlPolicy> resources defining who is allowed to access the <fanOutPoint> resource. | OA |
| memberTypeValidated | 0..1 | RO | Denotes if the resource types of all members resources of the group has been validated by the Hosting CSE. In the case that the memberType attribute of the <group> resource is not 'mixed', then this attribute shall be set.. | OA |
| consistencyStrategy | 1 | WO | This attribute determines how to deal with the <group> resource if the memberType validation fails. Its possible values are: - ABANDON_MEMBER. - ABANDON_GROUP. - SET_MIXED. Which means delete the inconsistent member if the attribute is ABANDON_MEMBER; delete the group if the attribute is ABANDON_GROUP; set the memberType to "mixed" if the attribute is SET_MIXED. If it is not given by the Originator at the creation procedure, default is " ABANDON_MEMBER". | OA |
| groupName | 0..1 | RW | Human readable name of the <group>. | OA |
| semanticSupportIndicator | 0..1 | RO | Indicator of support for sematic discovery functionality via <semanticFanOutPoint>. | OA |
| notifyAggregation | 0..1 | RW | This attribute specifies the number of messages and/or the duration that the group hosting CSE will aggregate notification messages when the subscriptions created specify aggregation of notifications i.e. specifying the notificationForwardingURI of the original <subscription> resource. | OA |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

Table 10.2.7.2-1: <group> CREATE

| <group> CREATE |  |
| --- | --- |
| Associated Reference Point | Mcc, Mca and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <CSEBase>, <AE>, or <remoteCSE> where the <group> resource is intended to be Created Content: The representation of the <group> resource for which the attributes are described in clause 9.6.13 |
| Processing at Originator before sending Request | The Originator shall request to Create a <group> resource by using the CREATE operation. The request shall address <CSEBase>, <remoteCSE> or <AE> resource of a Hosting CSE. The Request shall also provide memberIDs and may provide expirationTime attributes.For members which are of type <group>, the originator shall suffix the ‘/fopt' to that ‘memberID‘ during group creation if the originator wants to fan-out the group request to each member of that sub-<group>, else originator shall not suffix the ‘/fopt' to that ‘memberID‘. The Originator may be an AE or a CSE |
| Processing at Receiver | For the CREATE procedure, the Receiver shall: - Check if the Originator has CREATE permissions on the target resource - Check the validity of the provided attributes - Validate that there are no duplicate members present in the memberIDs attribute - Validate that the resource type of every member on each member Hosting CSE conforms to the memberType attribute in the request, if the memberType attribute of the <group> resource is not 'mixed'. Set the memberTypeValidated attribute to TRUE upon successful validation - If specializationType attribute is set and the memberType attribute of the <group> resource is not 'mixed', then validate that the resource type of every member on each member Hosting CSE conforms to the specialized resource type set in specializationType attribute. Set the memberTypeValidated attribute to TRUE upon successful validation. - Upon successful validation of the provided attributes, create a new group resource in the Hosting CSE. If the CSE supports semantic discovery functionality, the Hosting CSE shall also create and set the semanticSupportIndicator attribute to TRUE. - Conditionally, in the case that the group resource contains temporarily. unreachable Hosting CSE of sub-group resources as member resource, set the memberTypeValidated attribute of the <group> resource to FALSE - Respond to the Originator with the appropriate generic Response with the representation of the <group> resource if the memberTypeValidated attribute is FALSE, and the address of the created <group> resource if the CREATE was successful - As soon as any Hosting CSE that hosts the unreachable resource becomes reachable, the memberType validation procedure shall be performed. If the memberType validation fails, the Hosting CSE shall deal with the <group> resource according to the policy defined by the consistencyStrategy attribute of the <group> resource provided in the request. or by default if the attribute is not provided |
| Information in Response message | The representation of the <group> resource if the memberTypeValidated attribute is FALSE |
| Processing at Originator after receiving Response | None |
| Exceptions | No change from the basic procedure in clause 10.1.1 |

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

#### 10.2.7.4       Update <group>

This procedure shall be used for updating an existing <group> resource.

Table 10.2.7.4-1: <group> UPDATE

| <group> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the <group> resource |
| Processing at Originator before sending Request | The Originator shall request to update attributes of an existing <group> resource by using an UPDATE operation. The Request shall address the specific <group> resource of a CSE. If originator intends to update memberIDs attribute ,for members which are of type <group>, originator shall suffix the ‘/fopt' to that ‘memberID‘ during group updation if the originator wants to fan-out the group request to each member of that sub-<group> ,else originator shall not suffix the ‘/fopt' to that ‘memberID‘. The Originator may be an AE or a CSE |
| Processing at Receiver | The UPDATE procedure shall be: - Check if the Originator has UPDATE permissions on the <group> resource - Check the validity of provided attributes - Validate that there are no duplicated members present in the memberIDs attribute - Validate that the resource type of every member on each member Hosting CSE conforms to the memberType attribute in the request, if the memberType attribute of the <group> resource is not 'mixed'. Set the memberTypeValidated attribute to TRUE upon successful validation - Validate that the resource type of every member on each member Hosting CSE conforms to the specialized resource type set in specializationType attribute, if the memberType attribute of the <group> resource is not 'mixed'. Set the memberTypeValidated attribute to TRUE upon successful validation. - Upon successful validation of the provided attributes, update the <group> resource in the Hosting CSE - Conditionally, in the case that the <group> resource contains temporarily unreachable Hosting CSE of sub-group resources as members resource set the memberTypeValidated attribute of the <group> resource to FALSE - Respond to the Originator with the appropriate generic response with the representation of the <group> resource if the memberTypeValidated attribute is FALSE, and the address of the created <group> resource if the UPDATE is successful - As soon as any Hosting CSE that hosts unreachable resource becomes reachable, the memberType validation procedure shall be performed. If the memberType validation fails, the Hosting CSE shall deal with the <group> resource according to the policy defined by the consistencyStrategy attribute of the <group> resource provided in the request, or by default if the attribute is not provided |
| Information in Response message | The representation of the <group> resource if the memberTypeValidated attribute is FALSE |
| Processing at Originator after receiving Response | None |
| Exceptions | No change from the basic procedure in clause 10.1.3 |

### -----------------------End of change 3---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?