| Input Contribution |  |
| --- | --- |
| Meeting ID* | TP#39 |
| Title:* | TR-0050 ABAC Policy Resource Definition |
| Source:* | Wei Zhou, Datang, zhouwei@catt.cn |
| Date:* | 2019-02-18 |
| Input related to* | WI-0077 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) | TR-0055 Attribute Based Access Control Policy, V0_6_0 |
| Decision requested or recommendation:* | Approve the proposed text into the TR. |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

- Introduction

This contribution proposes a new resource that is used to store ABAC policy.

### -----------------------Start of change 1---------------------------------------------

### 6.3.x	Solution #1.y: ABAC policy resource

#### 6.3.x.1	Introduction

The resource defined in this clause is used to store ABAC policy defined in clause 6.3.8.

#### 6.3.x.2	Solution details

##### 6.3.x.2.1	Resource Type abacPolicy

The <abacPolicy> resource contains the child resource specified in table 6.3.x.2.1-1.

Table 6.3.x.2.1: Child resources of <abacPolicy> resource

| Child Resources of <accessControlPolicy> | Child Resource Type | Multiplicity | Description | <accessControlPolicyAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 in TS-0001 [i.1] | <subscription> |

The <abacPolicy> resource contains the attributes specified in table 6.3.x.2.1-2.

Table6.3.x.2.1-2: Attributes of <accessControlPolicy> resource

| Attributes of <accessControlPolicy> | Multiplicity | RW/ RO/ WO | Description | <accessControlPolicyAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in TS-0001 [i.1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 in TS-0001 [i.1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 in TS-0001 [i.1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 in TS-0001 [i.1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 in TS-0001 [i.1]. | MA |
| labels | 0..1(L) | RW | See clause 9.6.1.3 in TS-0001 [i.1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 in TS-0001 [i.1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in TS-0001 [i.1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 in TS-0001 [i.1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 in TS-0001 [i.1]. | NA |
| abacPrivileges | 1 | RW | See clause 6.3.8. | MA |
| adminPrivileges | 1 | RW | See clause 8.3.x.2. | MA |
| authorizationDecisionResourceIDs | 0..1 (L) | RW | A list of addresses of <authorizationDecision> resources. See clause 9.6.41 in TS-0001 [i.1] for further details. | MA |
| authorizationPolicyResourceIDs | 0..1 (L) | RW | A list of addresses of <authorizationPolicy> resources. See clause 9.6.42 in TS-0001 [i.1] for further details. | MA |
| authorizationInformationResourceIDs | 0..1 (L) | RW | A list of addresses of <authorizationInformation> resources. See clause 9.6.43 in TS-0001 [i.1] for further details. | MA |

### -----------------------End of change 1---------------------------------------------