| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | PRO #38 |
| Source:* | Miguel Angel Reina Ortega, miguelangel.reinaortega@etsi.org Peter Niblett, peter_niblett@uk.ibm.com |
| Date:* | 2018-12-06 |
| Reason for Change/s:* | nonblocking for CoAP proposal |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: PRO-2018-0261R02-TS-0008-nonBlocking_for_coap_proposal_R3 STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0008 v2.7.0 |
| Clauses * | Clauses 6.2.4, 6.3.2, 6.3.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes a solution for the nonblocking procedure applied when using CoAP.  The main idea is to send a non-blocking request using the CoAP POST method, and to use the response to that operation to pass the URI of the <request> object (if there is one).

The CR also extends the blocking mode, so that it is free to use the CoAP “separate mode”. This is the mode where the response is not piggybacked on the CoAP ack. This is to reduce the reduce the likelihood of the Originator having to make unnecessary retransmissions of the request.

The CR also adds some further RSCs and has a few other editorial changes.

### -----------------------Start of change 1-------------------------------------------

- 6.2.4	Response Codes Mapping

Table 6.2.4-1 defines a mapping between oneM2M Response Status Code parameter specified in [Error: Reference source not found] and CoAP Response Code.

In case of where multiple oneM2M Response Status Code parameters are mapped to single CoAP Status Code, Response Status Code parameter shall be specified in oneM2M-RSC Option.

Table 6.2.4-1 Mapping between oneM2M Response Status Code and CoAP Response Code

| oneM2M Response Status Code | Description | CoAP Response Code | Description |
| --- | --- | --- | --- |
| 1000 | ACCEPTED | None | Not used |
| 1001 | ACCEPTED for nonBlockingRequestSynch | 2.01 | Created (indicates that a <request> resource has been created |
| 1002 | ACCEPTED for nonBlockingRequestAsynch | 2.01 or 2.04 | 2.01 (Created) is used if a <request> resource was created, otherwise 2.04 (Changed) is used |
| 2000 (for RETRIEVE operation) | OK | 2.05 | Content |
| 2000 (for NOTIFY operation) | OK | 2.04 | Changed. CoAP payload shall be empty. |
| 2001 | CREATED | 2.01 | Created |
| 2002 | DELETED | 2.02 | Deleted |
| 2004 | UPDATED | 2.04 | Changed |
| 4000 | BAD_REQUEST | 4.00 | Bad Request |
| 4001 | RELEASE_VERSION_NOT_SUPPORTED | 4.00 | Bad Request |
| 4004 | NOT_FOUND | 4.04 | Not Found |
| 4005 | OPERATION_NOT_ALLOWED | 4.05 | Method Not Allowed |
| 4008 | REQUEST_TIMEOUT | 4.04 | Not Found |
| 4015 | UNSUPPORTED_MEDIA_TYPE | 4.15 | Unsupported Content-Format |
| 4101 | SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE | 4.03 | Forbidden |
| 4102 | CONTENTS_UNACCEPTABLE | 4.00 | Bad Request |
| 4103 | ORIGINATOR_HAS_NO_PRIVILEGE | 4.03 | Forbidden |
| 4104 | GROUP_REQUEST_IDENTIFIER_EXISTS | 4.00 | Bad Request |
| 4105 | CONFLICT | 4.03 | Forbidden |
| 4106 | ORIGINATOR_HAS_NOT_REGISTERED | 4.03 | Forbidden |
| 4107 | SECURITY_ASSOCIATION_REQUIRED | 4.03 | Forbidden |
| 4108 | INVALID_CHILD_RESOURCE_TYPE | 4.03 | Forbidden |
| 4109 | NO_MEMBERS | 4.03 | Forbidden |
| 4110 | GROUP_MEMBER_TYPE_INCONSISTENT | 4.00 | Bad Request |
| 4111 | ESPRIM_UNSUPPORTED_OPTION | 4.03 | Forbidden |
| 4112 | ESPRIM_UNKNOWN_KEY_ID | 4.03 | Forbidden |
| 4113 | ESPRIM_UNKNOWN_ORIG_RAND_ID | 4.03 | Forbidden |
| 4114 | ESPRIM_UNKNOWN_RECV_RAND_ID | 4.03 | Forbidden |
| 4115 | ESPRIM_BAD_MAC | 4.03 | Forbidden |
| 4116 | ESPRIM_IMPERSONATION_ERROR | 4.03 | Forbidden |
| 4117 | ORIGINATOR_HAS_ALREADY_REGISTERED | 4.03 | Forbidden |
| 5000 | INTERNAL_SERVER_ERROR | 5.00 | Internal Server Error |
| 5001 | NOT_IMPLEMENTED | 5.01 | Not Implemented |
| 5103 | TARGET_NOT_REACHABLE | 4.04 | Not Found |
| 5105 | RECEIVER_HAS_NO_PRIVILEGE | 4.03 | Forbidden |
| 5106 | ALREADY_EXISTS | 4.00 | Bad Request |
| 5203 | TARGET_NOT_ SUBSCRIBABLE | 4.03 | Forbidden |
| 5204 | SUBSCRIPTION_VERIFICATION_INITIATION_FAILED | 5.00 | Internal Server Error |
| 5205 | SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE | 4.03 | Forbidden |
| 5206 | NON_BLOCKING_SYNCH_REQUEST_NOT_SUPPORTED | 5.01 | Not Implemented |
| 5207 | NOT_ACCEPTABLE | 4.06 | Not Acceptable |
| 5208 | DISCOVERY_DENIED_BY_IPE | 4.03 | Forbidden |
| 5209 | GROUP_MEMBERS_NOT_RESPONDED | 5.00 | Internal Server Error |
| 5210 | ESPRIM_DECRYPTION_ERROR | 5.00 | Internal Server Error |
| 5211 | ESPRIM_ENCRYPTION_ERROR | 5.00 | Internal Server Error |
| 5212 | SPARQL_UPDATE_ERROR | 5.00 | Internal Server Error |
| 5220 | UNABLE_TO_RECALL_REQUEST | 5.03 | Service Unavailable |
| 6003 | EXTERNAL_OBJECT_NOT_REACHABLE | 4.04 | Not Found |
| 6005 | EXTERNAL_OBJECT_NOT_FOUND | 4.04 | Not Found |
| 6010 | MAX_NUMBER_OF_MEMBER_EXCEEDED | 4.00 | Bad Request |
| 6020 | MGMT_SESSION_CANNOT_BE_ESTABLISHED | 5.00 | Internal Server Error |
| 6021 | MGMT_SESSION_ESTABLISHMENT_TIMEOUT | 5.00 | Internal Server Error |
| 6022 | INVALID_CMDTYPE | 4.00 | Bad Request |
| 6023 | INVALID_ARGUMENTS | 4.00 | Bad Request |
| 6024 | INSUFFICIENT_ARGUMENTS | 4.00 | Bad Request |

The Receiver shall use this table to determine the CoAP response code that is to be used in the response, based on the value of the oneM2M Response Status Code parameter.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

#### 6.2.2.3	URI Options

This clause describes how to configure CoAP Uri-Host, Uri-Port, Uri-Path, and Uri-Query Options.

Host and port part of the address specified in pointOfAccess attribute of <remoteCSE> resource shall be mapped to Uri-Host and Uri-Port respectively.

If To parameter contains absolute format, then the first URI-Path Option shall contain a letter "_" and map To parameter removing starting "//" into next URI-Path Option(s).

If To parameter contains SP-relative format, then the first URI-Path Option shall contain a letter "~" and map To parameter removing starting "/" into next URI-Path Option(s).

If To parameter contains CSE-relative format, then To parameter shall be mapped to URI-Path Option(s).

Table 6.2.2.3-1 shows valid mappings between the To request primitive parameter and the Uri-Path of the CoAP.

CSEBase represents the resource name of a <CSEBase> resource, CSEBase/ae12/cont27/contInst696 represents a structured CSE-relative resource ID, and cin00856 an unstructured CSE-relative resource ID.

Table 6.2.2.3-1: Mapping examples between To parameter and Uri-Path of the CoAP

| Method |  | Request Scope |  |  |
| --- | --- | --- | --- | --- |
|  |  | CSE-Relative | SP-Relative | Absolute |
| Structured | To | CSEBase/ae12/cont27/contInst696 | /CSE178/CSEBase/ae12/cont27/contInst696 | //mym2msp.org/CSE178/CSEBase/ae12/cont27/contInst696 |
|  | Uri-Path | CSEBase |  | _ |
|  |  |  | ~ | mym2msp.org |
|  |  |  | CSE178 | CSE178 |
|  |  |  | CSEBase | CSEBase |
|  |  | ae12 | ae12 | ae12 |
|  |  | cont27 | cont27 | cont27 |
|  |  | contInst696 | contInst696 | contInst696 |
| Unstructured | To | cin00856 | /CSE178/cin00856 | //mym2msp.org/CSE178/cin00856 |
|  | Uri-Path | cin00856 |  | _ |
|  |  |  | ~ | mym2msp.org |
|  |  |  | CSE178 | CSE178 |
|  |  |  | cin00856 | cin00856 |
| NOTE: How to read this table: To primitive - from left to right, Uri-Path - from top to bottom. |  |  |  |  |

The responseTypeValue element of Response Type, and the Result Persistence, Delivery Aggregation, Result Content, parameters of Filter Criteria, Desired Identifier Result Type, Token Request Indicator, Tokens, Token IDs, Local Token IDs and Operation parameters shall be carried, if needed, in the Uri-Query Option in a short name form as specified in clause 8.2.2 of oneM2M TS-0004 [Error: Reference source not found].

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

## 6.3	Accessing Resources in CSEs

### 6.3.0	Introduction

This clause describes the behaviour of CoAP layer depending on Response Type parameter. Note that the CoAP messaging model defined in [1] applies to all message exchanges. In all cases the Originator shall resend requests until they have been acknowledged, and the Receiver shall resend confirmable responses until they have been acknowledged. The recipient (Receiver or Originator) shall take care to de-duplicate confirmable messages as described in [1].

-

### 6.3.1	Blocking case

- If Response Type parameter is configured as "blockingRequest" (blocking case), the Originator (CoAP client) shall use the Confirmable Method to send the request to the Receiver (CoAP server). The oneM2M Operation parameter shall be mapped to a CoAP Method according to Table 6.2.1-1.

- After processing the request, the Receiver shall send a CoAP response with a CoAP response code as given by Table 6.2.4-1. It may either piggyback this response to the request on the CoAP ACK message, or send a separate CoAP response message after the CoAP ACK.

- The Originator’s CoAP binding may generate a response primitive containing a oneM2M Response Status Code of "REQUEST_TIMEOUT" if it considers that it has taken too long for the CoAP response to come back from the Receiver. It shall ignore any response to the original request that it might receive after it has done this.

- 6.3.2	Non-Blocking Asynchronous case

1) If the Response Type parameter is configured as "nonBlockingRequestAsynch" (non-blocking asynchronous case), the Originator (CoAP client) shall use the Confirmable Method to send the request to the Receiver (CoAP server). This request shall be sent using a CoAP POST method, and shall include the Operation parameter, mapped as described in clause 6.2.2.3.

2) The Receiver, after validating the request and before processing it fully, shall return a CoAP response to the originator. It may either piggyback (2a) this response to the request on the CoAP ACK message, or send a separate CoAP response message after the CoAP ACK (2b).

- If the Receiver supports the <request> resource type, it shall respond with a 2.01 (Created) CoAP response code and a oneM2M Response Status Code of "ACCEPTED for nonBlockingRequestAsynch". The response shall include the URI of the new <request> resource in a sequence of one or more Location-Path and/or Location-Query Options.

- If the Receiver does not support the <request> resource type, it shall respond with a 2.04 (Changed) CoAP response code and a oneM2M Response Status Code of "ACCEPTED for nonBlockingRequestAsynch".

3) The Receiver, upon successful processing of the request, shall send a new CoAP Confirmable request message using POST method (NOTIFY primitive) and whose payload contains the response to the original request.

4) The Originator may either piggyback a response to this request (4a) or send it as a separate CoAP response after the acknowledgment message (4b). This response shall contain the appropriate CoAP response code as defined in table 6.2.4-1 and have an empty payload.

- 6.3.3	Non-Blocking Synchronous case

1) If the Response Type parameter is configured as "nonBlockingRequestSynch" (non-blocking synchronous case), the Originator (CoAP client) shall use the Confirmable Method for the resource to the Receiver (CoAP server). This request shall be sent using a CoAP POST method, and shall include the Operation parameter, mapped as described in clause 6.2.2.3.

2) The Receiver, after validating the request and before processing it fully, shall return a CoAP response to the originator. It may either piggyback this response (2a) on the CoAP ACK message or send a separate CoAP response message after the CoAP ACK (2b).

- If the Receiver supports the <request> resource type, it shall respond with a 2.01 (Created) CoAP response code and a oneM2M Response Status Code of "ACCEPTED for nonBlockingRequestSynch". The response shall include the URI of the new <request> resource in a sequence of one or more Location-Path and/or Location-Query Options.

- If the Receiver does not support the <request> resource type, it shall respond with a 5.01 (Not implemented) CoAP response code and a oneM2M Response Status Code of "NON_BLOCKING_REQUEST_NOT_SUPPORTED".

3) The Originator can use the <request> resource reference to synchronously retrieve the <request> resource that contains the response to the original request.

4) The Receiver, upon receipt of this retrieve request, shall handle it as in clause 6.3.1 since it is a non-blocking request.

NOTE:	If the Receiver is a Transit CSE, the Receiver acts as CoAP client and CoAP server.

- 6.3.4	Flex Blocking case

1) If the Response Type parameter is configured as "flex blocking", the Originator (CoAP client) shall use the Confirmable Method for the resource to the Receiver (CoAP server). This request shall be sent using a CoAP POST method, and shall include the Operation parameter, mapped as described in clause 6.2.2.3.

2) The Receiver shall determine whether to handle the request using "nonBlockingRequestSynch" or "nonBlockingRequestAsynch" mode:

- If the Receiver chooses "nonBlockingRequestAsynch" processing proceeds as described in clause 6.2.2, starting from step 2).

- If the Receiver chooses "nonBlockingRequestSynch" processing proceeds as described in clause 6.2.3, starting from step 2).

### -----------------------End of change 3---------------------------------------------

### -----------------------Start of change 4---------------------------------------------

## 6.2	Primitive Mapping to CoAP Message

### 6.2.0	Introduction

This clause describes where to map oneM2M parameters in a primitive to header, Option and payload fields in a CoAP message.

### 6.2.1	Header

This clause specifies how to configure CoAP header information:

- The Version field shall be configured as 1.

- The Type field shall be configured according to clause 6.3. The Reset message is used to send a error message in response to a malformed Confirmable message in CoAP layer.

- In case of a request, the Code field indicates CoAP Method. If the oneM2M operation is sent as a Blocking request the oneM2M Operation parameter shall be mapped to a CoAP Method according to the table 6.2.1-1. In non-blocking and flex blocking cases, the request shall use the CoAP POST method, and the Operation parameter shall be mapped as described in clause 6.2.2.3.

- In case of a response, the Code field indicates CoAP Response Code. The oneM2M Response Status Code parameter shall be mapped to CoAP Response Code as specified in clause 6.2.4.

- The Originator and Receiver shall set the 16 bit MessageId in accordance with the CoAP specification [1] and shall retry transmission of all unacknowledged Confirmable messages, as required by that specification.

Table 6.2.1-1: oneM2M Operation Parameter Mapping

| oneM2M Operation Parameter | CoAP Method | CoAP Method Code |
| --- | --- | --- |
| CREATE | POST | 0.02 |
| RETRIEVE | GET | 0.01 |
| UPDATE | PUT | 0.03 |
| DELETE | DELETE | 0.04 |
| NOTIFY | POST | 0.02 |

At the Receiver, a CoAP request message with POST method that does not carry an Operation parameter shall be mapped to oneM2M CREATE or NOTIFY operation in accordance with the existence of Resource Type parameter. If a Resource Type parameter exists then value of the Operation parameter is CREATE and if the Resource Type parameter does not exist, the value of the Operation parameter is NOTIFY.

### 6.2.2	Configuration of Token and Options

#### 6.2.2.0	Introduction

This clause describes configuration of Token and Options based on oneM2M parameters.

#### 6.2.2.1	Token

The CoAP token is used by the CoAP layer to match a response to a request, in a manner that is similar to the oneM2M Request Identifier. Due to size limitations, a Request Identifier can not be used directly as the CoAP Token.

The use of tokens by Originator and Receiver shall comply with requirements of the CoAP specification [1].

### -----------------------End of change 4---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?