| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #39 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com |
| Date:* | 2019-06-24 |
| Reason for Change/s:* | The CR is to add new feature of 3GPP QoS interworking. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0058 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 V4.1.0 |
| Clauses * | 7.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0001 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Add new feature to set up 3GPP QoS session to support End to End QoS session of oneM2M from TR-0024.

### -----------------------Start of change 1-------------------------------------------

## 7.x	Setting up an AS session with required QoS procedure

### 7.x.1 Overview

3GPP supports to set up an IP flow to a UE with a specific QoS  (e.g. low latency or jitter) and priority handling by the 3rd party service provider (AS/SCS session) via T8 API. The oneM2M may use this functionality for the communication management between IN-CSE and UE. AE may specify, for a set of target resource, a QoS level parameter which referss to pre-defined QoS information between oneM2M and 3GPP operator. Based on that, the oneM2M shall map the operation addressing those resources  from IN-CSE to the target UE to the 3GPP IP flow,  negotiate the QoS with 3GPP network, and request 3GPP network to deliver the operation with the negotiated QoS.

### 7.x.2 Resource Structure

Refer to the clause 9.6.xx Resource Type <e2eQosSession> of oneM2M TS-0001[1].

### 7.x.3 Procedures

#### 7.x.3.1 	Create/Update E2E QoS procedure

Figure 7.x.3.1-1: Create/Update E2E QoS procedure

Step 0: The IN-AE, ADN-AE or ASN/MN-CSE performs oneM2M registration.  The IN-CSE is pre-provisioned with 3GPP QoS Information based on a SLA with the MNO.

The IN-AE, ADN-AE or ASN/MN-CSE performs the oneM2M registration procedure. Then the IN-CSE gets the pointOfAccess network registration information of ASN/MN-CSE or ADN-AE.

The IN-CSE uses the pre-provisioned 3GPP QoS information to translate oneM2M QoS parameters defined in <e2eQosSession> resources into the QoS parameters defined by the MNO and used over the SCEF T8 interface.

Step 1: An AE or CSE sends a <e2eQosSession> CREATE/UPDATE request to the IN-CSE. The <e2eQosSession> CREATE/UPDATE request includes the following information:

- To parameter shall be configured with the Resource-ID of an <AE>, <remoteCSE> or <CSEBase> resource hosted by the IN-CSE

- From parameter shall be configured with the AE-ID or CSE-ID of the Originator

NOTE: The Originator of the <e2eQosSession> CREATE/UPDATE request may be an entity specified in the sessionEndpoints attribute.  Alternatively, it may be a different entity.

- sessionEndpoints attribute shall be set to a list consisting of one or more AE-IDs and/or CSE-IDs representing the endpoints of the E2E QoS session.

- e2eQosRequirements attribute shall be configured with a list of one or more tuples.  Each tuple in the list has the following elements:

  - qosLevel element shall be set to a value between 0 and 100.

  - resourceIDList element shall beset to a list of resource identifiers the QoS requirement applies to.

  - sessionSchedule element may be set.  If set, it consists of seven fields of second, minute, hour, day of month, month, day of week and year.

  - numOfRequests element may be set.  If set, it consists of the minimum number of requests required to be transferred at the specified qosLevel.

  - numOfBytes element may be set.  If set, it consists of the minimum number of bytes required to be transferred at the specified qosLevel.

- e2eQosPolicies attribute may be set.  If set, it is configured with 1 or more tuples.  Each tuple has the following elements:

  - status element shall be set with a value of FAILED, DISABLED or USAGE_EXHAUSTED

  - action element shall be set with a value of RE-ENABLE or DISABLE

Step 2: The IN-CSE validates the E2E QoS session endpoint entities.

The IN-CSE receives and validates the <e2eQosSession> CREATE/UPDATE request.  The IN-CSE validates the sessionEndpoints attribute.  The IN-CSE performs this check by first confirming that each AE-ID and/or CSE-ID configured within the sessionEndpoints attribute matches an AE-ID and CSE-ID of one of its Registree AEs or CSEs.

Editor’s Note: Support for E2E QoS sessions involving N-Hop deployments is FFS.  If support is defined, then the check to confirm that each AE-ID and/or CSE-ID configured within the sessionEndpoints attribute matches an AE-ID and CSE-ID of one of its Registree AEs or CSEs will need to be modified.

The IN-CSE also checks that at least one tuple is configured in the e2eQosRequirements attribute and that the qosLevel element and resourceIDList in this tuple is configured.  The IN-CSE also checks that all other mandatory attributes and parameters in the request are present and their values comply with their supported data types.  The IN-CSE also checks that the values of all other optional attributes and parameters in the request comply with their supported data types   and are properly formatted.  If these checks are successful, the IN-CSE creates/updates the <e2eQosSession> according to the request. Then the IN-CSE proceeds to Step 3.  Otherwise, the IN-CSE proceeds to Step 6 and returns a Response Status Code indicating BAD_REQUEST error.

Step 3: If necessary, the IN-CSE triggers the E2E QoS session endpoint entities.

If entities specified in the sessionEndpoints attribute are registered to the IN-CSE and use an underlying 3GPP network, the IN-CSE checks the connection (e.g. TCP) with the  sessionEndpoints. If the sessionEndpoints do not have an active 3GPP PDN connection to the IN-CSE (i.e. ASN/MN-CSE or ADN-AE pointOfAccess information is not configured or IN-CSE detects failed communication with ASN/MN-CSE or ADN-AE when the connection is based on TCP), the IN-CSE should send a device trigger request to the corresponding ASN/MN-CSE or ADN-AE to have it establish a connection (e.g. based on TCP) to the IN-CSE.

NOTE: If the sessionSchedule element is configured, the IN-CSE shall use the schedule information to determine whether to perform this step during the processing of the <e2eQosSession> CREATE/UPDATE request or sometime thereafter (e.g. closer to the time when scheduled communication via the session is required).

Step 4: The IN-CSE sends QoS Session Subscription Request(s) to the SCEF.

For each QoS session endpoint that connects to the IN-CSE via an underlying 3GPP network connection and that has an active PDN connection, the IN-CSE sends a AsSessionWithQoSSubscription Request to the SCEF.

NOTE: If the sessionSchedule element is configured, the IN-CSE shall use the schedule information to determine whether to perform this step during the processing of the <e2eQosSession> CREATE/UPDATE request or sometime thereafter (e.g. closer to the time when scheduled communication via the session is required).

Each AsSessionWithQoSSubscription Request from the IN-CSE to the SCEF contains information as specified in 3GPP TS 29.122 [4]. Such information includes:

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-as-session-with-qos/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload includes an AsSessionWithQoSSubscription data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - supportedFeatures shall be set to a string value of “0” indicating the IN-CSE does not support the QoS Session negotiable features specified in 3GPP TS 29.122 [4].

  - notificationDestination shall be set to a URI that the SCEF should target QoS session related notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - flowInfo includes the following attributes:

    - flowId shall be set to a integer value that describes the IP flow.  A value is assigned by the IN-CSE according local policy. The value that is used is unique within the scope of the IN-CSE.

    - flowDescriptions shall be an array of strings configured with two entries. The first entry in the array is an IP flow description for oneM2M requests and responses flowing from the IN-CSE to the QoS session endpoint applicable to this request.  The second entry in the array is an IP flow description for oneM2M requests and responses flowing in the reverse direction from the QoS session endpoint to the IN-CSE.

      - Entry #1 in the flowDescriptions array includes the following attributes:

        - direction shall be set to a value of “out”

        - source IP address shall be set to the IP address of the IN-CSE and destination IP address is set to the IP addresses configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.

        - protocol: shall be set to a value of “TCP” or “UDP” based on the underlying transport configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.  For example, “TCP” is set if the protocol configured in the pointOfAccess is HTTP, MQTT or WebSocket.  “UDP” is set if the protocol configured is CoAP.

        - source port shall be set to the port of the IN-CSE and destination port is set to the port configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.

      - Entry #2 in the flowDescriptions array includes the following attributes:

        - direction shall be set to a value of “in”

        - source IP address shall be set to the IP address configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request and destination IP address is set to the IP address of the IN-CSE.

- NOTE: The order is reversed from the order used in Entry #1.

        - protocol: shall be set to a value of “TCP” or “UDP” based on the underlying transport configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.  For example, “TCP” is set if the protocol configured in the pointOfAccess is HTTP, MQTT or WebSocket.  “UDP” is set if the protocol configured is CoAP.

        - source port shall be set to the port configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request and destination port is set to the port of the IN-CSE.

- NOTE: The order is reversed from the order used in Entry #1.

  - qosReference shall be set to a pre-provisioned string value that maps to the qosLevel.  This mapping is based on a SLA with the MNO.  The qosReference serves as an identifier of the pre-defined QoS information pre-provisioned into the IN-CSE based on a SLA with the MNO.

  - ueIpv4Addr shall be set to the IPv4 address (if applicable) configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.

  - ueIpv6Addr shall be set to the IPv6 address (if applicable) configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.

  - usageThreshold includes the following attributes:

    - duration shall be set to the amount of time in seconds that the QoS session is requested to remain active.  The IN-CSE computes this duration by inspecting all of the <e2eQosSession> resources that it hosts and that have a session endpoint that matches the session endpoint applicable to this request.  For any matches found, the IN-CSE will aggregate the   sessionSchedule elements (if any) configured within the list of e2eQosRequirements tuples of these <e2eQosSession> resources.  Based on the aggregated sessionSchedule elements and local policies, the IN-CSE will determine a duration of time to request.  If no sessionSchedule elements are configured, the IN-CSE will base its determination solely on local policies.

    - totalVolume shall be set to a total number of bytes of data that are required to be exchanged between the IN-CSE and the session endpoint applicable to this request.  The IN-CSE shall compute this number of bytes by inspecting all of the <e2eQosSession> resources that it hosts and that have a session endpoint that matches the session endpoint applicable to this request.  For any matches found, the IN-CSE will sum all of the numOfBytes elements (if any) configured within the list of e2eQosRequirements tuples of these <e2eQosSession> resources.  Based on the numOfBytes elements and local policies, the IN-CSE will determine an amount to request.  If no numOfBytes elements are configured, the IN-CSE will base its determination solely on local policies.

  - sponsorInfo may be set.  If set, the value is a string based on a SLA with the MNO.

  - ethFlowInfo, macAddr, requestTestNotification and websockNotifConfig are not supported by the present document and are not included.

Step 5: SCEF sends QoS Session Response(s) to IN-CSE.

The SCEF handles the AsSessionWithQoSSubscription Request together with the Mobile Core Network.  The SCEF sends an AsSessionWithQoSSubscription Response that contains information as specified in 3GPP TS 29.122 [4] to the IN-CSE.

The message includes the following information.

- A response code of 201 CREATED

- The URI of the QoS Session Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/ 3gpp-as-session-with-qos/v1/{scsAsId}/subscriptions/{subscriptionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {subscriptionId} segment is configured by the SCEF.

- The response payload may include a AsSessionWithQoSSubscription data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

  - self is configured with a URI to the resource created by the SCEF for the request

- The IN-CSE shall update the e2eQosStatus of the <e2eQosSession> resource according to the response from the SCEF.

Step 6: The IN-CSE returns response to Originator.

7.x.3.2 	QoS Session request processing procedure

Figure 7.x.3.2-1: Setting up 3GPP session with required QoS

Step 0: <e2eQosSession> created and IN-CSE creates QoS Session Subscription to SCEF

Step 1: The IN-AE sends a request that targets a E2E QoS session endpoint (e.g. ASN/MN-CSE)

The IN-AE sends a request to the IN-CSE that targets an entity that is configured as an E2E QoS session endpoint within a <e2eQosSession> resource hosted by the IN-CSE.

Step 2: The IN-CSE receives and processes the request and checks <e2eQosSession> resources

The IN-CSE processes the received request and obtains the targeted end point from the To parameter. Then IN-CSE checks the targeted end point and From parameters to see if they match with any sessionEndpoints configured within the <e2eQosSession> resources hosted by the IN-CSE.   If a match is found, the IN-CSE then checks To parameter in the request to see if the resource ID matches with any resourceIDList of the e2eQosRequirements configured within the same <e2eQosSession> resource.  If a match is found, the IN-CSE checks the e2eQosStatus whether the QoS session is enabled or not.  The IN-CSE also checks whether the entity targeted by the request has an active 3GPP PDN connection and is reachable by the IN-CSE.

CASE A: Both of these checks are successful.  The IN-CSE proceeds to Step 6.

CASE B: The targeted entity has an inactive 3GPP PDN connection (i.e. ASN/MN-CSE or ADN-AE pointOfAccess information is not configured or IN-CSE detects failed communication with ASN/MN-CSE or ADN-AE) then the IN-CSE proceeds to Step 3.

CASE C: The targeted entity has an active 3GPP PDN connection (i.e. ASN/MN-CSE or ADN-AE pointOfAccess information is configured), but the QoS session has been disabled (e2eQosStatus is set to DISABLED or USAGE_EXHAUSTED).  The IN-CSE proceeds to Step 4.

Step 3: The IN-CSE triggers the targeted E2E QoS session endpoint entity.

The targeted entity is registered to the IN-CSE and uses an underlying 3GPP network, but does not have an active 3GPP PDN connection to the IN-CSE, the IN-CSE sends a device trigger request to the entity to have it establish a 3GPP PDN connection to the IN-CSE.  The IN-CSE then checks whether the QoS session is enabled or not.  If the QoS session has been disabled (e2eQosStatus is set to DISABLED or USAGE_EXHAUSTED) the IN-CSE proceeds to Step 4, otherwise Step 6.

Step 4: The IN-CSE sends a request to update the QoS Session Subscription to re-enable the QoS session

If the configured e2eQosRequirements permit the IN-CSE to request that the QoS session be re-enabled when the e2eQosStatus is set to DISABLED or USAGE_EXHAUSTED, the IN-CSE may execute this step.  Otherwise, the IN-CSE shall either choose to continue processing the request in a best-effort fashion or go to Step 7 with a corresponding Response Status Code  indicating “NETWORK_QOS_CANNOT_ENABLE” by local policy.

For each QoS session endpoint that connects to the IN-CSE via an underlying 3GPP network connection and that has an active PDN connection, the IN-CSE sends a AsSessionWithQoSSubscription Request to the SCEF.

The IN-CSE sends a request to update to AsSessionWithQoSSubscription Request from the IN-CSE to the SCEF contains information as specified in 3GPP TS 29.122 [4]. Such information includes:

- An HTTP PUT method shall be used

- URI shall be set to {apiRoot}/3gpp-as-session-with-qos/v1/{scsAsId}/subscriptions/{subscriptionID}.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies. The {subscriptionId} segment is configured by the SCEF.

- The request payload includes an AsSessionWithQoSSubscription data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - supportedFeatures shall be set to a string value of “0” indicating the IN-CSE does not support the QoS Session negotiable features specified in 3GPP TS 29.122 [4].

  - notificationDestination shall be set to a URI that the SCEF should target QoS session related notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - flowInfo includes the following attributes:

    - flowId shall be set to a integer value that describes the IP flow.  A value is assigned by the IN-CSE according local policy. The value that is used is unique within the scope of the IN-CSE.

    - flowDescriptions shall be an array of strings configured with two entries. The first entry in the array is an IP flow description for oneM2M requests and responses flowing from the IN-CSE to the QoS session endpoint applicable to this request.  The second entry in the array is an IP flow description for oneM2M requests and responses flowing in the reverse direction from the QoS session endpoint to the IN-CSE.

      - Entry #1 in the flowDescriptions array includes the following attributes:

        - direction shall be set to a value of “out”

        - source IP address shall be set to the IP address of the IN-CSE and destination IP address is set to the IP addresses configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.

        - protocol: shall be set to a value of “TCP” or “UDP” based on the underlying transport configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.  For example, “TCP” is set if the protocol configured in the pointOfAccess is HTTP, MQTT or WebSocket.  “UDP” is set if the protocol configured is CoAP.

        - source port shall be set to the port of the IN-CSE and destination port is set to the port configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.

      - Entry #2 in the flowDescriptions array includes the following attributes:

        - direction shall be set to a value of “in”

        - source IP address shall be set to the IP address configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request and destination IP address shall be set to the IP address of the IN-CSE.

- NOTE: The order is reversed from the order used in Entry #1.

        - protocol: shall be set to a value of “TCP” or “UDP” based on the underlying transport configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.  For example, “TCP” is set if the protocol configured in the pointOfAccess is HTTP, MQTT or WebSocket.  “UDP” is set if the protocol configured is CoAP.

        - source port shall be set to the port configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request and destination port is set to the port of the IN-CSE.

- NOTE: The order is reversed from the order used in Entry #1.

  - qosReference shall be set to a pre-provisioned string value that maps to the qosLevel.  This mapping is based on a SLA with the MNO.  The qosReference serves as an identifier of the pre-defined QoS information pre-provisioned into the IN-CSE based on a SLA with the MNO.

  - ueIpv4Addr shall be set to the IPv4 address (if applicable) configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.

  - ueIpv6Addr shall be set to the IPv6 address (if applicable) configured in the pointOfAccess attribute of the <AE> or <remoteCSE> resource hosted on the IN-CSE and associated with the QoS session endpoint applicable to this request.

  - usageThreshold includes the following attributes:

    - duration shall be set to the amount of time in seconds that the QoS session is requested to remain active.  The IN-CSE computes this duration by inspecting all of the <e2eQosSession> resources that it hosts and that have a session endpoint that matches the session endpoint applicable to this request.  For any matches found, the IN-CSE will aggregate the   sessionSchedule elements (if any) configured within the list of e2eQosRequirements tuples of these <e2eQosSession> resources.  Based on the aggregated sessionSchedule elements and local policies, the IN-CSE will determine a duration of time to request.  If no sessionSchedule elements are configured, the IN-CSE will base its determination solely on local policies.

    - totalVolume shall be set to a total number of bytes of data that are required to be exchanged between the IN-CSE and the session endpoint applicable to this request.  The IN-CSE shall compute this number of bytes by inspecting all of the <e2eQosSession> resources that it hosts and that have a session endpoint that matches the session endpoint applicable to this request.  For any matches found, the IN-CSE will sum all of the numOfBytes elements (if any) configured within the list of e2eQosRequirements tuples of these <e2eQosSession> resources.  Based on the numOfBytes elements and local policies, the IN-CSE will determine an amount to request.  If no numOfBytes elements are configured, the IN-CSE will base its determination solely on local policies.

  - sponsorInfo may be set.  If set, the value is a string based on a SLA with the MNO.

  - ethFlowInfo, macAddr, requestTestNotification and websockNotifConfig are not supported by the present document and are not included.

Step 5: SCEF sends QoS Session Response(s) to IN-CSE.

The SCEF handles the AsSessionWithQoSSubscription Request together with the Mobile Core Network.  The SCEF sends an AsSessionWithQoSSubscription Response that contains information as specified in 3GPP TS 29.122 [4] to the IN-CSE.

The message includes the following information.

- A response code of 200 OK

- The response payload will include a AsSessionWithQoSSubscription data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

Step 6: The IN-CSE delivers the message with the required QoS and updates the <e2eQosSession> resource.

If the QoS session is enabled, the IN-CSE forwards the request to the targeted session endpoint entity.  After receiving a response back from the targeted session endpoint, the IN-CSE updates the e2eQosStatus of  the <e2eQosSession> resource and prepares a response for the Originator (IN-AE).

If the QoS session is disabled and the IN-CSE is un-successful in re-enabling it, then the IN-CSE shall either choose to continue processing the request in a best-effort fashion or go to Step 7 with a corresponding Response Status Code  indicating “NETWORK_QOS_CANNOT_ENABLE” by local policy.

Step 7: The IN-CSE returns response to Originator.

IN-CSE sends response to the IN-AE.

#### 7.x.3.3 	3GPP QoS status monitoring and report procedure

Figure 7.x.3.3-1: 3GPP QoS status report procedure

Step 1: The 3GPP network entities detect a bearer level event.

The 3GPP entities may notify the SCEF about bearer level events for the Rx session (e.g., transmission resources are released/lost) with an IP-CAN Session Modification as described in TS 23.203[i.11].

Step 2: SCEF sends Status notification message to IN-CSE.

When the SCEF receives information of a status change in step 1, the SCEF creates and sends an Event Notification message to the IN-CSE as specified in 3GPP TS 29.122 [4].

The Event Notification request includes the following:

- event indicates the event reported by the SCEF and is configured with one of the following enumerated values as specified in 3GPP TS 29.122 [4]

  - SESSION_TERMINATION, LOSS_OF_BEARER, RECOVERY_OF_BEARER, RELEASE_OF_BEARER, USAGE_REPORT.

- accumulatedUsage indicates the amount of time in seconds that the QoS session was used and the amount of data bytes transferred via the QoS session when event is USAGE_REPORT.   This notification is sent when the usage exceeds the values defined in the usageThreshold configured in the AsSessionWithQoSSubscription.

- flowIds is configured with the same value of flowId in the AsSessionWithQoSSubscription.

Step 3: IN-CSE performs the action according to the status

When the IN-CSE receives the Event Notification, the IN-CSE will map the event and accumulatedUsage to the QoS session status of <e2eQosSession> and performs the action according to the status.

| 3GPP event | oneM2M QoS session status |
| --- | --- |
| SESSION_TERMINATION | DISABLED |
| LOSS_OF_BEARER | DISABLED |
| RELEASE_OF_BEARER | DISABLED |
| RECOVERY_OF_BEARER | ENABLED |
| USAGE_REPORT | ENABLED (if accumaltedUsage indicates duration/volume has not been exhausted) USAGE_EXHAUSTED (if accumaltedUsage indicates duration/volume has been exhausted) |

The IN-CSE may update the e2eQosStatus of the resource <e2eQosSession> if there is no Step 4.

Step 4 (Optional): IN-CSE requests to delete QoS Session.

If the e2eQosPolicies attribute of the <e2eQosSession> resource is configured, then the IN-CSE evaluates and performs the configured action(s).

For example, if the configured action is DELETE when status equals DISABLED, then the IN-CSE will delete any existing QoS Session Subscription(s) to the underlying 3GPP network and also delete the <e2eQosSession> resource.

An IN-AE may also issue a request to delete an <e2eQosSession> resource.  When the IN-CSE receives this request, it will delete any existing QoS Session Subscription(s) to the underlying 3GPP network and also delete the <e2eQosSession> resource.

Step 5 (Optional): IN-CSE sends delete request(s) for any existing QoS Session Subscription(s) to the underlying 3GPP network

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this AsSessionWithQoSSubscription.

For each QoS session subscription associated with the <e2eQosSession> resource being deleted, the IN-CSE sends a AsSessionWithQoSSubscription DELETE Request to the SCEF. The request to delete a AsSessionWithQoSSubscription contains information as specified in 3GPP TS 29.122 [4]. Such information includes:

- An HTTP DELETE method is used

- URI shall be set to {apiRoot}/3gpp-as-session-with-qos/v1/{scsAsId}/subscriptions/{subscriptionID}.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies. The {subscriptionId} segment is configured by the SCEF.

Step 6 (Optional): SCEF sends QoS Session Response(s) to IN-CSE.

The SCEF handles the AsSessionWithQoSSubscription DELETE Request together with the Mobile Core Network.  The SCEF sends an AsSessionWithQoSSubscription DELETE Response that contains information as specified in 3GPP TS 29.122 [4] to the IN-CSE.

The message includes the following information.

- A response code of 204 No Content

If the IN-CSE receives a 204 No Content response code from the SCEF, it deletes the <e2eQosSession> resource.  Otherwise, the IN-CSE does not delete the <e2eQosSession> resource.

Step 7 (Optional): IN-CSE returns response to IN-AE.

The IN-CSE sends a DELETE response back to the IN-AE.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?