| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #41 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com Yongjing Zhang, Huawei Technologies Co., Ltd, zhangyongjing@huawei.com |
| Date:* | 2019-06-26 |
| Reason for Change/s:* | The CR is to add solution for PC5 mode of 3GPP V2X interworking |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0082 - 3GPP V2X Interworking MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0055 |
| Clauses * | 8.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TR-0024 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

### -----------------------Start of change 1-------------------------------------------

## 8.1 Solution A: Supporting PC5 communication

### 8.1.1 Solution Description

Each UE has a Layer-2 ID for the V2X communication over PC5 reference point, which is included in the source Layer-2 ID field of every frame that it sends on the layer-2 link.

In order to address the Key Issues 1, we can rely on the IN-CSE to configure the Layer-2 ID of PC5 for 3GPP V2X UEs.

### 8.1.2 Solution Applicability

This solution applies to Key Issue 1.

### 8.1.3 Solution Details

#### 8.1.3.1 Procedure

Pre-condition: The ADN-AE/ASN-CSE and ASN/MN-CSE have provisioned the certificates for PC5 communication.

Step 0:  The ADN-AE/ASN-CSE, ASN/MN-CSE, and IN-AE perform the oneM2M registration procedure.

Step 1: IN-CSE configures the Layer-2 ID of PC5 for each oneM2M field node. How the IN-CSE gets the Layer-2 ID of PC5 is outside the scope of the current document and may be based on policies and agreements with the mobile network operator.

Step 2: IN-AE sends operation request to the IN-CSE. In the request, the value of To parameter is set to the special value NULL, and the filter criteria condition of broadcastArea is set to the target location area for PC5 broadcasting.

Step 3: IN-CSE determines the transition ASN/MN-CSE (RSU/Vehicle) according to the broadcastArea, and sends the operation request to the transition ASN/MN-CSE (RSU/Vehicle) with Broadcast Indicator parameter being TRUE.

Step 4: The transition ASN/MN-CSE (RSU/Vehicle) determines to broadcast this operation request according to the Broadcast Indicator parameter in the request. If the Broadcast Indicator is TRUE, the ASN/MN-CSE gets the Layer-2 ID of PC5 from the <node> resource of itself, and prepares a PC5 broadcast request based on the received request while removing the Broadcast Indicator parameter. The prepared request should be protected by the signature using the PC5 certificate of the ASN/MN-CSE;

Step 5: The ASN/MN-CSE broadcasts the operation request by PC5 reference point without the Broadcast Indicator parameter.

Step 6: The ASN/MN-CSE sends the response to the IN-CSE to indicate that the request has been broadcast via PC5 suucessfully.

Step 7: The ADN-AE/ASN-CSE receives the broadcast request, validates the integrity of the request according to the signature of the PC5 certificate, and performs the operation as requested. The PC5 security procedure is specified in 3GPP TS 33.185[i.xx]

Step 8: The IN-CSE forwards operation response to the IN-AE.

#### 8.1.3.2 Resource

##### 8.1.3.2.1 <Node>

Table 9.6.18-2: Attributes of <node> resource

| Attributes of <node> | Multiplicity | RW/ RO/ WO | Description | <nodeAnnc> attributes |
| --- | --- | --- | --- | --- |
| broadcastAddress | 0..N | RO | Used for short range broadcasting. When the Node supports 3GPP PC5 conference point, the value is Layer-2 ID which is specified in 3GPP TS 23.285[2]. | OA |

##### 8.1.3.2.2 Request

Broadcast Indicator: Indicates the Receiver whether to broadcast the request or not.

Table 8.1.3.2.2-1: Summary of Request Message Parameters

| Request message parameter |  | Operation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Create | Retrieve | Update | Delete | Notify |
| Optional | Broadcast Indicator - whether to broadcast the request or not | O | O | O | O | O |

Table 8.1.3.2.2-2: Filter Criteria conditions

| Condition tag | Multiplicity | Description |
| --- | --- | --- |
| Matching Conditions |  |  |
| broadcastArea | 0..1 | Indicates area information where the broadcast message is applied. The area is a polygon and represented as a list of geographical coordinates that define the perimeter corner points of the polygon. |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

# 2	References

## 2.1	Normative references

As informative publications shall not contain normative references this clause shall remain empty.

[2]                    3GPP TS 23.285: Architecture enhancements for V2X services (Release 15)

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	oneM2M TS-0002: "Requirements".

[i.xx]           3GPP TS 33.185 Security aspect for LTE support of Vehicle-to-Everything (V2X) services (Release 15)

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?