| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #41 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com Yongjing Zhang, Huawei Technologies Co.,Ltd., zhangyongjing@huawei.com |
| Date:* | 2019-06-24 |
| Reason for Change/s:* | The CR is to add new feature of 3GPP V2X interworking. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0082 - 3GPP V2X Interworking MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0055 |
| Clauses * | 7.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | NA |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

### -----------------------Start of change 1-------------------------------------------

## 7.2 Key Issue 2: 3GPP V2X Downlink Continutiy of PC5 Communication

When the vehicle can only support PC5 mode, the vehicle communication with the IN-CSE via RSU. When the vehicle is moving, for example, on highway, the communication would go through different RSUs.

In the Annex A in 3GPP TS 23.285 [2], the RSU could be an 3GPP UE or eNodeB. In the subcause 6, the RSU is 3GPP UE and could be mapped to ASN/MN-CSE. The eNodeB type RSU is out of scope. The vehicle is 3GPP UE and could be mapped to ADN-AE or ASN-CSE.

Step 1: The ADN-AE/ASN-CSE sends the registration request to the IN-CSE via ASN/MN-CSE1 by PC5.

Step 2: The ASN/MN-CSE1 transfers the registration request to the IN-CSE.

Step 3: The IN-AE starts the The IN-CSE sends the continuity request to the ADN-AE/ASN-CSE via ASN/MN-CSE1, e.g. software downloading or fault diagnosis.

Step 4: The ASN/MN-CSE1 transfers the request to the ADN-AE/ASN-CSE by PC5.

Step 5: ADN-AE/ASN-CSE send response to the to the IN-CSE via ASN/MN-CSE2 by PC5.

Step 6: The ASN/MN-CSE1 transfers the response to the IN-CSE.

Step 7: The IN-CSE send the second request based on the result in the response to the ADN-AE/ASN-CSE via ASN/MN-CSE2.

Step 8: The ASN/MN-CSE2 transfers the request to the ADN-AE/ASN-CSE by PC5.

During the moving procedure, in Step 3 and 7, if the ADN-AE/ASN-CSE moves too fast, and moves out of the coverage scope of ASN/MN-CSE1/ASN/MN-CSE2, the IN-CSE sending request to the ADN-AE/ASN-CSE would fail.

This results the following issue:

The IN-CSE does not support the split one task into different subtasks based on the location policy.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?