| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #41 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com Yongjing Zhang, Huawei Technologies Co.,Ltd., zhangyongjing@huawei.com |
| Date:* | 2019-06-24 |
| Reason for Change/s:* | The CR is to add new feature of 3GPP V2X interworking. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0082 - 3GPP V2X Interworking MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0055 |
| Clauses * | 7.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | NA |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

### -----------------------Start of change 1-------------------------------------------

## 7.2 Key Issue 2: 3GPP V2X Downlink Continutiy of PC5 Communication

When the vehicle can only support PC5 mode, the vehicle communication with the IN-CSE via RSU. When the vehicle is moving, for example, on highway, the communication would go through different RSUs.

In the Annex A in 3GPP TS 23.285 [2], the RSU could be an 3GPP UE or eNodeB. In the subcause 6, the RSU is 3GPP UE and could be mapped to ASN/MN-CSE. The eNodeB type RSU is out of scope. The vehicle is 3GPP UE and could be mapped to ADN-AE or ASN-CSE.

Step 1: The ADN-AE/ASN-CSE sends the registration request to the IN-CSE via ASN/MN-CSE1 by PC5.

Step 2: The ASN/MN-CSE1 transfers the registration request to the IN-CSE.

Step 3: The IN-AE sends a request of a complex task to the ADN-AE/ASN-AE via the IN-CSE and requires the service of continuity. Such complex task could be pushing a software patch for upgrading the autonomous driving function or the HD (High Definition) map to the target vehicle (ADN-AE/ASN-CSE). This task should be done in a timely manner (which can be indicated by the request parameter of Operation Execution Time) for the sake of the driving safety, while the vehicle is moving across different RSUs (ASN/MN-CSE).

Depending on the speed of the vehicle and the coverage of the RSUs, the request may not be able to be delivered to the target vehicle within the coverage of a single RSU if the transmission time is too long for a big request message. In this case, the IN-CSE may split the task into a series of sub-tasks, e.g. split the software patch into multiple pieces, and generate a sub-task request for each piece, so that the transmission of each piece can be completed within the coverage of a certain RSU. To split the task in an appropriate way, the IN-CSE may need the information including the message size, the location, speed and direction of the vehicle, the coverage, network bandwidth and load of the RSUs, etc.

Step 4: The IN-CSE sends the first sub-task request to the ADN-AE/ASN-CSE via ASN/MN-CSE1.

Step 5: The ASN/MN-CSE1 transfers the first sub-task request to the ADN-AE/ASN-CSE by PC5.

Step 6: ADN-AE/ASN-CSE sends the first sub-task response to the IN-CSE via ASN/MN-CSE1 or ASN/MN-CSE2 via PC5 communication depending on the current network coverage.

Step 7: The ASN/MN-CSE2 transfers first the sub-task response to the IN-CSE in the case that the response is received at ASN/MN-CSE2.

Step 8: The IN-CSE sends the second sub-task request the ADN-AE/ASN-CSE via ASN/MN-CSE2. This should be done ahead of time, so that the target vehicle (ADN-AE/ASN-CSE) can receive the request as early as possible when it comes to the coverage of RSU2 (ASN/MN-CSE2). This step may happen before step 7 depending on whether the task requires sequential execution.

Step 9: The ASN/MN-CSE2 transfers the second sub-task request to the ADN-AE/ASN-CSE by PC5.

To support the procedure above,

the following key issues need to be resolved in oneM2M:

- oneM2M currently does not support registration over multiple hops (e.g. ADN-AE register with IN-CSE via a MN-CSE), so that a moving vehicle has no way to register to an stable registrar (e.g. the IN-CSE) but to an changing local registrar (e.g. MN-CSE)

- The IN-CSE does not support the service continuity by splitting one complex task into sub-tasks based on the location, speed and direction of the moving receiver and the underlying network resources in the PC5 communication case.

-----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?